CGCRS(12)04

Title: Modification to the draft report on ASA based on WGFM#73 output

Date: 3rd February 2012

Origin: CG Coordinator

Subject:

Study of the ASA concept

[Editor’s note: this report primarily aims to describe and assess the possible ability of the ASA concept to enhance the licensing/authorisation schemes of the NRAs in Europe. Consideration is also given on how cognitive radio technologies might contribute to the implementation of the ASA concept. One of the key objectives of this report is to identify further specific studies that would be needed in order to support possible implementation of the concept.]

# Background

During the 72nd Meeting of the WG FM in Miesbach, Germany 16 - 20 May 2011, a presentation was made on “An evolutionary spectrum authorisation scheme for sustainable economic growth and consumer benefit”. An Authorised Shared Access (ASA) concept was presented, developed with the aim to make a dynamic use of spectrum, whenever and wherever it is unused by incumbent users, on a shared and non-interference basis, subject to individual authorisation (i.e. licensed), in bands allocated to the Mobile Service by the ITU (and identified for IMT) and using cognitive radio capabilities, i.e. geolocation databases complemented, if required, by sensing. CEPT technical harmonisation work was referred to be of key importance for the implementation of the ASA concept. Initially, the spectrum sought for Mobile Service based applications authorized under the ASA concept was the 2.3 GHz band (Mobile shared with military/wireless cameras) and the 3.8 GHz band (Mobile with satellites).

WG FM tasked the correspondence group on CRS to evaluate the ASA concept, without linking it to any frequency band or existing concepts and to report back to WG FM during the 74th WG FM meeting on its findings.

# Introduction

Administrations and industry stakeholders are confronted worldwide with the constant challenge of ensuring more efficient use of the spectrum, and more specifically to identify more spectrum resource to respond to the growing demand for mobile broadband and the related traffic surge. The European Commission's communication on European Broadband proposes that wireless broadband should contribute to meeting the target that all EU citizens should have access to wireless broadband at sufficient speed by 2015 and at least 30 Mbps by 2020.

“Harmonisation” of additional spectrum at global (ITU-R) and regional level (CEPT, EU) has been so far the key enabler to respond to the needs of terrestrial mobile IMT in CEPT and to ensure that a suitable regulatory framework is available to give stability for the investment, to facilitate the creation of economies of scale, the development of standards and implementation of relevant regulation (authorisations) at national level. Consequently, harmonisation measures have been developed, are maintained or are under development within CEPT in the following frequency bands: 790-862 MHz; 880-915 MHz / 925-960 MHz (“900 MHz band”); 1710-1785 MHz / 1805-1880 MHz (“1800 MHz band”); 2500-2690 MHz (“2.6 GHz band”); 1900-1980 MHz, 2010-2025 MHz and 2110-2170 MHz (“2 GHz band”), 3.4-3.8GHz (“3.5 GHz band”).

The conventional harmonised approach will obviously continue to be essential to meet future demand, as illustrated by the large consensus at the ITU-R level to support [if seen appropriate] an Agenda Item at WRC-15/16 on terrestrial mobile broadband applications and IMT. Also, the initial work to enhance the efficient use of part of spectrum identified for IMT (e.g.: 1900-1920 MHz) will have to be continued.

Harmonisation has so far been combined with the clearing of such band from legacy services and users and subsequent exclusive allocation of such bands to mobile services/networks. The Authorized Spectrum Access (ASA) concept [might possibly be] / [is] a complementary regulatory approach. ASA was first introduced in response to the RSPG consultation on cognitive technologies in January 2011. The developer of the ASA concept describes it as a model to allow for a shared use of spectrum by ASA [users] / [licensees] whenever and wherever unused by incumbent users by using cognitive radio capabilities.   It is presented as being based on an individual authorisation model of spectrum use rights and could act as a regulatory enabler to making available, in a timely manner, additional harmonised spectrum required for mobile broadband while overcoming time, resource and political constraints associated with band clearing and while recognising the legitimate requirement for long term operation of incumbent users in certain bands.

This report proposes to analyse and evaluates of the ASA concept, the potentially applicable regulatory framework and how it could be implemented.

The report should in particular identify the necessary studies to be performed in order to define the harmonised sharing conditions between mobile broadband authorised under an ASA framework and identified incumbent users.

# Description of the ASA concept

Traditionally, the access and use of spectrum for radio communication services has been authorised in two ways: either licensed or licence-exempt

- “licensed” means that the usage rights are exclusively granted in time, frequency and geography. For example, a license might last for 25 years, apply to a frequency block and on a national basis. An example of licensed use is the 2 GHz spectrum for the provision of third generation mobile communication services,

- “licence-exempt” or “unlicensed” means that devices which meet certain technical conditions can share the spectrum, be used anywhere and at any time without guarantee of predictable quality of service. An example of licence-exempt use is the 2.4GHz spectrum for the provision of Wi-Fi access services.

Most of the frequencies in use for radio communication services are on a licensed basis.

The ASA concept is a framework to share spectrum between limited numbers of users. Under this concept, the initial user(s) (the “incumbent(s)” in this report) would share its spectrum (the ASA spectrum in this report) with one or several new users (the “incoming user(s)”) in accordance with a set of conditions to be defined by the regulation. These conditions may be “static” (e.g. exclusion zone, restricted time for operation) or more ‘dynamic’ (e.g. geographic/time sharing, on-demand authorisation of operators licensed under the ASA concept or on-demand restrictions by incumbents). In the latter case, a system for updating, maintaining and providing the access conditions is to be established. A key feature of ASA is to ensure a predictable quality of service for all spectrum use rights holders, network operators and for consumers.

The above mentioned dynamic implementation of the ASA concept may take advantages of the recent advances in cognitive technology allowing sharing spectrum in a more dynamic way including on a frequency, location and time sharing basis. More dynamic sharing does prevent neither a certain predictability (e.g., with the help of prediction models) nor a minimum guaranteed access if a certain level of spectrum availability is ensured (i.e. the primary incumbent user would guarantee access to a certain percentage of spectrum/time/location or would have to give sufficient time before significant restriction to access). In other words, the levels of priorities given to the primary user does not mean lack of certainty and guarantee for the incoming users and would be determined beforehand (e.g. during the course of an “ASA licensing process”).

Practically speaking, the following steps would need be followed by an ASA licensee:

- Potentially under the request of the administration that would then modify the incumbent authorisation accordingly, with the agreement of the later, the incumbent defines the conditions (frequency band, time, geographical area and potentially other technical conditions needed to ensure the proper operation of the incumbent usage in the time/space/frequency adjacent domain(s)) in which an ASA licensee could use part of the spectrum that this incumbent is duly authorised to use.

- A potential ASA licensee interested to use part of the spectrum not used by the incumbent applies for a ASA authorisation.

- According to the dynamical characteristic of the access to the spectrum for which the incumbent has an authorisation, this ASA licensee may need to have access to a data base where information about the area(s)/time of availability of (part of) the spectrum are given. If there is no change over a long time, this information can be given when the ASA licensee applies for its ASA authorisation.

- When the incumbent needs to have access to the part(s) of the band used by the ASA licensee, the later has to be informed and has to stop its use in accordance with the conditions defined in its ASA authorisation. The dynamical characteristic of this request, and the urgency of the later, may influence on the practical implementation for this step of information to the ASA licensee.

In summary, the advantage of the concept is that it would allow a more efficient use of spectrum and provide an alternative to a permanent segmentation and clearing/refarming of a band when there is a need to identify new spectrum, with a certain level of harmonisation. For the incumbent, the ASA concept shall give confidence that it can continue to use the spectrum with primary rights under specified conditions whenever and wherever necessary, while providing capacities to other incoming users that benefit from uncontested access to the spectrum resource they are authorised to exploit. The regulators would have to determine with the parties involved on a set of rules and practical implementation to share spectrum under the ASA concept.

The ASA concept as initially introduced, is primarily about granting “individual authorisations” to terrestrial mobile operators in bands used by other type of primary incumbent user(s). These authorisations would be based on explicit sharing agreement to ensure the protection of primary user(s) taking into account specific operational requirements.

If extrapolated to any type of incoming user, the basic key feature of ASA seem to be a sharing scheme where a secondary known user has access to the spectrum allocated to other user(s) on a primary basis, whether in a static or dynamic way.

- ]

# Analysis:

## Comparison between “conventional approach” and “ASA

The following table provides a comparison between “conventional approach” and “ASA concept” key features of the management of the radio spectrum from early steps in the regional harmonisation process to network operation and fine management of a specifically attributed frequency band:

| Conventional approach | ASA concept |
| --- | --- |
| Administration/ Regulator Spectrum licensee Multi-bands Device Incumbent Application/granting of spectrum use rights Conditions and timelines to free up the spectrum Base station controls device access to licensed spectrum Base station Base station Base station controls device access to licensed spectrum Licensed spectrum | Administration/ Regulator Spectrum licensee Incumbent Application/granting of ASA spectrum use rights Conditions of access to the “ASA spectrum” enabling protection of primary incumbent user Base station controls device access to licensed spectrum (licensed, ASA) Base station Base station Base station controls device access to licensed spectrum (licensed, ASA) Incumbent where/when ASA spectrum is available (Dynamic) Licensed spectrum Permitted ASA spectrum Multi-bands Device |
| [1/ Harmonisation process] / [ 1/Comparison of current and aimed situation] |  |
| Spectrum is identified for harmonised use in Europe long before technology is fully available and networks may be put in service. The process is relatively lengthy but the conditions for successful implementation and quality of service are quite well understood. It relies on a large and early consensus within Europe, with commitment from most administrations, that the spectrum will be freed within a certain timeframe to enable a new “primary user”. | After the conventional European harmonisation process, as described left, authorisations under the ASA concept could enhance the efficient use of spectrum by enabling additional users and provide additional flexibility and adaptability to national specificities. As a result, the conditions for successful implementation need to be assessed. Effective implementation and timelines could indeed vary significantly between administrations. In some countries, a frequency band “harmonised” under the ASA concept would become available to a ‘spectrum sharer’ only on a “secondary basis” for the delivery electronic communications services. Specific mechanisms need to be established to ensure that primary users are protected where and when necessary. In other countries, where there may be no incumbent user to be protected, the band could simply be made available on a “primary basis” and be designated under a conventional approach. The opportunity under the ASA concept is, where no “full harmonisation” is possible, to offer a sufficient market basis to enable economies of scale and subsequent investment from relevant industry stakeholders. The relevance of a candidate band would obviously depend upon the sharing conditions with primary users as they will determine the potential quality of service and its predictability. |
| 2/ Update of National frequency allocation table (NFAT) |  |
| The possibility to implement a European harmonised measure on a national basis is usually conditioned by appropriate reference in NFAT. This process should clarify the general conditions and timelines for the ‘incumbent’ to share the spectrum. | Conditions of access to the “ASA spectrum” enabling protection of primary incumbent user should be defined prior to the introduction of a [new] / [secondary use of a] band in the NFAT. |
| 3/ Licensing of spectrum |  |
| In case of mobile operators, the ‘spectrum licensee’ would typically have primary rights of use across the whole national territory on some blocks of European harmonised frequency bands. Licenses are attributed by the regulators usually for long duration (typically 15 – 20 years) and may include various obligations (e.g. geographical/demographic coverage ratio by certain timelines…) | The licensing could take various forms but would in any case clarify the general conditions to access the ASA spectrum. These conditions may be “static” (e.g. exclusion zone, time for operation …) or more dynamic. In the latter case, a system for updating the conditions and specifying explicitly “where/when ASA spectrum is available” is necessary. The fees for the spectrum use would likely be related to the portion of the territory and population that can be covered and the predictability of the quality of service that can offered for the “ASA licensee”. The question how to deal with the loss of “freedom”/disadvantage of the current licensee has to be solved. |
| 4/ Network operation and management of the licensed spectrum |  |
| Networks may be put in service once the licensed spectrum has been freed by the ‘incumbent’, usually at the end of a necessary refarming process. The ‘spectrum licensee’ has generally large freedom in the management of its licensed blocks of spectrum, within the boundaries of the terms set in its licence. Beyond e.g. obligations of notification or registration of high power base stations to the administration, little interaction is needed with the regulator in the fine management of the license spectrum. | In its network operation, the incoming users will have to comply with the conditions associated with its authorisation to use the ASA spectrum. If the access conditions to the ASA spectrum are static, the actual network management will not differ from the situation applicable to the conventional approach. However, if the spectrum access conditions can change over the time, the incoming users will have to update its network operation and management on the basis of these changes (e.g., reduction or increase of the geographical area were the spectrum can be used, reduction or increase of the bandwidth available in a given location,…). This dynamic approach must nevertheless be assessed with precaution. The impact for the new entrants of modifications of the conditions of spectrum access or the supplied quality of service should be estimated in its feasibility and its adequacy with the regulatory framework. The above mentioned dynamic implementation of the ASA concept will take advantages of the recent advances in cognitive technology allowing sharing spectrum in a more dynamic way including on a frequency, location and time sharing basis. |

## Stakeholders and responsibilities

Three main stakeholders would be at least involved in the licensing and management of spectrum authorised under an ASA framework: the Administration, the “incumbent” user and the ASA user.

This study proposes here to focus primarily on simple theoretical cases.

Not limited to, but in particular, with regard to the database(s) management, the following aspects should be considered:

- which entity provides and maintain it/them;

- which entity manage the authorisations and banning of frequency usages (in particular if the incumbent user is military and has obviously no interest in providing detailed information on its spectrum use in the considered band)

In the database architecture, the frequency use information may be separated from the available spectrum information. This allows for the administration to keep closer control on the frequency use information whilst facilitating – if needed - multiple database managers to handle the actual authorisations of available spectrum.

### Administration

The Administration is the national regulatory body, responsible for giving licenses on individual or general bases and for negotiating sharing conditions/ usage requirements on international and/or regional level. Also in its scope is the coordination procedure to be initiated among sharing partners in related bands as well as necessary border coordination.

ASA allows accommodating the incumbent spectrum users’ interests taking into account that spectrum use rights do not have to be withdrawn from an incumbent user to be granted to another user. While enabling swift availability of harmonised spectrum for the delivery of pan-European services, ASA will maintain governments’ sovereign rights over spectrum management and policy which are usually based on national requirements and needs. Indeed, the ASA approach is consistent with government exercising different spectrum management policy approaches.

### Incumbent users

The incumbent user is the current spectrum licensee. This could be a holder of an individual license (of any other radio communication service) or one or some governmental organisations which have priority rights on a given frequency band and manage it in order to deliver public services (e.g. defence, civil aviation).

The nature of the agreement between ASA user and the incumbent ensures that the incumbent may enjoy its primary rights for accessing spectrum under its licence whenever and wherever necessary. Accordingly, this approach ensure to the incumbent a long term access to the band for which it has priority rights.

t.b.c.

### Incoming users under an ASA framework

The ASA user must not interfere with the incumbent spectrum user whenever and wherever that user is exercising its right to access spectrum it has the license on. The ASA user also must operate within the emission and power limits stipulated to the particular band in question by the national regulatory authority.

The sharer (ASA user) can be held responsible for managing interference according to pre-defined/agreed criteria, giving additional security to the existing user.

Considering the initial target of the ASA concept to enable more spectrum resource to meet the growing demand for mobile broadband traffic, the most likely candidate for spectrum use authorised under the ASA concept would be a terrestrial mobile operator.

Currently, terrestrial mobile operators are the licence holders of “mobile bands” and generally manage their own networks. Additional spectrum granted by an “Authorised Shared Access” would be particularly attractive if it offers complementary capacity to meet the demand, e.g. in densely populated areas

In a second step, the relevance of the ASA concept should also be studied for other types of spectrum users which may seek additional local or temporary capacity for the delivery of e.g. PPDR, wireless local loop, satellite or even PMSE services.

## Typical scenarios: sharing conditions and quality of service

In the following, it is proposed to focus on selected typical “sharing scenarios”.

The shared use under the ASA concept is investigated here based on simple assumptions and independently from any frequency band.

The objective is to assess how sharing can be achieved with primary incumbent user and also which predictable quality of service could be anticipated under typical conditions.

The challenge is that the sharing conditions have to be sufficiently attractive and predictable for enabling the secondary user to invest in equipment and network.

[Note: implications on quality of service for the network operators, for the customers or for the incumbents could be considered].

### Fixed services

[General overview of challenges linked with sharing with equipment related to fixed services such as FS, FSS, BSS, RAS…]

### Mobile services

[General overview of challenges linked with sharing with equipment related to mobile services such as MS, MSS..].

### Temporary mobile applications

[General overview of challenges linked with sharing with ENG/OB, PMSE.]

## Issues at stake

It is increasingly difficult to clear out spectrum for new services. The rate at which existing regulatory processes are able to make wide bands of spectrum available is slow in comparison to the surging demand we are witnessing in e.g. mobile broadband uptake.

Clearing an existing exclusive user is controversial and costly. Defence sector in particular has large contiguous holdings coordinated and harmonised across countries. Many other primary users have narrowband allocations which makes any kind of refarming for clearing wide, contiguous bandwidth of spectrum very challenging. The current circumstances of spectrum use needs may also differ considerably from country to country, and finding harmonised spectrum bands available for clearing across multiple countries requires a lot of effort and may actually be impossible in some cases.

Sharing under the ASA concept provides administrations an additional tool that helps alleviate many of these problems.

# Existing

## Existing regulatory framework and tools:

The general review of the ASA concept suggests that it does not challenge the existing regulatory framework for the use of the spectrum, but basically complements it.

The radio spectrum is by nature part of public domain of states. Administrations manage the radio spectrum resource with a view to ensure quality to existing applications and enable the introduction of new ones.

The Radio Regulations (RR) sets rights and obligations for an individual state towards other states with respect to the use of the radio spectrum and orbital resources. Article 18 of the Radio Regulations stipulates that “no transmitting station may be established or operated by a private person or by any enterprise without a licence issued in an appropriate form and in conformity with the provisions of these Regulations by or on behalf of the government of the country to which the station in question is subject”.

The above term “licence” can be understood in its broad acceptance. This basically means that the use of spectrum must be explicitly permitted.

The RR allocates in the first place frequency bands to Radiocommunication Services, which as a result provides an international regulatory and economic environment that is favourable for the development of specific applications. In the area of mobile communication, the identification of specific bands for IMT is generally seen as a key enabler for the development of broadband mobile technologies.

At regional level, CEPT develops further harmonisation of the use of radio frequencies particularly in view of favouring economies of scale, border coordination and interoperability. In the area of broadband mobile communications, this is illustrated by numerous ECC Decisions setting a single or preferred channel plan for the use of designated bands.

Concerning the licensing process, the legal environment is created within the EU by Directive 2002/20/EC of the European Parliament and of the Council of 7 March 2002 on the authorisation of electronic communications networks and services (the “Authorisation Directive”).

Since the ASA concept is a framework to share spectrum between limited numbers of licensed users, it shall be clear the ASA users would use the spectrum under “individual authorisations”.

Article 7 of the Authorisation Directive provides the “Procedure for limiting the number of rights of use to be granted for radio frequencies”.

The proponents of the ASA concept advocate that ASA will maintain governments’ sovereign rights over spectrum management and policy which take into account national requirements, while enabling swift availability of harmonised spectrum for the delivery of pan-European services. It is also progressive – as the mobile industry adopts global mobile standards and service providers scale their service offerings over multiple jurisdictions, ASA can progressively enable spectrum to be brought into use more efficiently in a more dynamic manner to meet consumers' needs and demand.

Finally, it should be underlined that this report rather aims to focus on some basic key features of the ASA concept and to identify what type of specific studies and harmonisation measures might be needed to support its possible development in Europe, while recognizing that national implementation may consider more complex variations, notably concerning the way possible “ASA spectrum” could be assigned to new incoming users.

To summarise the concept in regulatory terms, the “ASA concept” is primarily about granting “individual authorisations” of the use of frequency band which are already licensed to another user. The additional authorisations to “ASA licensees” would specify the sharing conditions to ensure the protection of incumbent user(s).

## Existing sharing arrangements:

[Editor’s note: one administration underlined that there [may be] / [is] the case that basics of the concept are already used by the Administrations and proposed to develop a questionnaire so as to further investigate this matter. One alternative approach is to consider below some selected examples based on proposals from administrations.]

# Implementation of the ASA concept conditions (sharing conditions) in NRA authorisations:

[The NRA views should be sought on how the potentially complex sharing conditions associated with the ASA concept would be implemented in the authorisations they grant.]

Based on the description above, the following regulatory elements should be considered in the national authorisations schemes and within the European framework so as to allow the implementation of the ASA concept:

- Possibility for an incumbent user to allow the use by an ASA licensee of part of the spectrum for which this incumbent has usages rights,

  - Without being responsible in case of interferences created by the ASA licensee to a third party user (e.g., across the border)

  - And with the possibility of recovering the full use of the spectrum used by the ASA licensee when and where necessary, in an agreed delay

  - Can be subject to compensation (financial, infrastructure sharing etc.)

- Possibility for an administration to grant to an ASA licensee an authorisation with usage rights that are subject to change (time/frequency/space) with potentially a low predictability.

With regard to the sharing conditions, CEPT will have to consider the following ones:

- At the border between neighbouring administrations

  - in case of different uses with regard to the incumbent

  - in case of different conditions (frequency, time, technical) for the ASA licensee

- at the borders (time/frequency/space) between incumbent users and ASA licensee (at least to give guidelines to administrations).

- For specific bands, the sharing/compatibility conditions between incumbents and ASA licensee.

References:

Doc. FM(11)116: An evolutionary spectrum authorisation scheme for sustainable economic growth and consumer benefit, presentation at the 72nd Meeting of the WG FM (Miesbach, 16 – 20 May 2011)

Communication from the Commission to the European Parliament, the Council, the European Economic and Social Committee and the Committee of the Regions, ‘European Broadband: investing in digitally driven growth’, COM(2010) 472

See ECC Report 132, section 2, for a detailed description of the various regulatory regimes