|  |  | CPGPTA(2011)060 |
| --- | --- | --- |
| CPG-12 PTA Copenhagen (ECO), 5-7 October 2011 |  |  |
| Date issued: 28. 09. 2011 Source: Document CPG12(2011) 032 Annex V AI 1.13 Draft ECP Subject: Revision of draft ECP on AI 1.13 |  |  |

Summary:

This document is a proposal to revise the Preliminary Draft ECP on Agenda Item 1.13 regarding footnote RR5.530 and the Draft Resolution [BSS_earth_station_protection_21GHZ]

Proposal: At last CPG several concerns was raised regarding the protection of BSS receivers. Sweden believs that there is no need for a footnote requesting administration to avoid deploying fixed service applications which would prevent BSS deployment. Such footnot would have a restrictive influence of a use within the whole territory of country when its purpose obviously would be to restricte the Power flux-density value towards the border to another country. Therefore, it is suggested to delete fotnote RR 5.530. To protect BSS receivers RR 9.19 should be used with a condition stated in [BSS_earth_station_protection_21GHZ]. All proposed cahnges are highlighted in yellow.

Background:

CPG12(2011) 032 Annex  V  AI 1.13 Draft ECP

PRELIMINARY DRAFT EUROPEAN COMMON PROPOSAL

Proposal submitted by the following Administrations

[…, …, …]

Agenda item 1.13 (WRC-12)

Proposal

ARTICLE  5

Frequency allocations

Section IV – Table of Frequency Allocations
(See No. 2.1)

MOD	EUR/1.13/1

18.4-22 GHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 21.4-22 FIXED MOBILE BROADCASTING-SATELLITE 5.208B ADD X.XXX | 21.4-22 FIXED MOBILE | 21.4-22 FIXED MOBILE BROADCASTING-SATELLITE 5.208B 5.531 ADD X.XXX |

SUP	EUR/1.13/2

ADD

X.XXX Before an administration brings into use a station of the fixed or mobile service in this band, it shall ensure that the power flux-density (pfd) produced at 3 m above ground does not exceed [-114.3] dB(W/(m2 * 1 MHz)) for more than 20% of time at the border of the territory of any other administration. This limit may be exceeded on the territory of any country whose administration has so agreed.

Reason: A pfd threshold at the border of the territory of any other administrations in Regions 1 and 3 using BSS will ensure compatibility with terrestrial the BSS (broadcasting-satellite service) and therefore there is no need to limit the number of terrestrial stations. The protection of the BSS from terrestrial services could be ensured under the provisions of RR.9.19 or other appropriate procedures.

APPENDIX  5

Identification of administrations with which coordination is to be effected or 
agreement sought under the provisions of Article 9

TABLE  5-1

Technical conditions for coordination
(see Article 9)

MOD	EUR/1.13/18

| No. 9.19 Terrestrial, GSO, non-GSO/ GSO, non-GSO | Any transmitting station of a terrestrial service or a transmitting earth station in the FSS (Earth-to-space) in a frequency band shared on an equal primary basis with the BSS, with respect to typical earth stations included in the service area of a space station in the BSS | Bands listed in No. 9.11, the band 2 520-2 670 MHz, the band 11.7-12.7 GHz and the band 21.4-22.0 GHz [Editorial note: Proposal was made to modify this section under Agenda Item 7 (issue 2D)] | i) Necessary bandwidths overlap; and ii) the power flux-density (pfd) of the interfering station at the edge of the BSS service area exceeds the permissible level | Check by using the assigned frequencies and bandwidths | See also Article 6 of Appendix 30. |
| --- | --- | --- | --- | --- | --- |

Reason: The essence of Draft Resolution [BSS_EARTH_STATION_PROTECTION_21GHz] is transformed into a footnote X.XXX.

SUP	EUR/1.13/22

RESOLUTION 525 (Rev. WRC-07)

Introduction of high-definition television systems
of the broadcasting-satellite service in the
band 21.4-22.0 GHz in Regions 1 and 3

Reasons:	The regulatory content of the Resolution is  no longer valid. The regulatory situation is sorted out in the Table of Frequency Allocations.

ADD	EUR/1.13/23

[Editorial Note: The revision marks in this new draft Resolution are only revision to the text approved at the last CPG and not revision to an existing text]

Reason: The essence of Draft Resolution [BSS_EARTH_STATION_PROTECTION_21GHz] is transformed into a footnote X.XXX.