|  |  | CPGPTA(2011)072 |
| --- | --- | --- |
| CPG-12 PTA Copenhagen (ECO), 5-7 October 2011 |  |  |
| Date issued: 28 September 2011 Source: Luxembourg Subject: WRC-12 Agenda item 7 – New concept of “pairing” under RR appendices 30, 30A and 30B. |  |  |

Summary:

This contribution proposed to introduce a new regulatory mechanism allowing an administration wishing to coordinate a new incoming network that is not part of the Plan to reaches a temporary agreement with an administration having national allotments/assignments in the RR Appendices 30/30A/30B Plan with a certain orbital separation without seeing their protection margins permanently degraded. This mechanism will also permit to ensure the protection of this national/allotments during the temporary period with regards new submissions under Article 4 in Appendixes 30 and 30A and under Article 6 and 7 in Appendix 30B.

Proposal:

Luxembourg proposes that CPG PTA sends to CPG a draft ECP proposing this new concept of “pairing” under Appendices 30, 30A and 30B. Regulatory text is proposed in annex.

Background:

Annex

MOD		EUR/5A28/xx1

APPENDIX  30  (Rev.WRC-12)*

Provisions for all services and associated Plans and List1 for the broadcasting-satellite service in the frequency bands 11.7-12.2 GHz (in Region 3), 11.7-12.5 GHz (in Region 1) and 12.2-12.7 GHz (in Region 2) (WRC-03)

…

ARTICLE  4     (Rev.WRC-12)

Procedures for modifications to the Region 2 Plan or 
for additional uses in Regions 1 and 33

…

ADD		EUR/5A28/xx2

4.1.13bis	In application of 4.1.13, in the case of an agreement between an assignment in the List and an assignment in the Plan, and if both assignments are not separated by more than [6]° in the geostationary arc, both assignments should be paired. During this specific period, in the interference calculation to assignments that are part of the pair, only the interference contribution from assignments that are not part of the same pair are to be considered. On the other hand, for the interference calculation from assignments belonging to a pair into assignments that are not part of the same pair, only the worst interference contribution from that pair is to be taken into consideration.

MOD		EUR/5A28/xx3

APPENDIX  30A  (Rev.WRC-12)*

Provisions and associated Plans and List1 for feeder links for the broadcasting-satellite service (11.7-12.5 GHz in Region 1, 12.2-12.7 GHz in Region 2 and 11.7-12.2 GHz in Region 3) in the frequency bands 14.5-14.8 GHz2 and 17.3-18.1 GHz in Regions 1 and 3,
and 17.3-17.8 GHz in Region 2     (WRC-03)

…

ARTICLE  4     (Rev.WRC-12)

Procedures for modifications to the Region 2 feeder-link Plan 
or for additional uses in Regions 1 and 3

…

ADD		EUR/5A28/xx4

4.1. 13bis	In application of 4.1.13, in the case of an agreement between an assignment in the List and an assignment in the Plan, and if both assignments are not separated by more than [6]° in the geostationary arc, both assignments should be paired. During this specific period, in the interference calculation to assignments that are part of the pair, only the interference contribution from assignments that are not part of the same pair are to be considered. On the other hand, for the interference calculation from assignments belonging to a pair into assignments that are not part of the same pair, only the worst interference contribution from that pair is to be taken into consideration.

MOD		EUR/5A28/xx5

APPENDIX  30B  (Rev.WRC-12)

Provisions and associated Plan for the fixed-satellite service in the frequency bands 4 500-4 800 MHz, 6 725-7 025 MHz, 10.70-10.95 GHz,
11.20-11.45 GHz and 12.75-13.25 GHz

…

ARTICLE  6     (Rev.WRC-12)

Procedures for the conversion of an allotment into an assignment, for
the introduction of an additional system or for the modification of
an assignment in the List1, 2

…

ADD		EUR/5A28/xx6

6.16bis	If the agreement is required with respect to allotments in the Plan, the agreement of the administrations affected may also be obtained in accordance with this article, for a specific period. When this specific period of agreement expires, the assignment shall be maintained in the Plan until the end of the period referred to in 6.31. After that date this assignment shall lapse unless the agreement of the affected administration is renewed.

ADD		EUR/5A28/xx7

6.16ter	In application of 6.16bis, if the assignment and the allotment are not separated by more than [6]° in the geostationary arc, the assignment and the allotment should be paired. During this specific period, in the interference calculation to the assignment and the allotment that are part of the pair, only the interference contribution from assignments and allotments that are not part of the same pair are to be considered. On the other hand, for the interference calculation from assignments and allotments belonging to a pair into the assignment and the allotment that are not part of the same pair, only the worst interference contribution from that pair is to be taken into consideration.