|  |  | CPGPTA(2011)073 |
| --- | --- | --- |
| CPG-12 PTA Copenhagen (ECO), 5-7 October 2011 |  |  |
| Date issued: 28 September 2011 Source: Document CPG12(2011) 032 Annex V AI 1.13 Draft ECP Subject: Revision of draft ECP on AI 1.13 on Issue C |  |  |

Summary:

As agreed during the last CPG meeting, it was decided as compromise solution regarding Issue C (inter-service sharing issue) to further consider by project team A to solve the issue: in one hand, by technical regulations based on relaxed pfd limits for the satellite and pfd limits at the border to protect earth stations, and on the other hand, by a footnote encouraging administration to avoid deploying fixed service applications which would prevent BSS deployment. This document should be seen as a proposal to revise the Draft ECP on Agenda Item 1.13

Proposal:

Due to the very sensitivity of the BSS receiver as shown in different studies, WRC-92 proposed accordingly a super-primary status of the BSS. As a compromised solution, it was agreed during the last CPG to withdraw this super-primary status considering in parallel the restriction on terrestrial use in order to limit the harmful interference towards BSS receivers. Therefore, footnote 5.530 was modified accordingly. As recently discussed and agreed at the last WP4A, the Luxembourg administration proposes to endorse the PFD value derived in the Recommendation ITU-R BO.[PFD_TERRESTRIAL] as required hard limit to protect BSS receivers at the border. All proposed changes are highlighted in yellow. All these proposals should be considered as a package deal.

Background:

Annex

PRELIMINARY DRAFT EUROPEAN COMMON PROPOSAL

Proposal submitted by the following Administrations

[…, …, …]

Agenda item 1.13 (WRC-12)

to consider the results of ITU-R studies in accordance with Resolution 551 [COM6/13] (WRC-07) and decide on the spectrum usage of the 21.4-22 GHz band for the broadcasting-satellite service and the associated feeder-link bands in Regions 1 and 3;

Introduction

Change from interim to permanent procedures for use of the 21.4-22 GHz band by the Broadcasting-satellite service (BSS)

Europe proposes that WRC-12 adopts definitive provisions for the use of BSS in the 21.4-22 GHz band (i.e. suppression of Resolution 525) and modifies the articles and/or appendices of the Radio Regulations consequently and supports to improve transparency and accuracy of the due diligence procedure.

BSS intra-service sharing

Europe proposes that the applicable provisions to address the sharing between BSS networks in the 21.4-22 GHz band be the same as for the other bands which are not subject to a Plan.

In particular, Europe is opposed to a priori planning of the 21.4 – 22.0 GHz band as it would freeze access according to technological assumptions at the time of planning and then prevents flexible use taking account of real demand and developments.

Europe is of the view that the current types of protection criteria should be retained (i.e. a coordination arc contained in Appendix 5 of the Radio Regulations and a ΔT/T criterion to be included in the coordination if outside the arc through RR 9.41 and 9.42). Moreover, Europe proposes a coordination arc of 6° for geostationary-satellite orbit (GSO) BSS networks in the 21.4-22 GHz band.

Associated feeder-link bands

For a flexibility point of view, Europe is of the opinion that there should be no limitation on the FSS Earth-to-space bands which may be used for the associated feeder-links. However, a review of the availability of a new continuous block of 600 MHz of FSS spectrum for providing BSS feeder-links associated to the 22 GHz BSS band was undertaken to ensure a balance between uplink and downlink spectrum and that no shortage of uplink spectrum occurs.

In case of possible additional allocation of FSS spectrum, Europe considers the band 24.65-25.25 GHz.

In order to ensure that this sharing situation would not represent a heavy constraint for future FS development, Europe proposes to limit the use of this band to earth station in the fixed-satellite service (Earth-to-space) with a minimum antenna diameter of 3.5m to ensure that the number of FSS earth stations remains small. Europe is also of the view that there is no need to change RR 21.2.

Sharing between Regions 1 and 3 BSS and Region 2 terrestrial services

Europe is of the view that terrestrial services in Region 2 and the broadcasting-satellite service in Regions 1 & 3 should have the same primary status. Therefore, limits should be defined to ensure mutual-coexistence of the services with relevant modifications to the Radio Regulations.

Protection of the Region 2 terrestrial services from the Regions 1 and 3 BSS would then be ensured by pfd limits on the basis of Resolution 525 (Rev. WRC-03).

Protection of the Regions 1 and 3 BSS from the Region 2 terrestrial services, should be ensured by pfd hard limits.

Sharing between Regions 1 and 3 BSS and Regions 1 & 3 terrestrial services

Europe is of the view that terrestrial services and the broadcasting-satellite service in Regions 1 & 3 should co-exist under specific conditions.

Protection of terrestrial services from the BSS would then be ensured by pfd limits on the basis of RR Table 21-4 (Resolution 525 (Rev. WRC-03).

Protection of the BSS from terrestrial services, would be ensured by pfd hard limits.

Proposal

ARTICLE  5

Frequency allocations

Section IV – Table of Frequency Allocations
(See No. 2.1)

MOD	EUR/1.13/1

18.4-22 GHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 21.4-22 FIXED MOD 5.530 MOBILE MOD 5.530 BROADCASTING-SATELLITE 5.208B ADD 5.530bis | 21.4-22 FIXED MOBILE ADD 5.530bis | 21.4-22 FIXED MOD 5.530 MOBILE MOD 5.530 BROADCASTING-SATELLITE 5.208B ADD 5.530bis 5.531 |

MOD 	EUR/1.13/2

5.530	In Regions 1 and 3, Administrations should limit deployment of stations in Fixed and Mobiles services to terrestrial stations with a minimum antenna diameter of 2 m in order to ensure compatibility with the broadcasting satellite service,

Reasons:	To ensure mutual-coexistence of the services in the band 21.4-22.0 GHz.

ADD 	EUR/1.13/2bis

5.530bis	Before an administration brings into use transmitting stations of the fixed and mobile services in this band it shall ensure that the power flux-density (pfd) produced by a single fixed or mobile station, at 3 m above ground does not exceed -120.4 dB(W/(m2 ·1 MHz)) for not more than 20% of time at the border of the territory of any other administration. In order to ensure that the pfd limit at the border of the territory of any other administration is met, the calculations and verification shall be made, taking into account all relevant information, with the mutual agreement of both administrations (the administration responsible for the terrestrial station and the administration responsible for the earth station), with the assistance of the Bureau if so requested. In case of disagreement, the calculation and verification of the pfd shall be made by the Bureau, taking into account the information referred to above.

Reasons:	To ensure that BSS receivers will be adequately protected from the terrestrial services.

MOD	EUR/1.13/3

22-24.75 GHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 24.65-24.75 FIXED FIXED-SATELLITE (Earth-to-space) ADD5.A113 INTER-SATELLITE | 24.65-24.75 INTER-SATELLITE RADIOLOCATION- SATELLITE (Earth-to-space) | 24.65-24.75 FIXED FIXED-SATELLITE (Earth-to-space) ADD5.A113 INTER-SATELLITE MOBILE |
|  |  | 5.533 |

MOD	EUR/1.13/4

24.75-29.9 GHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 24.75-25.25 FIXED FIXED-SATELLITE (Earth-to-space) ADD5.A113 | 24.75-25.25 FIXED-SATELLITE (Earth-to-space) 5.535 | 24.75-25.25 FIXED FIXED-SATELLITE (Earth-to-space) 5.535 MOBILE |

ADD	EUR/1.13/5

5.A113	In the bands 24.65-25.25 GHz in Region 1 and 24.65-24.75 GHz in Region 3, an earth station in the fixed-satellite service (Earth-to-space) shall have a minimum antenna diameter of 3.5 m

Reasons:	To ensure that allocation of the band 24.65 – 25.25 GHz in Region 1 and the band 24.65 – 24.75 GHz in Region 3 in order to limit the number of Earth Stations

[Note to the Secretariat – As concerns the footnotes indicated in the title of Article 9, for reasons of economy, the texts are not reproduced as they remain unchanged.]

ARTICLE  9

MOD	EUR/1.13/6

Procedure for effecting coordination with or 
            obtaining agreement of other administrations1, 2, 3, 3, MOD4, 5, 6, 7, 8

Section II  –  Procedure for effecting coordination12, 13

Sub-Section IIA  –  Requirement and request for coordination

MOD	EUR/1.13/7

4	A.9.4	Resolution 49 (Rev.WRC-12) and Resolution [BSS_21GHz_Due diligence] (WRC-12) shall also be applied with respect to those satellite networks and satellite systems that are subject to it.

-

Reason:	Due diligence requirements for BSS networks in the 21.4-22 GHz band will be under the new resolution.

MOD	EUR/1.13/8

9.41		Following receipt of the BR IFIC referring to requests for coordination under Nos. 9.7 to 9.7B, an administration believing that it should have been included in the request or the initiating administration believing that an administration identified under No. 9.36 in accordance with the provisions of No. 9.7 (GSO/GSO) (items 1) to 9) of the frequency band column), No. 9.7A (GSO earth station/non-GSO system) or No. 9.7B (non-GSO system/GSO earth station) of Table 5-1 of Appendix 5 should not have been included in the request, shall, within four months of the date of publication of the relevant BR IFIC, inform the initiating administration or the identified administration, as appropriate, and the Bureau, giving its technical reasons for doing so, and shall request that its name be included or that the name of the identified administration be excluded, as appropriate.

Reasons:	Consequential changes to the modification of the coordination arc value for BSS networks in the 21.4-22 GHz band.

[Note to the Secretariat – As concerns the footnotes indicated in the title of Article 11, for reasons of economy, the texts are not reproduced as they remain unchanged.]

ARTICLE  11

MOD 	EUR/1.13/9

Notification and recording of frequency
assignments1, MOD 2, 3, 4, 5, 6, 7

Section II  –  Examination of notices and recording of frequency assignments
in the Master Register

MOD	EUR/1.13/10

2	A.11.2	Resolution 49 (Rev.WRC-12) and Resolution [BSS_21GHz_Due diligence] (WRC-12) shall also be applied with respect to those satellite networks and satellite systems that are subject to it.

-

Reason:	Due diligence requirements for BSS networks in the 21.4-22 GHz band will be under the new resolution.

MOD	EUR/1.13/11

18 11.37.2	When a frequency assignment to a space station in the broadcasting-satellite service in a non-planned band other than the 21.4-22 GHz band is recorded in the Master Register, a note shall be entered in the remarks column indicating that such recording does not prejudge in any way the decisions to be included in the agreements and associated plans referred to in Resolution 507.

Reasons:	As no a priori planning is foreseen for the 21.4-22 GHz band, Resolution 507 (Rev. WRC-07) should not be applicable to the 21.4-22 GHz band.

MOD	EUR/1.13/12

11.44	The notified date MOD 20 of bringing into use of any assignment to a space station of a satellite network shall be not later than seven years following the date of receipt by the Bureau of the relevant complete information under No. 9.1 or 9.2, as appropriate. Any frequency assignment not brought into use within the required period shall be cancelled by the Bureau after having informed the administration at least three months before the expiry of this period.

MOD	EUR/1.13/13

20 	11.44.1	In the case of space station frequency assignments that are brought into use prior to the completion of the coordination process, and for which the Resolution 49 (Rev.WRC-12) data or Resolution [BSS_21GHz_Due diligence] (WRC-12) data, as appropriate, have been submitted to the Bureau, the assignment shall continue to be taken into consideration for a maximum period of seven years from the date of receipt of the relevant information under No. 9.1. If the first notice for recording of the assignments in question under No. 11.15 has not been received by the Bureau by the end of this seven-year period, the assignments shall no longer be taken into account by the Bureau and administrations. The Bureau shall inform the notifying administration of its pending actions three months in advance.

In the case of satellite networks for which relevant advance publication information has been received prior to 22 November 1997, the corresponding period will be nine years from the date of publication of this information.

-

Reason:	Due diligence requirements for BSS networks in the 21.4-22 GHz band will be under the new resolution.

ARTICLE  21

Terrestrial and space services sharing frequency bands above 1 GHz

Section I  -  Choice of sites and frequencies

MOD	EUR/1.13/14

21.2 	§ 2	 1) As far as practicable, sites for transmittingMOD1, 3 stations, in the fixed or mobile service, employing maximum values of equivalent isotropically radiated power (e.i.r.p.) exceeding the values given in Table 21-1 in the frequency bands indicated, should be selected so that the direction of maximum radiation of any antenna will be separated from the geostationarysatellite orbit by at least the angle in degrees shown in the Table, taking into account the effect of atmospheric refraction2:

NOC	EUR/1.13/15

TABLE 21-1

MOD	EUR/1.13/16

1 21.2.1 For their own protection receiving stations in the fixed or mobile service operating in bands shared with space radiocommunication services (space-to-Earth) should also avoid directing their antennas towards the geostationary-satellite orbit if their sensitivity is sufficiently high that interference from space station transmissions may be significant. Especially, in the band 21.4-22.0 GHz, it is recommended a minimum separation angle with respect to geostationary-satellite orbit of 1.5°

Section V  –  Limits of power flux-density from space stations

MOD	EUR/1.13/17

TABLE  21-4

| Frequency band | Service* | Limit in dB(W/m2) for angles of arrival () above the horizontal plane |  |  | Reference bandwidth |
| --- | --- | --- | --- | --- | --- |
|  |  | 0-5 | 5-25 | 25-90 |  |
| 19.3-19.7 GHz 21.4-22 GHz (Region 2) 22.55-23.55 GHz 24.45-24.75 GHz 25.25-27.5 GHz 27.500-27.501 GHz | Fixed-satellite (space-to-Earth) Broadcasting-satellite Earth exploration-satellite (space-to-Earth) Inter-satellite Space research (space-to-Earth) | –115 13A | –115 + 0.5( – 5) 13A | –105 13A | 1 MHz |
| 21.4-22 GHz (Regions 1 and 3) | Broadcasting-satellite | XXX | YYY | ZZZ | 1 MHz |

Reason: 	To introduce pfd limits in Article 21 to protect terrestrial services in frequency band 21.4-22.0 GHz.

APPENDIX  5

Identification of administrations with which coordination is to be effected or 
agreement sought under the provisions of Article 9

TABLE  5-1

Technical conditions for coordination
(see Article 9)

MOD	EUR/1.13/18

| Reference of Article 9 | Case | Frequency bands (and Region) of the service for which coordination is sought | Threshold/condition | Calculation method | Remarks |
| --- | --- | --- | --- | --- | --- |
| No. 9.7 GSO/GSO | A station in a satellite network using the geostationary-satellite orbit (GSO), in any space radiocommunication service, in a frequency band and in a Region where this service is not subject to a Plan, in respect of any other satellite network using that orbit, in any space radiocommunication service in a frequency band and in a Region where this service is not subject to a Plan, with the exception of the coordination between earth stations operating in the opposite direction of transmission | 1) 3 400-4 200 MHz 5 725-5 850 MHz (Region 1) and 5 850-6 725 MHz 7 025-7 075 MHz | i) Bandwidth overlap, and ii) any network in the fixed-satellite service (FSS) and any associated space operation functions (see No. 1.23) with a space station within an orbital arc of 10° of the nominal orbital position of a proposed network in the FSS |  | With respect to the space services listed in the threshold/condition column in the bands in 1), 2), 3), 4), 5), 6), 7), 8) and 9), an administration may request, pursuant to No. 9.41, to be included in requests for coordination, indicating the networks for which the value of T/T calculated by the method in § 2.2.1.2 and 3.2 of Appendix 8 exceeds 6%. When the Bureau, on request by an affected administration, studies this information pursuant to No. 9.42, the calculation method given in § 2.2.1.2 and 3.2 of Appendix 8 shall be used |
|  |  | 2) 10.95-11.2 GHz 11.45-11.7 GHz 11.7-12.2 GHz (Region 2) 12.2-12.5 GHz (Region 3) 12.5-12.75 GHz (Regions 1 and 3) 12.7-12.75 GHz (Region 2) and 13.75-14.5 GHz | i) Bandwidth overlap, and ii) any network in the FSS or broadcasting-satellite service (BSS), not subject to a Plan, and any associated space operation functions (see No. 1.23) with a space station within an orbital arc of 9° of the nominal orbital position of a proposed network in the FSS or BSS, not subject to a Plan |  |  |

TABLE  5-1 (continued)     (WRC-07)

| Reference of Article 9 | Case | Frequency bands (and Region) of the service for which coordination is sought | Threshold/condition | Calculation method | Remarks |
| --- | --- | --- | --- | --- | --- |
| No. 9.7 GSO/GSO (cont.) |  | 3) 17.7-20.2 GHz, (Regions 2 and 3), 17.3-20.2 GHz (Region 1) and 27.5-30 GHz | i) Bandwidth overlap, and ii) any network in the FSS and any associated space operation functions (see No. 1.23) with a space station within an orbital arc of 8° of the nominal orbital position of a proposed network in the FSS |  |  |
|  |  | 4) 17.3-17.7 GHz (Regions 1 and 2) | i) Bandwidth overlap, and ii) a) any network in the FSS and any associated space operation functions (see No. 1.23) with a space station within an orbital arc of 8° of the nominal orbital position of a proposed network in the BSS, or b) any network in the BSS and any associated space operation functions (see No. 1.23) with a space station within an orbital arc of 8° of the nominal orbital position of a proposed network in the FSS |  |  |

TABLE  5-1 (continued)     (WRC-07)

| Reference of Article 9 | Case | Frequency bands (and Region) of the service for which coordination is sought | Threshold/condition | Calculation method | Remarks |
| --- | --- | --- | --- | --- | --- |
| No. 9.7 GSO/GSO (cont.) |  | 5) 17.7-17.8 GHz | i) Bandwidth overlap, and ii) a) any network in the FSS and any associated space operation functions (see No. 1.23) with a space station within an orbital arc of 8° of the nominal orbital position of a proposed network in the BSS, or b) any network in the BSS and any associated space operation functions (see No. 1.23) with a space station within an orbital arc of 8° of the nominal orbital position of a proposed network in the FSS NOTE – No. 5.517 applies in Region 2. |  |  |
|  |  | 6) 18.0-18.3 GHz (Region 2) 18.1-18.4 GHz (Regions 1 and 3) | i) Bandwidth overlap, and ii) any network in the FSS or meteorological-satellite service and any associated space operation functions (see No. 1.23) with a space station within an orbital arc of 8° of the nominal orbital position of a proposed network in the FSS or the meteorological-satellite service |  |  |

TABLE  5-1 (continued)     (WRC-07)

| Reference of Article 9 | Case | Frequency bands (and Region) of the service for which coordination is sought | Threshold/condition | Calculation method | Remarks |
| --- | --- | --- | --- | --- | --- |
| No. 9.7 GSO/GSO (cont.) |  | 7) Bands above 17.3 GHz, except those defined in § 3) and 6) | i) Bandwidth overlap, and ii) any network in the FSS and any associated space operation functions (see No. 1.23) with a space station within an orbital arc of 8° of the nominal orbital position of a proposed network in the FSS (see also Resolution 901 (Rev.WRC-07)) |  |  |
|  |  | 8) 21.4-22 GHz (Regions 1 and 3) | i) Bandwidth overlap, and ii) any network in the BSS and any associated space operation functions (see No. 1.23) with a space station within an orbital arc of 6° of the nominal orbital position of a proposed network in the BSS |  |  |
|  |  | 9) Bands above 17.3 GHz except those defined in § 4), 5) and 8) | i) Bandwidth overlap, and ii) any network in the FSS or BSS, not subject to a Plan, and any associated space operation functions (see No. 1.23) with a space station within an orbital arc of 16° of the nominal orbital position of a proposed network in the FSS or BSS, not subject to a Plan, except in the case of a network in the FSS with respect to a network in the FSS (see also Resolution 901 (Rev.WRC-07)) |  |  |

TABLE  5-1 (continued)     (WRC-07)

| Reference of Article 9 | Case | Frequency bands (and Region) of the service for which coordination is sought | Threshold/condition | Calculation method | Remarks |
| --- | --- | --- | --- | --- | --- |
| No. 9.7 GSO/GSO (cont.) |  | 10) All frequency bands, other than those in 1), 2), 3), 4), 5), 6), 7), 8) and 9), allocated to a space service, and the bands in 1), 2), 3), 4), 5), 6), 7), 8) and 9) where the radio service of the proposed network or affected networks is other than the space services listed in the threshold/ condition column, or in the case of coordination of space stations operating in the opposite direction of transmission | i) Bandwidth overlap, and ii) Value of T/T exceeds 6% | Appendix 8 | In application of Article 2A of Appendix 30 for the space operation functions using the guardbands defined in § 3.9 of Annex 5 of Appendix 30, the threshold/condition specified for the FSS in the bands in 2) applies. In application of Article 2A of Appendix 30A for the space operation functions using the guardbands defined in § 3.1 and 4.1 of Annex 3 of Appendix 30A, the threshold/condition specified for the FSS in the bands in 7) applies |
| … |  |  |  |  |  |
| No. 9.19 Terrestrial, GSO, non-GSO/ GSO, non-GSO | Any transmitting station of a terrestrial service or a transmitting earth station in the FSS (Earth-to-space) in a frequency band shared on an equal primary basis with the BSS, with respect to typical earth stations included in the service area of a space station in the BSS | Bands listed in No. 9.11, the band 2 520-2 670 MHz and the band 11.7-12.7 GHz [Editorial note: Proposal was made to modify this section under Agenda Item 7 (issue 2D)] | i) Necessary bandwidths overlap; and ii) the power flux-density (pfd) of the interfering station at the edge of the BSS service area exceeds the permissible level | Check by using the assigned frequencies and bandwidths | See also Article 6 of Appendix 30 and Resolution [BSS_earth_station_protection_21GHZ] |
| … |  |  |  |  |  |

Reasons: –	To introduce with § 8) a coordination arc value of ±6° in the band 21.4-22 GHz to trigger the coordination between BSS networks serving Regions 1 and 3.

APPENDIX  7

Methods for the determination of the coordination area around an earth
station in frequency bands between 100 MHz and 105 GHz

MOD	EUR/1.13/19

TABLE  7c

Parameters required for the determination of coordination distance for a transmitting earth station

| Transmitting space radiocommunication service designation |  | Fixed- satellite | … |
| --- | --- | --- | --- |
| Frequency bands (GHz) |  | 24.65-25.25 27.0-29.5 |  |
| Receiving terrestrial service designations |  | Fixed, mobile |  |
| Method to be used |  | § 2.1 |  |
| Modulation at terrestrial station 1 |  | N |  |
| Terrestrial station interference parameters and criteria | p0 (%) | 0.005 |  |
|  | n | 1 |  |
|  | p (%) | 0.005 |  |
|  | NL (dB) | 0 |  |
|  | Ms (dB) | 25 |  |
|  | W (dB) | 0 |  |
| Terrestrial station parameters | Gx (dBi) 4 | 50 |  |
|  | Te (K) | 2 000 |  |
| Reference bandwidth | B (Hz) | 106 |  |
| Permissible interference power | Pr( p) (dBW) in B | –111 |  |

Editorial Note: it may also be useful that CEPT administrations are invited to check the appropriateness of the current Appendix 7 characteristics to cover their FS stations.

Reasons:	To compute the coordination distances around an FSS earth station in the band 24.65 – 24.75 GHz in Regions 1 and 3.

MOD	EUR/1.13/20

RESOLUTION  49  (Rev.WRC-12)

Administrative due diligence applicable to some satellite
radiocommunication services

resolves

1	that the administrative due diligence procedure contained in Annex 1 to this Resolution shall be applied as from 22 November 1997 for a satellite network or satellite system of the fixed-satellite service, mobile-satellite service or broadcasting-satellite service, except the broadcasting-satellite service in the band 21.4-22.0 GHz, for which the advance publication information under No. 9.2B, or for which the request for modifications of the Region 2 Plan under Article 4, § 4.2.1 b) of Appendices 30 and 30A that involve the addition of new frequencies or orbit positions, or for which the request for modifications of the Region 2 Plan under Article 4, § 4.2.1 a) of Appendices 30 and 30A that extend the service area to another country or countries in addition to the existing service area, or for which the request for additional uses in Regions 1 and 3 under § 4.1 of Article 4 of Appendices 30 and 30A, or for which the submission of information under supplementary provisions applicable to additional uses in the planned bands as  defined in Article 2 of Appendix 30B (Section III of Article 6) has been received by the Bureau  from 22 November 1997, or for which submission under Article 6 of Appendix 30B (Rev.WRC-07) is received on or after 17 November 2007, with the exception of submissions of new Member States seeking the acquisition of their respective national allotments for inclusion in the Appendix 30B Plan;

See § 2.3 of Appendix 30B (Rev.WRC-07).

ANNEX 1 TO RESOLUTION 49 (Rev.WRC-12)

1	Any satellite network or satellite system of the fixed-satellite service, mobile-satellite service or broadcasting-satellite service with frequency assignments that are subject to coordination under Nos. 9.7, 9.11, 9.12, 9.12A and 9.13 and Resolution 33 (Rev.WRC-03), with the exception of broadcasting-satellite service submissions in the band 21.4-22.0 GHz, shall be subject to these procedures.

Reason:	Due diligence requirements for BSS networks in the 21.4-22 GHz band will be under the new resolution (i.e. Resolution [BSS_21GHz_DUE DILIGENCE] (WRC-12)). Consequently, Resolution 49 (Rev.WRC-12) will no longer be applicable for BSS networks in this band.

MOD	EUR/1.13/21

RESOLUTION 507 (Rev. WRC-03)

Establishment of agreements and associated plans
for the broadcasting-satellite service 1

1	This Resolution does not apply to the 21.4-22 GHz band.

Reasons:	As no a priori planning is foreseen for the 21.4-22 GHz band, Resolution 507 (Rev. WRC-03) should not be applicable to the 21.4-22 GHz band.

SUP	EUR/1.13/22

RESOLUTION 525 (Rev. WRC-07)

Introduction of high-definition television systems
of the broadcasting-satellite service in the
band 21.4-22.0 GHz in Regions 1 and 3

Reasons:	Action under this Resolution has been completed.

ADD	EUR/1.13/23

[Editorial Note: The revision marks in this new draft Resolution are only revision to the text approved at the last CPG and not revision to an existing text]

DRAFT RESOLUTION [BSS_21GHZ_DUE DILIGENCE] (WRC-12)

Long term access to and development in the band 21.4-22.0 GHz
in Regions 1 and 3

The World Radiocommunication Conference (Geneva, 2012),

considering

a)	that WARC-92 allocated the band 21.4-22.0 GHz in Regions 1 and 3 to the broadcasting-satellite service (BSS) to be implemented after 1 April 2007;

b)	that the use of the band since 1992 was subject to an interim procedure in accordance with Resolution 525 (WARC-92 and Rev.WRC-03);

c)	that Resolution 551 (WRC-07) instructs ITU-R to continue technical and regulatory studies on harmonization of spectrum usage, coordination procedures or other procedures, and BSS technologies, in preparation for WRC-12, in the 21.4-22 GHz band and the associated feeder-link bands in Regions 1 and 3;

d)	that Article 44 of the ITU Constitution sets out the basic principles for the use of the radio-frequency spectrum and the geostationary-satellite and other satellite orbits, taking into account the needs of developing countries,

e)	that a due diligence process was first adopted by WRC-97 with a view of providing as early as possible information on the industrial project behind a satellite network submitted to the ITU;

f)	that data concerning the manufacturer, launch service provider and launch date of a satellite will be more accurate and useful if submitted after the launch of the satellite,

resolves

1	that the procedures contained in this Resolution as well as the revised RR No A.9.4, A.11.2, 11.37.2 and 11.44.1 as adopted by this Conference shall enter into force as of 17 February 2012

2	that this Resolution applies to satellite network in the broadcasting-satellite service in the 21.4-22.0 GHz band;

3	that, for frequency assignments to such satellite networks for which the confirmation of the date of bringing into use was not received by the Bureau before 17 February 2012 or which were suspended at that date,   the procedures contained in the Annexes to this Resolution shall be applied:

i)	at the time of first bringing into use; or

ii)	when resuming use after a suspension;

4

that for an assignment of a satellite network of the broadcasting-satellite service in the 21.4-22.0 GHz band for which the confirmation of the date of bringing into use was received by the Bureau before 17 February 2012, the notifying administration shall submit to the Bureau not later than [17 August 2012] the complete information related to the operational situation as of 17 February 2012, in accordance with Annex 2 to this Resolution;

5	On receipt of the due diligence information under resolves 4 above, the Bureau shall promptly examine its completeness. If the information is found to be complete, the Bureau shall publish the complete information in a special section of the BR IFIC within two months. If the information is found to be incomplete, the Bureau shall request the notifying administration to submit the missing information within thirty days;

6	that if the complete information specified in resolves 4 above, is not received by the Bureau in accordance with resolves 4 and 5 above, the corresponding frequency assignments shall be cancelled by the Bureau;

7	that for frequency assignments to satellite networks as described in resolves 4 above, the provisions of § 4 to 8 of Annex 1 to this Resolution shall also apply,

instructs the Director of the Radiocommunication Bureau

to report to future competent world radiocommunication conferences on the results of the implementation of this Resolution.

Annex 1 to Resolution [BSS_21GHZ_DUE DILIGENCE]  (WRC-12)

1	Any frequency assignments to satellite networks referred to in resolves 2 shall be subject to these procedures either when their first bringing into use is declared or at the resumption of use following a suspension.

2	Within thirty days after the actual commencement, or resumption, of use of the frequency assignment to a satellite network subject to these procedures, the notifying administration shall send to the Bureau the the information specified in Annex 2 to this Resolution.

3	On receipt of the information under § 2 above, the Bureau shall promptly examine its completeness. If the information is found to be complete, the Bureau shall publish the complete information in a special section of the BR IFIC within two months. If the information is found to be incomplete, the Bureau shall request the notifying administration to submit the missing information within thirty days.

4	The information submitted in accordance with § 2 and resolves 3 above shall be updated and resubmitted to the Bureau by the notifying administration not later than thirty days after the end of life or the relocation of the spacecraft associated with the submission under § 2 and resolves 3 above .

5	In case of end of life of a spacecraft already used under this Resolution, the notifying administration shall inform accordingly the Bureau within thirty days after the event. The corresponding ITU number associated to such a spacecraft shall no longer be used.

6	On receipt of the information under § 4 above, the Bureau shall promptly examine its completeness. If the information is found to be complete, the Bureau shall publish the complete information in a special section of the BR IFIC within two months. If the information is found to be incomplete, the Bureau shall immediately request the administration to submit the missing information within thirty days.

7	If the complete information specified in § 4 above is not received by the Bureau within the time limits specified in §§ 4 and 6 above, the frequency assignments to the satellite network shall be considered as invalid. The Bureau shall immediately inform the notifying administration and take the appropriate measures under § 8, if required.

8	After the end of the periods referred to in No. 11.44, in No. 11.49, if the complete information under this Resolution is not yet received by the Bureau, the corresponding frequency assignments shall be cancelled by the Bureau.

Annex 2 to Resolution [BSS_21GHZ_DUE DILIGENCE] (WRC-12)

Due diligence information

1)	Identity of the satellite network

a)	Identity of the satellite network

b)	Name of the notifying administration

c)

Orbital characteristics.

d)	Reference to the advance publication information

e)	Reference to the request for coordinations, where applicable

f)	Frequency band(s) included in the relevant special sections of the satellite network

g)	Regulatory status (tick box)

- Satellite network under operation (only data listed in § 2should be provided)

Or

- Satellite network suspended (only data listed in § 3 should be provided)

2)	Identity of the spacecraft (if satellite network is under operation)

If data about the spacecraft were already submitted under this Resolution

a)	ITU ID number

If data about the spacecraft are submitted for the first time under this Resolution

a)	Spacecraft manufacturer

Name of the spacecraft manufacturer

b)	Launch services provider

-	Name of the launch vehicle provider

-	Launch date

-	Name of the launch vehicle

-	Name and location of the launch facility

c)	Frequency band(s) present on-board on the spacecraft (i.e. frequency bands that are able to be received or transmitted by a transponder located on-board the spacecraft).

3)	Suspension information (if satellite network filing is suspended)

a)	Date of suspension

b)	Reason of suspension:

-	Spacecraft moved to another orbital position, or

-	In-orbit failure of the spacecraft, or

-	Spacecraft de-orbited.

[-	other. Please specify]

Reasons:	Improvement of transparency and truthfulness of the due diligence procedure.

If data about the spacecraft are submitted for the first time under this Resolution, items “Spacecraft manufacturer”, “Launch services provider” and “Frequency band(s) present on-board on the spacecraft” shall be provided. Otherwise, if data about the spacecraft were already used under this Resolution , the ID number given by the Bureau at that time shall be indicated.

More than one spacecraft may be associated with a satellite network at the same orbital location.