[Note: text in BLUE can be deleted for submission to CPG]	TEMP17rev1

PRELIMINARY EUROPEAN COMMON PROPOSAL FOR

THE WORK OF THE CONFERENCE

PART 29

Agenda item 8.1

Sub-Part XB

WRC Resolution 547 (AI 8.1.1 Issue B)

Europe supports the relevant part of the [Draft] Report of the Director on the Activities of the Radiocommunication Sector - Part 6 – “Implementation of Resolution 547 (Rev.WRC-07)”-

[Note: how should we present the above statement? Through an ECP even when  there is no proposal for change of the Report?]

Sub-Part XD

WRC Resolution 80 (AI 8.1.3)

Introduction

Proposal

TBD

[Note: please refer to relevant parts of the BR Report namely the Report by the RRB to Resolution 80 (Rev.WRC-07)(Document R12-WRC12-C-0011!!MSW-E-1) for possible proposals].

Reasons:

Sub-Part XE

Difficulties or inconsistencies encountered in the application of the Radio Regulations (AI 8.1.2)

Introduction

Europe has encountered the following inconsistencies in the application of the Radio Regulations:

- The Radiocommunication Assembly 2007 adopted Resolution ITU-R 56 which resolves “that the term “IMT” be the root name that encompasses both IMT-2000 and IMT-Advanced collectively.” As a consequence, with the idea that the references in the Radio Regulations to “IMT-2000” should be changed to “IMT”, unless the specific reference to “IMT-2000” is still applicable, WRC-07 made appropriate changes to most of the Article 5 footnotes, Resolutions and Recommendations associated with IMT-2000. However some footnotes, as well as some considering and recognizing in some Resolutions and Recommendations still refer to IMT-2000 in a non-consistent manner.

- Resolution 124 was drafted at WRC-97 in order to further study pfd limits that were adopted during that Conference and applicable to GSO EESS satellites in the band 8025-8400 MHz in Region 1 and 3 (except Japan). These pfd limits are contained in No. 5.462A. Following WRC-97 former Study Group 9 and Study Group 7 worked together on this issue. The end result was the approval of Recommendation ITU-R F.1502 with recommended pfd limits less stringent than those contained in No. 5.462A. As a consequence of the approval of Recommendation ITU-R F.1502, WRC-2000 revised Resolution 124 to invite a future WRC to review No. 5.462A. However, no action was taken by the later Conferences (WRC-03 and WRC-07). Therefore, the provisional pfd mask in No 5.462A and the reference to the WRC-97 version of Resolution 124 are still valid, creating an inconsistency with the revised Resolution 124 of WRC-2000. Furthermore, the CPM report in the review of Resolutions under Agenda Item 4 proposes to delete Resolution 124 after the review of No. 5.462A under Agenda Item 8.1.2 of WRC-12.

- In addition, two editorial improvements to Appendix 4 are identified:

  - In the introduction of Appendix 4, include a reference to the radio astronomy service in addition to space services;

  - In footnote 2 to the title of Annex 2, replace the reference to “footnote 1” by the text of footnote 1 amended to apply to Annex 2 (i.e., by replacing “Terrestrial” by “Space”).

- Furthermore two editorial improvements to Article 5 are identified:

  - Delete No. 5.199 in band 235-267 MHz from the table of allocations in Article 5;

  - Merge the entries 200-202 GHz and 202-209 GHz.

For proposed modifications to Resolutions 223 (Rev.WRC-07), 646 (Rev.WRC-03), 716 (Rev.WRC-2000) and 734 (Rev.WRC-07) and Recommendation 206 (WRC-07), see ECP under Agenda item 4 (WRC-12 contribution 0005 Add. 27).

MOD	EUR/8.1E/1

5.388		The bands 1 885-2 025 MHz and 2 110-2 200 MHz are intended for use, on a worldwide basis, by administrations wishing to implement International Mobile Telecommunications (IMT). Such use does not preclude the use of these bands by other services to which they are allocated. The bands should be made available for IMT- in accordance with Resolution 212 (Rev.WRC-07). (See also Resolution 223 (Rev.WRC-07))

Reasons: 	No.5.388 refers to Resolution 212 (Rev.WRC-07), which was revised at WRC-07 and addresses IMT. Therefore, to be consistent with that WRC-07 decision, the reference to IMT-2000 has to be replaced by IMT in No. 5.388. Also the references to WRC Resolution 212 (Rev.WRC-07) and WRC Resolution 223 (Rev.WRC-07) need to be updated.

MOD	EUR/8.1E/2

5.388A		In Regions 1 and 3, the bands 1 885-1 980 MHz, 2 010-2 025 MHz and 2 110-2 170 MHz and, in Region 2, the bands 1 885-1 980 MHz and 2 110-2 160 MHz may be used by high altitude platform stations as base stations to provide International Mobile Telecommunications- (IMT-), in accordance with Resolution 221 (Rev.WRC-07). Their use by IMT- applications using high altitude platform stations as base stations does not preclude the use of these bands by any station in the services to which they are allocated and does not establish priority in the Radio Regulations.

Reasons: 	No. 5.388A refers to Resolution 221 (Rev.WRC-07), which was revised at WRC-07 and addresses IMT. Therefore, to be consistent with that WRC-07 decision, the reference to IMT-2000 has to be replaced by IMT in No. 5.388A. Also the reference to WRC Resolution 221 (Rev.WRC-07) needs to be updated.

MOD	EUR/8.1E/3

5.388B		In Algeria, Saudi Arabia, Bahrain, Benin, Burkina Faso, Cameroon, Comoros, Côte d’Ivoire, China, Cuba, Djibouti, Egypt, United Arab Emirates, Eritrea, Ethiopia, Gabon, Ghana, India, Iran (Islamic Republic of), Israel, the Libyan Arab Jamahiriya, Jordan, Kenya, Kuwait, Mali, Morocco, Mauritania, Nigeria, Oman, Uganda, Qatar, the Syrian Arab Republic, Senegal, Singapore, Sudan, Tanzania, Chad, Togo, Tunisia, Yemen, Zambia and Zimbabwe, for the purpose of protecting fixed and mobile services, including IMT- mobile stations, in their territories from co-channel interference, a high altitude platform station (HAPS) operating as an IMT- base station in neighbouring countries, in the bands referred to in No. 5.388A, shall not exceed a co-channel power flux-density of -127 dB(W/(m2 · MHz)) at the Earth’s surface outside a country’s borders unless explicit agreement of the affected administration is provided at the time of the notification of HAPS.

Reasons: 	No. 5.388B refers to No. 5.388A, which in turn refers to Resolution 223 (Rev.WRC-07), which was revised at WRC-07 and addresses IMT. Therefore, to be consistent with that WRC-07 decision, the reference to IMT-2000 has to be replaced by IMT in No. 5.388B.

MOD	EUR/8.1E/4

11.26A		Notices relating to assignments for high altitude platform stations operating as base stations to provide IMT- in the bands identified in 5.388A shall reach the Bureau not earlier than three years before the assignments are brought into use.

Reasons: 	No. 11.26A refers to No. 5.388A, which in turn refers to Resolution 223 (Rev.WRC-07), which was revised at WRC-07 and addresses IMT. Therefore, to be consistent with that WRC-07 decision, the reference to IMT-2000 has to be replaced by IMT in No. 11.26A.

ARTICLE  5

Frequency Allocations

MOD	EUR/XXA29E/5

5.462A		In Regions 1 and 3 (except for Japan), in the band 8 025-8 400 MHz, the Earth exploration-satellite service using geostationary satellites shall not produce a power flux-density in excess of the following values for angles of arrival (), without the consent of the affected administration:

135 dB(W/m2) in any 1 MHz band 	for       0°	        5º
–135 + 0.5 ( – 5) dB(W/m2) in any 1 MHz band	for       5º	      25º
–125 dB(W/m2) in any 1 MHz band	for     25º	      90º (WRC-12)

SUP	EUR/XXA29E/6

RESOLUTION  124  (Rev.WRC-2000)

Protection of the fixed service in the frequency band 8 025-8 400 MHz
sharing with geostationary-satellite systems of the Earth
exploration-satellite service (space-to-Earth)

Reasons: Noting the results of the ITU-R studies, which led to approval of Recommendation ITU-R F.1502 the pfd limits contained in No. 5.462A are not relevant anymore and create an inconsistency with Resolution 124. It is then proposed to adjust the pfd limits in No. 5.462A with the values contained in Recommendation ITU-R F.1502 and as a consequence delete Resolution 124.

APPENDIX  4  (Rev.WRC-07)

Consolidated list and tables of characteristics for use in the
application of the procedures of Chapter III

MOD	EUR/8.1E/7

1	The substance of this Appendix is separated into two parts: one concerning data and their use for terrestrial radiocommunication services and another concerning data and their use for space radiocommunication services and the radio astronomy service.

Reasons: 	Annex 2 to Appendix 4 is dealing with both space radiocommunication services and the radio astronomy service.

ANNEX  2

Characteristics of satellite networks, earth stations
or radio astronomy stations2

MOD	EUR/8.1E/8

_______________

2	The Radiocommunication Bureau shall develop and keep up-to-date forms of notice to meet fully the statutory provisions of this Appendix and related decisions of future conferences. Additional information on the items listed in this Annex together with an explanation of the symbols is to be found in the Preface to the BR IFIC (Space Services).

Reasons: 	Footnote 1 refers to the BR IFIC (Terrestrial services).

APPENDIX  5  (Rev.WRC-07)

Identification of administrations with which coordination is to be effected or agreement sought under the provisions of Article 9

ANNEX 1

TABLE 5-2

MOD	EUR/8.1E/9

NOTE 3 – The coordination thresholds in the band 2 160-2 270 MHz (Region 2) and 2 170-2 200 MHz (all Regions) to protect other terrestrial services do not apply to International Mobile Telecommunications (IMT-) systems, as the satellite and the terrestrial components are not intended to operate in the same area or on common frequencies within these bands.

Reasons: 	WRC-07 decided to use IMT as the root name for such applications.

ARTICLE 5

Frequency Allocations

MOD	EUR/8.1E/10

220 - 335.4 MHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 235-267 FIXED MOBILE 5.111 5.252 5.254 5.256 5.256A |  |  |

Reasons: 	WRC-07 decided to suppress No. 5.199.

MOD	EUR/8.1E/11

200 – 248 GHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 200-209 EARTH EXPLORATION-SATELLITE (passive) RADIO ASTRONOMY SPACE RESEARCH (passive) 5.340 5.341 5.563A |  |  |

Reasons: 	It is noted that the 200-202 GHz and the 202-209 GHz band have identical allocations in all 3 regions. Furthermore, the three corresponding footnotes in both bands (Nos. 5.340, 5.341 and 5.563A), do not distinguish between the bands. References were made to the 200-209 GHz band in total (i.e., Nos. 5.340 and 5.563A) and to an overall frequency range of 197-220 GHz (i.e., No. 5.341).

*	Note by the Secretariat:  This Resolution was revised by WRC-2000.