|  |  | TEMP25 |
| --- | --- | --- |
| CPG-12 PTA Copenhagen (ECO), 5-7 October 2011 |  |  |
| Date issued: 6 October 2011 Source: AI7 DG Subject: WRC-12 Agenda item 7 – Suppressing the 6-month period between an API and a coordination request in Article 9 of the Radio Regulations |  |  |

Summary:

The proposal of a definitive list under No. 9.36.2 has been endorsed at the last CPG meeting in Oxford. If this is also approved by WRC-12, there will be approximately 15 months between the receipt of an advance publication and the publication of the definitive list (6 months between API and coordination request, 3 months to publish the coordination request, 4 months to comment, approximately 2 months to publish the definitive list following the comments). Although this drawback was not identified in the CPM report, it is proposed to correct it by suppressing the 6-month period between the API receipt and the receipt of the coordination request. This would imply allocating only 9 months out of 7 years for the “publication” part of the coordination process. In order to provide transition measures, Europe proposes that the provisions below should enter into force 6 months after the end of the Conference.

Proposal:

Europe proposes to include the following regulatory provisions in a draft ECP on WRC-12 agenda item 7.

Background:

ARTICLE  9

Procedure for effecting coordination with or 
            obtaining agreement of other administrations1, 2, 3, 4, 5, 6, 7, 8

Section I  –  Advance publication of information on satellite
networks or satellite systems

General

MOD		EUR/5A28/xx

9.1		Before initiating any action under this Article or under Article 11 in respect of frequency assignments for a satellite network or a satellite system, an administration, or one9 acting on behalf of a group of named administrations, shall, prior to the coordination procedure described in Section II of Article 9 below, where applicable, send to the Bureau a general description of the network or system for advance publication in the International Frequency Information Circular (BR IFIC) not earlier than seven years and preferably not later than two years before the planned date of bringing into use of the network or system (see also No. 11.44). The characteristics to be provided for this purpose are listed in Appendix 4. The coordination or notification information may also be communicated to the Bureau at the same time. Where coordination is not required by Section II, notification shall be considered as having been received by the Bureau not earlier than six months after the date of publication of the advance publication information.

Reasons: To suppress the six-month period between the receipt of an advance publication and the receipt of the associated coordination request under Section II of Article 9, in order not to reduce the part dedicated to publication of special sections within the coordination process.

Sub-Section IB  –  Advance publication of information on satellite networks or satellite
systems that are subject to coordination procedure under Section II

MOD		EUR/5A28/yy

9.5B		If, upon receipt of the BR IFIC containing information published under No. 9.2B, any administration considers its existing or planned satellite systems or networks or terrestrial stations11 to be affected, it may send its comments to the publishing administration, so that the latter may take those comments into consideration during the coordination procedure. A copy of these comments may also be sent to the Bureau. Thereafter, both administrations shall endeavour to cooperate in joint efforts to resolve any difficulties, with the assistance of the Bureau, if so requested by either of the parties, and shall exchange any additional relevant information that may be available.     (WRC-2000)

Reasons: Consequential to the suppression of the six-month period because the coordination request can be sent before the publication of the advance publication.