|  |  |  | CPG PTC(11)070 |
| --- | --- | --- | --- |
| CPG PTC- 11 |  |  |  |
| Mainz, 27 – 30 September 2011 |  |  |  |
| Date issued: | 20 September 2011 |  |  |
| Source: | The Netherlands |  |  |
| Subject: | Proposed amendments to the draft ECP on agenda item 1.15 |  |  |

N

Password protection required? (Y/N)

Summary:

This proposal is to modify the draft ECP based on CPG12(2011) 032 Annex V AI 1.15. Proposed amendments are highlighted in turquoise. Furthermore the draft ECP text highlighted in green needs to be removed from the draft text and the yellow parts needs to be accepted to bring the draft text in line with the existing RR text.

Proposal:

PT-C is requested to take this proposal into account for further development of the draft ECP WRC-12 AI 1.15.

Background:

CPG12(2011) 032 Annex  V  AI 1.15 Draft ECP

PRELIMINARY DRAFT EUROPEAN COMMON PROPOSALS FOR THE WORK OF THE CONFERENCE

AGENDA ITEM 1.15

1.15	to consider possible allocations in the range 3-50 MHz to the radiolocation service for oceanographic radar applications, taking into account the results of ITU-R studies, in accordance with Resolution 612 (WRC-07);

Introduction

This agenda item covers the following issue:

Resolution 612 (WRC-2007): Use of the radiolocation service between 3 and 50 MHz to support high-frequency oceanographic radar operations

CEPT proposes new primary allocations for the radiolocation service in portions of the 3 to 50 MHz band identified as suitable for current and future oceanographic radar operations. CEPT also proposes technical and regulatory conditions to protect other services in these frequency bands, including the emission of a call sign to properly identify oceanographic systems.

Proposals

MOD	EUR/XXA15/1

5 003-7 450 kHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| … |  |  |
| 5 060-5 160 FIXED MOBILE except aeronautical mobile RADIOLOCATION 5.XXX |  |  |
| 5 160-5 250 FIXED MOBILE except aeronautical mobile |  |  |
| ... |  |  |

7 450 – 13 360 kHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| …/… |  |  |
| 9 040-9 200 FIXED |  |  |
| 9 200-9 400 FIXED RADIOLOCATION 5.XXX |  |  |
| …/… |  |  |
| 12 100-12 200 FIXED RADIOLOCATION 5.XXX |  |  |
| 12 200-12 230 FIXED |  |  |
| …/… |  |  |

13 360 - 18 030 kHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| …/… |  |  |
| 13 410-13 510 FIXED MOBILE except aeronautical mobile (R) RADIOLOCATION 5.XXX 5.150 |  |  |
| 13 510-13 570 FIXED MOBILE except aeronautical mobile (R) 5.150 |  |  |
| …/… |  |  |
| 13 870-14 000 FIXED MOBILE except aeronautical mobile (R). …/…. 14 350-14 990 FIXED MOBILE except aeronautical mobile (R). |
| …/… |  |  |
| 15 800-16 000 FIXED 5.153 |  |  |
| 16 000-16 200 FIXED RADIOLOCATION 5.XXX 5.153 |  |  |
| 16 200-16 360 FIXED 5.153 |  |  |
| …/… |  |  |

18 030–23 350 kHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| …/… |  |  |
| 18 168 – 18 780 FIXED MOBILEexcept aeronautical mobile |  |  |
| …/… |  |  |
| 23 000 – 23 200 FIXED MOBILE except aeronautical mobile 5.156 |  |  |

23 350-27 500 kHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| …/… |  |  |
| 26 175 - 27 200 FIXED MOBILE except aeronautical mobile 5.150 |  |  |
| 27 200 -27 500 FIXED MOBILE exceptaeronautical mobile RADIOLOCATION 5.XXX 5.150 |  |  |

27.5-47 MHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| …/… |  |  |
| 38.25-39.00 FIXED MOBILE |  |  |
| 39.00-39.5 FIXED MOBILE RADIOLOCATION 5.XXX |  |  |
| 39.5-39.986 FIXED MOBILE |  |  |
| …/… |  |  |

40.02-40.52 FIXED MOBILE RADIOLOCATION 5.XXX 5.150

40.52-40.98 FIXED MOBILE 5.150

…/…

ADD	EUR/XXA15/2

5.XXX	Radiolocation stations operating in the radiolocation service in the bands 5 060-5 160 kHz, -9 200-9 400 kHz, 12 100-12 200 kHz, 13 410-13 510 kHz, 16 000-16 200 kHz,  27 200-27 500 kHz, 39-39.5 MHz and 40.02-40.52 MHz  shall comply with the provisions of Resolution 612 (Rev. WRC-12).     (WRC-12)

ADD	EUR/XXA15/3

| AP4-4 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TABLE 1 |  |  |  |  |  |  |  |  |  |  |
| Characteristics for terrestrial services |  |  |  |  |  |  |  |  |  |  |
| 3 |  |  | CALL SIGN AND STATION IDENTIFICATION |  |  |  |  |  |  |  |  |  |
| 3.1 |  | 3A1 | the call sign used in accordance with Article 19 | O | O | + |  |  |  | O | 3A1 |  |
|  |  |  | In the case of a transmitting station, for the fixed service below 28 MHz, mobile service, meteorological aids service, radiolocation service between 3 and 50 MHz (operating in accordance with Resolution 612 (Rev.WRC 12)),, or standard frequency and time signal service, in the application of Article 11, required if the station identification (3A2) is not provided |  |  |  |  |  |  |  |  |  |

ARTICLE  19

Identification of stations

Section I  –  General provisions

MOD	EUR/XXA15/4

19.1.1	In the present state of the technique, it is recognized nevertheless that the transmission of identifying signals for certain radio systems (e.g. radiodetermination, radio relay systems and space systems) is not always possible. However, for stations in the radiolocation service, notified to the Bureau or brought into use after 17 February 2012, in bands between 3 and 50 MHz (operating in accordance with Resolution 612 (WRC-12)), the provisions of No. 19.51 shall be applied.

MOD	EUR/XXA15/5

RESOLUTION 612 (Rev.WRC-12)

Use of the radiolocation service between 3 and 50 MHz to
support oceanographic radar operations

The World Radiocommunication Conference (Geneva, 2012),

considering

a)	that there is increasing interest, on a global basis, in the operation of oceanographic radars for measurement of coastal sea surface conditions to support environmental, oceanographic, meteorological, climatological, maritime and disaster mitigation operations;

b)	that oceanographic radars are also known in parts of the world as HF ocean radars, HF wave height sensing radars or HF surface wave radars;

c)	that oceanographic radars operate through the use of ground-wave propagation over seas;

d)	that oceanographic radar technology has applications in global maritime domain awareness by allowing the long-range sensing of surface vessels, which provides a benefit to the global safety and security of shipping and ports;

e)	that operation of oceanographic radars provides benefits to society through environmental protection, disaster preparedness, public health protection, improved meteorological operations, increased coastal and maritime safety and enhancement of national economies;

f )	that oceanographic radars have been operated on an experimental basis around the world, providing an understanding of spectrum needs and spectrum sharing considerations, as well as an understanding of the benefits these systems provide;

g)	that performance and data requirements dictate the regions of spectrum that can be used by oceanographic radar systems for ocean observations,

recognizing

a)	that oceanographic radars have been operated  under provision N°4.4 since the 1970s by several administrations..

b)	that developers of the experimental systems have implemented techniques to make the most efficient use of the spectrum and mitigate interference to other radio services;

-

c)	that for the purpose of protecting existing services from harmful interference, received interference by those services from oceanographic radars shall not exceed a power flux-density with an I/N ratio of −6 dB in an area where its radio noise is categorized to the rural and the quiet rural defined in Recommendation ITU-R P.372-10;

d)	that for the purpose of protecting existing services from harmful interference, oceanographic radars’ impact via ground-wave propagation can be checked by Report ITU-R M.[RLS 3-50 MHz SHARING] based on Recommendation ITU-R P.368-9,

resolves

-

-

--

- that in accordance with No. 11.2 of the RR oceanographic radars shall be notified to the Bureau and shall contain the station identification (call sign) (Appendix 4 and Article 19)

- that the peak e.i.r.p of  oceanographic radars shall not exceed 25 dBW;

- that for transmission of the station identification machine readable code shall be used (No.19.18);

4	that the separation distance between an oceanographic radar and the border of a neighbouring country shall be higher than the distances specified in the following table unless otherwise agreed between neighbouring countries:

| Frequency band | Land path |  | Sea path or mixed |  |
| --- | --- | --- | --- | --- |
|  | Rural* | Quiet Rural* | Rural* | Quiet Rural* |
| 5060-5160 kHz | 120 Km | 170 Km | 790 Km | 920 Km |
| 9 200-9400 KHz | 100 Km | 130 Km | 590 Km | 670 Km |
| 12100-12200 kHz and 13410-13510 kHz | 100 Km | 110 Km | 480 Km | 520 Km |
| 16000-16200 kHz | 80 Km | 100 Km | 390 Km | 450 Km |
| 27200-27500 kHz | 80 Km | 100 Km | 280 Km | 320 Km |
| 39-39.5 MHz and 40.02-40.52 MHz | 80 Km | 100 km | 200 Km | 230 Km |

-

-

Reasons: CEPT supports new primary allocations for the radiolocation service in portions of the 3 to 50 MHz band identified as suitable for oceanographic radar operations. CEPT also supports technical and regulatory conditions to protect other services in these frequency bands.

[NOTE: CPG PTC decided to continue its work based on Option 1 of the draft ECP as presented in CPG-6 (Berlin). The complete text of Option 2 is therefore proposed to be deleted.]

Given the difficulty in predicting the harmful interference from the sky-wave propagation the separation distance must be understood to be the minimum required separation distance beyond which an oceanographic radar will not cause harmful interference into co-primary services receivers via ground-wave propagation. The rural or quiet rural environment applies to the location of the fixed or mobile service receiver, not the radiolocation system location.