|  |  |  | CPG PTC(11)077 |
| --- | --- | --- | --- |
| CPG PTC- 11 |  |  |  |
| Mainz, 27 – 30 September 2011 |  |  |  |
| Date issued: | 23 September 2011 |  |  |
| Source: | Germany |  |  |
| Subject: | CPG12(2011) 032 Annex V AI 1.3 Draft ECP |  |  |

N

Password protection required? (Y/N)

Summary:

The possible use of Satellite Systems intended for FSS Operation is proposed to be part of the CEPT Position.

Proposal:

PTC is invited to consider the proposed amendments

Background:

PRELIMINARY DRAFT EUROPEAN COMMON PROPOSALS FOR THE WORK OF THE CONFERENCE

AGENDA ITEM 1.3

1.3	to consider spectrum requirements and possible regulatory actions, including allocations, in order to support the safe operation of unmanned aircraft systems (UAS), based on the results of ITU-R studies, in accordance with Resolution 421 (WRC-07).

CONTENTS

Page

Introduction	1

1	Sub-Part 3A	3

2	Sub-Part 3B	10

Introduction

This agenda item considers the spectrum requirements to support the safe operation of unmanned aircraft systems in non-segregated airspace.

The safe flight operation of UAS needs reliable communication links and associated spectrum, especially for the remote pilot to command and control the flight and to relay the air traffic control communications. In addition the safe flight operation of UAS necessitates advanced techniques to detect and track nearby aircraft, terrain and obstacles to navigation in order to ensure the UAS avoids these objects in a manner equivalent to that achieved by manned aircraft.

A significant increase of the worldwide use of unmanned aircraft systems is expected in the future. The seamless operation of unmanned aircraft with piloted aircraft in non-segregated airspaces is becoming vital for the further development of unmanned aircraft applications that will fill many diverse requirements. Therefore, globally harmonized spectrum is required to satisfy this need. WRC-12 Agenda item 1.3 seeks to identify spectrum that can be used to meet this demand. The envisioned unmanned aircraft systems infrastructure will be composed of terrestrial and satellite components.

Report ITU-R M.2171 provides the analyses for determining the amount of spectrum required for the operation of a prospected number of unmanned aircraft systems sharing non-segregated airspace with manned aircraft as required by Resolution 421 (WRC-07) and in response to WRC-12 Agenda item 1.3.The operation of unmanned aircraft systems will require access to both terrestrial and satellite spectrum.

The maximum amount of spectrum required for unmanned aircraft systems are:

–	34 MHz for the terrestrial component,

–	56 MHz for the satellite component.

For the satellite component, some consequential modifications related to the coordination mechanism of the existing AMS(R)S allocation in 5 030-5 091 MHz in case of the new AM(R)S allocation are proposed in the same band (see sub-part 3A). No new allocations are required as there are enough frequency bands to accommodate the spectrum requirement for the satellite component. However, the requirement for resilient links demand the use dissimilar satellite links for redundancy. To provide the appropriate flexibility Europe proposes to also consider the use of systems in the FSS for the control and non payload communication (CNPC) of unmanned aircraft systems under certain conditions.

For the terrestrial component, Europe is of the view that AM(R)S allocations should be used. Studies showed that there is a lack of spectrum to accommodate the spectrum requirement for the terrestrial component. [Europe proposed to allocate the band 5 030-5 091 MHz to AM(R)S to satisfy the Agenda Item, along with consequential regulatory and technical considerations to protect the existing services.]

The proposal that follows is divided into two parts: sub-part 3A for the terrestrial component and sub-part 3B for the satellite component.

# Sub-Part 3A

Terrestrial component

[Taking into account the results of conducted studies the European countries propose the following solution:

- the frequency band 960-1164 MHz allocated to the aeronautical mobile (R) service (AM(R)S) shall not be used for UAS control links terrestrial component ;-

- [5000] 5150 MHz – [TBD];

- the AM(R)S frequency band 15.4-15.5 GHz should not be allocated to the UAS terrestrial component due to extreme difficulty of sharing between the CNPC applications and services allocated in co-frequency and adjacent frequency bands. ]

[Editorial Note: sharing studies are still ongoing. If no new AM(R)S allocation is proposed proposals A3/1 until A3/7 are obsolete]]

ARTICLE 5

Frequency Allocations

Section IV  –  Table of Frequency Allocations

[MOD	EUR/1.3/1

890-1300 MHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 960–1 164 | AERONAUTICAL RADIONAVIGATION 5.328 AERONAUTICAL MOBILE (R) MOD 5.327А |  |

MOD		EUR/1.3/2

5.327A	The use of the band 960-1 164 MHz by the aeronautical mobile (R) service is limited to systems that operate in accordance with recognized international aeronautical standards. Such use shall be in accordance with Resolution 417 (Rev.WRC- 12). The band 960-1164 MHz shall not be used by the unmanned aircraft systems.

EUR/1.3/3

NOC

ARTICLE 5

Frequency Allocations

Section IV  –  Table of Frequency Allocations

15.4 – 18.4 GHz

ARTICLE  5

Frequency allocations

Section IV  –  Table of Frequency Allocations
(See No. 2.1)

[MOD	EUR/A3/1

4 800-5 570 MHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| ... |  |  |
| 5 030-5 091 AERONAUTICAL RADIONAVIGATION AERONAUTICAL MOBILE-SATELLITE (R) ADD 5.C103 AERONAUTICAL MOBILE (R) ADD 5.D103 5.444 |  |  |
| ... |  |  |

Reasons: Sharing studies have shown that an AM(R)S allocation is feasible in the band 5030-5091 MHz, provided that power limitations are associated to these allocations.

ADD	EUR/A3/2

5.C103	The use of the band 5 030-5 091 MHz by aeronautical mobile-satellite (R) service is limited to internationally standardized aeronautical systems and is subject to coordination under No. 9.11A.

Reasons: This new footnote replaces the application of 9.21 for the AMS(R)S allocation by the application of 9.11A, as a consequence of the new aeronautical mobile (R) allocation.

ADD	EUR/A3/3

5.D103	The use of the band 5 030-5 091 MHz by the aeronautical mobile (R) service is limited to internationally standardized aeronautical systems. In order to protect the radionavigation-satellite service (Space-to-earth) in the adjacent band 5 010-5 030 MHz, the e.i.r.p. density of any AM(R)S station shall not exceed -75 dBW/MHz in the band 5 010-5 030 MHz.

Reasons: This new footnote limits the use of the AM(R)S allocation to systems developed and standardized by ICAO to facilitate the development of such an international system and the coordination between ICAO systems in this band. The out-of-band e.i.r.p. density limit proposed will protect RNSS receivers in 5010-5030 MHz.

MOD	EUR/A3/4

  - Additional allocation:  The bands 1 610-1 626.5 MHz, 5 000-5 030 MHz and 5 091-5 150 MHz are also allocated to the aeronautical mobile-satellite (R) service on a primary basis, subject to agreement obtained under No. 9.21.

Reasons: Modification to this footnote is a consequence to the new footnote 5.C103.

APPENDIX  5  (Rev.WRC-12)

Identification of administrations with which coordination is to be effected or
agreement sought under the provisions of Article 9

MOD	EUR/A3/5

TABLE  5-1     (WRC-07)

Technical conditions for coordination
(see Article 9)

Reasons: A new condition is introduced in Appendix 5 to implement coordination under No. 9.14 for AMS(R)S in the band 5 030-5 091 MHz (See modification in the Table 5-1 below). The level of the accepted interference by the identified administration will be fixed by agreement between administrations concerned using the relevant ITU-R Recommendations as a guide.

| Reference of Article 9 | Case | Frequency bands (and Region) of the service for which coordination is sought | Threshold/condition | Calculation method | Remarks |
| --- | --- | --- | --- | --- | --- |
| No. 9.14 Non-GSO/ terrestrial, GSO/ terrestrial | A space station in a satellite network in the frequency bands for which a footnote refers to No. 9.11A or to No. 9.14, in respect of stations of terrestrial services where threshold(s) is (are) exceeded | 1) Frequency bands for which a footnote refers to No. 9.11A; or 2) 11.7-12.2 GHz (Region 2 GSO FSS) 3) 5 030-5 091 MHz | 1) See § 1 of Annex 1 to this Appendix; In the bands specified in No. 5.414A, the detailed conditions for the application of No. 9.14 are provided in No. 5.414A for MSS networks or 2) In the band 11.7-12.2 GHz (Region 2 GSO FSS): –124 dB(W/(m2 · MHz)) for 0° 5 –124 + 0.5 ( – 5) dB(W/(m2 · MHz)) for 5° < 25 –114 dB(W/(m2 · MHz)) for > 25 where is the angle of arrival of the incident wave above the horizontal plane (degrees) 3) In the band 5 030-5 091 MHz (AMS(R)S), bandwidth overlap is the threshold. | 1) See § 1 of Annex 1 to this Appendix |  |

APPENDIX  7  (Rev.WRC-12)

Methods for the determination of the coordination area around an earth
station in frequency bands between 100 MHz and 105 GHz

MOD	EUR/A3/6

Reasons: New parameters are introduced in Appendix 7 in Table 7B and 9A to implement coordination under No. 9.15 to 9.18 for AMS(R)S in the band 5 030-5 091 MHz (See modification in the two Tables below) .

TABLE  7b     (WRC-07)

Parameters required for the determination of coordination distance for a transmitting earth station

| Transmitting space radiocommunication service designation |  | Fixed-satellite, mobile-satellite | Aeronautical mobile satellite (R) service | Aeronautical mobile satellite (R) service | Fixed- satellite |  | Fixed- satellite |  | Fixed- satellite |  | Fixed- satellite |  |  |  | Space operation, space research |  |  |  | Fixed-satellite, mobile-satellite, meteorological- satellite |  |  |  | Fixed- satellite |  |  |  | Fixed- satellite |  |  |  | Fixed- satellite |  | Fixed- satellite 3 |  | Fixed- satellite |  | Fixed- satellite 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Frequency bands (GHz) |  | 2.655-2.690 | 5.030-5.091 | 5.030-5.091 | 5.091-5.150 |  | 5.091-5.150 |  | 5.725-5.850 |  | 5.725-7.075 |  |  |  | 7.100-7.235 5 |  |  |  | 7.900-8.400 |  |  |  | 10.7-11.7 |  |  |  | 12.5-14.8 |  |  |  | 13.75-14.3 |  | 15.43-15.65 |  | 17.7-18.4 |  | 19.3-19.7 |  |
| Receiving terrestrial service designations |  | Fixed, mobile | Aeronautical radio- navigation | Aeronautical mobile (R) | Aeronautical radio- navigation |  | Aeronautical mobile (R) |  | Radio-location |  | Fixed, mobile |  |  |  | Fixed, mobile |  |  |  | Fixed, mobile |  |  |  | Fixed, mobile |  |  |  | Fixed, mobile |  |  |  | Radiolocation radionavigation (land only) |  | Aeronautical radionavigation |  | Fixed, mobile |  | Fixed, mobile |  |
| Method to be used |  | § 2.1 | § 2.1, § 2.2 | § 2.1, § 2.2 |  |  |  |  | § 2.1 |  | § 2.1 |  |  |  | § 2.1, § 2.2 |  |  |  | § 2.1 |  |  |  | § 2.1 |  |  |  | § 2.1, § 2.2 |  |  |  | § 2.1 |  |  |  | § 2.1, § 2.2 |  | § 2.2 |  |
| Modulation at terrestrial station 1 |  | A |  |  |  |  |  |  |  |  | A |  | N |  | A |  | N |  | A |  | N |  | A |  | N |  | A |  | N |  | – |  |  |  | N |  | N |  |
| Terrestrial station interference parameters and criteria | p0 (%) | 0.01 |  |  |  |  |  |  |  |  |  | 0.01 |  | 0.005 |  | 0.01 |  | 0.005 |  | 0.01 |  | 0.005 |  | 0.01 |  | 0.005 |  | 0.01 |  | 0.005 |  | 0.01 |  |  |  | 0.005 |  | 0.005 |  |
|  | n | 2 |  |  |  |  |  |  |  |  |  | 2 |  | 2 |  | 2 |  | 2 |  | 2 |  | 2 |  | 2 |  | 2 |  | 2 |  | 2 |  | 1 |  |  |  | 2 |  | 2 |  |
|  | p (%) | 0.005 |  |  |  |  |  |  |  |  |  | 0.005 |  | 0.0025 |  | 0.005 |  | 0.0025 |  | 0.005 |  | 0.0025 |  | 0.005 |  | 0.0025 |  | 0.005 |  | 0.0025 |  | 0.01 |  |  |  | 0.0025 |  | 0.0025 |  |
|  | NL (dB) | 0 |  |  |  |  |  |  |  |  |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  |  |  | 0 |  | 0 |  |
|  | Ms (dB) | 26 2 |  |  |  |  |  |  |  |  |  | 33 |  | 37 |  | 33 |  | 37 |  | 33 |  | 37 |  | 33 |  | 40 |  | 33 |  | 40 |  | 1 |  |  |  | 25 |  | 25 |  |
|  | W (dB) | 0 |  |  |  |  |  |  |  |  |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  |  |  | 0 |  | 0 |  |
| Terrestrial station parameters | Gx (dBi) 4 | 49 2 | 6 | 10 |  | 6 |  | 6 |  |  |  | 46 |  | 46 |  | 46 |  | 46 |  | 46 |  | 46 |  | 50 |  | 50 |  | 52 |  | 52 |  | 36 |  |  |  | 48 |  | 48 |  |
|  | Te (K) | 500 2 |  |  |  |  |  |  |  |  |  | 750 |  | 750 |  | 750 |  | 750 |  | 750 |  | 750 |  | 1 500 |  | 1 100 |  | 1 500 |  | 1 100 |  | 2 636 |  |  |  | 1 100 |  | 1 100 |  |
| Reference bandwidth | B (Hz) | 4  103 | 150  103 | 37.5 x 103 |  | 150  103 |  | 106 |  |  |  | 4  103 |  | 106 |  | 4  103 |  | 106 |  | 4  103 |  | 106 |  | 4  103 |  | 106 |  | 4  103 |  | 106 |  | 107 |  |  |  | 106 |  | 106 |  |
| Permissible interference power | Pr( p) (dBW) in B | –140 | –160 | –154 |  | –160 |  | –143 |  |  |  | –131 |  | –103 |  | –131 |  | –103 |  | –131 |  | –103 |  | –128 |  | –98 |  | –128 |  | –98 |  | –131 |  |  |  | –113 |  | –113 |  |

MOD	EUR/A3/7

TABLE  9a     (WRC-12)

Parameters required for the determination of coordination distance for a transmitting earth station
in bands shared bidirectionally with receiving earth stations

| Space service designation in which the transmitting earth station operates |  | Land mobile- satellite | Mobile-satellite | Land mobile-satellite | Earth exploration-satellite, meteorological-satellite | Mobile-satellite |  | Fixed-satellite, mobile-satellite | Aeronautical mobile satellite (R) service |  | Fixed- satellite 3 |  | Fixed-satellite | Fixed-satellite, meteorological- satellite | Fixed-satellite |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Frequency bands (GHz) |  | 0.1499-0.15005 | 0.272-0.273 | 0.3999-0.40005 | 0.401-0.402 | 1.670-1.675 |  | 2.655-2.690 | 5.030-5.091 |  | 5.150-5.216 |  | 6.700-7.075 | 8.025-8.400 | 8.025-8.400 |
| Space service designation in which the receiving earth station operates |  | Radio-navigation-satellite | Space operation | Radio-navigation-satellite | Space operation | Meteorological-satellite |  | Fixed-satellite, broadcasting-satellite | Aeronautical mobile satellite (R) service |  | Fixed-satellite | Radiodetermination-satellite | Fixed-satellite | Earth exploration- satellite | Earth exploration- satellite |
| Orbit 6 |  |  | Non-GSO |  | Non-GSO | Non-GSO | GSO |  | Non-GSO | GSO | Non-GSO |  | Non-GSO | Non-GSO | GSO |
| Modulation at receiving earth station 1 |  |  | N |  | N | N | N |  |  |  |  |  | N | N | N |
| Receiving earth station interference parameters and criteria | p0 (%) |  | 1.0 |  | 0.1 | 0.006 | 0.011 |  |  |  |  |  | 0.005 | 0.011 | 0.083 |
|  | n |  | 1 |  | 2 | 3 | 2 |  |  |  |  |  | 3 | 2 | 2 |
|  | p (%) |  | 1.0 |  | 0.05 | 0.002 | 0.0055 |  |  |  |  |  | 0.0017 | 0.0055 | 0.0415 |
|  | NL (dB) | 0 | 0 | 0 | 0 | 0 | 0 |  |  |  |  |  | 1 | 0 | 1 |
|  | Ms (dB) | 2 | 1 | 2 | 1 | 2.8 | 0.9 | 2 |  |  | 2 | 2 | 2 | 4.7 | 2 |
|  | W (dB) | 0 | 0 | 0 | 0 | 0 | 0 |  |  |  |  |  | 0 | 0 | 0 |
| Receiving earth station parameters | Gm (dBi) 2 | 0 | 20 | 0 | 20 | 30 | 45 |  | 45 | 45 | 48.5 |  | 50.7 |  |  |
|  | Gr (dBi) 4 | 0 | 19 | 0 | 19 | 19 9 | 8 |  | 8 | 8 | 10 |  | 10 | 10 | 8 |
|  | min 5 | 3 | 10 | 3 | 10 | 5 | 3 | 3 | 10° | 10° | 3 | 3 | 3 | 5 | 3 |
|  | Te (K) 7 | 200 | 500 | 200 | 500 | 370 | 118 | 75 | 340 | 340 | 75 | 75 | 75 |  |  |
| Reference bandwidth | B (Hz) | 4  103 | 103 | 4  103 | 1 | 106 | 4  103 |  | 37.5 x103 | 37.5 x103 |  |  | 106 | 106 | 106 |
| Permissible interference power | Pr( p) (dBW) in B | –172 | –177 | –172 | –208 | –145 | –178 |  | -163.5 | -163.5 |  |  | –151 | –142 | –154 |

# Sub-Part 3B

Satellite component

[NOTE: The following text highlighted in grey is intended to replace the deleted text above.]

mod EUR/A3/8

ARTCLE 5

Frequency Allocations

7 250-8 500 MHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 7 250-7 300 FIXED FIXED-SATELLITE (space-to-Earth) 5.YYY MOBILE 5.461 |  |  |
| 7 300-7 450 FIXED FIXED-SATELLITE (space-to-Earth) 5.YYY MOBILE except aeronautical mobile 5.461 |  |  |
| 7 450-7 550 FIXED FIXED-SATELLITE (space-to-Earth) METEOROLOGICAL-SATELLITE (space-to-Earth) 5.YYY MOBILE except aeronautical mobile 5.461A |  |  |
| 7 550-7 750 FIXED FIXED-SATELLITE (space-to-Earth) 5.YYY MOBILE except aeronautical mobile |  |  |
| … |  |  |
| 7 900-8 025 FIXED FIXED-SATELLITE (Earth-to-space) 5.ZZZ MOBILE 5.461 |  |  |
| 8 025-8 175 EARTH EXPLORATION-SATELLITE (space-to-Earth) FIXED FIXED-SATELLITE (Earth-to-space) 5.ZZZ MOBILE 5.463 5.462A |  |  |
| 8 175-8 215 EARTH EXPLORATION-SATELLITE (space-to-Earth) FIXED FIXED-SATELLITE (Earth-to-space) 5.ZZZ METEOROLOGICAL-SATELLITE (Earth-to-space) MOBILE 5.463 5.462A |  |  |
| 8 215-8 400 EARTH EXPLORATION-SATELLITE (space-to-Earth) FIXED FIXED-SATELLITE (Earth-to-space) 5.ZZZ MOBILE 5.463 5.462A |  |  |
| … |  |  |

Reason:

This proposal opens adequate flexibility for the operation of reliant CMPC links for unmanned aircrafts for an appropriate degree of redundancy.

MOD EUR/1.3/9

10-11.7 GHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| … |  |  |
| 10.7-11.7 FIXED FIXED-SATELLITE (space-to-Earth) 5.441 5.484A 5.YYY (Earth-to-space) 5.484 MOBILE except aeronautical mobile | 10.7-11.7 FIXED FIXED-SATELLITE (space-to-Earth) 5.441 5.484A 5.YYY MOBILE except aeronautical mobile |  |

Reason:

This proposal intends to achieve an adequate flexibility for the operation of reliant CNPC links for unmanned aircrafts for an appropriate degree of redundancy.

MOD EUR/1.3/10

11.7-14 GHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 11.7-12.5 FIXED MOBILE except aeronautical mobile BROADCASTING BROADCASTING-SATELLITE 5.492 | 11.7-12.1 FIXED 5.486 FIXED-SATELLITE (space-to-Earth) 5.484A 5.488 5.YYY Mobile except aeronautical mobile 5.485 | 11.7-12.2 FIXED MOBILE except aeronautical mobile BROADCASTING BROADCASTING-SATELLITE 5.492 |
|  | 12.1-12.2 FIXED-SATELLITE (space-to-Earth) 5.484A 5.488 5.YYY |  |
|  | 5.485 5.489 | 5.487 5.487A |
|  | 12.2-12.7 FIXED MOBILE except aeronautical mobile BROADCASTING BROADCASTING-SATELLITE 5.492 | 12.2-12.5 FIXED FIXED-SATELLITE (space-to-Earth) MOBILE except aeronautical mobile BROADCASTING |
| 5.487 5.487A |  | 5.484A 5.487 |
| 12.5-12.75 | 5.487A 5.488 5.490 | 12.5-12.75 |
| FIXED-SATELLITE (space-to-Earth) 5.484A 5.YYY (Earth-to-space) 5.494 5.495 5.496 | 12.7-12.75 FIXED FIXED-SATELLITE (Earth-to-space) MOBILE except aeronautical mobile | FIXED FIXED-SATELLITE (space-to-Earth) 5.484A 5.YYY MOBILE except aeronautical mobile BROADCASTING- SATELLITE 5.493 |
| … |  |  |
| 13.75-14 FIXED-SATELLITE (Earth-to-space) 5.484A 5.ZZZ RADIOLOCATION Earth exploration-satellite Standard frequency and time signal-satellite (Earth-to-space) Space research 5.499 5.500 5.501 5.502 5.503 |  |  |

Reason:

This proposal opens adequate flexibility for the operation of reliant CMPC links for unmanned aircrafts for an appropriate degree of redundancy.

MOD EUR/1.3/11

| 14-15.4 GHz |  |  |
| --- | --- | --- |
| Allocation to services |  |  |
| Region 1 | Region 2 | Region 3 |
| 14-14.25 FIXED-SATELLITE (Earth-to-space) 5.457A 5.457B 5.484A 5.506 5.506B 5.ZZZ RADIONAVIGATION 5.504 Mobile-satellite (Earth-to-space) 5.504B 5.504C 5.506A Space research 5.504A 5.505 |  |  |
| 14.25-14.3 FIXED-SATELLITE (Earth-to-space) 5.457A 5.457B 5.484A 5.506 5.506B 5.ZZZ RADIONAVIGATION 5.504 Mobile-satellite (Earth-to-space) 5.504B 5.506A 5.508A Space research 5.504A 5.505 5.508 |  |  |
| 14.3-14.4 FIXED FIXED-SATELLITE (Earth-to-space) 5.457A 5.457B 5.484A 5.506 5.506B 5.ZZZ MOBILE except aeronautical mobile Mobile-satellite (Earth-to-space) 5.504B 5.506A 5.509A Radionavigation-satellite 5.504A | 14.3-14.4 FIXED-SATELLITE (Earth-to-space) 5.457A 5.484A 5.506 5.506B 5.ZZZ Mobile-satellite (Earth-to-space) 5.506A Radionavigation-satellite 5.504A | 14.3-14.4 FIXED FIXED-SATELLITE (Earth-to-space) 5.457A 5.484A 5.506 5.506B 5.ZZZ MOBILE except aeronautical mobile Mobile-satellite (Earth-to-space) 5.504B 5.506A 5.509A Radionavigation-satellite 5.504A |
| 14.4-14.47 FIXED FIXED-SATELLITE (Earth-to-space) 5.457A 5.457B 5.484A 5.506 5.506B 5.ZZZ MOBILE except aeronautical mobile Mobile-satellite (Earth-to-space) 5.504B 5.506A 5.509A Space research (space-to-Earth) 5.504A |  |  |
| 14.47-14.5 FIXED FIXED-SATELLITE (Earth-to-space) 5.457A 5.457B 5.484A 5.506 5.506B 5.ZZZ MOBILE except aeronautical mobile Mobile-satellite (Earth-to-space) 5.504B 5.506A 5.509A Radio astronomy 5.149 5.504A |  |  |
| … |  |  |

Reason:

This proposal opens adequate flexibility for the operation of reliant CMPC links for unmanned aircrafts for an appropriate degree of redundancy.

MOD EUR/1.3/12

15.4-18.4 GHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| … |  |  |
| 17.3-17.7 FIXED-SATELLITE (Earth-to-space) 5.516 (space-to-Earth) 5.516A 5.516B 5.YYY Radiolocation | 17.3-17.7 FIXED-SATELLITE (Earth-to-space) 5.516 BROADCASTING-SATELLITE Radiolocation | 17.3-17.7 FIXED-SATELLITE (Earth-to-space) 5.516 Radiolocation |
| 5.514 | 5.514 5.515 | 5.514 |
| 17.7-18.1 FIXED FIXED-SATELLITE (space-to-Earth) 5.484A 5.YYY (Earth-to-space) 5.516 MOBILE | 17.7-17.8 FIXED FIXED-SATELLITE (space-to-Earth) 5.517 5.YYY (Earth-to-space) 5.516 BROADCASTING-SATELLITE Mobile 5.515 | 17.7-18.1 FIXED FIXED-SATELLITE (space-to-Earth) 5.484A 5.YYY (Earth-to-space) 5.516 MOBILE |
|  | 17.8-18.1 FIXED FIXED-SATELLITE (space-to-Earth) 5.484A 5.YYY (Earth-to-space) 5.516 MOBILE 5.519 |  |
| 18.1-18.4 FIXED FIXED-SATELLITE (space-to-Earth) 5.484A 5.516B 5.YYY (Earth-to-space) 5.520 MOBILE 5.519 5.521 |  |  |

Reason:

This proposal opens adequate flexibility for the operation of reliant CMPC links for unmanned aircrafts for an appropriate degree of redundancy.

MOD EUR/1.3/13

18.4-22 GHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 18.4-18.6 FIXED FIXED-SATELLITE (space-to-Earth) 5.484A 5.516B 5.YYY MOBILE |  |  |
| 18.6-18.8 EARTH EXPLORATION-SATELLITE (passive) FIXED FIXED-SATELLITE (space-to-Earth) 5.522B 5.YYY MOBILE except aeronautical mobile Space research (passive) | 18.6-18.8 EARTH EXPLORATION- SATELLITE (passive) FIXED FIXED-SATELLITE (space-to-Earth) 5.516B 5.522B 5:YYY MOBILE except aeronautical mobile SPACE RESEARCH (passive) | 18.6-18.8 EARTH EXPLORATION-SATELLITE (passive) FIXED FIXED-SATELLITE (space-to-Earth) 5.522B 5.YYY MOBILE except aeronautical mobile Space research (passive) |
| 5.522A 5.522C | 5.522A | 5.522A |
| … |  |  |

Reason:

This proposal opens adequate flexibility for the operation of reliant CMPC links for unmanned aircrafts for an appropriate degree of redundancy.

MOD EUR/1.3/14

24.75-29.9 GHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| … |  |  |
| 27.5-28.5 FIXED 5.537A FIXED-SATELLITE (Earth-to-space) 5.484A 5.516B 5.539 5.ZZZ MOBILE 5.538 5.540 |  |  |
| 28.5-28.6 FIXED FIXED-SATELLITE (Earth-to-space) 5.484A 5.516B 5.523A 5.539 5.ZZZ MOBILE Earth exploration-satellite (Earth-to-space) 5.541 5.540 |  |  |
| 28.6-29.1 FIXED FIXED-SATELLITE (Earth-to-space) 5.484A 5.516B 5.523A 5.539 5 MOBILE Earth exploration-satellite (Earth-to-space) 5.541 5.540 |  |  |
| 29.1-29.5 FIXED FIXED-SATELLITE (Earth-to-space) 5.516B 5.523C 5.523E 5.535A 5.539 5.541A MOBILE Earth exploration-satellite (Earth-to-space) 5.541 5.540 |  |  |
| 29.5-29.9 FIXED-SATELLITE (Earth-to-space) 5.484A 5.516B 5.539 5.ZZZ Earth exploration-satellite (Earth-to-space) 5.541 Mobile-satellite (Earth-to-space) | 29.5-29.9 FIXED-SATELLITE (Earth-to-space) 5.484A 5.516B 5.539 MOBILE-SATELLITE (Earth-to-space) Earth exploration-satellite (Earth-to-space) 5.541 | 29.5-29.9 FIXED-SATELLITE (Earth-to-space) 5.484A 5.516B 5.539 5.ZZZ Earth exploration-satellite (Earth-to-space) 5.541 Mobile-satellite (Earth-to-space) |
| 5.540 5.542 | 5.525 5.526 5.527 5.529 5.540 5.542 | 5.540 5.542 |

Reason:

This proposal opens adequate flexibility for the operation of reliant CMPC links for unmanned aircrafts for an appropriate degree of redundancy.

ADD EUR/1.3/15

5.YYY: In the bands 7.25 – 7.75 GHz, 10.95 – 11.20 GHz, 11.45 – 11.70 GHz, 11.70 – 12.20 GHz (in Region 2 only), 12.20 – 12.50 GHz (in Region 3 only), 12.50 – 12.75 GHz (in Regions 1 and 3 only), 17.30 – 17.70 GHz (in Region 1 only), 17.70 – 18.80 GHz and 19.70 – 21.20 GHz, Earth stations on board unmanned aircraft (UA) and their associated UA control stations  may receive from geostationary satellite systems operated on a primary basis in the fixed satellite service (space-to-Earth) in accordance with Resolution [SAT-UAS-FSS] (WRC-12).

Reason:

This proposal opens adequate flexibility for the operation of reliant CMPC links for unmanned aircrafts for an appropriate degree of redundancy.

ADD EUR/1.3/16

5.ZZZ	In the bands 7.9 – 8.4 GHz, [13.75 – 14.00 GHz], 14.00 – 14.50 GHz and 27.50 – 28.6 GHz and 29.50 – 31.00 GHz Earth stations on board unmanned aircraft and their associated control stations  may transmit to geostationary satellite systems on a primary basis in the fixed- satellite service (Earth-to-space) in accordance with Resolution [SAT-UAS-FSS] (WRC-12).

Reason:

This proposal opens adequate flexibility for the operation of reliant CMPC links for unmanned aircrafts for an appropriate degree of redundancy.

ADD EUR/1.3/17

RESOLUTION [SAT-UAS-FSS] (WRC-12)

Use of frequency bands allocated to the Fixed-Satellite Service not subject to Appendix 30A/30B for the command and non-payload communications (CNPC) of unmanned aircraft systems in non-segregated airspaces

The World Radiocommunication Conference (Geneva, 2012),

considering

a)	that unmanned aircraft need to operate seamlessly with manned aircraft in non-segregated airspace;

b)	that the command and control, the ATC voice relay, and the sense and avoid information of UAs operating in non-segregated airspace require reliable and safe communication links;

c)	that the operation of UAs in non-segregated airspace has to comply with international aeronautical standards to achieve airworthiness certifications.on a worldwide basis;

d)	that satellite systems operating in the fixed-satellite service (FSS) have proven the capability of providing reliable and safe communication links as demanded in considering b);

further considering

a)	that there is a need to limit the number of communication equipments onboard an UA;

b)	that there exists a variety of proven technical methods that can be applied to provide reliable and safe digital communication links;

resolves

that for the radiocommunication links to command and control unmanned aircraft (UA), relay of Air Traffic Control (ATC) voice communications, and transmit sense and avoid data between an UA and the associated UA control station (UACS) via geostationary satellites, the frequency band(s) allocated on a primary basis to the FSS (except those covered by Appendices 30A and 30B) may be used, provided that such FSS satellite systems and the Earth Station on board the UA and the UACS meet the technical requirements contained in Annex 1 of this Resolution;

requests the Secretary-General

to bring this Resolution to the attention of ICAO.

Annex 1

Technical characteristics of fixed-satellite service systems to support
control communication links of unmanned aircraft systems (UAS)

# 1	Introduction

Umanned Aircrafts flying beyond-line-of-sight (BLOS) of the Earth Station controlling it are expected to maintain connectivitiy to the UACS responsible for the flight control through  satellite communication links These links comprise data for aircraft control, relay of Air Traffic Control (ATC) voice communications, and communicate sense and avoid data between the UA and the UA Control Station (UA CS).  It is likely that UA will primarily utilize terrestrial line-of-sight radio communications for all low-altitude operations, such as take-off, approach and landing, but will apply satellite communications for all operations during the beyond line of sight cruising time. Satellite links can provide all features required for a high link availability in order to meet national and international aviation certification requirements when flying in non-segregated airspaces.

This annex contains the minimum required performance criteria that are to be met and typical link characteristics of UAS control links to meet them.

The UACS Earth Station and UA Earth Station are operated under the same regulatory frame conditions as conventional FSS Earth Station.

ATC

UA

UACS ES

1

4

3

2

Satellite

Forward link:

1. UACS ES to satellite

2: satellite to UA ES

Return link:

3: UA ES to satellite

4: satellite to UA CS ES

5

6

FIGURE 1: UA Earth Station, UACS Earth Station and Satellite Links

# 2	Technical requirements

The technical characteristics of typical UAS communication links that describe the forward and return (UAS) link performance of a typical FSS satellite is provided below. .

a)	Frequency bands.

Space-to-Earth

[	7.25 – 7.75 GHz

10.95 – 11.20 GHz

11.45 – 11.70 GHz

11.70 – 12.20 GHz [Region 2 only]

12.20 – 12.50 GHz [Region 3 only]

12.50 – 12.75 GHz [Regions 1 and 3 only]

17.30 – 17.70 GHz in Region 1

17.70 – 18.8 GHz

19.70 – 21.20 GHz]

Earth-to-Space

[	7.9 – 8.4 GHz

13.75 – 14.00 GHz

14.00 – 14.50 GHz

27.50 – 28.60 GHz

29.50 – 31.0 GHz]

b) 	Protection criteria

The protection criteria of incumbent services in the frequency bands mentioned under a) for the Earth-to-space case (uplink) have to be in accordance with appropriate regulations of RR, Article 22, in particular with:

- Section II – Control of interference to geostationary-satellite systems

- Section VI – Off-axis power limits on earth stations of a geostationary-satellite network in the fixed-satellite service

c)	The rain conditions (i.e. rain rates) in which the links must operate.

UA CS Earth stations and UA Earth stations should be designed to achieve the required link availability taking radio channel propagation conditions into account as given by . Recommendation ITU-R P-837

d)	Pointing accuracy of the UA Earth Station antenna.

The UA Earth Station antenna  main beam deviation from the space station shall be smaller than the 1 dB main beam off-axis angle at any specified frequency over the full range of azimuth and elevation antenna direction angles.

Reason:

This proposal opens adequate flexibility for the operation of reliant CMPC links for unmanned aircrafts for an appropriate degree of redundancy