|  |  |  | CPG PTC(11) TEMP 040R1 |
| --- | --- | --- | --- |
| CPG PTC- 11 |  |  |  |
| Mainz, 27 – 30 September 2011 |  |  |  |
| Date issued: | 26 September 2011 |  |  |
| Source: | Coordinator |  |  |
| Subject: | Proposed amendments to the draft Brief on agenda item 1.4 |  |  |

N

Password protection required? (Y/N)

Summary:

Proposal:

PTC is invited to consider this document to further develop the draft CEPT Brief on WRC-12 agenda item 1.4.

Background:

Draft CEPT Brief on agenda item 1.4

1.4	 to consider, based on the results of ITU-R studies, any further regulatory measures to facilitate introduction of new aeronautical mobile (R) service (AM(R)S) systems in the bands 112-117.975 MHz, 960-1 164 MHz and 5 000-5 030 MHz in accordance with Resolutions 413 (Rev.WRC-07), 417(WRC-07) and 420(WRC-07);

## Issue

This agenda item covers the following issues:

During the WRC-07 the aeronautical industry has been provided with provisional AM(R)S allocations in order to introduce new technology in the frequency bands 112-117.975 MHz (Resolution 413 (WRC-2007) and 960-1 164 MHz (Resolution 417 (WRC-2007)). In-band and adjacent band sharing studies needs to be developed in order to execute the implementation of these new technologies. This needs to be done as soon as possible to respond to the foreseen lack of capacity. The conference decided to have monitoring actions in order to take necessary regulatory actions.

Furthermore sharing studies are required, if necessary, to introduce new AM(R)S allocations for surface applications at airports in the 5 000-5 030 MHz band (Resolution 420 (WRC-2007)) in case the existing allocation in the 5 091-5 150 MHz band is not sufficient.

Preliminary CEPT position

Resolution 413 (WRC-2007):

Based on ICAO/ITU-R studies, DNR ITU-R M.[FMBC-AM(R)S], CEPT is of the opinion that no harmful interference will arise from the introduction of AM(R)S systems in the band 112-117.975 MHz into FM broadcasting receivers below 108 MHz. Resolution 413 should be amended to reflect the results of those studies.

Studies are still ongoing concerning the interference from digital broadcasting sound systems into AM(R)S. CEPT is proposing that this matter will be further pursued under traditional ITU-R activities and outside the WRC process. Resolution 413 should be amended to reflect this.

Resolution 417 (WRC-2007):

- Sharing studies with non ICAO ARNS systems. (Resolution 417 (WRC-2007) Considering f)

CEPT is of the opinion that sharing the 960-1164 MHz frequency band between networks in the aeronautical mobile (R) service and non-ICAO national systems in the aeronautical radionavigation service would be feasible with frequency off-set and/or distance separation subject to coordination of the AM(R)S networks with the non ICAO ARNS systems.

CEPT is of the opinion that Resolution 417 (WRC-07) should be modified to detail the operational and technical means to facilitate sharing between AM(R)S systems operating in the band 960-1 164 MHz and non ICAO ARNS systems.

- Sharing studies with non ICAO ARNS systems. (Resolution 417 (WRC-2007) Considering g)

CEPT is of the opinion that sharing the 960-1164 MHz frequency band between networks in the aeronautical mobile (R) service and non-ICAO national systems in the aeronautical radionavigation service is feasible and is a matter which can be dealt with in ICAO.

- Sharing studies with RNSS operating in the band 1164-1215 MHz

With the current technologies, and with a sufficient frequency offset and/or distance separation, CEPT is of the opinion that limits can be defined for the AM(R)S out-of-band emission in order to protect the RNSS receiver operating in the 1164-1215 MHz.

Resolution 420 (WRC-2007):

Based on the conducted studies within ITU-R, CEPT is of the opinion that spectrum needed to fulfil airport surface network requirements can be met in the current allocation. Therefore, CEPT does not support any new AM(R)S allocation to cover this particular need. Furthermore, CEPT notes that there is currently little flexibility in the frequency band 5 091-5 150 MHz and that the previously identified need for aeronautical security system intended to operate in this band is no longer required. CEPT is therefore of the view that the radio regulatory provisions related to the aeronautical security system should be deleted.

Background

General:

Reason for the European proposal is that a new system in the Aeronautical Mobile (R) Service (AM(R)S) and additional spectrum resource are required in order to overcome expected shortage in spectrum for line-of-sight air-ground communications. ICAO is considering frequency bands, already allocated to aeronautical services, for the introduction of new technologies to support air navigation, including airborne and ground surveillance applications.

ICAO have yet to select the technology solution to support the future communications system(s) and have still to conclude on the amount of spectrum required for a future communications system(s). Preliminary indication of ICAO was 60 MHz in the band 960 – 1024 MHz and between 60 and 100 MHz in the frequency range 5000 – 5150 MHz (source document PT3(06)15). The selection of the spectral bands (other than mentioned above) to meet the spectrum requirement for the future communication system(s) has yet to be made. More time is needed to allow ICAO to complete studies to resolve the above issues.

Not all necessary sharing studies with existing systems within the identified bands have been concluded within ITU.

Resolution 413 (WRC-2007):

- Compatibility studies between broadcasting and AM(R)S that may arise from the introduction of AM(R)S systems in the band 112-117.975 MHz.

Studies have been completed on the investigation of compatibility between the broadcasting and AM(R) services due to the introduction of AM(R)S systems in the band 112-117.975 MHz.

Perceivable interference (not harmful interference) might only be expected when the aircraft are operating at lower altitudes and the FM-radio is in close vicinity of the aircraft. 
This may happen only during final approach and landing of an aircraft (or during take-off). The nature of the interference is transient. It should also be noted that in most cases AM(R)S is likely to operate at frequencies higher than 112 MHz. Furthermore, the separation distances are calculated with the minimum FM broadcasting signal level, which is normally present at the edge of the FM broadcasting service area.

On the basis of the measurement results, it may be safely concluded that the introduction of AM(R)S at frequencies above 112 MHz is not expected to produce harmful interference into FM broadcasting receivers.

- Compatibility studies between broadcasting and AM(R)S in the band 108-117.975 MHz that may arise from the introduction of appropriate digital sound broadcasting systems.

Studies are still ongoing concerning the interference from digital broadcasting sound systems into AM(R)S. CEPT is proposing that this matter will be further pursued under traditional ITU-R activities and outside the WRC process.

Resolution 417 (WRC-2007):

Note: sharing with services below 960 MHz has not been considered as it is not requested within Resolution 417. This is being addressed within other CEPT groups.

- Sharing studies with non ICAO ARNS systems. (Resolution 417 (WRC-2007) Considering f)

Based on the report ITU-R M.[AM(R)S 1GHZ_SHARING], CEPT is of the opinion that sharing the 960-1164 MHz frequency band between networks in the aeronautical mobile (R) service and non-ICAO national systems in the aeronautical radionavigation service would be feasible with frequency off-set and/or distance separation subject to coordination of the AM(R)S networks with the ARNS systems.

CEPT is of the opinion that Resolution 417 (WRC-07) should be modified to detail the operational and technical means to facilitate sharing between AM(R)S and non ICAO ARNS systems operating in the band 960-1 164 MHz .

Protection distances that ensure interference-free operation of ARNS systems are 124-935 km for “aircraft-aircraft” radiolinks and 124-495 km for “aircraft-Earth” radiolinks. The maximum protection distances can be used as coordination distances applied for identification of the affected Administrations. In frequency planning the less stringent protection distances can be used. Their values are determined by provisions reached in the coordination process

In the proposed ECP of Resolution 417, Resolves 2 contains a list of administrations with which coordination agreements need to be obtained when AM(R)S systems are planned to be operated in a certain distance to the border of these countries. To reduce constraints on the new AM(R)S systems deployment only those countries which indicate to operate non-ICAO standardised systems in this band are listed.

- Sharing studies with non ICAO ARNS systems. (Resolution 417 (WRC-2007) Considering g)

Based on the report ITU-R M.[AM(R)S 1GHZ_SHARING], CEPT is of the opinion that sharing the 960-1164 MHz frequency band between networks in the aeronautical mobile (R) service and non-ICAO national systems (TACAN) in the aeronautical radionavigation service would be feasible with frequency off-set and/or distance separation. In case of co-channel operation, protection distances that ensure free-interference operation of ARNS (TACAN) systems are 935 km for “aircraft-aircraft” radiolinks and 572 km for “aircraft-Earth” radiolinks. With a frequency off-set of 4 MHz for an aircraft interrogator and [4] MHz for a beacon, no separation distance is required.

CEPT is of the opinion that Resolution 417 (WRC-07) should be modified to detail the operational and technical means to facilitate sharing between AM(R)S systems operating in the band 960-1 164 MHz and non ICAO ARNS systems.

CEPT believes that the coordination of these systems will be dealt within ICAO, as it is currently the case (TACAN and DME are without differences by administrations). Nevertheless, CEPT is of the view that characteristics of non ICAO ARNS systems identified under considering g) should be taken into account when deploying new AM(R)S systems to ensure compatibility. These characteristics are contained in one of the annex of Recommendation M.[Charlie] and are referred to in the revised Resolution 417 (in resolves 3)). In case this Recommendation is not adopted before the Conference, CEPT would propose to add these characteristics in a new Annex to the Resolution.

CEPT is of the opinion that Resolution 417 (WRC-07) should be modified to detail the operational and technical means to facilitate sharing between AM(R)S systems operating in the band 960-1 164 MHz and non ICAO ARNS systems.

- Sharing studies with RNSS operating in the band 1164-1215 MHz

Based on the report ITU-R M.[AM(R)S 1GHZ_SHARING], CEPT is of the opinion that with the current technologies, and with a sufficient frequency offset and/or distance separation, the AM(R)S out-of-band emission can be mitigated to protect the RNSS receiver operating in the 1164-1215 MHz.

Administrations intending to implement AM(R)S in the band 960-1 164 MHz in order not to cause harmful interference to the radionavigation-satellite service in the band 1 164-1 215 MHz shall utilize the criteria set forth below:

−	.any ground station operating under the AM(R)S allocation in the band 960-1 164 MHz, shall limit its equivalent isotropically radiated power (e.i.r.p.) to the values presented in the following table:

| Emissions in the band 960-1 164 MHz (Total e.i.r.p. in the band 960-1 164 MHz as a function of the carrier central frequency) | Emissions in the band 1 164-1 215 MHz |  |
| --- | --- | --- |
| AM(R)S centre frequency 1 146.45-1 164 MHz | 1 164-1 197.6MHz | 1 197.6-1 215 MHz |
| Linearly decreasing from 34 to −62.9dBW | −90.8 dBW in any 1 MHz of the band 1 164-1 197.6 MHz | −90.8 dBW in any 1 MHz of the band 1 197.6-1 215 MHz |

−	any aircraft station operating under the AM(R)S allocation in the band 960-1 164 MHz, shall limit its equivalent isotropically radiated power (e.i.r.p.) to the values presented in the following table:

| Emissions in the band 960-1 164 MHz (Total e.i.r.p. in the band 960-1 164 MHz as a function of the carrier central frequency) | Emissions in the band 1 164-1 215 MHz |  |
| --- | --- | --- |
| AM(R)S centre frequency 1 146.45-1 164 MHz | 1 164-1 197.6 MHz | 1 197.6-1 215MHz |
| Linearly decreasing from 37.75 to −59.2dBW | −84 dBW in any 1 MHz of the band 1 164-1 197.6 MHz | −92.4 dBW in any 1 MHz of the band 1 197.6-1 215 MHz |

Resolution 420 (WRC-2007):

- AM(R)S spectrum requirements for surface applications in the 5 GHz range in order to determine if they can be fulfilled in the band 5 091-5 150 MHz;

At WRC-07 studies performed on the compatibility of AM(R)S with RNSS in the bands 
5 000-5 010 and 5 010-5 030 MHz were not considered mature enough to make AM(R)S allocations in those bands and it was not certain that the AM(R)S requirement for surface applications could not be completely fulfilled in the band 5 091-5 150 MHz which was allocated to AM(R)S at WRC-07. Instead, Agenda item 1.4 and Resolution 420 (WRC-07) were adopted to determine if AM(R)S spectrum requirements for surface applications at airports could be satisfied in the already-allocated 5 091-5 150 MHz band and/or in case that this is not possible to  “further investigate the feasibility of an allocation of AM(R)S for surface applications at airports, study the technical and operational issues relating to the protection of RNSS in the bands between 5 000 and 5 030 MHz and of the radio astronomy service in the band 4 990-5 000 MHz from AM(R)S, and develop appropriate Recommendations ”regulatory measures” were necessary to “facilitate introduction of new AM(R)S in the band(s) … 5 000-5 030 MHz”. Studies including AM(R)S spectrum requirements and compatibility with existing systems were performed during the 2007-2012 study cycle to address this question.

These ITU-R studies conclude that there is a requirement for up to 130 MHz of spectrum to support surface applications at airports. Of the 130 MHz identified, a maximum of 50 MHz is required for AM(R)S, It should be noted that the additional identified spectrum requirement other than AM(R)S needs to be met by other means such as from allocations to the corresponding services other than AM(R)S.. The answer to resolves 1 of Resolution 420 (WRC-07) is that AM(R)S spectrum requirements for surface applications at airports in the 5 GHz range can be fulfilled in the band 5 091-5 150 MHz. Therefore AM(R)S spectrum needed to fulfil airport surface network requirements can be met in the current allocation and for this reason, CEPT does not support any new allocation in 5 000-5 030 MHz.

As far the results of ITU-R studies concerning the technical and operational issues relating to the protection of RNSS in the bands 5 000–5 030 MHz, CEPT is of the view that the protection of the RNSS uplink in 5 000-5 010 MHz can be ensured by appropriate e.i.r.p. limitation in every direction per airport. The protection level is based on a T/T of 2% (6% apportioned between three services). Studies and conclusions are contained in report ITU-R M.2168-1.

Since the RNSS signal characteristics are not sufficiently defined in the band 5 010-5 030 MHz to make a credible estimate of input parameters, it is not possible to reach a conclusion. As a result, no allocation is proposed for the AM(R)S in this band.

.

The frequency band 5 091-5 150 MHz is allocated to a number of services and that having to take account of all of these services reduces the flexibility in planning and implementing an airport surface safety communication system.  Additionally it is noted that the aeronautical security system for which there are provisions included in the Radio Regulations under footnote 5.444B is no longer required. Therefore in order to increase flexibility for the deployment of an airport surface safety communication system, CEPT supports the amendment of footnote 5.444B to remove the provisions for the aeronautical security system and the suppression of Resolution 419.

The operation of AM(R)S applications for airport surface in the band 5000-5030 MHz requires a separation distance of 72 km for the 5000 – 5010 MHz band and 52 km for the 5010-5030 MHz band for flat terrain in order not to exceed the protection limits for radio astronomy in the band 4990-5000 MHz. Operating these AM(R)S applications at the higher frequencies will result in a reduction of the protection radius to 52 km Knowing that a number of observatories are situated closer than the indicated distances, Radio astronomy services may be affected by emissions of the AM(R)S applications for airport surfaces located with 72/52 km of the observatories operating in the band 4990-5000 MHz.

The frequency band 5 091-5 150 MHz is allocated to a number of services and that having to take account of all of these services reduces the flexibility in planning and implementing an airport surface safety communication system.  Additionally it is noted that the aeronautical security system for which there are provisions included in the Radio Regulations under footnote 5.444B is no longer required. Therefore in order to increase flexibility for the deployment of an airport surface safety communication system, CEPT supports the amendment of footnote 5.444B to remove the provisions for the aeronautical security system and the suppression of Resolution 419.

List of relevant documents

- CPM on WRC-12 AGENDA ITEM 1.4

Resolution 413

- Recommendation ITU-R BS.412, ITU-R BS.450, ITU-R BS.704 and ITU R BS.1114,

- Report ITU-R M.2147

Resolution 417

- PDNR ITU-R M [AM(R)S_1GHz_SHARING]

- Working paper on the introduction of the aeronautical future communications system and the coexistence with TACAN, dated 24 March 2010 (NLD)

- Recommandations: ITU-R M.1787, ITU-R M.1318 and ITU-R M.[CHAR-RX3]

Resolution 420

- PDNR ITU-R M.[AMRS-RNSS]

- ITU-R M.2168-1

- PDR ITU-R M 2120

- Preliminary Draft New Report ITU-R M.[5GHz-SURF]

- Recommendation ITU-R M.1318,

- Recommendations ITU-R M.[S-E RX+TX] and ITU-R M.[E-S TX+RX]

- R(10)065R1_CPG12 PTC-7 RAS_RNSS_in_5GHz

## Actions to be taken

## Relevant information from outside CEPT

## To be updated with the latest information available

European Union

Regional telecommunication organisations

APT (Presented September 2011)

Resolution 413

APT Members support Method A as no further studies on protection of FM broadcasting receivers operating below 108 MHz are required.  APT Members support the suppression of “invitesITU-R 1 & 3, and resolves 6” in Resolution 413 (WRC-07) as a result of the studies shown in Report ITU-R M.2147.  APT Members also support the compatibility studybetween digital soundbroadcasting below 108 MHz and AM(R)S, in accordance with Resolutions 413 (Rev.WRC-07) taking into account Recommendation ITU-R BS.1114.

Resolution 417

APT Members support ITU-R studies on the protection of RNSS and non-ICAO ARNS systems and accordingly support Method B in the CPM Report.

APT Members support the amendment of Resolution 417 (WRC-07) based on ITU-R studies in order to introduce operational and technical means to facilitate sharing and coordination process between administrations planning to operate AM(R)S systems in the band 960-1164 MHz and some administrations listed in RR No. 5.312 operating non-ICAO ARNS systems.

Some APT Members are concerned that the protection distances for non-ICAO aeronautical radio navigation systems of countries listed in RR No.5.312 will severely limit the deployment and development of AM(R)S by countries in the Asia Pacific region. APT members urge that practical operational measures be developed to facilitate the coordination between AM(R)S systems and non-ICAO ARNS systems (considering e).

Resolution 420

Generally, APT Members support Method C1 in the CPM Report and are of the view that:

- It has yet to be conclusively shown that spectrum requirements to serve applications supporting Article 44.1 1-6 cannot be met in the existing 5 091–5 150 MHz allocation.

- Whilst ITU-R studies including Report ITU-R M.2168-1 show sharing is possible between AM(R)S and existing services in the band 5 000-5 010 MHz, studies considering AM(R)S allocations in the 5GHz bands are also being considered for WRC-12 agenda item 1.3, which may make it difficult to ensure protection of  RNSS in the band 5 000-5 010 MHz.

- No changes to Article 5 of the Radio Regulations are required as a result of studies conducted within the ITU-R in response toWRC-12 Agenda item 1.4 Resolution 420 (WRC-07).

- They support the suppression of Resolution 420 (WRC-07).

Note

Some APT Members support Method C2 in the CPM Report and are of the view that:

- ITU-R studies shows that sharing is possible between AM(R) S and existing services (including RNSS and adjacent band RAS) in the 5000-5010 MHz band and would not place undue constraints

- A primary AM(R)S allocation should be added in the band 5 000-5 010 MHz, with no changes to the band 5 010-5 030 MHz, to address the emerging requirements for surface applications at airports.

- In regards to Method C2, the e.i.r.p. limits of 40.6 dBm/10 MHz below 5 degree elevations and 37.1 dBm/10 MHz at or above 5 degree elevations is appropriate.

.

ATU (date of proposal)

Arab Group (Presented 4-5 March 2010)

Preliminary ASMG Views

Resolution 413:

Support the current studies and any regulatory measures to facilitate the introduction of the AM(R)S in the bands 112-117.975 MHz.

Resolution 417:

Support the current studies to identify technical and operational requirements facilitate the sharing between the AM(R)S and the ARNS in the band 960-1164 MHz

Support the current studies to identify technical and operational requirements facilitate the sharing between the AM(R)S in the band 960-1164 MHz and the RNSS in the band 1164-1215 MHz.

Support having additional Regulatory considerations to  facilitate the introduction of AMRS in the band 960-1164 MHz

Resolution 420:

Support the additional allocation to AM(R)S in the band 5000-5030, provided that the AM(R)S will not place any constraints on the existing services and that the studies prove that the current allocation in the band 5090-5150 MHz is not sufficient

CITEL (Presented 3 September 2010)

Preliminary View CAN and USA

Res 413 – Draft CPM Method A
Canada and the United States also seek to maintain compatibility with services in frequency bands adjacent to the 112-117.975 MHz band.  In this regard, the United States supports no additional constraints on the broadcasting service in the 87-108 MHz band consistent with the provisions of Resolution 413(WRC-07). Consequently, the United States supports no changes to the allocations in the 108-117.975 MHz band or to Resolution 413(WRC-07).

Res 417 – Draft CPM Method B2
Support compatibility studies between AM(R)S systems operating in the band 960-1 164 MHz and non-ICAO standardized ARNS systems, and based on the results of studies, will consider if further regulatory measures are required to facilitate introduction of new AM(R)S systems in the band.

Res 420 – Draft CPM Method C2 
If the spectrum requirements for surface applications at airports cannot be fully accommodated within the 5 091-5 150 MHz band, and if compatibility studies identified in Resolution 420(WRC-07) ensure protection of RNSS and RAS from AM(R)S surface applications, supports a new allocation to the AM(R)S in the band 5 000-5 030 MHz. (AMS(R)S Allocation in 5000-5010)

Preliminary Proposal (2323-1-04_i)

Corresponding proposal to the View

Supported by CAN

RCC (October 2010)

Resolution 413 (Rev. WRC-07)

RCC CAs do no object to the usage of the frequency band 112 – 117.975 MHz by AM(R)S systems since the conducted studies showed the possibility of its compatibility with the analogue broadcasting systems operating in the bands below 108 MHz. The AM(R)S usage should be limited by the systems which should operate in accordance with the acknowledged international aviation standards.

RCC CAs support the appropriate modifications of Resolution 413.

(see also Method A of the draft CPM Report)

Resolution 417 (WRC-07)

1. RCC CAs are of the view that protection of ARNS stations from systems of AM(R)S in the frequency band 960-1164 MHz, as well as compatibility of AM(R)S systems in the band 960-1164 МHz with RNSS stations operating in the adjacent band 1164-1215 МHz shall be ensured.

2. RCC CAs support the modification of Resolution 417 (Rev.WRC-12) with respect of the conditions of protection of ARNS operating in the frequency band 960 – 1164 MHz and the stations of RNSS operating in the adjacent frequency bands 1164 – 1215 MHz from the interference caused by the stations of AM(R)S operating in the frequency band 960 – 1164 MHz.

(see also Method B1 of the draft CPM Report).

Resolution 420 (WRC-07)

RCC CAs do not object the allocation of the frequency band 5 000 – 5 010 MHz to AM(R)S provided that the compatibility with the existing radio services in this and adjacent frequency bands under adopted restrictions is ensured.

(see also Method C2 of the draft CPM Report)

International organisations

IATA (October 2009)

To support as required, based on the results of studies, the inclusion in the Radio Regulations of further regulatory measures that will facilitate the introduction of

•	future AM(R)S systems in the band 112 - 117.975 MHz.

•	future AM(R)S systems in the band 960 - 1 164 MHz.

If the spectrum requirements for surface applications at airports cannot be fully accommodated within the 5 091 - 5 150 MHz band, and based on the results of successful compatibility studies, support a new allocation to the AM(R)S in portions of the band 5 000 - 5 030 MHz subject to technical compatibility.

ITU (date of proposal)

Progress To date

- Resolution 413

  - New Report M.[FMBC-AM(R)S] (Doc. 5B/296 Annex 11)

  - WD towards a PDN Report M.[COMPAT-VHF](Doc. 5B/296 Annex 12)

- Resolution 417

  - PDN Report M.[AM(R)S 1GHz SHARING](Doc.5B/417 Annex 22)

- Resolution 420

  - PDN Report M.[AM(R)S-RNSS](Doc.5B/417 Annex 25)

  - DN Report M.[AM(R)S-RNSS-RAS] (Doc. 5B/189

  - PD Revision ITU-R M.2120 ”Initial Estimate of Spectrum Requirement” (Doc5B/296 Annex )

- Workplan (Doc. 5B/417 Annex 03)

- Draft CPM text (Doc. 5B/417 Annex 4)

- Progress report (Doc. 5/194 Section 4)

ICAO (June 2011)

To support as required, based on the results of studies identified in Resolution 413, the inclusion in the Radio Regulations of further regulatory measures that will facilitate the introduction of future AM(R)S systems in the band 112 - 117.975 MHz.

To support as required, based on the results of studies identified in Resolution 417, the inclusion in the Radio Regulations of further regulatory measures that will facilitate the introduction of future AM(R)S systems in the band 960 - 1 164 MHz.

If the spectrum requirements for surface applications at airports cannot be fully accommodated within the 5 091 - 5 150 MHz band, and based on the results of successful compatibility studies identified in Resolution 420, support a new allocation to the AM(R)S in the band 5 000 - 5 030 MHz.

NATO (October, 2011)

Preliminary NATO Military Position

Resolution 413 (WRC-2007):

NATO supports Method A of the CPM 11-2 Report.

Resolution 417 (WRC-2007):

NATO supports Method B of the CPM 11-2 Report.

Resolution 420 (WRC-2007):

In the band 5 000 - 5 010 MHz, an allocation could be considered only if the spectrum requirements cannot be fulfilled in the band 5 091-5 150 MHz and if AM(R)S would not create undue constraints on the existing radiocommunication Services.

In the band 5010 - 5030, NATO supports no change because neither the AM(R)S operational environment nor the RNSS signal characteristics are sufficiently defined to finalize ITU-R compatibility studies between the two services.

Military Importance:  High

SFCG (September 10)

SFCG supports the protection of existing RNSS allocations in the band 5000-5030 MHz. RNSS systems operating in this band are under development by some SFCG member agencies.

SFCG has no particular view on Methods in the Draft CPM text since they provide the necessary protection of RNSS operations.

Regional organisations

ESA (November 08)

Same as SFCG position, the band 5010-5030 MHz is planned to be used for next-generation Galileo user terminals and sharing of these terminals with aeronautical mobile systems appears problematic because of the ubiquitous nature of both applications.

ASFCG (February 2011)

To support as required, based on the results of studies, the inclusion in the Radio Regulations of further regulatory measures that will facilitate the introduction of

•	future AM(R)S systems in the band 112 - 117.975 MHz.

•	future AM(R)S systems in the band 960 - 1 164 MHz.

To accept that no harmful interference may arise from the introduction of AM(R)S systems in the band 112-117/975 MHz into FM broadcasting receivers below 108 MHz.

To require that no harmful interference may arise from the introduction of appropriate digital sound broadcasting systems into AM(R )S receiver in the band 108-117.975 MHz.

If the spectrum requirements for surface applications at airports cannot be fully accommodated within the 5 091 - 5 150 MHz band, and based on the results of successful compatibility studies, support a new allocation to the AM(R)S in portions of the band 5 000 - 5 030 MHz subject to technical compatibility

[Other relevant information]

____________________