Editor’s note: the modifications relative to the output of last CPG in Oxford are in red track changes. Furthermore, the modifications stemming from discussions during PTC(11) are highlighted in yellow.

Draft CEPT Brief on agenda item 1.15

1.15	to consider possible allocations in the range 3-50 MHz to the radiolocation service for oceanographic radar applications, taking into account the results of ITU R studies, in accordance with Resolution 612 (WRC 07);

## Issue

This agenda item covers the following issues:

Resolution 612 (WRC-07):

1	to invite ITU-R to identify high-frequency oceanographic radar system applications between 3 and 50 MHz, including bandwidth requirements, appropriate portions of this band for these applications, and other characteristics necessary to conduct sharing studies;

2	to invite ITU-R to conduct sharing analyses between the radiolocation service applications identified under resolves 1 and incumbent services in the bands identified to be suitable for the operation of high-frequency oceanographic radar systems;

3	that, if compatibility with existing services is confirmed under resolves 2, to recommend that WRC-12 consider allocations to the radiolocation service in several suitable bands between 3 and 50 MHz, as determined in the ITU-R studies, each band not exceeding 600 kHz, for the operation of oceanographic radars,

Preliminary CEPT position

CEPT supports new primary and secondary allocations for the radiolocation service in portions of the 3 to 50 MHz band identified as suitable for oceanographic radar operations.

CEPT also supports technical and regulatory conditions to protect incumbent services in the frequency bands between 3 and 50 MHz where new allocations to radiolocation are introduced.

Background

WRC-12 Agenda item 1.15 calls for the consideration of the creation of radiolocation allocations in the 3 to 50 MHz range. These allocations will be used for the operation of oceanographic radars that monitor the sea surface for wave heights, currents and tracking of large objects. These radars will have an operational range up to 300 km. Oceanographic radars have been operating in the 3 to 50 MHz range since 1970 on an experimental, non-interference basis. Increased reliance on the data from these systems for maritime safety, oceanographic, climatological, meteorological and disaster response operations have driven the need to improve the regulatory status of the spectrum which is used by Oceanographic radars while taking into account the protection of existing allocated services. WRC-12 Agenda item 1.15 was established with the understanding that spectrum would be allocated on a shared basis. Reallocation of spectrum from an existing allocated radio service to the radiolocation services is not the intent.

Multiple sub-bands are required within the range of 3 to 50 MHz for providing long range data as well as high resolution data. Lower frequencies support long-range operation whereas higher frequency ranges support high resolution data collection. High resolution data collection at long ranges is not achievable with current technology as they are mutually exclusive characteristics. Manufacturers and researchers have settled on frequencies near 4.5 MHz±1 MHz, 9 MHz±1 MHz, 13 MHz±1 MHz, 16 MHz ±1 MHz,  26 MHz±3 MHz and 43 MHz±3 MHz to meet the various scientific and operational requirements. This does not imply that 2 to 6 MHz is required in each range for operation of a network of high-frequency oceanographic ocean observing radars. The actual spectrum requirements are much lower:

For very long-range operation (low frequencies around 4.5 MHz), 2 separate bands of around 50 kHz are needed in each range of operation.

For long-range operation (9 MHz, 13 MHz and 16 MHz) 2 separate bands of around 100 kHz are required in each range of operation for optimal use.

For short-range operation around 26 MHz, 2 separate bands of around 150 kHz are needed in each range of operation.

For short-range operations at high frequencies around 43 MHz, 2 separate bands of around 500 kHz are needed in each range of operation.

Considering that sharing will be easier with terrestrial services as oceanographic radars will emit towards the ocean, it is proposed to focus the sharing studies in the bands used by Fixed Service and/or land Mobile Service only. Such bands, considered as the most suitable bands, are listed in Table below (see Annex A).

| Band | Needs for HF radars | Most suitable bands | Services | Proposed allocations |
| --- | --- | --- | --- | --- |
| 4.5MHz ±1MHz | 2*50 kHz joined | 4 438 -4 650 kHz 5 060 -5 450 kHz | Fixed, Mobile exc AM(R) Fixed, Mobile exc AM(R) | 5060-5160 kHz |
| 9 MHz ±1 MHz | 2*100 kHz joined | 7 450 -8 100 kHz 9 040 -9 400 kHz 9 900 -9 995 kHz | Fixed, Mobile exc AM(R) Fixed Fixed | 9200-9300 kHz 9300-9400 kHz ** |
| 13 MHz ±1 MHz | 2*100 kHz joined | 12 100 -12230 kHz 13 410 -13 570 kHz 13 870 -14 000 kHz | Fixed Fixed, Mob exc AM(R) second Fixed, Mob exc AM(R) second | 12100-12200 kHz and 13 410-13 510 kHz ** |
| 16 MHz ±1 MHz | 2*100 kHz joined | 14 350 -14 990 kHz 15 800 - 16 350 kHz | Fixed, Mob exc AM(R) second Fixed | 16000-16100 kHz ** 16100-16200 kHz |
| 26 MHz ±3 MHz | 2*150 kHz joined | 22 855 -23 200 kHz 24 000 -24 890 kHz 25 010 -25 070 kHz 25 210 -25 550 kHz 26 175 -27 500 kHz 29 700 -30 000 kHz. | Fixed Fixed, Mob exc AM Fixed, Mob exc AM Fixed, Mob exc AM Fixed, Mob exc AM Fixed, Mobile | 27 200-27 500 kHz |
| 43 MHz ±4 MHz | 2*500 kHz joined | 39 -39.986 MHz 40.02 -40.98 MHz [41.015 - 47 MHz] | Fixed, Mobile Fixed, Mobile Fixed, Mobile | 39.0-39.5 MHz |

** radiolocation allocation with a secondary status

Note 1: For frequency bands 9, 13 and 16 MHz, 2 bands of around 100 kHz are required for an optimal operation of oceanographic radars. However, in case of frequency bandwidth limitations, oceanographic radars can operate with a single band of around 100 kHz.

- About the requirement of 2 times the amount of spectrum in each HF sub band, some rationale for this is:

- Considering one system, 1/ current vector maps needs 2 close radars with twice the spectrum in order not work properly, and 2/ Possibilities for innovation in future systems (better resolution, 25 kHz only allows 6 km resolution)

- Coexistence of 2 or more systems. Growing needs for such systems (pollution monitoring, see the European program TOSCA for instance), it is expected that the need for such systems will increase.

- About the proposal for contiguous bands, some rationale for this is:

  - physical reasons: current measurements via 2 radial projections are better around the same frequency (slightly delayed).

  - possibility for different systems to co exist with locally increase resolution (chirps interleavings)

  - increase possibility to avoid some noisy frequency bands

Sharing studies with the Fixed and Land Mobile Services in the bands4.5 MHz ±1 MHz, 9 MHz ±1 MHz, 13MHz ± 1MHz, 16MHz ± 1MHz, 26 MHz ±3 MHz and 43 MHz ±4 MHz

The conducted studies which do not consider the topography effect show that:

- Skywave interference from oceanographic radars to fixed and mobile systems and vice versa can occur as a function of the characteristics which are associated with skywave propagation. However, it is believed that the sharing situation with oceanographic radars is not significantly different from the sharing situation between presently co-allocated HF fixed and mobile services.

- Receivers from systems of the Fixed and Land Mobile Services will not suffer harmful interferences from oceanographic radars by ground wave propagation when using adequate protection distances as follows:

| Frequency band | Land path |  | Sea path or mixed |  |
| --- | --- | --- | --- | --- |
|  | Quiet Rural* | Rural* | Quiet Rural* | Rural* |
| 4.5 MHz | 170 m | 120 Km | 920 Km | 790 Km |
| 9 MHz | 130 Km | 100 Km | 670 Km | 590 Km |
| 13 MHz | 110 Km | 100 Km | 520 Km | 480 Km |
| 16 MHz | 100 Km | 80 Km | 450 Km | 390 Km |
| 26 MHz | 100 Km | 80 Km | 320 Km | 280 Km |
| 43 MHz | 100 Km | 80 Km | 230 Km | 200 Km |

* The “rural” or “quiet rural” environment applies to the location of the fixed or mobile service receiver, not the radiolocation system location

However, in order to protect all incumbent services, CEPT proposes to upgrade the mobile service allocations to a primary status in bands where radiolocation allocation is introduced as well as in adjacent/neighbouring bands.

List of relevant documents

ITU-R Recommendation M.1874 Technical and operational characteristics of oceanographic radars operating in sub-bands within the frequency range 3 50 MHz

ITU-R WP 5B Working Document towards a DNR ITU-R M.[RLS 3-50 MHz Sharing]

ITU-R CPM Report on WRC-12 Agenda item 1.15

## Actions to be taken

## Relevant information from outside CEPT

European Union

Regional telecommunication organisations

APT (September 2011)

APT members support new allocations to the RLS for oceanographic radar applications with appropriate regulatory provisions for a possible sharing with existing services to enable the most efficient use of the spectrum and mitigate interference to existing services.

APT members recognize that the combination of allocation on primary and secondary basis is practical on sharing and protecting with various existing services in multiple sub-bands from lower frequencies for supporting long-range operation to higher frequencies for supporting higher resolution in short-range operation.

Therefore, APT Members support Method C in the CPM report.

ATU (July 2011)

The preliminary ATU Common position is to support Method C in the CPM report.

Arab Group (December 2010)

ASMG support the additional allocation of radiolocation for oceanographic radar provided adequate protection to the existing services and without any additional constraint on it.

All ASMG administrations are requested to submit their positions on each of the sub-bands proposed (secondary, primary or NOC).

CITEL (June 2011)

Preliminary Views

Brazil and Mexicosupport Method A of CPM report, with the following modifications:

• MOD Appendix 4, Table 1

• MOD Article 19, specifying application of provisions in 19.51

• MOD Resolution 612 (WRC‐07)

The US support Method A of the CPM Report, with:

• MOD Article 5 to provide primary allocation in limited bands

• ADD new Footnote: “Stations in the radiolocation service shall not cause harmful interference to, nor claim protection from, stations operating in the fixed and mobile services. Resolution 612 (Rev. WRC‐12) shall apply. (WRC‐12)”

• MOD Resolution 612 (WRC‐07)

RCC (April 2011)

Possible allocations in the band 3-50 MHz for radiolocation service should not affect the existing radiocommunication services operation.

The RCC final position will be determined based on the results of the studies.

EBU (January 2010)

The EBU is pleased to note that the draft CEPT brief for this agenda item proposes to focus the sharing studies for oceanographic radars in the bands used by Fixed Service and/or land Mobile Service only and that the bands listed for consideration exclude those bands allocated to the broadcasting service

International organisations

ICAO (July 2011)

Ensure that any allocation made as a result of this agenda item shall not cause harmful interference to the operation of existing and planned aeronautical systems that operate in or adjacent to the frequency band 3 - 50 MHz

IMO (January 2011)

The Group noted the information provided by the Secretariat with regard to WRC-12 Agenda item 1.15 (IMO/ITU EG 4/5/4) and considered that the use of the radiolocation service between 3 and 50 MHz to support HF oceanographic radar operations of sea states and wave conditions. The Group noted that the advertised studies could be of benefit to the maritime community. However, the Group further noted that this was a complex issue with many questions still pending. In particular, it was recognized that some consideration was being given to higher output powers and surveillance for surface objects out to a range of 200 NM. It was considered that such use could close off the development of short range and/or high bandwidth HF applications under agenda items 1.9 and 1.10. It was further considered to be of utmost importance to watch any developments regarding this agenda item very closely.

IMO position

Ensure that any allocation made as a result of this agenda item does not affect the operation of existing and planned maritime systems that operate in or adjacent to the frequency  range 3 to50 MHz.

It should be noted that HF ocean sensing radar capable of measuring wave and current information, including detecting Tsunami waves and assisting authorities in search and rescue planning, benefits the safety of life.

NATO (September 2011)

Preliminary NATO Military Position

NATO appreciates the ITU-R sharing studies between the proposed radiolocation service (RLS) and incumbent services in portions of the 3 to 50 MHz band identified for possible high-frequency oceanographic radar use.

Based on the results of these studies, NATO can support new allocations to the RLS in RR Article 5 in parts of the band 3 - 50 MHz (except the bands in the range of 30-50 MHz which are included in the NJFA) under the condition that sufficient regulatory provisions are developed to ensure incumbent services are adequately protected.

Military Importance:  High

SFCG (October 09)

SFCG supports the protection of existing space science service allocations.  No allocation changes to support HF oceanographic radar operations should be made in space science service bands unless acceptable sharing criteria are developed.

WMO (October 09)

WMO confirms the importance of oceanographic radars for meteorology, climatology and environmental activities and supports relevant radiolocation service allocation within the 3-50 MHz band to enable the implementation and operation of such radars.

European Aeronautical Common Position – EACP (October 2009)

Ensure that any allocation made as a result of this agenda item does not adversely affect the operation of existing and planned aeronautical systems that operate in or adjacent to the frequency band 3 - 50 MHz.

Regional organisations

ESA (November 08)

Same as SFCG position

EUMETNET (September 08)

Same as WMO position

EACP (European Aeronautical Common Position) (October 09)

Ensure that any allocation made as a result of this agenda item does not adversely affect the operation of existing and planned aeronautical systems that operate in or adjacent to the frequency band 3 - 50 MHz.

[Other relevant information]

Annex : List of services in the considered bands

|  |  |  |  |  |  |  |  | Service allocations between 3.5 and 5.5 MHz |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| kHz | 3 500 – 3 750 | 3 750 - 3 800 | 3 800 - 3 900 | 3 900 - 3 950 | 3 950 - 4 000 | 4 000 - 4 063 | 4 063 - 4 438 | 4 438 - 4 650 | 4 650 - 4 700 | 4 700 - 4 750 | 4 750 - 4 850 | 4 850 - 4 995 | 4 995 - 5 003 | 5 003 - 5005 | 5 005 - 5 060 | 5 060 - 5 250 | 5 250 - 5 450 | 5 450 - 5 480 | 5 480 - 5 680 |
| REGION 1 |  |  |  | OR |  |  |  |  | R | OR |  |  | SF TS | SF TS* |  |  |  |  | R |
|  |  |  | OR |  |  |  |  | Exc R |  |  | OR |  |  |  |  | s Exc Aero | Exc Aero | OR |  |
|  | Exc Aero | Exc Aero |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| REGION 2 |  |  |  |  |  |  |  |  | R | OR |  |  | SF TS | SF TS* |  |  |  | R | R |
|  |  |  |  |  |  |  |  | Exc R |  |  | Exc R |  |  |  |  | s Exc Aero | Exc Aero |  |  |
|  |  | Exc R | Exc R | Exc R | Exc R |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| REGION 3 |  |  |  |  |  |  |  |  | R | OR |  |  | SF TS | SF TS* |  |  |  |  | R |
|  |  |  |  |  |  |  |  | Exc Aero |  |  |  |  |  |  |  | s Exc Aero | Exc Aero | OR |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | * = Space Research |  |  |  |  |  |
|  | Amateur |  | AS | Amateur-satellite |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Fixed |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Mobile | Exc R | Mobile except aeronautical mobile (R) |  |  | Exc OR | Mobile except aeronautical mobile (OR) |  |  |  | Exc Aero | Mobile except aeronautical mobile |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | s Exc Aero | Mobile except aeronautical mobile (secondary allocation) |  |  |  |  |  |  |  |
| OR | Aeronautical Mobile (OR) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| R | Aeronautical Mobile (R) |  |  | Aeronautical Mobile |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Land Mobile |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Maritime Mobile |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Broadcasting |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SF TS | Standard frequency and time signal |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

|  |  |  |  |  |  | Service allocations from 8 to 10 MHz |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| kHz | 7 450 - 8 100 | 8 100 - 8 195 | 8 195 - 8 815 | 8 815 - 8 965 | 8 965 - 9 040 | 9 040 - 9 400 | 9 400 - 9 500 | 9 500 - 9 900 | 9 900 - 9 995 | 9 995 - 10 003 |  |
| REGION 1 |  |  |  | R | OR |  |  |  |  | SF TS |  |
|  | Exc R |  |  |  |  |  |  |  |  |  |  |
| REGION 2 |  |  |  | R | OR |  |  |  |  | SF TS |  |
|  | Exc R |  |  |  |  |  |  |  |  |  |  |
| REGION 3 |  |  |  | R | OR |  |  |  |  | SF TS |  |
|  | Exc R |  |  |  |  |  |  |  |  |  |  |

| LEGEND |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Amateur |  | AS | Amateur-satellite |  |  |  |  |  |  |  |  |
|  | Fixed |  |  |  |  |  |  |  |  |  |  |  |
|  | Mobile | Exc R | Mobile except aeronautical mobile (R) |  |  | Exc OR | Mobile except aeronautical mobile (OR) |  |  | Exc Aero | Mobile except aeronautical mobile |  |
|  |  | s Exc R | Mobile except aeronautical mobile (R) secondary allocation |  |  |  |  |  |  | s Exc Aero | Mobile except aeronautical mobile (secondary allocation) |  |
| OR | Aeronautical Mobile (OR) |  |  |  |  |  |  |  |  |  |  |  |
| R | Aeronautical Mobile (R) |  |  | Aeronautical Mobile |  |  |  |  |  |  |  |  |
|  | Land Mobile |  |  |  |  |  |  |  |  |  |  |  |
|  | Maritime Mobile |  |  |  |  |  |  |  |  |  |  |  |
|  | Broadcasting |  |  |  |  |  |  |  |  |  |  |  |
| SF TS | Standard frequency and time signal |  |  |  |  |  |  |  |  |  |  |  |
|  | Radio Astronomy |  |  |  |  |  |  |  |  |  |  |  |

|  |  |  |  |  |  | Service allocation from 12 to 14 MHz |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 11 650 - 12 050 | 12 050 - 12 100 | 12 100 - 12 230 | 12 230 - 13 200 | 13 200 - 13 260 | 13 260 - 13 360 | 13 360 - 13 410 | 13 410 - 13 570 | 13 570 - 13 600 | 13 600 - 13 800 | 13 800 - 13 870 | 13 870 - 14 000 |
| REGION 1 |  |  |  |  | OR | R |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | s Exc R |  |  |  | s Exc R |
| REGION 2 |  |  |  |  | OR | R |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | s Exc R |  |  |  | s Exc R |
| REGION 3 |  |  |  |  | OR | R |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | s Exc R |  |  |  | s Exc R |

| LEGEND |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Amateur |  | AS | Amateur-satellite |  |  |  |  |  |  |  |  |
|  | Fixed |  |  |  |  |  |  |  |  |  |  |  |
|  | Mobile | Exc R | Mobile except aeronautical mobile (R) |  |  | Exc OR | Mobile except aeronautical mobile (OR) |  |  | Exc Aero | Mobile except aeronautical mobile |  |
|  |  | s Exc R | Mobile except aeronautical mobile (R) secondary allocation |  |  |  |  |  |  | s Exc Aero | Mobile except aeronautical mobile (secondary allocation) |  |
| OR | Aeronautical Mobile (OR) |  |  |  |  |  |  |  |  |  |  |  |
| R | Aeronautical Mobile (R) |  |  | Aeronautical Mobile |  |  |  |  |  |  |  |  |
|  | Land Mobile |  |  |  |  |  |  |  |  |  |  |  |
|  | Maritime Mobile |  |  |  |  |  |  |  |  |  |  |  |
|  | Broadcasting |  |  |  |  |  |  |  |  |  |  |  |
| SF TS | Standard frequency and time signal |  |  |  |  |  |  |  |  |  |  |  |
|  | Radio Astronomy |  |  |  |  |  |  |  |  |  |  |  |

|  |  |  |  |  |  |  | Service allocation from 15 to 17 MHz |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 14 350 -14 990 | 14 990 – 15 005 | 15 005- 15 010 | 15 010 – 15 100 | 15 100 - 15 600 |  | 15 600 - 15 800 |  | 15 800- 16 360 | 16 360 – 17 410 |  |  |  |  |
| REGION 1 |  | SF TS | SF TS | OR |  |  |  |  |  |  |  |  |  |  |
|  | s Exc R |  | Space research |  |  |  |  |  |  |  |  |  |  |  |
| REGION 2 |  | SF TS | SF TS | OR |  |  |  |  |  |  |  |  |  |  |
|  | s Exc R |  | Space research |  |  |  |  |  |  |  |  |  |  |  |
| REGION 3 |  | SF TS | SF TS | OR |  |  |  |  |  |  |  |  |  |  |
|  | s Exc R |  | Space research |  |  |  |  |  |  |  |  |  |  |  |
|  | Amateur |  | AS | Amateur-satellite |  |  |  |  |  |  |  |  |  |  |
|  | Fixed |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Mobile | Exc R | Mobile except aeronautical mobile (R) |  |  |  | Exc OR |  | Mobile except aeronautical mobile (OR) |  |  | Exc Aero | Mobile except aeronautical mobile |  |
|  |  | s Exc R | Mobile except aeronautical mobile (R) secondary allocation |  |  |  |  |  |  |  |  | s Exc Aero | Mobile except aeronautical mobile (secondary allocation) |  |
| OR | Aeronautical Mobile (OR) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| R | Aeronautical Mobile (R) |  |  | Aeronautical Mobile |  |  |  |  |  |  |  |  |  |  |
|  | Land Mobile |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Maritime Mobile |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Broadcasting |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SF TS | Standard frequency and time signal |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Radio Astronomy |  |  |  |  |  |  |  |  |  |  |  |  |  |

|  |  |  |  |  |  |  | Service allocation from 22 to 30 MHz |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 22 855 – 23 000 | 23 000 – 23 200 | 23 200 – 23 350 | 23 350- 24 000 | 24 000- 24 890 | 24 890- 24 990 | 24 990- 25 005 | 25 005- 25 210 | 25 010- 25 070 | 25 070- 25 210 | 25 210- 25 550 | 25 550- 25 670 | 25 670 – 26 100 | 26 100 – 26 175 | 26 175 – 27 500 | 27 500 – 28 000 | 28 000- 29 700 | 29 700- 30 000 |
| REGION 1 |  |  |  |  |  |  | SF TS | SF TS |  |  |  |  |  |  |  | METOC |  |  |
|  |  | Exc Aero |  | Exc Aero |  | AS |  |  | Exc Aero |  | Exc Aero |  |  |  | Exc Aero |  | AS |  |
| REGION 2 |  |  |  |  |  |  | SF TS | SF TS |  |  |  |  |  |  |  | METOC |  |  |
|  |  | Exc Aero |  | Exc Aero |  | AS |  |  | Exc Aero |  | Exc Aero |  |  |  | Exc Aero |  | AS |  |
| REGION 3 |  |  |  |  |  |  | SF TS | SF TS |  |  |  |  |  |  |  | METOC |  |  |
|  |  | Exc Aero |  | Exc Aero |  | AS |  |  | Exc Aero |  | Exc Aero |  |  |  | Exc Aero |  | AS |  |
|  | Amateur |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Fixed |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Mobile |  | Exc R | Mobile except aeronautical mobile (R) |  |  |  | Exc Aero | Mobile except aeronautical mobile |  |  |  |  |  |  |  |  |  |
|  |  |  | s Exc R | Idem above, secondary allocation |  |  |  | s Exc Aero | Idem above, secondary allocation |  |  |  |  |  |  |  |  |  |
| OR | Aeronautical Mobile (OR) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| R | Aeronautical Mobile (R) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Maritime Mobile |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Broadcasting |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SF TS | Standard frequency and time signal |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | RAS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

|  |  |  |  |  |  | Service allocation from 39 to 50 MHz |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 39-39.986 | 39.986-40.02 | 40.02-40.98 | 40.98-41.015 | 41.015-44 | 44-47 | 47-50 |  |  |  |  |
| REGION 1 |  |  |  |  |  | 5.162A | 5.162A |  | 46-68 MHz has additional allocation in most european countries to the radioloc service on a secondary basis, limited to the operation of wind profiler radars in accordance with res 217 |  |  |
|  |  | * |  | * |  |  |  |  |  |  |  |
| REGION 2 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | * |  | * |  |  |  |  |  |  |  |
| REGION 3 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | * |  | * |  |  |  |  |  |  |  |
|  | Amateur |  | AS | Amateur-satellite |  | * = Space Research |  |  |  |  |  |
|  | Fixed |  |  |  |  |  |  |  |  |  |  |
|  | Mobile | Exc R | Mobile except aeronautical mobile (R) |  |  | Exc OR | Mobile except aeronautical mobile (OR) |  |  | Exc Aero | Mobile except aeronautical mobile |
|  |  | s Exc R | Mobile except aeronautical mobile (R) secondary allocation |  |  |  |  |  |  | s Exc Aero | Mobile except aeronautical mobile (secondary allocation) |
| OR | Aeronautical Mobile (OR) |  |  |  |  |  |  |  |  |  |  |
| R | Aeronautical Mobile (R) |  |  | Aeronautical Mobile |  |  |  |  |  |  |  |
|  | Land Mobile |  |  |  |  |  |  |  |  |  |  |
|  | Maritime Mobile |  |  |  |  |  |  |  |  |  |  |
|  | Broadcasting |  |  |  |  |  |  |  |  |  |  |
| SF TS | Standard frequency and time signal |  |  |  |  |  |  |  |  |  |  |
|  | Radio Astronomy |  |  |  |  |  |  |  |  |  |  |

Resolution 612 (WRC-07) states that high-frequency oceanographic radars operate with peak power levels in the order of 50 Watts (recognizing d), and through the use of ground-wave propagation (considering c).