CPG PTC(11)082 Annex 4

PRELIMINARY DRAFT EUROPEAN COMMON PROPOSALS FOR THE WORK OF THE CONFERENCE

AGENDA ITEM 1.3

1.3	to consider spectrum requirements and possible regulatory actions, including allocations, in order to support the safe operation of unmanned aircraft systems (UAS), based on the results of ITU-R studies, in accordance with Resolution 421 (WRC-07).

CONTENTS

Page

Introduction	1

1	Sub-Part 3A	3

2	Sub-Part 3B	13

Introduction

This agenda item considers the spectrum requirements to support the safe operation of unmanned aircraft systems in non-segregated airspace.

The safe flight operation of UAS needs reliable communication links and associated spectrum, especially for the remote pilot to command and control the flight and to relay the air traffic control communications. In addition the safe flight operation of UAS necessitates advanced techniques to detect and track nearby aircraft, terrain and obstacles to navigation in order to ensure the UAS avoids these objects in a manner equivalent to that achieved by manned aircraft.

A significant increase of the worldwide use of unmanned aircraft systems is expected in the future. The seamless operation of unmanned aircraft with piloted aircraft in non-segregated airspaces is becoming vital for the further development of unmanned aircraft applications that will fill many diverse requirements. Therefore, globally harmonized spectrum is required to satisfy this need. WRC-12 Agenda item 1.3 seeks to identify spectrum that can be used to meet this demand. The envisioned unmanned aircraft systems infrastructure will be composed of terrestrial and satellite components.

Report ITU-R M.2171 provides the analyses for determining the amount of spectrum required for the operation of a prospected number of unmanned aircraft systems sharing non-segregated airspace with manned aircraft as required by Resolution 421 (WRC-07) and in response to WRC-12 Agenda item 1.3. Deployment of unmanned aircraft systems will require access to both terrestrial and satellite spectrum.

The maximum amount of spectrum required for unmanned aircraft systems are:

–	34 MHz for the terrestrial component,

–	56 MHz for the satellite component.

For the satellite component, except for some consequential modifications related to the coordination mechanism of the existing AMS(R)S allocation in 5 030-5 091 MHz in case of the new AM(R)S allocation proposed in the same band (see sub-part 3A), Europe is of the view that no change is required to the Radio Regulations for the satellite component. Indeed, no new allocations are required as there are enough frequency bands to accommodate the spectrum requirement for the satellite component and no new or modified procedures/provisions are needed for the existing satellite allocations.

For the terrestrial component, Europe is of the view that AM(R)S allocations should be used. Studies showed that there is a lack of spectrum to accommodate the spectrum requirement for the terrestrial component. Europe proposed to allocate the band 5 030-5 091 MHz [and 15.4-15.5 GHz] to AM(R)S to satisfy the Agenda Item, along with consequential regulatory and technical considerations to protect the existing services.

Moreover, Europe is of the view that the band 960-1164 MHz is not appropriate to accommodate the CNPC link of unmanned aircraft systems, considering the numerous existing or planned applications.

The proposal that follows is divided into two parts: sub-part 3A for the terrestrial component and sub-part 3B for the satellite component.

# Sub-Part 3A

Terrestrial component

ARTICLE  5

Frequency allocations

Section IV  –  Table of Frequency Allocations
(See No. 2.1)

MOD	EUR/5A3/1

1 610-1 660 MHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 1 610-1 610.6 MOBILE-SATELLITE (Earth-to-space) 5.351A AERONAUTICAL RADIONAVIGATION | 1 610-1 610.6 MOBILE-SATELLITE (Earth-to-space) 5.351A AERONAUTICAL RADIONAVIGATION RADIODETERMINATION- SATELLITE (Earth-to-space) | 1 610-1 610.6 MOBILE-SATELLITE (Earth-to-space) 5.351A AERONAUTICAL RADIONAVIGATION Radiodetermination-satellite (Earth-to-space) |
| 5.341 5.355 5.359 5.364 5.366 MOD 5.367 5.368 5.369 5.371 5.372 | 5.341 5.364 5.366 MOD 5.367 5.368 5.370 5.372 | 5.341 5.355 5.359 5.364 5.366 MOD 5.367 5.368 5.369 5.372 |
| 1 610.6-1 613.8 MOBILE-SATELLITE (Earth-to-space) 5.351A RADIO ASTRONOMY AERONAUTICAL RADIONAVIGATION | 1 610.6-1 613.8 MOBILE-SATELLITE (Earth-to-space) 5.351A RADIO ASTRONOMY AERONAUTICAL RADIONAVIGATION RADIODETERMINATION-SATELLITE (Earth-to-space) | 1 610.6-1 613.8 MOBILE-SATELLITE (Earth-to-space) 5.351A RADIO ASTRONOMY AERONAUTICAL RADIONAVIGATION Radiodetermination-satellite (Earth-to-space) |
| 5.149 5.341 5.355 5.359 5.364 5.366 MOD 5.367 5.368 5.369 5.371 5.372 | 5.149 5.341 5.364 5.366 MOD 5.367 5.368 5.370 5.372 | 5.149 5.341 5.355 5.359 5.364 5.366 MOD 5.367 5.368 5.369 5.372 |
| 1 613.8-1 626.5 MOBILE-SATELLITE (Earth-to-space) 5.351A AERONAUTICAL RADIONAVIGATION Mobile-satellite (space-to-Earth) 5.208B | 1 613.8-1 626.5 MOBILE-SATELLITE (Earth-to-space) 5.351A AERONAUTICAL RADIONAVIGATION RADIODETERMINATION- SATELLITE (Earth-to-space) Mobile-satellite (space-to-Earth) 5.208B | 1 613.8-1 626.5 MOBILE-SATELLITE (Earth-to-space) 5.351A AERONAUTICAL RADIONAVIGATION Mobile-satellite (space-to-Earth) 5.208B Radiodetermination-satellite (Earth-to-space) |
| 5.341 5.355 5.359 5.364 5.365 5.366 MOD 5.367 5.368 5.369 5.371 5.372 | 5.341 5.364 5.365 5.366 MOD 5.367 5.368 5.370 5.372 | 5.341 5.355 5.359 5.364 5.365 5.366 MOD 5.367 5.368 5.369 5.372 |

MOD	EUR/5A3/2

4 800-5 570 MHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 5 000-5 010 AERONAUTICAL RADIONAVIGATION RADIONAVIGATION-SATELLITE (Earth-to-space) ADD 5.B13 |  |  |
| 5 010-5 030 AERONAUTICAL RADIONAVIGATION RADIONAVIGATION-SATELLITE (space-to-Earth) (space-to-space) 5.328B 5.443B ADD 5.B13 |  |  |
| 5 030-5 091 AERONAUTICAL RADIONAVIGATION AERONAUTICAL MOBILE (R) ADD 5. A13 ADD 5.B13 5.444 |  |  |
| 5 091-5 150 AERONAUTICAL RADIONAVIGATION AERONAUTICAL MOBILE 5.444B ADD 5.B13 5.444 5.444A |  |  |

Reasons: Sharing studies have shown that an AM(R)S allocation is feasible in the band 5030-5091 MHz, provided that power limitations are associated to these allocations.

ADD	EUR/5A3/3

5.A13	The use of the band 5 030-5 091 MHz by the aeronautical mobile (R) service is limited to internationally standardized aeronautical systems. In order to protect the radionavigation-satellite service (Space-to-earth) in the band 5 010-5 030 MHz, the e.i.r.p. density of any AM(R)S station shall not exceed -75 dBW/MHz in the band 5 010-5 030 MHz.

Reasons: This new footnote limits the use of the AM(R)S allocation to systems developed and standardized by ICAO to facilitate the development of such an internationl system and the coordination between ICAO systems in this band. The out-of-band e.i.r.p. density limit proposed will protect RNSS receivers in 5010-5030 MHz.

MOD	EUR/5A3/4

  - Additional allocation:  The band 1 610-1 626.5 MHz is also allocated to the aeronautical mobile-satellite (R) service on a primary basis, subject to agreement obtained under No. 9.21.

Reasons: This proposed revision to the footnote removes references to the 5 GHz allocation as this is proposed to be treated separately in proposed new footnote 5.B13.

ADD	EUR/5A3/5

5.B13	Additional allocation:  The bands, 5 000-5 030 and 5091 - 5150 MHz are also allocated to the aeronautical mobile-satellite (R) service on a primary basis subject to agreement obtained under No. 9.21. The band, 5 030 - 5091 MHz is also allocated to the aeronautical mobile-satellite (R) service on a primary basis subject to agreement obtained under No. 9.11A. The use of the bands is limited to internationally standardized aeronautical systems

Reasons: This new footnote replaces the application of 9.21 for the AMS(R)S allocation in the band 5030 -5091 MHZ by the application of 9.11A, as a consequence of the new aeronautical mobile (R) allocation.

[MOD	EUR/5A3/6

15.4-18.4 GHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 15.4-15.43 AERONAUTICAL RADIONAVIGATION AERONAUTICAL MOBILE (R) ADD 5.C13 5.511D |  |  |
| 15.43-15.5 FIXED-SATELLITE (Earth-to-space) 5.511A AERONAUTICAL RADIONAVIGATION AERONAUTICAL MOBILE (R) ADD 5.C13 5.511C |  |  |
| 15.5-15.63 FIXED-SATELLITE (Earth-to-space) 5.511A AERONAUTICAL RADIONAVIGATION 5.511C |  |  |

ADD	EUR/5A3/7

5.C13	The use of the band 15.4-15.5 GHz by aeronautical mobile (R) service is limited to internationally standardized aeronautical systems. In order to protect the Radio Astronomy in the band 15.35-15.4 GHz, the e.i.r.p. density of any AM(R)S station shall not exceed -68 dBW/50MHz in the band 15.35-15.4 GHz. For the protection of aeronautical radionavigation systems operating in 15.4-15.5 GHz, any Administration operating AM(R)S systems in this band shall obtain the coordination agreement from other Administrations whose territory is located at less than 903 km from the border of the territory of the Administration intending to operate the AM(R)S.]

APPENDIX  5  (Rev.WRC-12)

Identification of administrations with which coordination is to be effected or
agreement sought under the provisions of Article 9

MOD	EUR/5A3/8

TABLE  5-1     (WRC-12)

Technical conditions for coordination
(see Article 9)

Reasons: A new condition is introduced in Appendix 5 to implement coordination under No. 9.14 for AMS(R)S in the band 5 030-5 091 MHz (See modification in the Table 5-1 below). The level of the accepted interference by the identified administration will be fixed by agreement between administrations concerned using the relevant ITU-R Recommendations as a guide.

| Reference of Article 9 | Case | Frequency bands (and Region) of the service for which coordination is sought | Threshold/condition | Calculation method | Remarks |
| --- | --- | --- | --- | --- | --- |
| No. 9.14 Non-GSO/ terrestrial, GSO/ terrestrial | A space station in a satellite network in the frequency bands for which a footnote refers to No. 9.11A or to No. 9.14, in respect of stations of terrestrial services where threshold(s) is (are) exceeded | 1) Frequency bands for which a footnote refers to No. 9.11A; or 2) 11.7-12.2 GHz (Region 2 GSO FSS) 3) 5 030-5 091 MHz | 1) See § 1 of Annex 1 to this Appendix; In the bands specified in No. 5.414A, the detailed conditions for the application of No. 9.14 are provided in No. 5.414A for MSS networks or 2) In the band 11.7-12.2 GHz (Region 2 GSO FSS): –124 dB(W/(m2 · MHz)) for 0° 5 –124 + 0.5 ( – 5) dB(W/(m2 · MHz)) for 5° < 25 –114 dB(W/(m2 · MHz)) for > 25 where is the angle of arrival of the incident wave above the horizontal plane (degrees) 3) In the band 5 030-5 091 MHz (AMS(R)S), bandwidth overlap is the threshold. | 1) See § 1 of Annex 1 to this Appendix |  |

APPENDIX  7  (Rev.WRC-12)

Methods for the determination of the coordination area around an earth
station in frequency bands between 100 MHz and 105 GHz

MOD	EUR/5A3/9

Reasons: New parameters are introduced in Appendix 7 in Table 7B and 9A to implement coordination under No. 9.15 to 9.18 for AMS(R)S in the band 5 030-5 091 MHz (See modification in the two Tables below) .

TABLE  7b     (WRC-12)

Parameters required for the determination of coordination distance for a transmitting earth station

| Transmitting space radiocommunication service designation |  | Fixed-satellite, mobile-satellite | Aeronautical mobile satellite (R) service | Aeronautical mobile satellite (R) service | Fixed- satellite |  | Fixed- satellite |  | Fixed- satellite |  | Fixed- satellite |  |  |  | Space operation, space research |  |  |  | Fixed-satellite, mobile-satellite, meteorological- satellite |  |  |  | Fixed- satellite |  |  |  | Fixed- satellite |  |  |  | Fixed- satellite |  | Fixed- satellite 3 |  | Fixed- satellite |  | Fixed- satellite 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Frequency bands (GHz) |  | 2.655-2.690 | 5.030-5.091 | 5.030-5.091 | 5.091-5.150 |  | 5.091-5.150 |  | 5.725-5.850 |  | 5.725-7.075 |  |  |  | 7.100-7.235 5 |  |  |  | 7.900-8.400 |  |  |  | 10.7-11.7 |  |  |  | 12.5-14.8 |  |  |  | 13.75-14.3 |  | 15.43-15.65 |  | 17.7-18.4 |  | 19.3-19.7 |  |
| Receiving terrestrial service designations |  | Fixed, mobile | Aeronautical radio- navigation | Aeronautical mobile (R) | Aeronautical radio- navigation |  | Aeronautical mobile (R) |  | Radio-location |  | Fixed, mobile |  |  |  | Fixed, mobile |  |  |  | Fixed, mobile |  |  |  | Fixed, mobile |  |  |  | Fixed, mobile |  |  |  | Radiolocation radionavigation (land only) |  | Aeronautical radionavigation |  | Fixed, mobile |  | Fixed, mobile |  |
| Method to be used |  | § 2.1 | § 2.1, § 2.2 | § 2.1, § 2.2 |  |  |  |  | § 2.1 |  | § 2.1 |  |  |  | § 2.1, § 2.2 |  |  |  | § 2.1 |  |  |  | § 2.1 |  |  |  | § 2.1, § 2.2 |  |  |  | § 2.1 |  |  |  | § 2.1, § 2.2 |  | § 2.2 |  |
| Modulation at terrestrial station 1 |  | A |  |  |  |  |  |  |  |  | A |  | N |  | A |  | N |  | A |  | N |  | A |  | N |  | A |  | N |  | – |  |  |  | N |  | N |  |
| Terrestrial station interference parameters and criteria | p0 (%) | 0.01 |  |  |  |  |  |  |  |  |  | 0.01 |  | 0.005 |  | 0.01 |  | 0.005 |  | 0.01 |  | 0.005 |  | 0.01 |  | 0.005 |  | 0.01 |  | 0.005 |  | 0.01 |  |  |  | 0.005 |  | 0.005 |  |
|  | n | 2 |  |  |  |  |  |  |  |  |  | 2 |  | 2 |  | 2 |  | 2 |  | 2 |  | 2 |  | 2 |  | 2 |  | 2 |  | 2 |  | 1 |  |  |  | 2 |  | 2 |  |
|  | p (%) | 0.005 |  |  |  |  |  |  |  |  |  | 0.005 |  | 0.0025 |  | 0.005 |  | 0.0025 |  | 0.005 |  | 0.0025 |  | 0.005 |  | 0.0025 |  | 0.005 |  | 0.0025 |  | 0.01 |  |  |  | 0.0025 |  | 0.0025 |  |
|  | NL (dB) | 0 |  |  |  |  |  |  |  |  |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  |  |  | 0 |  | 0 |  |
|  | Ms (dB) | 26 2 |  |  |  |  |  |  |  |  |  | 33 |  | 37 |  | 33 |  | 37 |  | 33 |  | 37 |  | 33 |  | 40 |  | 33 |  | 40 |  | 1 |  |  |  | 25 |  | 25 |  |
|  | W (dB) | 0 |  |  |  |  |  |  |  |  |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  |  |  | 0 |  | 0 |  |
| Terrestrial station parameters | Gx (dBi) 4 | 49 2 | 6 | 10 |  | 6 |  | 6 |  |  |  | 46 |  | 46 |  | 46 |  | 46 |  | 46 |  | 46 |  | 50 |  | 50 |  | 52 |  | 52 |  | 36 |  |  |  | 48 |  | 48 |  |
|  | Te (K) | 500 2 |  |  |  |  |  |  |  |  |  | 750 |  | 750 |  | 750 |  | 750 |  | 750 |  | 750 |  | 1 500 |  | 1 100 |  | 1 500 |  | 1 100 |  | 2 636 |  |  |  | 1 100 |  | 1 100 |  |
| Reference bandwidth | B (Hz) | 4  103 | 150  103 | 37.5 x 103 |  | 150  103 |  | 106 |  |  |  | 4  103 |  | 106 |  | 4  103 |  | 106 |  | 4  103 |  | 106 |  | 4  103 |  | 106 |  | 4  103 |  | 106 |  | 107 |  |  |  | 106 |  | 106 |  |
| Permissible interference power | Pr( p) (dBW) in B | –140 | –160 | –154 |  | –160 |  | –143 |  |  |  | –131 |  | –103 |  | –131 |  | –103 |  | –131 |  | –103 |  | –128 |  | –98 |  | –128 |  | –98 |  | –131 |  |  |  | –113 |  | –113 |  |

MOD	EUR/5A3/10

TABLE  9a     (WRC-12)

Parameters required for the determination of coordination distance for a transmitting earth station
in bands shared bidirectionally with receiving earth stations

| Space service designation in which the transmitting earth station operates |  | Land mobile- satellite | Mobile-satellite | Land mobile-satellite | Earth exploration-satellite, meteorological-satellite | Mobile-satellite |  | Fixed-satellite, mobile-satellite | Aeronautical mobile satellite (R) service |  | Fixed- satellite 3 |  | Fixed-satellite | Fixed-satellite, meteorological- satellite | Fixed-satellite |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Frequency bands (GHz) |  | 0.1499-0.15005 | 0.272-0.273 | 0.3999-0.40005 | 0.401-0.402 | 1.670-1.675 |  | 2.655-2.690 | 5.030-5.091 |  | 5.150-5.216 |  | 6.700-7.075 | 8.025-8.400 | 8.025-8.400 |
| Space service designation in which the receiving earth station operates |  | Radio-navigation-satellite | Space operation | Radio-navigation-satellite | Space operation | Meteorological-satellite |  | Fixed-satellite, broadcasting-satellite | Aeronautical mobile satellite (R) service |  | Fixed-satellite | Radiodetermination-satellite | Fixed-satellite | Earth exploration- satellite | Earth exploration- satellite |
| Orbit 6 |  |  | Non-GSO |  | Non-GSO | Non-GSO | GSO |  | Non-GSO | GSO | Non-GSO |  | Non-GSO | Non-GSO | GSO |
| Modulation at receiving earth station 1 |  |  | N |  | N | N | N |  |  |  |  |  | N | N | N |
| Receiving earth station interference parameters and criteria | p0 (%) |  | 1.0 |  | 0.1 | 0.006 | 0.011 |  |  |  |  |  | 0.005 | 0.011 | 0.083 |
|  | n |  | 1 |  | 2 | 3 | 2 |  |  |  |  |  | 3 | 2 | 2 |
|  | p (%) |  | 1.0 |  | 0.05 | 0.002 | 0.0055 |  |  |  |  |  | 0.0017 | 0.0055 | 0.0415 |
|  | NL (dB) | 0 | 0 | 0 | 0 | 0 | 0 |  |  |  |  |  | 1 | 0 | 1 |
|  | Ms (dB) | 2 | 1 | 2 | 1 | 2.8 | 0.9 | 2 |  |  | 2 | 2 | 2 | 4.7 | 2 |
|  | W (dB) | 0 | 0 | 0 | 0 | 0 | 0 |  |  |  |  |  | 0 | 0 | 0 |
| Receiving earth station parameters | Gm (dBi) 2 | 0 | 20 | 0 | 20 | 30 | 45 |  | 45 | 45 | 48.5 |  | 50.7 |  |  |
|  | Gr (dBi) 4 | 0 | 19 | 0 | 19 | 19 9 | 8 |  | 8 | 8 | 10 |  | 10 | 10 | 8 |
|  | min 5 | 3 | 10 | 3 | 10 | 5 | 3 | 3 | 10° | 10° | 3 | 3 | 3 | 5 | 3 |
|  | Te (K) 7 | 200 | 500 | 200 | 500 | 370 | 118 | 75 | 340 | 340 | 75 | 75 | 75 |  |  |
| Reference bandwidth | B (Hz) | 4  103 | 103 | 4  103 | 1 | 106 | 4  103 |  | 37.5 x103 | 37.5 x103 |  |  | 106 | 106 | 106 |
| Permissible interference power | Pr( p) (dBW) in B | –172 | –177 | –172 | –208 | –145 | –178 |  | -163.5 | -163.5 |  |  | –151 | –142 | –154 |

# Sub-Part 3B

Satellite component

EUR/5A3/11

NOC to the Radio Regulations for the satellite component for the Control and non PayLoad communication link of UAS under AI 1.3