CPG PTC(11)082 Annex 14

PRELIMINARY DRAFT EUROPEAN COMMON PROPOSALS FOR THE WORK OF THE CONFERENCE

AGENDA ITEM 1.15

1.15	to consider possible allocations in the range 3-50 MHz to the radiolocation service for oceanographic radar applications, taking into account the results of ITU-R studies, in accordance with Resolution 612 (WRC-07);

Introduction

This agenda item covers the following issue:

Resolution 612 (WRC-2007): Use of the radiolocation service between 3 and 50 MHz to support high-frequency oceanographic radar operations

CEPT proposes new primary allocations for the radiolocation service in portions of the 3 to 50 MHz band identified as suitable for current and future oceanographic radar operations. CEPT also proposes technical and regulatory conditions to protect other services in these frequency bands, including the emission of a call sign to properly identify oceanographic systems.

ARTICLE 5

Frequency allocations

Section IV – Table of Frequency Allocations
(See No. 2.1)

MOD	EUR/XXA15/1

5 003-7 450 kHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| … |  |  |
| 5 060-5 160 FIXED MOBILE except aeronautical mobile RADIOLOCATION ADD 5.A115 |  |  |
| 5 160-5 250 FIXED MOBILEexcept aeronautical mobile |  |  |
| ... |  |  |

7 450 – 13 360 kHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| … |  |  |
| 9 040-9 200 FIXED |  |  |
| 9 200-9 300 FIXED RADIOLOCATION ADD 5.A115 |  |  |
| 9 300-9 400 FIXED Radiolocation ADD 5.A115 |  |  |
| … |  |  |
| 12 100-12 200 FIXED RADIOLOCATION ADD 5.A115 |  |  |
| 12 200-12 230 FIXED |  |  |
| … |  |  |

13 360 - 18 030 kHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| … |  |  |
| 13 410-13 510 FIXED MOBILEexcept aeronautical mobile (R) Radiolocation ADD 5.A115 5.150 |  |  |
| 13 510-13 570 FIXED MOBILE except aeronautical mobile (R) 5.150 |  |  |
| … |  |  |
| 13 870-14 000 FIXED MOBILEexcept aeronautical mobile (R). …. 14 350-14 990 FIXED MOBILEexcept aeronautical mobile (R). |
| … |  |  |
| 15 800-16 000 FIXED 5.153 |  |  |
| 16 000-16 100 FIXED Radiolocation ADD 5.A115 5.153 |  |  |
| 16 100-16 200 FIXED RADIOLOCATION ADD 5.A115 5.153 |  |  |
| 16 200-16 360 FIXED 5.153 |  |  |
| … |  |  |

23 350-27 500 kHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| … |  |  |
| 26 175 - 27 200 FIXED MOBILE except aeronautical mobile 5.150 |  |  |
| 27 200 -27 500 FIXED MOBILE except aeronautical mobile RADIOLOCATION ADD 5.A115 5.150 |  |  |

27,5-47 MHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| … |  |  |
| 38.25-39 FIXED MOBILE |  |  |
| 39-39.5 FIXED MOBILE RADIOLOCATION ADD 5.A115 |  |  |
| 39.5-39.986 FIXED MOBILE |  |  |
| … |  |  |

Reasons: CEPT supports new primary and secondary allocations for the radiolocation service in portions of the 3 to 50 MHz band identified as suitable for oceanographic radar operations. As a consequence of the introduction of the radiolocation service in some bands, in order to protect all incumbent services, CEPT proposes to upgrade the mobile service allocations to a primary status in bands where radiolocation allocation is introduced as well as in adjacent/neighbouring bands.

SUP EUR/5A15/2

5.133	Different category of service:  in Armenia, Azerbaijan, Belarus, the Russian Federation, Georgia, Kazakhstan, Latvia, Lithuania, Uzbekistan, Kyrgyzstan, Tajikistan, Turkmenistan and Ukraine, the allocation of the band 5 130-5 250 kHz to the mobile, except aeronautical mobile, service is on a primary basis (see No. 5.33). (WRC-07)

Reasons: The suppression of 5.133 is the consequence of the upgrade of the mobile service allocation to a primary status in the band 5 060 kHz to 5 250 kHz.

ADD	EUR/5A15/3

5.A115	Radiolocation stations operating in the radiolocation service in the bands 5 060-5 160 kHz, 9 200-9 400 kHz, 12 100-12 200 kHz, 13 410-13 510 kHz, 16 000-16 200 kHz, 27 200-27 500 kHz, and 39-39.5 MHz shall comply with the provisions of Resolution 612 (Rev. WRC-12).     (WRC-12)

Reasons: CEPT proposes technical and regulatory conditions to protect other services in the frequency bands where new allocations to radiolocation between 3 and 50 MHz are introduced.

APPENDIX  4  (Rev.WRC-12)

Consolidated list and tables of characteristics for use in the
application of the procedures of Chapter III

…

ANNEX 1

Characteristics of stations in the terrestrial services

…

TABLE 1

Characteristics for terrestrial services

ADD	EUR/5A15/4

| Column No. | Item identifier | Notice related to Description of data items and requirements | Broadcasting (sound and television) stations in the VHF/UHF bands up to 960 MHz, for the application of No. 11.2 and No. 9.21 | Broadcasting (sound) stations in the LF/MF bands, for the application of No. 11.2 | Transmitting stations (except broadcasting stations in the planned LF/MF bands, in the HF bands governed by Article 12, and in the VHF/UHF bands up to 960 MHz), for the application of No. 11.2 and No. 9.21 | Receiving land stations, for the application of No. 11.9 and No. 9.21 | Typical transmitting stations, for the application of No. 11.17 | Maritime mobile frequency allotment, for the application of plan modification under Appendix 25 (Nos. 25/1.1.1, 25/1.1.2, 25/1.25) | Broadcasting stations in the HF bands, for the application of No. 12.16 | Item identifier |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| … | … | … | … |  |  |  |  |  |  |  |
| … | … | … | … | … | … | … | … | … | … | … |
| 3 |  | CALL SIGN AND STATION IDENTIFICATION |  |  |  |  |  |  |  |  |
| 3.1 | 3A1 | the call sign used in accordance with Article 19 In the case of a transmitting station, for the fixed service below 28 MHz, mobile service, meteorological aids service, radiolocation service between 3 and 50 MHz (operating in accordance with Resolution 612 (Rev.WRC 12)), or standard frequency and time signal service, in the application of Article 11, required if the station identification (3A2) is not provided | O | O | + |  |  |  | O | 3A1 |
| … | … | … |  |  |  |  |  |  |  |  |

Reasons: CEPT supports the emission of a call sign to properly identify oceanographic systems with a primary or secondary allocation between 3 and 50 MHz.

MOD	EUR/5A15/5

RESOLUTION 612 (Rev.WRC-12)

Use of the radiolocation service between 3 and 50 MHz to
support oceanographic radar operations

The World Radiocommunication Conference (Geneva,2012),

considering

a)	that there is increasing interest, on a global basis, in the operation of oceanographic radars for measurement of coastal sea surface conditions to support environmental, oceanographic, meteorological, climatological, maritime and disaster mitigation operations;

b)	that oceanographic radars are also known in parts of the world as HF ocean radars, HF wave height sensing radars or HF surface wave radars;

c)	that oceanographic radars operate through the use of ground-wave propagation over seas;

d)	that oceanographic radar technology has applications in global maritime domain awareness by allowing the long-range sensing of surface vessels, which provides a benefit to the global safety and security of shipping and ports;

e)	that operation of oceanographic radars provides benefits to society through environmental protection, disaster preparedness, public health protection, improved meteorological operations, increased coastal and maritime safety and enhancement of national economies;

f )	that oceanographic radars have been operated on an experimental basis around the world, providing an understanding of spectrum needs and spectrum sharing considerations, as well as an understanding of the benefits these systems provide;

g)	that performance and data requirements dictate the regions of spectrum that can be used by oceanographic radar systems for ocean observations,

recognizing

a)	that oceanographic radars have been operated  under provision N°4.4 since the 1970s by several administrations;

b)	that developers of the experimental systems have implemented techniques to make the most efficient use of the spectrum and mitigate interference to other radio services;

-

c)	that for the purpose of protecting existing services from harmful interference, received interference by those services from oceanographic radars shall not exceed a power flux-density with an I/N ratio of −6 dB in an area where its radio noise is categorized to the rural and the quiet rural defined in Recommendation ITU-R P.372-10;

d)	that for the purpose of protecting existing services from harmful interference, oceanographic radars’ impact via ground-wave propagation can be checked by Report ITU-R M.[RLS 3-50 MHz SHARING] based on Recommendation ITU-R P.368-9,

resolves

-

-

--

- that in accordance with No. 11.2 of the RR oceanographic radars shall be notified to the Bureau and shall contain the station identification (call sign) (Appendix 4 and Article 19)

- that the peak e.i.r.p of oceanographic radars shall not exceed 25 dBW;

- that for transmission of the station identification machine readable code shall be used (No.19.18);

4	that the separation distance between an oceanographic radar and the border of a neighbouring country shall be higher than the distances specified in the following table unless otherwise agreed between neighbouring countries:

| Frequency band | Land path |  | Sea path or mixed |  |
| --- | --- | --- | --- | --- |
|  | Rural* | Quiet Rural* | Rural* | Quiet Rural* |
| 5 060-5 160 kHz | 120 Km | 170 Km | 790 Km | 920 Km |
| 9 200-9 400 kHz | 100 Km | 130 Km | 590 Km | 670 Km |
| 12 100-12 200 kHz and 13 410-13 510 kHz | 100 Km | 110 Km | 480 Km | 520 Km |
| 16 000-16 200 kHz | 80 Km | 100 Km | 390 Km | 450 Km |
| 27 200-27 500 kHz | 80 Km | 100 Km | 280 Km | 320 Km |
| 39-39.5 MHz | 80 Km | 100 km | 200 Km | 230 Km |

-

-

Reasons: CEPT supports technical and regulatory conditions to protect other services in the frequency bands between 3 and 50 MHz where new primary and secondary allocations to radiolocation are introduced.

It has to be noted that provision 19.1.1 does not apply.

Given the difficulty in predicting the harmful interference from the sky-wave propagation the separation distance must be understood to be the minimum required separation distance beyond which an oceanographic radar will not cause harmful interference into co-primary services receivers via ground-wave propagation. The rural or quiet rural environment applies to the location of the fixed or mobile service receiver, not the radiolocation system location.