CPG PTC(11)082 Annex 16

EUROPEAN COMMON PROPOSALS
FOR WRC-12 WORK

Agenda Item 1.21

1.21 to consider a primary allocation to the radiolocation service in the band 15.4-

15.7 GHz, taking into account the results of ITU-R studies, in accordance with

Resolution 614 (WRC-07);

Resolution 614 (WRC-07): Use of the band 15.4-15.7 GHz by the radiolocation service

Introduction

The results of the ITU studies demonstrate that the operation of radiolocation systems in the 15.4-15.7 GHz band is compatible with adjacent-band and co-frequency allocations, provided that appropriate mitigation techniques are adopted.

OPTION 1 (Allocation to Radiolocation of 300 MHz in 15.4-15.7 GHz)

Proposal

The European countries proposes a new primary allocation for the RLS in the 15.4-15.7 GHz frequency band and a possible resolution providing further clarification on the use of this band as well as taking practical steps to protect ARNS and RAS in the adjacent 15.35-15.4 GHz frequency band.

This will provide a primary allocation to RLS, contiguous across 15.4-17.3 GHz, with sufficient bandwidth to meet emerging requirements for increased image resolution and range accuracy, and assure long-term operating and development environment for radiolocation systems, while maintaining protection in the Radio Regulations for ARNS and RAS systems.

The addition of a primary allocation for the RLS in the 15.4-15.7 GHz band and examples of new footnotes protecting ARNS and RAS systems are given below.

MOD  EUR/1.21/01

ARTICLE  5

Frequency allocations

Section IV – Table of Frequency Allocations
(See No. 2.1)

15.4-18.4 GHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 15.4-15.43 AERONAUTICAL RADIONAVIGATION RADIOLOCATION ADD 5.A121 ADD 5.B121 5.511D |  |  |
| 15.43-15.63 FIXED-SATELLITE (Earth-to-space) 5.511A AERONAUTICAL RADIONAVIGATION RADIOLOCATION ADD 5.A121 ADD 5.B121 5.511C |  |  |
| 15.63-15.7 AERONAUTICAL RADIONAVIGATION RADIOLOCATION ADD 5.A121 ADD 5.B121 5.511D |  |  |
| ... |  |  |

ADD EUR/1.21/02

5.A121	In the band 15.4-15.7 GHz, stations operating in the radiolocation service shall not cause harmful interference to, nor claim protection from, stations operating in the aeronautical radionavigation service.

ADD EUR/1.21/03

5.B121	In order to protect the radio astronomy service in the band 15.35-15.4 GHz, transmissions from radiolocation stations operating in the band 15.4-15.7 GHz shall not exceed the power flux-density level of −156 dB(W/m2) in a 50 MHz bandwidth into the band 15.35-15.4 GHz, at any radio astronomy observatory site for more than 2% of the time.

SUP EUR/1.21/04

RESOLUTION 614 (WRC-07)

Use of the band 15.4-15.7 GHz by the radiolocation service

OPTION 2 (Allocation to Radiolocation of 200 MHz in 15.5-15.7 GHz)

Proposal

The European countries proposes a new primary allocation for the RLS in the 15.5-15.7 GHz frequency band and a possible resolution providing further clarification on the use of this band as well as taking practical steps to protect ARNS.

This will provide a primary allocation to RLS, contiguous across 15.5-17.3 GHz, with sufficient bandwidth to meet emerging requirements for increased image resolution and range accuracy, and assure long-term operating and development environment for radiolocation systems, while maintaining protection in the Radio Regulations for ARNS and RAS systems.

The addition of a primary allocation for the RLS in the 15.5-15.7 GHz band and examples of a new footnote protecting ARNS systems is given below.

MOD  EUR/1.21/01

ARTICLE  5

Frequency allocations

Section IV – Table of Frequency Allocations
(See No. 2.1)

15.4-18.4 GHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 15.4-15.43 AERONAUTICAL RADIONAVIGATION 5.511D |  |  |
| 15.43-15.5 FIXED-SATELLITE (Earth-to-space) 5.511A AERONAUTICAL RADIONAVIGATION 5.511C |  |  |
| 15.5-15.63 FIXED-SATELLITE (Earth-to-space) 5.511A AERONAUTICAL RADIONAVIGATION RADIOLOCATION ADD 5.A121 5.511C |  |  |
| 15.63-15.7 AERONAUTICAL RADIONAVIGATION RADIOLOCATION ADD 5.A121 5.511D |  |  |
| ... |  |  |

ADD EUR/1.21/02

5.A121	In the band 15.5-15.7 GHz, stations operating in the radiolocation service shall not cause harmful interference to, nor claim protection from, stations operating in the aeronautical radionavigation service.

SUP EUR/1.21/03

RESOLUTION 614 (WRC-07)

Use of the band 15.4-15.7 GHz by the radiolocation service