| World Radiocommunication Conference (WRC-12) Geneva, 23January - 17February 2012 |  |
| --- | --- |
| PLENARY MEETING | Addendum 25 to Document 5-E |
|  | 2011 |
|  | Original: English |
| European common PROPOSAl FOR THE WORK OF THE CONFERENCE |  |
| PART 25 |  |
| Agenda item 1.25 |  |

1.25	to consider possible additional allocations to the mobile-satellite service, in accordance with Resolution 231 (WRC-07);

Introduction

Resolution 231 (WRC-07) resolves: to complete, for WRC-11, studies of possible bands for new allocations to the mobile-satellite service (MSS) in the Earth-to-space and space-to-Earth directions, with particular focus on the range 4 GHz to 16 GHz, taking into account sharing and compatibility, without placing undue constraints on existing services in this band.

The frequency range 4 and 16 GHz has been examined with regard to Agenda Item 1.25 and more closely studied a number of frequency bands for potential new MSS allocations. Those bands are:

| Frequency band | MSS direction |
| --- | --- |
| 5 150-5 250 MHz | Downlink |
| 7 055-7 250 MHz | Downlink |
| 8 400-8 500 MHz | Uplink |
| 10.5-10.6 GHz | Downlink |
| 13.25-13.4 GHz | Downlink |
| 15.43-15.63 GHz | Uplink |

Frequency band 5 150-5 250 MHz

From the technical point of view it could be possible in some cases to reach a sharing between existing services, operating in 5150 – 5250 MHz, and new MSS (s-E) allocation. However there is a view that there is no demand for additional MSS allocation due to current low spectrum use.

Frequency band 7 055-7 250 MHz

Taking into account the cumulative impact of all aspects identified during the studies the sharingbetween incumbent services and new MSS applications would practically not be feasible in the frequency band 7 055-7 250 MHz due to severe operational constraints that MSS systems may suffer to achieve compatibility with affected current and future systems of other services and due to interference that may be caused by MSS to other services to which the frequency band is allocated. Furthermore provisions of Resolution 231 (WRC-07)inviting ITU-R to place “no undue constraints on existing systems operating in accordance with the Radio Regulations” could not be complied with considering the constraints which would have to be imposed on existing services in order to allow for viable MSS operations.

Frequency band 8 400-8 500 MHz

Taking into account the cumulative impact of all aspects identified during the studies the sharingbetween incumbent services and new MSS applications would practically not be feasible in the frequency band 8 400-8 500 MHz due to severe operational constraints that MSS systems may suffer to achieve compatibility with affected current and future systems of other services and due to interference that may be caused by MSS to other services to which the frequency band is allocated. Furthermore provisions of Resolution 231 (WRC-07)inviting ITU-R to place “no undue constraints on existing systems operating in accordance with the Radio Regulations” could not be complied with considering the constraints which would have to be imposed on existing services in order to allow for viable MSS operations.

Frequency band 10.5 – 10.6 GHz

Based on the result of studies the sharing between MSS (s-E) with the stations of existing services is not feasible in the frequency band 10.5-10.6GHz.

Frequency band 13.25 – 13.4 GHz

Based on the result of studies the sharing between aeronautical radionavigation service (ARNS) operating in a number of administrations and new MSS applications (s-E) is not feasible in the frequency band 13.25-13.4GHz.

Frequency band 15.43 – 15.63 GHz

The band 15.43-15.63 GHz is allocated to the aeronautical radionavigation service (ARNS) on a primary basis and provision RR No. 4.10 applies (safety related).  Based on the result of studies the sharing between ARNS operating in a number of administrations and new MSS applications (E-s) is not feasible in the frequency band 15.43 -15.63GHz.

Based on the results of studies regarding Agenda Item 1.25, in particular on the frequency band mentioned above, Europe proposes the following:

ARTICLE 5

Frequency allocations

Section IV  –  Table of Frequency Allocations
(See No. 2.1)

NOC	EUR/5A25/1

to the Table of Frequency Allocations with regard to Agenda Item 1.25.

Reasons: Studies relevant for the frequency band 4-16 GHz conducted under this Agenda Item shows that unduly constrains to existing, planned and future radio applications operating in the existing services can notbe avoided.

SUP	EUR/5A25/2

RESOLUTION  231  (WRC-07)

Additional allocations to the mobile-satellite service with
particular focus on the bands between 4 GHz and 16 GHz

Reasons: Resolution 231 (WRC-07) is no longer required.

________________

_______________