|  | ECC/CPG12(2011)046 Annex 5 Source:CPG12(2011) 032 Annex IV AI 1.24 |
| --- | --- |

Draft CEPT Brief on agenda item 1.24

1.24		to consider the existing allocation to the meteorological-satellite service in the band 7 750-7 850 MHz with a view to extending this allocation to the band 7 850-7 900 MHz, limited to non-geostationary meteorological satellites in the space-to-Earth direction, in accordance with Resolution 672 (WRC-07).

## Issue

Extension of the existing primary allocation to the meteorological-satellite service in the band 7750-7850 MHz to the band 7850 – 7900 MHz, limited to non-geostationary meteorological satellites in the space-to-Earth direction, in order to enable next generation MetSat systems to transmit higher data rates.

CEPT position

CEPT supports the allocation of the frequency band 7 850 – 7 900 MHz to the MetSat, because the compatibility assessment has shown that sharing with the FS (including ENG/OB) under the same regulatory conditions as existing in the currently allocated band 7 750-7 850 MHz is feasible. In this band 7 850 – 7 900 MHz the pfd limits already applicable to the band 7 750-7 850 MHz, as contained in Table 21-4 of Article 21, shall be applied. Furthermore, for the determination of coordination distances for receiving MetSat earth stations the same parameters as already contained in Table 8c of Appendix 7 shall also be applied for this band 7 850 – 7 900 MHz. CEPT has not been made aware of use of this band by mobile systems other than ENG/OB.

Background

The European EUMETSAT Polar System (EPS) with its series of three Metop satellites (Metop-A is operational since October 2006) is using the band 7750 – 7850 MHz for the downlink of the gathered instrument data of a full orbit over dedicated Earth stations in Svalbard (Spitsbergen) and McMurdo (Antarctica). It is the European contribution to the Initial Joint Polar-Orbiting Operational Satellite System (IJPS), a joint European-US polar satellite system. This system, which observes the Earth's weather, climate and environment, has significantly improved operational meteorology, in particular the Numerical Weather Prediction (NWP).

Europe will have to provide continuity to the current system in the areas of operational meteorology, climate monitoring, and detection of global climatic changes with a follow-on system to EPS referred to as EPS Second Generation (EPS-SG). Furthermore, it also has the objective to provide continuity of the services of EPS, in the framework of the European-US follow-on to IJPS, the Joint Polar System (JPS).

The required extension of the MetSat allocation into the band 7850 – 7900 MHz would only concern the same radiocommunication services, namely the FIXED and MOBILE (except aeronautical mobile) service, like in the band 7750 – 7850 MHz where MetSat (limited to NGSO) is already allocated on a primary basis.

The results of the sharing studies are contained in Report ITU-R SA.2164 on “Compatibility between the meteorological satellite and the fixed service in the band 7 850-7 900 MHz”. Regarding the potential of interference into FS receivers, the compatibility studies with non-geostationary MetSat systems concluded that even under worst case assumptions, a positive margin of 11 dB with respect to the FDP fixed service interference criterion remains, matching well with the results of the sharing studies that were performed in preparation for WRC-97. In order to respond to the concern raised by Iran at CPM regarding FS links with elevation angles >5°, an additional sharing study was performed in the framework of ITU WP7B, resulting in modifications to Report ITU-R SA.2164. This sharing study has confirmed that for fixed service links at mid-latitudes with elevation angles above 5° such as in Iran have margins of around 30 dB.

Considering the potential of interference of transmitting FS stations into MetSat earth station receivers, separation distances for typical constellations are in the order of 5-20 km and are generally determined by the first obstacle in the line-of-sight transmission path. Only in a line-of-sight mainbeam-to-mainbeam scenario using maximum FS EIRP density levels as contained in Recommendation ITU-R F.758 and very worst case constellations, the separation distance can extend up to around 40-50 km.

It can be concluded that the potential extension band 7 850-7 900 MHz can be shared under the same conditions as the current allocation in the band 7 750-7 850 MHz. The current pfd limits contained in Table 21-4 of Article 21 and Table 8c of Appendix 7 for the band 7 750-7 850 MHz are adequate for proper operation of future meteorological satellite systems as well as protection of fixed service systems.

Also for ENG/OB applications potentially operated in some countries the sharing situation is not expected to be significantly different to that for FS systems.

List of relevant documents

Resolution 672 (WRC-07) – Extension of the allocation to the meteorological-satellite service in the band 7750 – 7850 MHz;

Recommendation ITU-R F.758-4 – Considerations in the development of criteria for sharing between the terrestrial fixed service and other services;

## Report ITU-R SA.2164 – Compatibility between the meteorological satellite and the fixed service in the band 7 850-7 900 MHz;

## CPM Report, chapter 4, pages 35 – 38.

## Actions to be taken

## ---

## Relevant information from outside CEPT

European Union

Note from Presidency to Coreper, dd 12 May 2011:

the following objectives to be achieved at WRC-12 in view of the successful implementation of relevant Union policies, taking into account the Commission's Communication:

- to promote new allocations and protection for scientific services, in particular services helping to combat climate change and ensuring the security of citizens; to support studies on the role of Earth Observation for monitoring and predicting climate change, for disaster prediction and for understanding all aspects of climate change (WRC-12 Agenda Item 1.6/1.8/1.24/8.1.1);

Regional telecommunication organisations

APT (2September 2011)

- APT Members support the results of ITU-R studies invited by Resolution 672 (WRC-07) to address sharing of Metsat services with the existing and planned fixed and mobile services in the band 7 850-7 900 MHz

- APT Members support the single method outlined in the CPM Report, in accordance with the sections 4/1.24/5 and 4/1.24/6. The Preliminary APT Common Proposals are in line with the ECPs on this agenda item.

- One APT member conditionally supports the single method outlined in CPM Report, waiting for the results of simulations for FS stations with antenna elevation angles greater than 5° to be included in the Report ITU-R SA.2164 by WP-7B which will be held at the end of September 2011, clearly illustrating the protection of such FS stations with respect to Non-GSO Metsat systems.

ATU (July 2011)

ATU members support the only method but that power limits be always respected and on similar provisions.

ASMG (October 2011)

The preliminary position of ASMG is to support an additional allocation to non-geostationary meteorological satellites (s-E) in the 7 850-7 900 MHz, subject to regulatory, procedural, technical measures and conditions provided in Article 21 and Appendix 7 of the RR, similar to the ones imposed in the band 7750-7850 MHz.

CITEL (May 2011)

The IAPs (Argentine Republic, Brazil (Federative Republic of), Canada, Chile, Colombia (Republic of), Costa Rica, Dominican Republic, Ecuador, United States of America, Guatemala (Republic of), Mexico, Uruguay (Eastern Republic of), Venezuela (Bolivarian Republic of)) are in line with the ECPs on this agenda item.

CITEL supports the only Method in the CPM report

- MOD Article 5 to extend MetSat to 7850-7900 MHz

- MOD footnote 5.461B

- MOD Table 21-4

- MOD Appendix 7 Table 8c

- SUP Resolution 672 (WRC-07)

RCC (September 2011)

RCC CAs do not object to allocation of the band 7850-7900 MHz to the space-Earth meteorological satellite service (limited to non-geostationary networks) on a primary basis under the following provisions:

- pfd limits contained in Table 21-4 of RR Article 21 currently applicable to the band 7750-7850 MHz shall be applied in the frequency band 7850 – 7900 MHz;

- to apply the same parameters that are indicated for the frequency band 7750 – 7850 MHz for the frequency band 7850 – 7900 MHz in Table 8c of RR Appendix 7.

International organisations

NATO (October 2011)

NATO supports the allocation of the frequency band 7 850 – 7 900 MHz to the MetSat as proposed in the CPM 11-2 Report, because the compatibility assessment has shown that sharing with the FS (including ENG/OB) under the same regulatory conditions as existing in the currently allocated band 7 750-7 850 MHz is feasible.

SFCG (June 2011)

SFCG supports the single Method described in the CPM Report on WRC-12 Agenda item 1.24.

The mission requirements for next generation non-GSO meteorological satellites in terms of observations, instruments and user-services clearly show a need to transmit higher data rates compared to current systems. SFCG supports the extension of the existing allocation of the 7 750-7 850 MHz band to the meteorological-satellite service (space-to-Earth) for use by non-geostationary satellites into the 7 850-7 900 MHz band. SFCG notes that sharing scenarios in the frequency band 7 850-7 900 MHz are similar to ones in the frequency band 7 750-7 850 MHz.

WMO (October 2011)

WMO stresses that the same services are allocated in the 7750-7850 MHz and 7850-7900 MHz bands hence justifying similar sharing conditions with the METSAT service. WMO supports the METSAT allocation extension in the 7850-7900 MHz under the same regulatory conditions as in the 7750-7850 MHz band as described in the only Method in the CPM Report.

Regional organisations

ESA (June 2011)

Same as SFCG position.

EUMETNET  (October 2011)

Same as WMO position.

EUMETSAT  (June 2011)

Same as WMO and SFCG.