|  |  | Doc. ECC/CPG12(2011) 049 |
| --- | --- | --- |
| CPG Bucharest, 01-04 November 2011 |  |  |
| Date issued: 25th October 2011 Source: France Subject: WRC-12 Agenda Item 1.18 |  |  |

Summary:

PTD minutes mentioned that: “A French input at WP 4C has highlighted a correction required to a study on the RDSS impacts into MSS and which is included in the CPM text. This has no impact on the ECP, but may influence discussions about relaxing the MSS coordination trigger level. In the 4C discussions, it was suggested that CEPT could make a contribution to WRC with the corrected study. PT-D agreed to this in principle, but the paper would need to be sent to CPG for approval”. This document proposes a CEPT input to WRC-12 explaining the mistake done and gives additional technical information. Two annexes contain additional studies which show that by using the polarisation isolation between the two systems or by increasing both their PFD threshold level, the sharing is feasible.

Proposal:

CPG is invited to consider this document to approve it and send it to WRC-12.

Background:

- to consider extending the existing primary and secondary radiodetermination-satellite service (space-to-Earth) allocations in the band 2 483.5-2 500 MHz in order to make a global primary allocation, and to determine the necessary regulatory provisions based upon the results of ITU-R studies, in accordance with Resolution 613 (WRC-07);

| World Radiocommunication Conference (WRC-12) Geneva, 23 January - 17 February 2012 |  |
| --- | --- |
| PLENARY MEETING | Document XXX-E |
|  | Original: English |
| CEPT |  |
| PROPOSALS FOR THE WORK OF THE CONFERENCE |  |
| Sharing between MSS and RDSS in the band 2 483.5-2 500 MHz in relation with Agenda Item 1.18 |  |

- Introduction

WRC-12 Agenda item 1.18 calls for the conduct of appropriate technical, operational and regulatory studies leading to technical and procedural recommendations to WRC-12 enabling it to decide whether a global primary allocation for the radiodetermination-satellite service (RDSS) in the frequency band 2 483.5-2 500 MHz (Space to Earth) is compatible with other services in the band.

- Summary and Discussion:

The purpose of this contribution is to correct the technical material regarding the sharing studies between RDSS and MSS and to give additional information to the CPM text. A computation error related to the studies on the impact of RDSS emissions on MSS receivers was discovered and brought forward at WP 4C for consideration (see document 4C/654): in previous studies, this impact was computed based on a methodology using a spectral separation coefficient (SSC) calculated based on a pre-correlation bandwidth of 16.5 MHz (i.e. the entire MSS allocation) instead of 1.23 MHz (i.e. the MSS receiver bandwidth). This error does not affect the general validity of the conclusions related to the feasibility for RDSS and MSS systems to share the 2 483.5-2 500 MHz band, but the attention of administrations is drawn that additional measures may be needed to overcome the additional degradation caused by RDSS systems on MSS receivers.

The results of these new calculations indicate that interference could be caused to MSS system if the RDSS system and the MSS system are operated on a co-frequency and co-coverage basis (see tables 3&5 of Annex A of this document). But these calculations have been done in a worst case considering 12 RDSS satellites in visibility with the same PFD value for each satellite. In reality, in many locations on Earth, the number of satellite in visibility is less important and the terrain shielding will add attenuation for satellites seen from low elevation angles.

- Moreover CEPT proposes new technical information for the Conference consideration to balance the MSS degradation due to RDSS:Annex B studies the possibility to overcome the MSS C/No degradation by increasing its threshold level. It shows that by increasing the MSS and RDSS threshold levels (respectively -129 and -126 dBW/m²/MHz in the CPM text) respectively by 1.5 dB and 0.5 dB, the interference between the two systems stays very reasonable and acceptable values

- Conclusion

This contribution has presented the results of potential interference from RDSS systems operating in the 2 483.5-2 500 MHz band into MSS systems. Measures exist to overcome the additional degradation caused by RDSS system on MSS receiver as shown in this document.

The Conference is invited to consider this new information when deriving the PFD threshold for RDSS. However, it should be noted that more than 4 dB difference between the RDSS and the MSS coordination threshold would prevent the use of the band by RDSS systems.

ANNEX A

MSS C/No degradation due to RDSS

# 1. Mobile and Galileo Characteristics

Table 1 provides the characteristics of the service downlinks of the GLOBALSTAR system, which is currently the only MSS system using the band 2 483.5-2 500 MHz. The characteristics of this system are available from Recommendation ITU-R M.1184.

| System name | HIBLEO-4 |
| --- | --- |
| Altitude | 1 414 km |
| Inclination | 52 degrees |
| Number of orbital planes | 8 |
| Number of satellites per plane | 6 spaced every 60 degrees |
| Phasing | 7.5 degrees |
| Antenna type | Multi-beam |
| Gain | 13.5 dBi nominal |
| 3 dB beamwidth | 28 degrees |
| Polarization | Left-hand circular |
| e.i.r.p. per user | 0.3 dBW |
| Signal bandwidth | 1.23 MHz |
| Signal centre frequency | 2 491.77 MHz |
| Receiver noise temperature | 300 K |
| Peak receiver antenna gain | 2.7 dBi |
| Receiver antenna beamwidth | 126 degrees |

Table 1: Mobile service system parameters

It is proposed to retain a criteria based on an increase in noise of 6% at the GLOBALSTAR Mobile Earth Station (MES) antenna port, corresponding to an I/N ratio of -12 dB.

Globalstar operates so as its power flux density received on the ground is below the threshold given in Appendix 5 of the Radio Regulations: -126 dBW/m²/MHz. Interferences calculations in this document will be performed, taking into account this reference value.

| System name | Galileo RDSS |
| --- | --- |
| Altitude | 23 616 km |
| Inclination | 56 degrees |
| Number of orbital planes | 3 |
| Number of satellites per plane | 9 spaced every 40 degrees |
| Phasing | 13 1/3 degrees |
| Antenna type | Iso-flux |
| Peak gain | 13.3 dBi |
| 3 dB beamwidth | 40 degrees peak-to-peak |
| Polarization | Right-hand circular |
| Signal bandwidth | 16.5 MHz |
| Signal center frequency | 2 491.75 MHz |
| Proposed power flux density limit | –129 dBW/square meter * MHz |
| Implied e.i.r.p. per spacecraft | 40.5 dBW |

Table 2 : Galileo characteristics

# 2. Methodology used

The C/No degradation is calculated as the difference between the C/No of the interfered system when there is no external interference and the C/No taking into account the interfering system.

Where:

- No is the thermal noise floor (W/Hz).

- Po is the intra-system interference (W/Hz).

- Io is the external interference (W/Hz).

The external interference level is calculated as follows:

I’o(dBW/Hz)= PSD Interf  + Bw + GAnt + SSC

Where:

- PSD Interf is the power spectral density of the signal corresponding to the interfering system at the receiver antenna (dBW/MHz).

- Bw the receiver bandwidth (MHz).GAnt the receiver antenna gain. Two values have been taken into account: a maximum value of 3 dBi and a middle value of 0 dBi (dB).

- SSC is the Spectral Separation Coefficient between the interfering and the interfered signals (dB/Hz).

Simulations have been conducted in order to assess the intra-system interference of Galileo-like RNSS system and Globalstar constellations in S band. The obtained results for the RNSS system show that it is always below -222 dBW/Hz, so this value is assumed as a worst case, while for Globalstar it is below -220 dBW/Hz 95% of the time, and reaches a maximum value of -205 dBW/Hz.

# 3. Spectral Separation Coefficient

In general, Spectral Separation Coefficient between the interfering and interfered signals is computed through the formula:

Where:

- is the normalized power spectral density of the interfering signal.

- is the power spectral density of the interfered signal, normalized in the receiver bandwidth.

- BW  is the bandwidth

4. C/No degradation

In this part, the interference from a RNSS system to Globalstar has been assessed assuming a receiver bandwidth equal to one Globalstar FDMA channel width (1.23 MHz).

Calculations in the tables below are done using the methodology described in Annex A:

- Globalstar C/No degradation (3 dB antenna gain)

|  | BPSK(1) | BPSK(4) | BPSK(8) | BOC(1,1) |
| --- | --- | --- | --- | --- |
| Galileo pfd per satellite (dBW/m²/MHz) | -129 | -129 | -129 | -129 |
| Galileo PSD (dBW/MHz) | -158,39 | -158,39 | -158,39 | -158,39 |
| Gant (dB) | 3,00 | 3,00 | 3,00 | 3,00 |
| Number of satellites in view | 12,00 | 12,00 | 12,00 | 12,00 |
| BW (MHz) | 1,23 | 1,23 | 1,23 | 1,23 |
| SSC (dB/Hz) | -60,66 | -60,05 | -60,01 | -61,40 |
| Io (dBW/Hz) | -204,36 | -203,75 | -203,71 | -205,10 |
| Po (dBW/Hz) | -220,00 | -220,00 | -220,00 | -220,00 |
| No (dBW/Hz) | -203,83 | -203,83 | -203,83 | -203,83 |
| C/No deg. (dB) Po=-220 dBW/Hz (95% of time) | 2,70 | 3,00 | 3,02 | 2,38 |
| C/No deg. (dB) Po=-205 dBW/Hz (5% of time) | 1,77 | 1,98 | 1,99 | 1,53 |

Table 3: Globalstar C/No degradation due to RNSS emission

For comparison the results of the previous study:

|  | BPSK(1) | BPSK(4) | BPSK(8) | BOC(1,1) |
| --- | --- | --- | --- | --- |
| SSC (dB/Hz) | -72,80 | -77,10 | -79,90 | -77,10 |
| C/No deg. (dB) Po=-220 dBW/Hz (95% of time) | 0,22 | 0,08 | 0,04 | 0,08 |
| C/No deg. (dB) Po=-205 dBW/Hz (5% of time) | 0,13 | 0,05 | 0,03 | 0,05 |

Table 4: Globalstar C/No degradation due to RNSS emission in the previous study

- Globalstar C/No degradation (0 dB antenna gain)

|  | BPSK(1) | BPSK(4) | BPSK(8) | BOC(1,1) |
| --- | --- | --- | --- | --- |
| Galileo pfd per satellite (dBW/m²/MHz) | -129 | -129 | -129 | -129 |
| Galileo PSD (dBW/MHz) | -158,39 | -158,39 | -158,39 | -158,39 |
| Gant (dB) | 0,00 | 0,00 | 0,00 | 0,00 |
| Number of satellites in view | 12,00 | 12,00 | 12,00 | 12,00 |
| BW (MHz) | 1,23 | 1,23 | 1,23 | 1,23 |
| SSC (dB/Hz) | -60,66 | -60,05 | -60,01 | -61,40 |
| Io (dBW/Hz) | -207,36 | -206,75 | -206,71 | -208,10 |
| Po (dBW/Hz) | -220,00 | -220,00 | -220,00 | -220,00 |
| No (dBW/Hz) | -203,83 | -203,83 | -203,83 | -203,83 |
| C/No deg. (dB) Po=-220 dBW/Hz (95% of time) | 1,56 | 1,76 | 1,77 | 1,35 |
| C/No deg. (dB) Po=-205 dBW/Hz (5% of time) | 0,97 | 1,10 | 1,11 | 0,84 |

Table 5: Globalstar C/No degradation due to RNSS emission

For comparison the results of the previous study:

|  | BPSK(1) | BPSK(4) | BPSK(8) | BOC(1,1) |
| --- | --- | --- | --- | --- |
| SSC (dB/Hz) | -72,80 | -77,10 | -79,90 | -77,10 |
| C/No deg. (dB) Po=-220 dBW/Hz (95% of time) | 0,11 | 0,04 | 0,02 | 0,04 |
| C/No deg. (dB) Po=-205 dBW/Hz (5% of time) | 0,07 | 0,02 | 0,01 | 0,02 |

Table 6: Globalstar C/No degradation due to RNSS emission in the previous study

ANNEX B

Increasing MSS and RDSS threshold levels to overcome the C/No degradation

1. Methodology

The following methodology takes into account the carrier of the interfered signal.

The degradation can be minimized by increasing the Power Flux Density of the interfered signal when an interfering signal is present.

The C/No degradation is calculated as the difference between the C/No of the interfered system when there is no external interference and the C’/No (C’(dB)=C (dB)+offset (dB)) taking into account the interfering system and  the new offset for the interfered signal.

2. Increase MSS and RDSS threshold level to balance the degradation effect

The figure 1 shows the MSS degradation due to RDSS emission (antenna gain = 0dB, No=-220 dBW/Hz and worst case BPSK (8) modulation) and also the degradation can be balanced by increasing the interfered signal Power Flux Density.

Figure 1 : MSS degradation due to RDSS emission

The figure 2 shows the MSS degradation due to RDSS emission (worst case BPSK (8) modulation) and also the degradation can be balanced by increasing the interfered signal.

Figure 2 : RDSS degradation due to MSS emission