|  |  |  | Doc. ECC/CPG12(2011) 052 |
| --- | --- | --- | --- |
| CPG12 – 8 |  |  |  |
| Bucharest, 1st - 4th November 2011 |  |  |  |
| Date issued: | 24th October 2011 |  |  |
| Source: | France |  |  |
| Subject: | Change to the draft ECP on WRC12 agenda item 1.3 (subpart 3A – terrestrial component) |  |  |

N

Password protection required? (Y/N)

Summary: This document contains a modification to the draft ECP on WRC12 agenda item 1.3 (subpart 3A – terrestrial component). It is proposed to include, directly in the table of frequency allocations, the AMS(R)S allocations in the 5000-5150 MHz band previously contained in RR 5.367. This is justified since this is a worldwide allocation, whereas an "additional allocation" as defined in 5.35 addresses allocations "in an area smaller than a Region or in a particular country".

Proposal:

CPG is invited to consider the proposed modifications to EUR/5A3/2, EUR/5A3/4 and EUR/5A3/5 contained in this document.

Background: Within PTC, there was some discussion on the most appropriate way to present in the RR, the existing worldwide allocation to the AMS(R)S in the 5000-5150 MHz band, previously contained in RR 5.367, taking into account the proposals in the ECP to modify RR 5.367 and the proposed new allocation for the AM(R)S in the 5030-5091 MHz band. France is of the view that the AMS(R)S allocations in the 5000-5150 MHz band should be directly included in the table of frequency allocations, rather than within an “additional allocation” footnote. This is justified since this is a worldwide allocation, whereas an "additional allocation" as defined in 5.35 addresses allocations "in an area smaller than a Region or in a particular country".

PRELIMINARY DRAFT EUROPEAN COMMON PROPOSALS FOR THE WORK OF THE CONFERENCE

AGENDA ITEM 1.3

1.3	to consider spectrum requirements and possible regulatory actions, including allocations, in order to support the safe operation of unmanned aircraft systems (UAS), based on the results of ITU-R studies, in accordance with Resolution 421 (WRC-07).

….

# Sub-Part 3A

Terrestrial component

….

ARTICLE  5

Frequency allocations

Section IV  –  Table of Frequency Allocations
(See No. 2.1)

MOD	EUR/5A3/2

4 800-5 570 MHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 5 000-5 010 AERONAUTICAL RADIONAVIGATION AERONAUTICAL MOBILE-SATELLITE (R) ADD 5.B13 RADIONAVIGATION-SATELLITE (Earth-to-space) |  |  |
| 5 010-5 030 AERONAUTICAL RADIONAVIGATION AERONAUTICAL MOBILE-SATELLITE (R) ADD 5.B13 RADIONAVIGATION-SATELLITE (space-to-Earth) (space-to-space) 5.328B 5.443B |  |  |
| 5 030-5 091 AERONAUTICAL RADIONAVIGATION AERONAUTICAL MOBILE-SATELLITE (R) ADD 5.B13 AERONAUTICAL MOBILE (R) ADD 5. A13 5.444 |  |  |
| 5 091-5 150 AERONAUTICAL RADIONAVIGATION AERONAUTICAL MOBILE 5.444B AERONAUTICAL MOBILE-SATELLITE (R) ADD 5.B13 5.444 5.444A |  |  |

Reasons:

- New AM(R)S allocation: Sharing studies have shown that an AM(R)S allocation is feasible in the band 5030-5091 MHz, provided that power limitations are associated to these allocations.

- In addition, it is proposed to include, directly in the table of frequency allocations, the AMS(R)S allocations previously contained in RR 5.367. This is justified since this is a worldwide allocation, whereas an "additional allocation" as defined in 5.35 addresses allocations "in an area smaller than a Region or in a particular country".

ADD	EUR/5A3/3

5.A13	The use of the band 5 030-5 091 MHz by the aeronautical mobile (R) service is limited to internationally standardized aeronautical systems. In order to protect the radionavigation-satellite service (Space-to-earth) in the band 5 010-5 030 MHz, the e.i.r.p. density of any AM(R)S station shall not exceed -75 dBW/MHz in the band 5 010-5 030 MHz.

Reasons: This new footnote limits the use of the AM(R)S allocation to systems developed and standardized by ICAO to facilitate the development of such an internationl system and the coordination between ICAO systems in this band. The out-of-band e.i.r.p. density limit proposed will protect RNSS receivers in 5010-5030 MHz.

MOD	EUR/5A3/4

  - Additional allocation:  The band 1 610-1 626.5 MHz is also allocated to the aeronautical mobile-satellite (R) service on a primary basis, subject to agreement obtained under No. 9.21.

Reasons: This proposed revision to the footnote removes references to the 5 GHz allocation as this is proposed to be treated separately in the table of allocation and in the proposed new footnote 5.B13.

ADD	EUR/5A3/5

5.B13	In the bands 5 000-5 030 and 5091 - 5150 MHz, the aeronautical mobile-satellite (R) service is subject to agreement obtained under No. 9.21. In the band 5 030 - 5091 MHz, the aeronautical mobile-satellite (R) service is subject to coordination under No. 9.11A. The use of these bands is limited to internationally standardized aeronautical systems.

Reasons: This new footnote replaces the application of 9.21 for the AMS(R)S allocation in the band 5030 -5091 MHZ by the application of 9.11A, as a consequence of the new aeronautical mobile (R) allocation.

…..