Annex

PRELIMINARY DRAFT EUROPEAN COMMON PROPOSAL

Proposal submitted by the following Administrations

[…, …, …]

Agenda item 1.13 (WRC-12)

1.13 to consider the results of ITU-R studies in accordance with Resolution 551  (WRC-07) and decide on the spectrum usage of the 21.4-22 GHz band for the broadcasting-satellite service and the associated feeder-link bands in Regions 1 and 3;

Introduction

Under WRC-12 Agenda item 1.13, Europe proposes that WRC-12 adopts definitive provisions for the use of the broadcasting-satellite service (BSS) in the 21.4-22 GHz band , modifies the Articles and Appendices of the Radio Regulations accordingly and improves transparency and accuracy of the due diligence procedure.

Following the structure of the CPM report, Europe presents its proposals in three sub-parts (each of them corresponding to a separate addendum to this document):

– 	Sub-Part A – Sharing between satellite networks in the broadcasting-satellite service (BSS) based on the use of the procedures of Articles 9 and 11 together with a reduction of the size of the coordination arc to ±6° and a Resolution about information on satellites, identical to the one proposed by Europe under WRC-12 Agenda item 7,

– 	Sub-Part B – New allocation to the fixed-satellite service (Earth-to-space) in the bands 24.65-25.25 GHz in Region 1 and 24.65-24.75 GHz in Region 3, together with a minimum earth station antenna size of 3.5 m,

– 	Sub-Part C – Sharing between the broadcasting-satellite service and terrestrial services:

– 	Case of terrestrial services in Region 2: protection of the terrestrial services by including a pfd mask of -115/-105 dBW/(m².MHz) over Region 2 and protection of BSS earth stations from terrestrial services by including a pfd limit to be met at the border of Regions 1 and 3 countries,

– 	Case of terrestrial services in Regions 1 and 3: protection of terrestrial services by including a pfd mask of -115/-105 dBW/(m².MHz) over Regions 1 and 3 and protection of BSS earth stations from terrestrial services by including a pfd limit to be met at the border of Regions 1 and 3 countries. Additional provision is included to facilitate the development of the BSS.

SUB-PART A – SHARING BETWEEN BSS SATELLITE NETWORKS

Europe proposes to use the procedures of Articles 9 and 11 of the Radio Regulations  to address sharing between BSS satellite networks in the 21.4-22 GHz band.

In particular, Europe is opposed to any a priori planning of the 21.4 – 22 GHz band as it would freeze access according to technological assumptions at the time of planning and therefore prevents its flexible use, taking account of real demand and developments.

In the band 21.4-22 GHz, Europe proposes to reduce the size of the coordination arc from ±16° to ±6° for applying RR No. 9.7 (coordination between geostationary satellite networks) .

Europe also proposes a Resolution to improve the transparency and accuracy of the due diligence procedure, in a manner identical to what Europe proposes under WRC-12 Agenda item 7.

As a consequence of these proposals, Europe finally proposes to revise Resolutions 507 (Rev. WRC-03) and 526 (WARC-92) and to suppress Resolutions 525 (Rev.WRC-07) and 551 (WRC-07).

ARTICLE  9

MOD	EUR/5A13/1

Procedure for effecting coordination with or 
            obtaining agreement of other administrations1, 2, 3, 3, MOD-4, 5, 6, 7, 8

Section II  –  Procedure for effecting coordination12, 13

Sub-Section IIA  –  Requirement and request for coordination

MOD	EUR/5A13/2

4	A.9.4	Resolution 49 (Rev.WRC-12) and Resolution [BSS_21GHz_Due diligence] (WRC-12) shall also be applied with respect to those satellite networks and satellite systems that are subject to it.

-

Reasons:	Due diligence requirements for BSS satellite networks in the 21.4-22 GHz band will be provided under the proposed new Resolution.

MOD	EUR/5A13/3

9.41		Following receipt of the BR IFIC referring to requests for coordination under Nos. 9.7 to 9.7B, an administration believing that it should have been included in the request or the initiating administration believing that an administration identified under No. 9.36 in accordance with the provisions of No. 9.7 (GSO/GSO) (items 1) to 9) of the frequency band column), No. 9.7A (GSO earth station/non-GSO system) or No. 9.7B (non-GSO system/GSO earth station) of Table 5-1 of Appendix 5 should not have been included in the request, shall, within four months of the date of publication of the relevant BR IFIC, inform the initiating administration or the identified administration, as appropriate, and the Bureau, giving its technical reasons for doing so, and shall request that its name be included or that the name of the identified administration be excluded, as appropriate.

Reason:		Consequential changes to the modification of the coordination arc value for BSS satellite networks in the 21.4-22 GHz band.

ARTICLE  11

MOD 	EUR/5A13/4

Notification and recording of frequency
assignments1, MOD-2, 3, 4, 5, 6, 7

Section II  –  Examination of notices and recording of frequency assignments
in the Master Register

MOD	EUR/5A13/5

2	A.11.2	Resolution 49 (Rev.WRC-12) and Resolution [BSS_21GHz_Due diligence] (WRC-12) shall also be applied with respect to those satellite networks and satellite systems that are subject to it.

-

Reasons:	Due diligence requirements for BSS satellite networks in the 21.4-22 GHz band will be provided under the proposed new Resolution.

MOD	EUR/5A13/6

18 11.37.2	When a frequency assignment to a space station in the broadcasting-satellite service in a non-planned band other than the 21.4-22 GHz band is recorded in the Master Register, a note shall be entered in the remarks column indicating that such recording does not prejudge in any way the decisions to be included in the agreements and associated plans referred to in Resolution 507.

Reason:		As no a priori planning is foreseen for the 21.4-22 GHz band, Resolution 507 (Rev. WRC-03) should not be applicable to the 21.4-22 GHz band.

MOD	EUR/5A13/7

11.44	The notified date MOD 20 of bringing into use of any assignment to a space station of a satellite network shall be not later than seven years following the date of receipt by the Bureau of the relevant complete information under No. 9.1 or 9.2, as appropriate. Any frequency assignment not brought into use within the required period shall be cancelled by the Bureau after having informed the administration at least three months before the expiry of this period.

MOD	EUR/5A13/8

20 	11.44.1	In the case of space station frequency assignments that are brought into use prior to the completion of the coordination process, and for which the Resolution 49 (Rev.WRC-12) data or Resolution [BSS_21GHz_Due diligence] (WRC-12) data, as appropriate, have been submitted to the Bureau, the assignment shall continue to be taken into consideration for a maximum period of seven years from the date of receipt of the relevant information under No. 9.1. If the first notice for recording of the assignments in question under No. 11.15 has not been received by the Bureau by the end of this seven-year period, the assignments shall no longer be taken into account by the Bureau and administrations. The Bureau shall inform the notifying administration of its pending actions three months in advance.

In the case of satellite networks for which relevant advance publication information has been received prior to 22 November 1997, the corresponding period will be nine years from the date of publication of this information.

-

Reasons:	Due diligence requirements for BSS satellite networks in the 21.4-22 GHz band will be provided under the proposed new Resolution.

APPENDIX  5

Identification of administrations with which coordination is to be effected or 
agreement sought under the provisions of Article 9

TABLE  5-1

Technical conditions for coordination
(see Article 9)

MOD	EUR/5A13/9

| Reference of Article 9 | Case | Frequency bands (and Region) of the service for which coordination is sought | Threshold/condition | Calculation method | Remarks |
| --- | --- | --- | --- | --- | --- |
| No. 9.7 GSO/GSO | A station in a satellite network using the geostationary-satellite orbit (GSO), in any space radiocommunication service, in a frequency band and in a Region where this service is not subject to a Plan, in respect of any other satellite network using that orbit, in any space radiocommunication service in a frequency band and in a Region where this service is not subject to a Plan, with the exception of the coordination between earth stations operating in the opposite direction of transmission | 1) 3 400-4 200 MHz 5 725-5 850 MHz (Region 1) and 5 850-6 725 MHz 7 025-7 075 MHz | i) Bandwidth overlap, and ii) any network in the fixed-satellite service (FSS) and any associated space operation functions (see No. 1.23) with a space station within an orbital arc of 10° of the nominal orbital position of a proposed network in the FSS |  | With respect to the space services listed in the threshold/condition column in the bands in 1), 2), 3), 4), 5), 6), 7), 8) and 9), an administration may request, pursuant to No. 9.41, to be included in requests for coordination, indicating the networks for which the value of T/T calculated by the method in § 2.2.1.2 and 3.2 of Appendix 8 exceeds 6%. When the Bureau, on request by an affected administration, studies this information pursuant to No. 9.42, the calculation method given in § 2.2.1.2 and 3.2 of Appendix 8 shall be used |
|  |  | 2) 10.95-11.2 GHz 11.45-11.7 GHz 11.7-12.2 GHz (Region 2) 12.2-12.5 GHz (Region 3) 12.5-12.75 GHz (Regions 1 and 3) 12.7-12.75 GHz (Region 2) and 13.75-14.5 GHz | i) Bandwidth overlap, and ii) any network in the FSS or broadcasting-satellite service (BSS), not subject to a Plan, and any associated space operation functions (see No. 1.23) with a space station within an orbital arc of 9° of the nominal orbital position of a proposed network in the FSS or BSS, not subject to a Plan |  |  |

TABLE  5-1 (continued)     (WRC-07)

| Reference of Article 9 | Case | Frequency bands (and Region) of the service for which coordination is sought | Threshold/condition | Calculation method | Remarks |
| --- | --- | --- | --- | --- | --- |
| No. 9.7 GSO/GSO (cont.) |  | 3) 17.7-20.2 GHz, (Regions 2 and 3), 17.3-20.2 GHz (Region 1) and 27.5-30 GHz | i) Bandwidth overlap, and ii) any network in the FSS and any associated space operation functions (see No. 1.23) with a space station within an orbital arc of 8° of the nominal orbital position of a proposed network in the FSS |  |  |
|  |  | 4) 17.3-17.7 GHz (Regions 1 and 2) | i) Bandwidth overlap, and ii) a) any network in the FSS and any associated space operation functions (see No. 1.23) with a space station within an orbital arc of 8° of the nominal orbital position of a proposed network in the BSS, or b) any network in the BSS and any associated space operation functions (see No. 1.23) with a space station within an orbital arc of 8° of the nominal orbital position of a proposed network in the FSS |  |  |

TABLE  5-1 (continued)     (WRC-07)

| Reference of Article 9 | Case | Frequency bands (and Region) of the service for which coordination is sought | Threshold/condition | Calculation method | Remarks |
| --- | --- | --- | --- | --- | --- |
| No. 9.7 GSO/GSO (cont.) |  | 5) 17.7-17.8 GHz | i) Bandwidth overlap, and ii) a) any network in the FSS and any associated space operation functions (see No. 1.23) with a space station within an orbital arc of 8° of the nominal orbital position of a proposed network in the BSS, or b) any network in the BSS and any associated space operation functions (see No. 1.23) with a space station within an orbital arc of 8° of the nominal orbital position of a proposed network in the FSS NOTE – No. 5.517 applies in Region 2. |  |  |
|  |  | 6) 18.0-18.3 GHz (Region 2) 18.1-18.4 GHz (Regions 1 and 3) | i) Bandwidth overlap, and ii) any network in the FSS or meteorological-satellite service and any associated space operation functions (see No. 1.23) with a space station within an orbital arc of 8° of the nominal orbital position of a proposed network in the FSS or the meteorological-satellite service |  |  |

TABLE  5-1 (continued)     (WRC-07)

| Reference of Article 9 | Case | Frequency bands (and Region) of the service for which coordination is sought | Threshold/condition | Calculation method | Remarks |
| --- | --- | --- | --- | --- | --- |
| No. 9.7 GSO/GSO (cont.) |  | 7) Bands above 17.3 GHz, except those defined in § 3) and 6) | i) Bandwidth overlap, and ii) any network in the FSS and any associated space operation functions (see No. 1.23) with a space station within an orbital arc of 8° of the nominal orbital position of a proposed network in the FSS (see also Resolution 901 (Rev.WRC-07)) |  |  |
|  |  | 8) 21.4-22 GHz (Regions 1 and 3) | i) Bandwidth overlap, and ii) any network in the BSS and any associated space operation functions (see No. 1.23) with a space station within an orbital arc of 6° of the nominal orbital position of a proposed network in the BSS |  |  |
|  |  | 9) Bands above 17.3 GHz except those defined in § 4), 5) and 8) | i) Bandwidth overlap, and ii) any network in the FSS or BSS, not subject to a Plan, and any associated space operation functions (see No. 1.23) with a space station within an orbital arc of 16° of the nominal orbital position of a proposed network in the FSS or BSS, not subject to a Plan, except in the case of a network in the FSS with respect to a network in the FSS (see also Resolution 901 (Rev.WRC-07)) |  |  |

TABLE  5-1 (continued)     (WRC-07)

| Reference of Article 9 | Case | Frequency bands (and Region) of the service for which coordination is sought | Threshold/condition | Calculation method | Remarks |
| --- | --- | --- | --- | --- | --- |
| No. 9.7 GSO/GSO (cont.) |  | 10) All frequency bands, other than those in 1), 2), 3), 4), 5), 6), 7), 8) and 9), allocated to a space service, and the bands in 1), 2), 3), 4), 5), 6), 7), 8) and 9) where the radio service of the proposed network or affected networks is other than the space services listed in the threshold/ condition column, or in the case of coordination of space stations operating in the opposite direction of transmission | i) Bandwidth overlap, and ii) Value of T/T exceeds 6% | Appendix 8 | In application of Article 2A of Appendix 30 for the space operation functions using the guardbands defined in § 3.9 of Annex 5 of Appendix 30, the threshold/condition specified for the FSS in the bands in 2) applies. In application of Article 2A of Appendix 30A for the space operation functions using the guardbands defined in § 3.1 and 4.1 of Annex 3 of Appendix 30A, the threshold/condition specified for the FSS in the bands in 7) applies |

Reasons: –	To introduce with § 8) a coordination arc value of ±6° in the band 21.4-22 GHz to trigger coordination between BSS satellite networks serving Regions 1 and 3.

MOD	EUR/5A13/10

RESOLUTION  49  (Rev.WRC-12)

Administrative due diligence applicable to some satellite
radiocommunication services

resolves

1	that the administrative due diligence procedure contained in Annex 1 to this Resolution shall be applied as from 22 November 1997 for a satellite network or satellite system of the fixed-satellite service, mobile-satellite service or broadcasting-satellite service, except the broadcasting-satellite service in the band 21.4-22.0 GHz, for which the advance publication information under No. 9.2B, or for which the request for modifications of the Region 2 Plan under Article 4, § 4.2.1 b) of Appendices 30 and 30A that involve the addition of new frequencies or orbit positions, or for which the request for modifications of the Region 2 Plan under Article 4, § 4.2.1 a) of Appendices 30 and 30A that extend the service area to another country or countries in addition to the existing service area, or for which the request for additional uses in Regions 1 and 3 under § 4.1 of Article 4 of Appendices 30 and 30A, or for which the submission of information under supplementary provisions applicable to additional uses in the planned bands as  defined in Article 2 of Appendix 30B (Section III of Article 6) has been received by the Bureau  from 22 November 1997, or for which submission under Article 6 of Appendix 30B (Rev.WRC-07) is received on or after 17 November 2007, with the exception of submissions of new Member States seeking the acquisition of their respective national allotments for inclusion in the Appendix 30B Plan;

See § 2.3 of Appendix 30B (Rev.WRC-07).

ANNEX 1 TO RESOLUTION 49 (Rev.WRC-12)

1	Any satellite network or satellite system of the fixed-satellite service, mobile-satellite service or broadcasting-satellite service with frequency assignments that are subject to coordination under Nos. 9.7, 9.11, 9.12, 9.12A and 9.13 and Resolution 33 (Rev.WRC-03), with the exception of broadcasting-satellite service submissions in the band 21.4-22.0 GHz, shall be subject to these procedures.

Reason:		Due diligence requirements for BSS satellite networks in the 21.4-22 GHz band will be provided under the proposed new resolution (i.e. Resolution [BSS_21GHz_DUE DILIGENCE] (WRC-12). Consequently, Resolution 49 (Rev.WRC-12) will no longer be applicable for BSS satellite networks in this band.

MOD	EUR/5A13/11

RESOLUTION 507 (Rev. WRC-03)

Establishment of agreements and associated plans
for the broadcasting-satellite service ADD 1

ADD	EUR/5A13/12

1	This Resolution does not apply to the 21.4-22 GHz band.

Reasons:	As no a priori planning is foreseen for the 21.4-22 GHz band, Resolution 507 (Rev. WRC-03) should not be applicable to the 21.4-22 GHz band.

SUP	EUR/5A13/13

RESOLUTION 525 (Rev. WRC-07)

Introduction of high-definition television systems
of the broadcasting-satellite service in the
band 21.4-22.0 GHz in Regions 1 and 3

Reasons:	This Resolution is not needed any longer.

MOD	EUR/5A13/14

RESOLUTION 526 (Rev. WRC-12)

Future adoption of procedures to ensure flexibility in the use of the frequency band allocated to the broadcasting-satellite service (BSS) for wide RF-band high-definition television (HDTV) and to the associated feeder links1

The World Radiocommunication Conference (Geneva, 2012),

considering

a)	that WARC-92 has added an allocation to the BSS in the band 17.3-17.8 GHz in Region 2 for use by wide RF-band HDTV;

b)	that in the longer term regulatory provisions designed to ensure flexible and equitable use of the BSS (HDTV) and associated feeder-link allocations will be necessary,

resolves to urge all administrations

to study the development of future regulatory provisions for BSS (HDTV) to ensure flexibility in the use of the band 17.3-17.8 GHz in Region 2, having regard to the interests of all countries and the state of technical development of this new service,

instructs the Secretary-General

to bring this Resolution to the attention of the Council with a view to placing an appropriate item on the agenda of a future world radiocommunication conference.

Reasons:	Studies are completed with regard to the band 21.4-22 GHz. During WRC-12, Region 2 countries may decide to retain or suppress the part of Resolution 526 concerning the band 17.3-17.8 GHz.

SUP	EUR/5A13/15

RESOLUTION  551  (WRC-07)

Use of the band 21.4-22 GHz for broadcasting-satellite service
and associated feeder-link bands in Regions 1 and 3

Reasons:	Studies under WRC-12 Agenda item 1.13 are completed.

ADD	EUR/5A13/16

DRAFT RESOLUTION [BSS_21GHZ_DUE DILIGENCE] (WRC-12)

Long term access to and development in the band 21.4-22.0 GHz
in Regions 1 and 3

-

The World Radiocommunication Conference (Geneva, 2012),

considering

a)	that Article 44 of the ITU Constitution sets out the basic principles for the use of the radio-frequency spectrum and the geostationary-satellite and other satellite orbits, taking into account the needs of developing countries;

b)	that a due diligence process was first adopted by WRC-97 with a view to providing as early as possible information on the industrial project behind a satellite network submitted to ITU;

c)	that providing information required under this due diligence process was a prerequisite to qualifying for a two-year extension of the regulatory period to bring into use a satellite network in non-planned bands;

d)	that WRC-03 decided to remove the two-year extension by setting the regulatory period to bring into use a satellite network in non-planned bands to seven years;

e)	that data concerning the manufacturer, launch service provider and launch date of a satellite will be more accurate and useful if submitted after the launch of the satellite,

resolves

1	that this Resolution applies to geostationary satellite networks in the broadcasting-satellite service in the 21.4-22 GHz band;

2	that for frequency assignments to such satellite networks for which the confirmation of the date of bringing into use was not received by the Bureau before 17 February 2012 or which were suspended at that date, the procedures contained in Annex 1 to this Resolution shall be applied:

i) 	at the time of first bringing into use; or

ii)	when resuming use after a suspension;

3	that for frequency assignments to satellite networks as described in resolves 1 for which the confirmation of the date of bringing into use was received by the Bureau before 17 February 2012, the notifying administration shall submit to the Bureau not later than 17 August 2012 the complete information related to the operational situation as of 17 February 2012, in accordance with Annex 2 to this Resolution;

4	that on receipt of the information under resolves 3 above, the Bureau shall promptly examine its completeness. If the information is found to be complete, the Bureau shall publish the complete information in a special section of the BR IFIC within two months. If the information is found to be incomplete, the Bureau shall request the notifying administration to submit the missing information within thirty days;

5	that if the complete information specified in resolves 3 is not received by the Bureau in accordance with resolves 3 and 4, the corresponding frequency assignments shall be cancelled by the Bureau;

6	that for frequency assignments to satellite networks as described in resolves 3 above, the provisions of §§ 4 to 8 of Annex 1 to this Resolution shall also apply,

instructs the Director of the Radiocommunication Bureau

to report to future competent world radiocommunication conferences on the results of the implementation of this Resolution.

Annex 1 to draft Resolution [BSS_21GHZ_DUE DILIGENCE] (WRC-12)

1	Any frequency assignments to satellite networks referred to in resolves 1 shall be subject to these procedures either when their first bringing into use is declared or at the resumption of use following a suspension.

2	Within thirty days after the actual commencement, or resumption, of use of the frequency assignments to a satellite network subject to these procedures, the notifying administration shall send to the Bureau the information specified in Annex 2 to this Resolution.

3	On receipt of the information under § 2 above, the Bureau shall promptly examine its completeness. If the information is found to be complete, the Bureau shall publish the complete information in a special section of the BR IFIC within two months. If the information is found to be incomplete, the Bureau shall request the notifying administration to submit the missing information within thirty days.

4	The information submitted in accordance with § 2 and resolves 3 above shall be updated and resubmitted to the Bureau by the notifying administration not later than thirty days after the end of life or the relocation of the spacecraft associated with the submission under § 2 or resolves 3 above.

5	In case of end of life of a spacecraft already submitted under this Resolution, the notifying administration shall inform accordingly the Bureau within thirty days after the event. The corresponding ITU number associated to such a spacecraft shall no longer be used.

6	On receipt of the information under § 4 above, the Bureau shall promptly examine its completeness. If the information is found to be complete, the Bureau shall publish the complete information in a special section of the BR IFIC within two months. If the information is found to be incomplete, the Bureau shall request the notifying administration to submit the missing information within thirty days.

7	If the complete information specified in § 4 above is not received by the Bureau within the time-limits specified in §§ 4 and 6 above, the frequency assignments to the satellite network shall be considered as invalid. The Bureau shall immediately inform the notifying administration and take appropriate measures under § 8, if required.

8	After the end of the periods referred to in No. 11.44 and in No. 11.49, if the complete information under this Resolution is not yet received by the Bureau, the corresponding frequency assignments shall be cancelled by the Bureau.

Annex 2 to draft Resolution [BSS_21GHZ_DUE DILIGENCE] (WRC-12)

Information to be submitted

1)	Identity of the satellite network

a)	Identity of the satellite network

b)	Name of the notifying administration

c)	Orbital characteristics

d)	Reference to the advance publication information

e)	Reference to the request for coordination

f)	Reference to the notification, when available

g)	Frequency band(s) included in the relevant special sections of the satellite network

h)	Regulatory status

–	Satellite network under operation (only data listed in § 2 shall be provided), or

–	Satellite network suspended (only data listed in § 3 shall be provided)

2)	Identity of the spacecraft2, 3 (if satellite network filing is under operation)

a)	ITU ID number, or

a)	Spacecraft manufacturer

–	Name of the spacecraft manufacturer

b)	Launch services provider

–	Name of the launch vehicle provider

–	Name of the launch vehicle

–	Name and location of the launch facility

–	Launch date

c)	Frequency band(s) present on board the spacecraft (i.e. frequency bands that are able to be received or transmitted by a transponder located on-board the spacecraft)

3)	Suspension information (if satellite network filing is suspended)

a)	Date of suspension

b)	Reason of suspension:

–	Spacecraft moved to another orbital position, or

–	In-orbit failure of the spacecraft, or

–	Spacecraft de-orbited.

Reasons: 	to improve the transparency and accuracy of the due diligence procedure, in a manner identical to what Europe proposes under WRC-12 Agenda item 7.

SUB-PART B – NEW ALLOCATIONS TO 
THE FIXED-SATELLITE SERVICE (EARTH-TO-SPACE)

In order to maintain flexibility in the design of satellite systems, Europe considers that there should be no limitation on the FSS Earth-to-space bands which may be used for the feeder-links of the BSS in the 21.4-22 GHz band.

However, to ensure a balance between uplink and downlink spectrum within the 30/20 GHz range in Regions 1 and 3, Europe proposes to allocate to the fixed-satellite service (Earth-to-space) the bands 24.65-25.25 GHz in Region 1 and 24.65-24.75 GHz in Region 3. Europe also proposes to adopt a minimum earth station antenna size of 3.5 m to facilitate sharing with the terrestrial services in these bands. Europe is also of the view that there is no need to change RR Table 21-1.

ARTICLE  5

Frequency allocations

Section IV – Table of Frequency Allocations
(See No. 2.1)

MOD	EUR/5A13/17

22-24.75 GHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 24.65-24.75 FIXED FIXED-SATELLITE (Earth-to-space) ADD5.A113 INTER-SATELLITE | 24.65-24.75 INTER-SATELLITE RADIOLOCATION- SATELLITE (Earth-to-space) | 24.65-24.75 FIXED FIXED-SATELLITE (Earth-to-space) ADD5.A113 INTER-SATELLITE MOBILE |
|  |  | 5.533 |

Reasons:	To ensure a balance between uplink and downlink spectrum within the 30/20 GHz range in Regions 1 and 3.

MOD	EUR/5A13/18

24.75-29.9 GHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 24.75-25.25 FIXED FIXED-SATELLITE (Earth-to-space) ADD5.A113 | 24.75-25.25 FIXED-SATELLITE (Earth-to-space) 5.535 | 24.75-25.25 FIXED FIXED-SATELLITE (Earth-to-space) 5.535 MOBILE |

Reasons:	To ensure a balance between uplink and downlink spectrum within the 30/20 GHz range in Regions 1 and 3.

ADD	EUR/5A13/19

5.A113	In the bands 24.65-25.25 GHz in Region 1 and 24.65-24.75 GHz in Region 3, an earth station in the fixed-satellite service (Earth-to-space) shall have a minimum antenna diameter of 3.5 m

Reasons:	To facilitate sharing of the fixed-satellite service (Earth-to-space) with the terrestrial services in these bands by de facto limiting the number of deployed earth stations

NOC	EUR/5A13/20

TABLE 21-1

APPENDIX  4  (Rev.WRC-07)

Consolidated list and tables of characteristics for use in the
application of the procedures of Chapter III

MOD	EUR/5A13/21

| Items in Appendix | A _ GENERAL CHARACTERISTICS OF THE SATELLITE NETWORK, EARTH STATION OR RADIO ASTRONOMY STATION | Advance publication of a geostationary-satellite network | Advance publication of a non-geostationary-satellite network subject to coordination under Section II of Article 9 | Advance publication of a non-geostationary-satellite network not subject to coordination under Section II of Article 9 | Notification or coordination of a geostationary-satellite network (including space operation functions under Article 2A of Appendices 30 or 30A) | Notification or coordination of a non-geostationary-satellite network | Notification or coordination of an earth station (including notification under Appendices 30A or 30B) | Notice for a satellite network in the broadcasting-satellite service under Appendix 30 (Articles 4 and 5) | Notice for a satellite network (feeder-link) under Appendix 30A (Articles 4 and 5) | Notice for a satellite network in the fixed-satellite service under Appendix 30B (Articles 6 and 8) | Items in Appendix | Radio astronomy |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A.7.f | the antenna diameter, in metres |  |  |  |  |  | + 1 |  |  |  | A.7.f |  |
|  | Required only for fixed-satellite service earth stations operating in the frequency bands 13.75-14 GHz, 24.65-25.25 GHz (Region 1) and 24.65-24.75 GHz (Region 3) |  |  |  |  |  |  |  |  |  |  |  |
| … |  |  |  |  |  |  |  |  |  |  |  |  |
| C.10.d.7 |  | the antenna diameter, in metres |  |  |  |  |  |  |  |  | + |  | + |  |  |  |  |  | X |  |  |  | C.10.d.7 |  |  |
|  | In cases other than Appendix 30A, required for fixed-satellite service networks operating in the frequency bands 13.75-14 GHz, 24.65-25.25 GHz (Region 1) and 24.65-24.75 GHz (Region 3) and for maritime mobile-satellite service networks operating in the frequency band 14-14.5 GHz |  |  |  |  |  |  |  |  |  |  |  |

APPENDIX  7

Methods for the determination of the coordination area around an earth
station in frequency bands between 100 MHz and 105 GHz

MOD	EUR/5A13/22

TABLE  7c

Parameters required for the determination of coordination distance for a transmitting earth station

| Transmitting space radiocommunication service designation |  | Fixed- satellite | … |
| --- | --- | --- | --- |
| Frequency bands (GHz) |  | 24.65-25.25 27.0-29.5 |  |
| Receiving terrestrial service designations |  | Fixed, mobile |  |
| Method to be used |  | § 2.1 |  |
| Modulation at terrestrial station 1 |  | N |  |
| Terrestrial station interference parameters and criteria | p0 (%) | 0.005 |  |
|  | n | 1 |  |
|  | p (%) | 0.005 |  |
|  | NL (dB) | 0 |  |
|  | Ms (dB) | 25 |  |
|  | W (dB) | 0 |  |
| Terrestrial station parameters | Gx (dBi) 4 | 50 |  |
|  | Te (K) | 2 000 |  |
| Reference bandwidth | B (Hz) | 106 |  |
| Permissible interference power | Pr( p) (dBW) in B | –111 |  |

Reasons:	To compute the coordination distances around an FSS earth station in the band 24.65 – 24.75 GHz in Regions 1 and 3 (the case of the band 24.75 – 25.25 GHz was already addressed because of the existing allocation to the fixed-satellite service in Regions 2 and 3).

SUB-PART C – SHARING BETWEEN THE BROADCASTING-SATELLITE SERVICE AND TERRESTRIAL SERVICES

Sharing between Regions 1 and 3 BSS and Region 2 terrestrial services

Europe is of the view that terrestrial services in Region 2 and the broadcasting-satellite service in Regions 1 and 3 should enjoy the same primary status. Therefore, Europe proposes to include pfd limits to ensure mutual-coexistence of these services.

Protection of the Region 2 terrestrial services from the Regions 1 and 3 BSS is proposed to be ensured by the following pfd limits in Table 21-4 of Article 21 of the Radio Regulations:

−115 dB(W/(m2 · MHz)) for 0° ≤ θ ≤ 5°

−115 + 0.5(θ − 5) dB(W/(m2 · MHz)) for 5° ≤ θ ≤ 25°

−105 dB(W/(m2 · MHz)) for θ > 25°,

where θ is the angle of arrival above the horizontal plane.

These values were initially contained inResolution 525 (Rev. WRC-03).

Protection of the Regions 1 and 3 BSS Earth station from Region 2 terrestrial services is proposed to be ensured through a pfd limit of – 120.4 dBW/(m².MHz) to be met at the border of Regions 1 and 3 countries (this value is based on a minimum elevation angle of 10°).

Sharing between Regions 1 and 3 BSS and Regions 1 & 3 terrestrial services

Europe is of the view that fixed services and the broadcasting-satellite service in Regions 1 & 3 should co-exist under specific conditions on the same territory. Therefore Europe proposes that administrations in Regions 1 and 3 should not deploy stations in the mobile service and should limit the deployment of stations in the fixed service to point-to-point links.

Protection of the Regions 1 and 3 terrestrial services from the Regions 1 and 3 BSS is proposed to be ensured by the following pfd limits in Table 21-4 of Article 21 of the Radio Regulations:

-115 dB(W/(m2 · MHz)) for 0° ≤ θ ≤ 5°

-115 + 0.5(θ − 5) dB(W/(m2 · MHz)) for 5° ≤ θ ≤ 25°

-105 dB(W/(m2 · MHz)) for θ > 25°,

where θ is the angle of arrival above the horizontal plane.

Europe proposes that the protection of BSS earth stations from terrestrial services be ensured through a pfd limit of    – 120.4 dBW/(m².MHz) to be met at the border of Regions 1 and 3 countries (this value is based on a minimum elevation angle of 10°).

ARTICLE  5

Frequency allocations

Section IV – Table of Frequency Allocations
(See No. 2.1)

MOD	EUR/5A13/23

18.4-22 GHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 21.4-22 FIXED MOBILE BROADCASTING-SATELLITE 5.208B MOD 5.530 ADD 5.B113 | 21.4-22 FIXED MOBILE ADD 5.530 | 21.4-22 FIXED MOBILE BROADCASTING-SATELLITE 5.208B MOD 5.530 5.531 ADD 5.B113 |

MOD 	EUR/5A13/24

5.530	Any station in the fixed or mobile services of one administration shall not produce a power flux-density in excess of –120.4 dB(W/(m².MHz)) at 3 m above the ground of any point of the territory of any other administration in Regions 1 and 3 for more than 20% of the time (calculated in accordance with the latest version of Recommendation ITU-R P.452), unless otherwise agreed between the concerned administrations.

Reasons:	To ensure that BSS receivers will be adequately protected from the terrestrial services.

ADD 	EUR/5A13/25

5.B113		In the band 21.4-22 GH, in order to facilitate the development of the broadcasting-satellite service, administrations in Regions 1 and 3 are encouraged not to deploy stations in the mobile service and are encouraged to limit the deployment of stations in the fixed service to point-to-point links.

Reasons:	To ensure protection of BSS earth stations from terrestrial services, as well as facilitating the development of the BSS in the 21.4-22 GHz band.

ARTICLE  21

Terrestrial and space services sharing frequency bands above 1 GHz

Section I  -  Choice of sites and frequencies

MOD	EUR/5A13/26

21.2 	§ 2	 1) As far as practicable, sites for transmittingMOD1, 3 stations, in the fixed or mobile service, employing maximum values of equivalent isotropically radiated power (e.i.r.p.) exceeding the values given in Table 21-1 in the frequency bands indicated, should be selected so that the direction of maximum radiation of any antenna will be separated from the geostationary satellite orbit by at least the angle in degrees shown in the Table, taking into account the effect of atmospheric refraction2:

MOD	EUR/5A13/27

1 21.2.1 For their own protection receiving stations in the fixed or mobile service operating in bands shared with space radiocommunication services (space-to-Earth) should also avoid directing their antennas towards the geostationary-satellite orbit if their sensitivity is sufficiently high that interference from space station transmissions may be significant. Especially, in the band 21.4-22.0 GHz, it is recommended a minimum separation angle with respect to geostationary-satellite orbit of 1.5°

Section V  –  Limits of power flux-density from space stations

MOD	EUR/5A13/28

TABLE  21-4

| Frequency band | Service* | Limit in dB(W/m2) for angles of arrival () above the horizontal plane |  |  | Reference bandwidth |
| --- | --- | --- | --- | --- | --- |
|  |  | 0-5 | 5-25 | 25-90 |  |
| 19.3-19.7 GHz 21.4-22 GHz 22.55-23.55 GHz 24.45-24.75 GHz 25.25-27.5 GHz 27.500-27.501 GHz | Fixed-satellite (space-to-Earth) Broadcasting-satellite Earth exploration-satellite (space-to-Earth) Inter-satellite Space research (space-to-Earth) | –115 13A | –115 + 0.5( – 5) 13A | –105 13A | 1 MHz |

Reason: 	To introduce pfd limits in Article 21 to protect terrestrial services in frequency band 21.4-22.0 GHz.

| - - |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |

1	WRC-97 made editorial amendments to this Resolution.

2	If data about the spacecraft are submitted for the first time under this Resolution, items “Spacecraft manufacturer”, “Launch services provider” and “Frequency band(s) present on-board on the spacecraft” shall be provided. Otherwise, if data about the spacecraft were already submitted under this Resolution, the ID number given by the Bureau to this spacecraft at that time shall be indicated.

3	More than one spacecraft may be associated with a satellite network at the same orbital location and one spacecraft may be associated with more than one satellite network at the same orbital location.