Doc. ECC/CPG12(2011) 036 Annex 7

DRAFT EUROPEAN COMMON PROPOSALS
FOR WRC-12 WORK

Agenda Item 1.14 (Option 2)

1.14	to consider requirements for new applications in the radiolocation service and review allocations or regulatory provisions for implementation of the radiolocation service in the range 30-300 MHz, in accordance with Resolution 611 (WRC-07);

Introduction

Agenda item 1.14 invites to consider requirements for new applications in the radiolocation service and review allocations or regulatory provisions for implementation of the radiolocation service in the range 30-300 MHz.

Europe does not see a need for a global allocation in the Table of article 5 under this agenda item but could accept an additional allocation to the radiolocation service on a primary basis, through a footnote of Article 5,  for specific countries expressing this need at WRC-12.

Europe is of the opinion that, since the results of ITU-R studies show that compatibility with other services is feasible, such primary allocation of the frequency band 154-156 MHz to radiolocation service in some countries should be limited to systems with characteristics specified in Recommendation ITU-R М.1802 and under the following conditions:

- The usage of the frequency band 154-156 MHz by the radiolocation service shall be limited by systems based on space-object detection applications operating from terrestrial locations.

- The operation of stations in the radiolocation service in the band 154-156 MHz shall be subject to agreement obtained under No. 9.21 with administrations whose services, operating in accordance with the Table of Frequency Allocations, may be affected. For the identification of potentially affected administrations the field-strength value of 12 dB(μV/m) for 10% of time produced at 10 m above ground level in the 25 kHz reference frequency band at the border of the territory of any other administration shall be used.

- For providing protection of maritime mobile service safety systems operating in the frequency bands 156.7625 – 156. 8375 MHz, 156.5125 – 156.5375 MHz, 161.9625 – 161.9875 MHz, 162.0125 – 162.0375 MHz out-of-band EIRP of space surveillance radars in the specified frequency bands shall not exceed -16 dBW.

However, Europe proposes the suppression of Resolution 611:

SUP 	EUR/1.14/2

RESOLUTION 611 (WRC-07)

Use of a portion of the VHF band by the radiolocation service