| World Radiocommunication Conference (WRC-12) Geneva, 23 January - 17 February 2012 |  |
| --- | --- |
| PLENARY MEETING | Addendum 2 to Document 5(Add 13)-E |
|  | 17 November2011 |
|  | Original: English |
| EUROPEAN COMMON PROPOSALS FOR THE WORK OF THE CONFERENCE |  |
| part 13 Addendum 2 |  |
| Agenda item 1.13 Subpart B New allocations to the Fixed-Satellite Services (Earth-to-Space) |  |

Introduction

In order to maintain flexibility in the design of satellite systems, Europe considers that there should be no limitation on the FSS Earth-to-space bands which may be used for the feeder-links of the BSS in the 21.4-22 GHz band.

However, to ensure a balance between uplink and downlink spectrum within the 30/20 GHz range in Regions 1 and 3, Europe proposes to allocate to the fixed-satellite service (Earth-to-space) the bands 24.65-25.25 GHz in Region 1 and 24.65-24.75 GHz in Region 3. Europe also proposes to adopt a minimum earth station antenna size of 3.5 m to facilitate sharing with the terrestrial services in these bands. Europe is also of the view that there is no need to change RR Table 21-1.

ARTICLE  5

Frequency allocations

Section IV – Table of Frequency Allocations
(See No. 2.1)

MOD	EUR/5A13/17

22-24.75 GHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 24.65-24.75 FIXED FIXED-SATELLITE (Earth-to-space) ADD 5.A113 INTER-SATELLITE | 24.65-24.75 INTER-SATELLITE RADIOLOCATION- SATELLITE (Earth-to-space) | 24.65-24.75 FIXED FIXED-SATELLITE (Earth-to-space) ADD 5.A113 INTER-SATELLITE MOBILE |
|  |  | 5.533 |

Reasons:	To ensure a balance between uplink and downlink spectrum within the 30/20 GHz range in Regions 1 and 3.

MOD	EUR/5A13/18

24.75-29.9 GHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 24.75-25.25 FIXED FIXED-SATELLITE (Earth-to-space) ADD 5.A113 | 24.75-25.25 FIXED-SATELLITE (Earth-to-space) 5.535 | 24.75-25.25 FIXED FIXED-SATELLITE (Earth-to-space) 5.535 MOBILE |

Reasons:	To ensure a balance between uplink and downlink spectrum within the 30/20 GHz range in Regions 1 and 3.

ADD	EUR/5A13/19

5.A113	In the bands 24.65-25.25 GHz in Region 1 and 24.65-24.75 GHz in Region 3, an earth station in the fixed-satellite service (Earth-to-space) shall have a minimum antenna diameter of 3.5 m

Reasons:	To facilitate sharing of the fixed-satellite service (Earth-to-space) with the terrestrial services in these bands by de facto limiting the number of deployed earth stations

ARTICLE  21

Terrestrial and space services sharing frequency bands above 1 GHz

Section I  –  Choice of sites and frequencies

NOC	EUR/5A13/20

TABLE 21-1

APPENDIX  4  (Rev.WRC-07)

Consolidated list and tables of characteristics for use in the
application of the procedures of Chapter III

MOD	EUR/5A13/21

| Items in Appendix | A _ GENERAL CHARACTERISTICS OF THE SATELLITE NETWORK, EARTH STATION OR RADIO ASTRONOMY STATION | Advance publication of a geostationary-satellite network | Advance publication of a non-geostationary-satellite network subject to coordination under Section II of Article 9 | Advance publication of a non-geostationary-satellite network not subject to coordination under Section II of Article 9 | Notification or coordination of a geostationary-satellite network (including space operation functions under Article 2A of Appendices 30 or 30A) | Notification or coordination of a non-geostationary-satellite network | Notification or coordination of an earth station (including notification under Appendices 30A or 30B) | Notice for a satellite network in the broadcasting-satellite service under Appendix 30 (Articles 4 and 5) | Notice for a satellite network (feeder-link) under Appendix 30A (Articles 4 and 5) | Notice for a satellite network in the fixed-satellite service under Appendix 30B (Articles 6 and 8) | Items in Appendix | Radio astronomy |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A.7.f | the antenna diameter, in metres |  |  |  |  |  | + 1 |  |  |  | A.7.f |  |
|  | Required only for fixed-satellite service earth stations operating in the frequency bands 13.75-14 GHz, 24.65-25.25 GHz (Region 1) and 24.65-24.75 GHz (Region 3) |  |  |  |  |  |  |  |  |  |  |  |
| … |  |  |  |  |  |  |  |  |  |  |  |  |
| C.10.d.7 |  | the antenna diameter, in metres |  |  |  |  |  |  |  |  | + |  | + |  |  |  |  |  | X |  |  |  | C.10.d.7 |  |  |
|  | In cases other than Appendix 30A, required for fixed-satellite service networks operating in the frequency bands 13.75-14 GHz, 24.65-25.25 GHz (Region 1) and 24.65-24.75 GHz (Region 3) and for maritime mobile-satellite service networks operating in the frequency band 14-14.5 GHz |  |  |  |  |  |  |  |  |  |  |  |

APPENDIX  7  (Rev.WRC-07)

Methods for the determination of the coordination area around an earth
station in frequency bands between 100 MHz and 105 GHz

MOD	EUR/5A13/22

TABLE  7c

Parameters required for the determination of coordination distance for a transmitting earth station

| Transmitting space radiocommunication service designation |  | Fixed- satellite | … |
| --- | --- | --- | --- |
| Frequency bands (GHz) |  | 24.65-25.25 27.0-29.5 |  |
| Receiving terrestrial service designations |  | Fixed, mobile |  |
| Method to be used |  | § 2.1 |  |
| Modulation at terrestrial station 1 |  | N |  |
| Terrestrial station interference parameters and criteria | p0 (%) | 0.005 |  |
|  | n | 1 |  |
|  | p (%) | 0.005 |  |
|  | NL (dB) | 0 |  |
|  | Ms (dB) | 25 |  |
|  | W (dB) | 0 |  |
| Terrestrial station parameters | Gx (dBi) 4 | 50 |  |
|  | Te (K) | 2 000 |  |
| Reference bandwidth | B (Hz) | 106 |  |
| Permissible interference power | Pr( p) (dBW) in B | –111 |  |

Reasons:	To compute the coordination distances around an FSS earth station in the band 24.65 – 24.75 GHz in Regions 1 and 3 (the case of the band 24.75 – 25.25 GHz was already addressed because of the existing allocation to the fixed-satellite service in Regions 2 and 3).

______________