| World Radiocommunication Conference (WRC-12) Geneva, 23 January - 17 February 2012 |  |
| --- | --- |
| PLENARY MEETING | Addendum 3 to Document 5(Add 13)-E |
|  | 17 November 2011 |
|  | Original: English |
| EUROPEAN COMMON PROPOSALS FOR THE WORK OF THE CONFERENCE |  |
| part 13 Addendum 3 |  |
| Agenda item 1.13 Subpart C Sharing between the Broadcasting Satellite Service and Terrestrial services |  |

Introduction

Sharing between Regions 1 and 3 BSS and Region 2 terrestrial services

Europe is of the view that terrestrial services in Region 2 and the broadcasting-satellite service in Regions 1 and 3 should enjoy the same primary status. Therefore, Europe proposes to include pfd limits to ensure mutual-coexistence of these services.

Protection of the Region 2 terrestrial services from the Regions 1 and 3 BSS is proposed to be ensured by the following pfd limits in Table 21-4 of Article 21 of the Radio Regulations:

−115 dB(W/(m2 · MHz)) for 0° ≤ θ ≤ 5°

−115 + 0.5(θ − 5) dB(W/(m2 · MHz)) for 5° ≤ θ ≤ 25°

−105 dB(W/(m2 · MHz)) for θ > 25°,

where θ is the angle of arrival above the horizontal plane.

These values were initially contained inResolution 525 (Rev. WRC-03).

Protection of the Regions 1 and 3 BSS Earth station from Region 2 terrestrial services is proposed to be ensured through a pfd limit of – 120.4 dBW/(m².MHz) to be met at the border of Regions 1 and 3 countries (this value is based on a minimum elevation angle of 10°).

Sharing between Regions 1 and 3 BSS and Regions 1 & 3 terrestrial services

Europe is of the view that fixed services and the broadcasting-satellite service in Regions 1 & 3 should co-exist under specific conditions on the same territory. Therefore Europe proposes that administrations in Regions 1 and 3 should not deploy stations in the mobile service and should limit the deployment of stations in the fixed service to point-to-point links.

Protection of the Regions 1 and 3 terrestrial services from the Regions 1 and 3 BSS is proposed to be ensured by the following pfd limits in Table 21-4 of Article 21 of the Radio Regulations:

-115 dB(W/(m2 · MHz)) for 0° ≤ θ ≤ 5°

-115 + 0.5(θ − 5) dB(W/(m2 · MHz)) for 5° ≤ θ ≤ 25°

-105 dB(W/(m2 · MHz)) for θ > 25°,

where θ is the angle of arrival above the horizontal plane.

Europe proposes that the protection of BSS earth stations from terrestrial services be ensured through a pfd limit of    – 120.4 dBW/(m².MHz) to be met at the border of Regions 1 and 3 countries (this value is based on a minimum elevation angle of 10°).

ARTICLE  5

Frequency allocations

Section IV – Table of Frequency Allocations
(See No. 2.1)

MOD	EUR/5A13/23

18.4-22 GHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 21.4-22 FIXED MOBILE BROADCASTING-SATELLITE 5.208B MOD 5.530 ADD 5.B113 | 21.4-22 FIXED MOBILE ADD 5.530 | 21.4-22 FIXED MOBILE BROADCASTING-SATELLITE 5.208B MOD 5.530 5.531 ADD 5.B113 |

MOD 	EUR/5A13/24

5.530	Any station in the fixed or mobile services of one administration shall not produce a power flux-density in excess of –120.4 dB(W/(m².MHz)) at 3 m above the ground of any point of the territory of any other administration in Regions 1 and 3 for more than 20% of the time (calculated in accordance with the latest version of Recommendation ITU-R P.452), unless otherwise agreed between the concerned administrations.

Reasons:	To ensure that BSS receivers will be adequately protected from the terrestrial services.

ADD	EUR/5A13/25

5.B113	In the band 21.4-22 GH, in order to facilitate the development of the broadcasting-satellite service, administrations in Regions 1 and 3 are encouraged not to deploy stations in the mobile service and are encouraged to limit the deployment of stations in the fixed service to point-to-point links.

Reasons:	To ensure protection of BSS earth stations from terrestrial services, as well as facilitating the development of the BSS in the 21.4-22 GHz band.

ARTICLE  21

Terrestrial and space services sharing frequency bands above 1 GHz

Section I  –  Choice of sites and frequencies

MOD	EUR/5A13/26

21.2 	§ 2	 1) As far as practicable, sites for transmittingMOD1, 3 stations, in the fixed or mobile service, employing maximum values of equivalent isotropically radiated power (e.i.r.p.) exceeding the values given in Table 21-1 in the frequency bands indicated, should be selected so that the direction of maximum radiation of any antenna will be separated from the geostationary satellite orbit by at least the angle in degrees shown in the Table, taking into account the effect of atmospheric refraction2:

MOD	EUR/5A13/27

1 21.2.1 For their own protection receiving stations in the fixed or mobile service operating in bands shared with space radiocommunication services (space-to-Earth) should also avoid directing their antennas towards the geostationary-satellite orbit if their sensitivity is sufficiently high that interference from space station transmissions may be significant. Especially, in the band 21.4-22.0 GHz, it is recommended a minimum separation angle with respect to geostationary-satellite orbit of 1.5°

Section V  –  Limits of power flux-density from space stations

MOD	EUR/5A13/28

TABLE  21-4

| Frequency band | Service* | Limit in dB(W/m2) for angles of arrival () above the horizontal plane |  |  | Reference bandwidth |
| --- | --- | --- | --- | --- | --- |
|  |  | 0-5 | 5-25 | 25-90 |  |
| 19.3-19.7 GHz 21.4-22 GHz 22.55-23.55 GHz 24.45-24.75 GHz 25.25-27.5 GHz 27.500-27.501 GHz | Fixed-satellite (space-to-Earth) Broadcasting-satellite Earth exploration-satellite (space-to-Earth) Inter-satellite Space research (space-to-Earth) | –115 13A | –115 + 0.5( – 5) 13A | –105 13A | 1 MHz |

Reason: 	To introduce pfd limits in Article 21 to protect terrestrial services in frequency band 21.4-22.0 GHz

______________