| World Radiocommunication Conference (WRC-12) Geneva, 23 January - 17 February 2012 |  |
| --- | --- |
| PLENARY MEETING | Addendum 21 to Document 5-E |
|  | 17 November 2011 |
|  | Original: English |
| EUROPEAN COMMON PROPOSALS FOR THE WORK OF THE CONFERENCE |  |
| PART 21 Agenda item 1.21 1.21 to consider a primary allocation to the radiolocation service in the band 15.4-15.7 GHz, taking into account the results of ITU-R studies, in accordance with Resolution 614 (WRC-07); |  |

Introduction

The results of the ITU studies demonstrate that the operation of radiolocation systems in the 15.5-15.7 GHz band is compatible with adjacent-band and co-frequency allocations, provided that appropriate mitigation techniques are adopted.

Proposal

Given the European proposal for a new AM(R)S allocation under agenda item 1.3 and in order to facilitate the protection of EESS and radioastronomy below 15.4 GHz, Europe proposes a new primary allocation for the RLS limited to the 15.5-15.7 GHz frequency band together with a footnote to protect ARNS.

This will provide a primary allocation to RLS, contiguous across 15.5-17.3 GHz, with sufficient bandwidth to meet emerging requirements for increased image resolution and range accuracy, and assure long-term operating and development environment for radiolocation systems, while maintaining protection in the Radio Regulations for ARNS and RAS systems.

ARTICLE  5

Frequency allocations

Section IV – Table of Frequency Allocations
(See No. 2.1)

MOD	EUR/5A21/1

15.4-18.4 GHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 15.43-15.5 FIXED-SATELLITE (Earth-to-space) 5.511A AERONAUTICAL RADIONAVIGATION 5.511C |  |  |
| 15.5-15.63 FIXED-SATELLITE (Earth-to-space) 5.511A AERONAUTICAL RADIONAVIGATION RADIOLOCATION ADD 5.A121 5.511C |  |  |
| 15.63-15.7 AERONAUTICAL RADIONAVIGATION RADIOLOCATION ADD 5.A121 5.511D |  |  |

ADD	EUR/5A21/2

5.A121	In the band 15.5-15.7 GHz, stations operating in the radiolocation service shall not cause harmful interference to, nor claim protection from, stations operating in the aeronautical radionavigation service.

SUP	EUR/5A21/3

RESOLUTION 614 (WRC-07)

Use of the band 15.4-15.7 GHz by the radiolocation service

____________________