| World Radiocommunication Conference (WRC-12) Geneva, 23 January - 17 February 2012 |  |
| --- | --- |
| PLENARY MEETING | Addendum 23 to Document 5-E |
|  | 17 November 2011 |
|  | Original: English |
| EUROPEAN COMMON PROPOSALS FOR THE WORK OF THE CONFERENCE |  |
| PART 23 Agenda item 1.23 1.23 to consider an allocation of about 15 kHz in parts of the band 415-526.5 kHz to the amateur service on a secondary basis, taking into account the need to protect existing services; |  |

# Introduction

This part of the spectrum is interesting to radio amateurs because its propagation properties fill a significant gap between existing amateur allocations at LF and HF.

The following proposal takes into consideration the compatibility and sharing studies that have been carried out together with current occupancy surveys. Report ITU-R M.2203 (compatibility of amateur service stations with existing services in the range 415-526.5 kHz) confirms this to be a valid Method.  An earlier Report carried out in 1986, ITU-R M.910 (Sharing between the maritime mobile service and the aeronautical radionavigation service in the band 415-526.5 kHz), concluded that coexistence between the two services was possible when account was taken of protection requirements.  Given the low e.i.r.p. level likely to be used by the amateur service this conclusion would support the findings of the more recent study.

Selecting the range 472-480 kHz means that the top end of the frequency allocation is 10 kHz away from the NAVTEX service at 490 kHz.

ARTICLE 5

Frequency allocations

Section IV – Table of Frequency Allocations 
(See No. 2.1)

MOD	EUR/5A23/1

200-495 kHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 415-435 MARITIME MOBILE5.79 AERONAUTICAL RADIONAVIGATION 5.72 | 415-472 MARITIME MOBILE 5.79 Aeronautical radionavigation 5.80 5.77 5.78 5.82 |  |
| 435-472 MARITIME MOBILE 5.79 Aeronautical radionavigation 5.72 5.82 |  |  |
| 472-480 MARITIME MOBILE 5.79 Aeronautical radionavigation Amateur ADD 5.A123 5.72 5.82 | 472-480 MARITIME MOBILE 5.79 Aeronautical radionavigation 5.80 Amateur ADD 5.A123 5.77 5.82 |  |
| 480-495 MARITIME MOBILE 5.79 5.79A Aeronautical radionavigation 5.72 5.82 | 480-495 MARITIME MOBILE 5.79 5.79A Aeronautical radionavigation 5.80 5.77 5.82 |  |

Reasons:	An 8 kHz secondary allocation to amateur service is proposed, in the range 472-480 kHz, as it meets the objective of the Agenda Item, whilst ensuring the need to protect the operation of existing primary maritime mobile services.

ADD	EUR/5A23/2

5.A123	Stations in the amateur service using frequencies in the band 472-480 kHz shall not exceed a maximum radiated power of 5 W (e.i.r.p.) and shall not cause harmful interference to nor claim protection from stations of the Aeronautical radionavigation service.

Reasons:	 This footnote recognises that the secondary aeronautical radionavigation service has existed for a number of years, and therefore should be protected from the new incoming secondary amateur radio service.

_______________________