| World Radiocommunication Conference (WRC-12) Geneva, 23 January - 17 February 2012 |  |
| --- | --- |
| PLENARY MEETING | Addendum 1 to Document 5(Add.3)-E |
|  | 17 November 2011 |
|  | Original: English |
| EUROPEAN COMMON PROPOSALS FOR THE WORK OF THE CONFERENCE |  |
| PART 3 ADDENDUM 1 |

Agenda item 1.3

Terrestrial component and Satellite component in the 5 GHz band

For the terrestrial component (subpart 3A), Europe is of the view that AM(R)S allocations should be used. Studies showed that there is a lack of spectrum to accommodate the spectrum requirement for the terrestrial component. Europe proposes to allocate the band 5 030-5 091 MHz to AM(R)S to satisfy the Agenda Item, along with consequential regulatory and technical considerations to protect the existing services.

For the satellite component (subpart 3B), except for some consequential modifications related to the coordination mechanism of the existing aeronautical mobile (route) satellite service (AMS(R)S) allocation in 5 030-5 091 MHz in case of the new aeronautical mobile (route) service (AM(R)S) allocation proposed in the same band (see subpart 3A), Europe is of the view that no change is required to the Radio Regulations for the satellite component. Indeed, no new allocations are required as there are enough frequency bands to accommodate the spectrum requirement for the satellite component and no new or modified procedures/provisions are needed for the existing satellite allocations.

Subpart 3A   -   Terrestrial component

ARTICLE 5

Frequency allocations

Section IV – Table of Frequency Allocations
(See No. 2.1)

MOD	EUR/5A3/1

1 610-1 660 MHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 1 610-1 610.6 MOBILE-SATELLITE (Earth-to-space) 5.351A AERONAUTICAL RADIONAVIGATION | 1 610-1 610.6 MOBILE-SATELLITE (Earth-to-space) 5.351A AERONAUTICAL RADIONAVIGATION RADIODETERMINATION- SATELLITE (Earth-to-space) | 1 610-1 610.6 MOBILE-SATELLITE (Earth-to-space) 5.351A AERONAUTICAL RADIONAVIGATION Radiodetermination-satellite (Earth-to-space) |
| 5.341 5.355 5.359 5.364 5.366 MOD 5.367 5.368 5.369 5.371 5.372 | 5.341 5.364 5.366 MOD 5.367 5.368 5.370 5.372 | 5.341 5.355 5.359 5.364 5.366 MOD 5.367 5.368 5.369 5.372 |
| 1 610.6-1 613.8 MOBILE-SATELLITE (Earth-to-space) 5.351A RADIO ASTRONOMY AERONAUTICAL RADIONAVIGATION | 1 610.6-1 613.8 MOBILE-SATELLITE (Earth-to-space) 5.351A RADIO ASTRONOMY AERONAUTICAL RADIONAVIGATION RADIODETERMINATION-SATELLITE (Earth-to-space) | 1 610.6-1 613.8 MOBILE-SATELLITE (Earth-to-space) 5.351A RADIO ASTRONOMY AERONAUTICAL RADIONAVIGATION Radiodetermination-satellite (Earth-to-space) |
| 5.149 5.341 5.355 5.359 5.364 5.366 MOD 5.367 5.368 5.369 5.371 5.372 | 5.149 5.341 5.364 5.366 MOD 5.367 5.368 5.370 5.372 | 5.149 5.341 5.355 5.359 5.364 5.366 MOD 5.367 5.368 5.369 5.372 |
| 1 613.8-1 626.5 MOBILE-SATELLITE (Earth-to-space) 5.351A AERONAUTICAL RADIONAVIGATION Mobile-satellite (space-to-Earth) 5.208B | 1 613.8-1 626.5 MOBILE-SATELLITE (Earth-to-space) 5.351A AERONAUTICAL RADIONAVIGATION RADIODETERMINATION- SATELLITE (Earth-to-space) Mobile-satellite (space-to-Earth) 5.208B | 1 613.8-1 626.5 MOBILE-SATELLITE (Earth-to-space) 5.351A AERONAUTICAL RADIONAVIGATION Mobile-satellite (space-to-Earth) 5.208B Radiodetermination-satellite (Earth-to-space) |
| 5.341 5.355 5.359 5.364 5.365 5.366 MOD 5.367 5.368 5.369 5.371 5.372 | 5.341 5.364 5.365 5.366 MOD 5.367 5.368 5.370 5.372 | 5.341 5.355 5.359 5.364 5.365 5.366 MOD 5.367 5.368 5.369 5.372 |

MOD	EUR/5A3/2

5.367	Additional allocation:  The band 1 610-1 626.5 MHz is also allocated to the aeronautical mobile-satellite (R) service on a primary basis, subject to agreement obtained under No. 9.21.

Reasons: This proposed revision to the footnote removes references to the 5 GHz allocation as this is proposed to be treated separately in proposed new footnote 5.B13.

MOD	EUR/5A3/3

4 800-5 570 MHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 5 000-5 010 AERONAUTICAL RADIONAVIGATION AERONAUTICAL MOBILE-SATELLITE (R) ADD 5.B13 RADIONAVIGATION-SATELLITE (Earth-to-space) |  |  |
| 5 010-5 030 AERONAUTICAL RADIONAVIGATION AERONAUTICAL MOBILE-SATELLITE (R) ADD 5.B13 RADIONAVIGATION-SATELLITE (space-to-Earth) (space-to-space) 5.328B 5.443B |  |  |
| 5 030-5 091 AERONAUTICAL RADIONAVIGATION AERONAUTICAL MOBILE-SATELLITE (R) ADD 5.B13 AERONAUTICAL MOBILE (R) ADD 5. A13 5.444 |  |  |
| 5 091-5 150 AERONAUTICAL RADIONAVIGATION AERONAUTICAL MOBILE-SATELLITE (R) ADD 5.B13 AERONAUTICAL MOBILE 5.444B 5.444 5.444A |  |  |

Reasons:

- New AM(R)S allocation: Sharing studies have shown that an AM(R)S allocation is feasible in the band 5030-5091 MHz, provided that power limitations are associated to these allocations.

- In addition, it is proposed to include, directly in the table of frequency allocations, the AMS(R)S allocations previously contained in No 5.367. This is justified since this is a worldwide allocation, whereas an "additional allocation" as defined in No 5.35 addresses allocations "in an area smaller than a Region or in a particular country".

ADD	EUR/5A3/4

5.A13	The use of the band 5 030-5 091 MHz by the aeronautical mobile (R) service is limited to systems operated in accordance with recognised international aeronautical standards. In order to protect the radionavigation-satellite service (Space-to-earth) in the band 5 010-5 030 MHz, the e.i.r.p. density of any AM(R)S station shall not exceed -75 dBW/MHz in the band 5 010-5 030 MHz.     (WRC-12)

Reasons: This new footnote limits the use of the AM(R)S allocation to systems developed and standardized by ICAO to facilitate the development of such an internationl system and the coordination between ICAO systems in this band. The out-of-band e.i.r.p. density limit proposed will protect RNSS receivers in 5010-5030 MHz.

ADD	EUR/5A3/5

5.B13	 In the bands 5 000-5 030 and 5091 - 5150 MHz, the aeronautical mobile-satellite (R) service  is subject to agreement obtained under No. 9.21. In the band, 5 030 - 5091 MHz the aeronautical mobile-satellite (R) service is subject to coordination under No. 9.11A. The use of these bands is limited to systems operated in accordance with recognised international aeronautical standards.     (WRC-12)

Reasons: This new footnote replaces the application of 9.21 for the AMS(R)S allocation in the band 5 030 -5 091 MHZ by the application of 9.11A, as a consequence of the new aeronautical mobile (R) allocation.

MOD	EUR/5A3/6

APPENDIX  5  (Rev.WRC-12)

Identification of administrations with which coordination is to be effected or
agreement sought under the provisions of Article 9

MOD	EUR/5A3/7

TABLE  5-1     (WRC-12)

Technical conditions for coordination
(see Article 9

| Reference of Article 9 | Case | Frequency bands (and Region) of the service for which coordination is sought | Threshold/condition | Calculation method | Remarks |
| --- | --- | --- | --- | --- | --- |
| No. 9.14 Non-GSO/ terrestrial, GSO/ terrestrial | A space station in a satellite network in the frequency bands for which a footnote refers to No. 9.11A or to No. 9.14, in respect of stations of terrestrial services where threshold(s) is (are) exceeded | 1) Frequency bands for which a footnote refers to No. 9.11A; or 2) 11.7-12.2 GHz (Region 2 GSO FSS) 3) 5 030-5 091 MHz | 1) See § 1 of Annex 1 to this Appendix; In the bands specified in No. 5.414A, the detailed conditions for the application of No. 9.14 are provided in No. 5.414A for MSS networks or 2) In the band 11.7-12.2 GHz (Region 2 GSO FSS): –124 dB(W/(m2 · MHz)) for 0° 5 –124 + 0.5 ( – 5) dB(W/(m2 · MHz)) for 5° < 25 –114 dB(W/(m2 · MHz)) for > 25 where is the angle of arrival of the incident wave above the horizontal plane (degrees) 3) In the band 5 030-5 091 MHz (AMS(R)S), bandwidth overlap is the threshold. | 1) See § 1 of Annex 1 to this Appendix |  |

Reasons: A new condition is introduced in Appendix 5 to implement coordination under No. 9.14 for AMS(R)S in the band 5 030-5 091 MHz (See modification in the Table 5-1 below). The level of the accepted interference by the identified administration will be fixed by agreement between administrations concerned using the relevant ITU-R Recommendations as a guide.

MOD	EUR/5A3/8

APPENDIX  7  (Rev.WRC-12)

Methods for the determination of the coordination area around an earth
station in frequency bands between 100 MHz and 105 GHz

MOD	EUR/5A3/9

Reasons: New parameters are introduced in Appendix 7 for Table 7B and 9A to implement coordination under Nos. 9.15 to 9.18 for AMS(R)S in the band 5 030-5 091 MHz (See modification in the two Tables below)

TABLE  7b     (WRC-12)

Parameters required for the determination of coordination distance for a transmitting earth station

| Transmitting space radiocommunication service designation |  | Fixed-satellite, mobile-satellite | Aeronautical mobile satellite (R) service | Aeronautical mobile satellite (R) service | Fixed- satellite |  |  | Fixed- satellite | Fixed- satellite |  | Fixed- satellite |  |  |  | Space operation, space research |  |  |  | Fixed-satellite, mobile-satellite, meteorological- satellite |  |  |  | Fixed- satellite |  |  |  | Fixed- satellite |  |  |  | Fixed- satellite |  | Fixed- satellite 3 |  | Fixed- satellite |  | Fixed- satellite 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Frequency bands (GHz) |  | 2.655-2.690 | 5.030-5.091 | 5.030-5.091 | 5.091-5.150 |  |  | 5.091-5.150 | 5.725-5.850 |  | 5.725-7.075 |  |  |  | 7.100-7.235 5 |  |  |  | 7.900-8.400 |  |  |  | 10.7-11.7 |  |  |  | 12.5-14.8 |  |  |  | 13.75-14.3 |  | 15.43-15.65 |  | 17.7-18.4 |  | 19.3-19.7 |  |
| Receiving terrestrial service designations |  | Fixed, mobile | Aeronautical radio- navigation | Aeronautical mobile (R) | Aeronautical radio- navigation |  |  | Aeronautical mobile (R) | Radio-location |  | Fixed, mobile |  |  |  | Fixed, mobile |  |  |  | Fixed, mobile |  |  |  | Fixed, mobile |  |  |  | Fixed, mobile |  |  |  | Radiolocation radionavigation (land only) |  | Aeronautical radionavigation |  | Fixed, mobile |  | Fixed, mobile |  |
| Method to be used |  | § 2.1 | § 2.1, § 2.2 | § 2.1, § 2.2 |  |  |  |  | § 2.1 |  | § 2.1 |  |  |  | § 2.1, § 2.2 |  |  |  | § 2.1 |  |  |  | § 2.1 |  |  |  | § 2.1, § 2.2 |  |  |  | § 2.1 |  |  |  | § 2.1, § 2.2 |  | § 2.2 |  |
| Modulation at terrestrial station 1 |  | A |  |  |  |  |  |  |  |  | A |  | N |  | A |  | N |  | A |  | N |  | A |  | N |  | A |  | N |  | – |  |  |  | N |  | N |  |
| Terrestrial station interference parameters and criteria | p0 (%) | 0.01 |  |  |  |  |  |  |  |  |  | 0.01 |  | 0.005 |  | 0.01 |  | 0.005 |  | 0.01 |  | 0.005 |  | 0.01 |  | 0.005 |  | 0.01 |  | 0.005 |  | 0.01 |  |  |  | 0.005 |  | 0.005 |  |
|  | n | 2 |  |  |  |  |  |  |  |  |  | 2 |  | 2 |  | 2 |  | 2 |  | 2 |  | 2 |  | 2 |  | 2 |  | 2 |  | 2 |  | 1 |  |  |  | 2 |  | 2 |  |
|  | p (%) | 0.005 |  |  |  |  |  |  |  |  |  | 0.005 |  | 0.0025 |  | 0.005 |  | 0.0025 |  | 0.005 |  | 0.0025 |  | 0.005 |  | 0.0025 |  | 0.005 |  | 0.0025 |  | 0.01 |  |  |  | 0.0025 |  | 0.0025 |  |
|  | NL (dB) | 0 |  |  |  |  |  |  |  |  |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  |  |  | 0 |  | 0 |  |
|  | Ms (dB) | 26 2 |  |  |  |  |  |  |  |  |  | 33 |  | 37 |  | 33 |  | 37 |  | 33 |  | 37 |  | 33 |  | 40 |  | 33 |  | 40 |  | 1 |  |  |  | 25 |  | 25 |  |
|  | W (dB) | 0 |  |  |  |  |  |  |  |  |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  |  |  | 0 |  | 0 |  |
| Terrestrial station parameters | Gx (dBi) 4 | 49 2 | 6 | 10 |  | 6 | 6 |  |  |  |  | 46 |  | 46 |  | 46 |  | 46 |  | 46 |  | 46 |  | 50 |  | 50 |  | 52 |  | 52 |  | 36 |  |  |  | 48 |  | 48 |  |
|  | Te (K) | 500 2 |  |  |  |  |  |  |  |  |  | 750 |  | 750 |  | 750 |  | 750 |  | 750 |  | 750 |  | 1 500 |  | 1 100 |  | 1 500 |  | 1 100 |  | 2 636 |  |  |  | 1 100 |  | 1 100 |  |
| Reference bandwidth | B (Hz) | 4  103 | 150  103 | 37.5 x 103 |  | 150  103 | 106 |  |  |  |  | 4  103 |  | 106 |  | 4  103 |  | 106 |  | 4  103 |  | 106 |  | 4  103 |  | 106 |  | 4  103 |  | 106 |  | 107 |  |  |  | 106 |  | 106 |  |
| Permissible interference power | Pr( p) (dBW) in B | –140 | –160 | –154 |  | –160 | –143 |  |  |  |  | –131 |  | –103 |  | –131 |  | –103 |  | –131 |  | –103 |  | –128 |  | –98 |  | –128 |  | –98 |  | –131 |  |  |  | –113 |  | –113 |  |

TABLE  9a     (WRC-12)

Parameters required for the determination of coordination distance for a transmitting earth station
in bands shared bidirectionally with receiving earth stations

| Space service designation in which the transmitting earth station operates |  | Land mobile- satellite | Mobile-satellite | Land mobile-satellite | Earth exploration-satellite, meteorological-satellite | Mobile-satellite |  | Fixed-satellite, mobile-satellite | Aeronautical mobile satellite (R) service |  | Fixed- satellite 3 |  | Fixed-satellite | Fixed-satellite, meteorological- satellite | Fixed-satellite |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Frequency bands (GHz) |  | 0.1499-0.15005 | 0.272-0.273 | 0.3999-0.40005 | 0.401-0.402 | 1.670-1.675 |  | 2.655-2.690 | 5.030-5.091 |  | 5.150-5.216 |  | 6.700-7.075 | 8.025-8.400 | 8.025-8.400 |
| Space service designation in which the receiving earth station operates |  | Radio-navigation-satellite | Space operation | Radio-navigation-satellite | Space operation | Meteorological-satellite |  | Fixed-satellite, broadcasting-satellite | Aeronautical mobile satellite (R) service |  | Fixed-satellite | Radiodetermination-satellite | Fixed-satellite | Earth exploration- satellite | Earth exploration- satellite |
| Orbit 6 |  |  | Non-GSO |  | Non-GSO | Non-GSO | GSO |  | Non-GSO | GSO | Non-GSO |  | Non-GSO | Non-GSO | GSO |
| Modulation at receiving earth station 1 |  |  | N |  | N | N | N |  |  |  |  |  | N | N | N |
| Receiving earth station interference parameters and criteria | p0 (%) |  | 1.0 |  | 0.1 | 0.006 | 0.011 |  |  |  |  |  | 0.005 | 0.011 | 0.083 |
|  | n |  | 1 |  | 2 | 3 | 2 |  |  |  |  |  | 3 | 2 | 2 |
|  | p (%) |  | 1.0 |  | 0.05 | 0.002 | 0.0055 |  |  |  |  |  | 0.0017 | 0.0055 | 0.0415 |
|  | NL (dB) | 0 | 0 | 0 | 0 | 0 | 0 |  |  |  |  |  | 1 | 0 | 1 |
|  | Ms (dB) | 2 | 1 | 2 | 1 | 2.8 | 0.9 | 2 |  |  | 2 | 2 | 2 | 4.7 | 2 |
|  | W (dB) | 0 | 0 | 0 | 0 | 0 | 0 |  |  |  |  |  | 0 | 0 | 0 |
| Receiving earth station parameters | Gm (dBi) 2 | 0 | 20 | 0 | 20 | 30 | 45 |  | 45 | 45 | 48.5 |  | 50.7 |  |  |
|  | Gr (dBi) 4 | 0 | 19 | 0 | 19 | 19 9 | 8 |  | 8 | 8 | 10 |  | 10 | 10 | 8 |
|  | min 5 | 3 | 10 | 3 | 10 | 5 | 3 | 3 | 10° | 10° | 3 | 3 | 3 | 5 | 3 |
|  | Te (K) 7 | 200 | 500 | 200 | 500 | 370 | 118 | 75 | 340 | 340 | 75 | 75 | 75 |  |  |
| Reference bandwidth | B (Hz) | 4  103 | 103 | 4  103 | 1 | 106 | 4  103 |  | 37.5 x103 | 37.5 x103 |  |  | 106 | 106 | 106 |
| Permissible interference power | Pr( p) (dBW) in B | –172 | –177 | –172 | –208 | –145 | –178 |  | -163.5 | -163.5 |  |  | –151 | –142 | –154 |

Subpart 3B     -    Satellite component

NOC  	EUR/5A3/10

to the Radio Regulations for the satellite component for the Control and non-Payload communication link of UAS under AI 1.3

_____________