| World Radiocommunication Conference (WRC-12) Geneva, 23 January - 17 February 2012 |  |
| --- | --- |
| PLENARY MEETING | Addendum 2 to Document 5(Add.3)-E |
|  | 17 November 2011 |
|  | Original: English |
| EUROPEAN COMMON PROPOSALS FOR THE WORK OF THE CONFERENCE |  |
| PART 3 ADDENDUM 2 |

Agenda item 1.3

Subpart 3C   -  Terrestrial component in the 15.4 – 15.5 GHz

Introduction

For the terrestrial component, Europe is of the view that AM(R)S allocations should be used. Studies showed that there is a lack of spectrum to accommodate the spectrum requirement for the terrestrial component. In addition to the proposal to allocate the band 5030-5091 to AM(R)S (see Addendum 1 to Addendum 3, subpart 3A), Europe proposes to allocate the band 15.4-15.5 GHz to AM(R)S to satisfy the Agenda Item, along with consequential regulatory and technical considerations to protect the existing services.

ARTICLE  5

Frequency allocations

MOD	EUR/5A3/11

15.4-18.4 GHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 15.4-15.43 AERONAUTICAL RADIONAVIGATION AERONAUTICAL MOBILE (R) ADD 5.C13 5.511D |  |  |
| 15.43-15.5 FIXED-SATELLITE (Earth-to-space) 5.511A AERONAUTICAL RADIONAVIGATION AERONAUTICAL MOBILE (R) ADD 5.C13 5.511C |  |  |
| 15.5-15.63 FIXED-SATELLITE (Earth-to-space) 5.511A AERONAUTICAL RADIONAVIGATION 5.511C |  |  |

ADD	EUR/5A3/12

5.C13	The use of the band 15.4-15.5 GHz by aeronautical mobile (R) service is limited to systems operated in accordance with recognised international aeronautical standards. In order to protect the Radio Astronomy in the band 15.35-15.4 GHz, the e.i.r.p. density of any AM(R)S station shall not exceed -68 dBW/50MHz in the band 15.35-15.4 GHz. For the protection of aeronautical radionavigation systems operating in 15.4-15.5 GHz, any Administration operating AM(R)S systems in this band shall obtain the coordination agreement from other Administrations whose territory is located at less than 903 km from the border of the territory of the Administration intending to operate the AM(R)S.

Reasons: In order to add more flexibility in assigning frequencies, Europe additionally proposes to allocate the band 15.4 – 15.5 GHz to AM(R)S with associated provisions to protect the existing services.

______________