| World Radiocommunication Conference (WRC-12) Geneva, 23 January - 17 February 2012 |  |
| --- | --- |
| PLENARY MEETING | Addendum 7 to Document 5(Add.28)-E |
|  | 17 November 2011 |
|  | Original: English |
| EUROPEAN COMMON PROPOSALS FOR THE WORK OF THE CONFERENCE |  |
| part 28 Addendum 7 |  |
| Agenda item 7 Subpart D (continuation of Addenddum 4) Miscellaneous issues |  |

Introduction

Proposals within this subpart are related to Issue 6A of the CPM report on WRC-12 Agenda item 7 as well as new issues not contained in the CPM report. Even if they are largely independent, they are grouped altogether for convenience. This Addendum is related to Addendum 4 to Addendum 28 to Document 5.

Studies on Complementary Ground Components

Issue 6A of the CPM report deals with “Interim procedures for notification and recording of complementary ground components of integrated MSS systems in the 1 525-1 559 MHz and 1 626.5-1 660.5 MHz bands”. Europe does not support any change in the regulatory procedures for coordination, notification and recording of frequency assignments to Complementary Ground Components (CGC) of integrated MSS systems, before the sharing and compatibility studies related to the impact of CGC on other systems and services are completed. Therefore it is proposed that Recommendation 206 (WRC-07) should be modified in order to focus only on the 1.6/1.5 GHz L-band (because other MSS bands are either already allocated to the mobile service or do not attract interest for deploying CGC) and to request ITU-R to perform studies on this issue in a more explicit manner, bearing however in mind that, ultimately, ITU-R is a contribution-driven organisation.

Suppressing the 6-month period between an API and a coordination request in Article 9 of the Radio Regulations

The European proposal of a definitive list under No. 9.36.2 will result in a period of approximately 15 months between the receipt of an advance publication and the publication of the definitive list (6 months between API and coordination request, 3 months to publish the coordination request, 4 months to comment, approximately 2 months to publish the definitive list following the comments). Although this drawback was not identified in the CPM report, it is proposed to correct it by suppressing the 6-month period between the API receipt and the receipt of the coordination request. This would imply allocating only 9 months out of 7 years for the “publication” part of the coordination process.

In order to provide transition measures, Europe proposes that the provisions below should enter into force 6 months after the end of the Conference.

Facilitating the conclusion of interim agreements in the bands governed by Appendices 30, 30A and 30B.

Europe proposes facilitating the conclusion of interim agreements in the bands governed by Appendices 30, 30A and 30B. Regulatory text is proposed afterwards.

It is proposed to introduce a regulatory mechanism allowing administrations having no immediate plans to use their planned assignments and allotments in the Appendices 30, 30A and 30B to give their agreements on a temporary basis to administrations wishing to operate assignments in the List without seeing their protection margins permanently degraded. Such a mechanism would guarantee the continued protection of national resources in the space Plans, improve the coordination mechanisms for assignments in the List and increase the accuracy of notified characteristics of the space systems.

Use of modern means of electronic communications in the administrative correspondence related to coordination and notification of satellite networks.

Europe proposes a Resolution to instruct the BR to develop a mechanism allowing fax to be replaced by secured email exchanges in a coordinated manner. This would simplify the administrative correspondence related to coordination and notification of satellite networks.

| Topic | Related issue in the CPM text on WRC-12 Agenda item 7 | Method from the CPM text that Europe supports, if any | List of European proposals |  |
| --- | --- | --- | --- | --- |
|  |  |  | Intent of the proposal | Proposal number (EUR/5A28/) |
| Interim procedures for notification and recording of complementary ground components of integrated MSS systems in the 1 525-1 559 MHz and 1 626.5-1 660.5 MHz bands | Issue 6A |  | Modify Recommendation 206 (WRC-07) to focus only on the 1.6/1.5 GHz L-band and to request ITU-R to perform studies on this issue in a more explicit manner | 92 |
| 6-month period between an API and a coordination request in Article 9 |  |  | Suppressing the 6-month period between an API and a coordination request in Article 9 of the Radio Regulations | 93, 94 |
| Interim agreements in the bands governed by Appendices 30, 30A and 30B |  |  | Introduction of a new regulatory mechanism allowing administrations having no immediate plans to use their planned assignments and allotments in the Appendices 30, 30A and 30B to give their agreements on a temporary basis to administrations wishing to operate assignments in the List without seeing their protection margins permanently degraded. | 95, 96, 97, 98, 99, 100, 101 |
| Use of modern means of electronic communications in the administrative correspondence related to coordination and notification of satellite networks. | - | - | New Resolution to instruct the BR to develop a mechanism allowing fax to be replaced by secured email exchanges in a coordinated manner. | 102 |

Studies on Complementary Ground Components

MOD		EUR/5A28/92

RECOMMENDATION  206  (WRC-12)

Studies on the possible use of integrated mobile-satellite service and ground component systems in 
the bands 1 525-1 544 MHz, 1 545-1 559 MHz, 1 626.5-1 645.5 MHz, 1 646.5-1 660.5 MHz

The World Radiocommunication Conference (Geneva, 2012),

considering

a)	that mobile-satellite service (MSS) systems may provide service to a wide area;

b)	that integrated MSS systems are employing a satellite component and a ground component where the ground component is complementary to the satellite component and operates as, and is, an integral part of the MSS system. In such systems, the ground component is controlled by the satellite resource and network management system. Further, the ground component uses the same portions of MSS frequency bands as the associated operational mobile-satellite system.”

c)	that MSS systems have a limited capacity for providing reliable radiocommunication services in urban areas on account of natural or man-made obstacles and that the ground component of an integrated MSS system can mitigate blockage areas, as well as allow for indoor service coverage;

d)	that MSS systems can improve coverage of rural areas, thus being one element that can bridge the digital divide in terms of geography;

e)	that MSS systems are suitable for public protection and disaster relief communications, as stated in Resolution 646 (WRC-03);

-

f )	that the bands 1 525-1 544 MHz, 1 545-1 559 MHz, 1 626.5-1 645.5 MHz, 1 646.5-1 660.5 MHz are allocated on a primary basis to the mobile-satellite services and to other services but that not all of them are allocated to the mobile service;

--

g)	that in some of the bands identified in considering f), some administrations consider to authorize integrated MSS systems

;

h)	that ITU-R has performed frequency sharing studies and has determined that the coexistence between independent systems in the MSS and systems in the mobile services in the same spectrum without harmful interference is not feasible in the same or adjacent geographical area,

recognizing

a)	that ITU-R has not yet performed studies on sharing, technical or regulatory issues with regard to integrated MSS systems;

b)	that the radionavigation-satellite service in the 1 559-1 610 MHz band and the radio astronomy service in the bands 1 610.6-1 613.8 MHz and 1 660-1 670 MHz need to be protected from harmful interference;

c)	that the MSS needs to be protected from harmful interference that may be caused by the introduction of the ground component of Integrated Systems;

d)	that Nos. 5.353A and 5.357A are applicable to MSS systems in different portions of the bands 1 525-1 559 MHz and 1 626.5-1 660.5 MHz with respect to the spectrum requirements and prioritization of communications for the Global Maritime Distress and Safety System and the aeronautical mobile-satellite (R) service,

noting

-

that there are a limited number of frequency bands allocated to the MSS, that these bands are already congested, and that the introduction of integrated ground components may in some instances make spectrum access for other MSS systems more difficult;

recommends

that ITU-R should conduct studies on the possible use of integrated MSS systems in the bands 1 525-1 544 MHz, 1 545-1 559 MHz, 1 626.5-1 645.5 MHz, 1 646.5-1 660.5 MHz, as appropriate, taking into account existing systems and those proposed to be used soon and the above considering, recognizing and noting,

invites administrations

to participate as necessary in the ITU-R studies taking into account recognizing a).

Reasons: To focus only on the 1.6/1.5 GHz L-band and to request ITU-R to perform studies on this issue in a more explicit manner

Suppressing the 6-month period between an API and a coordination request in Article 9 of the Radio Regulations

ARTICLE  9

Procedure for effecting coordination with or 
            obtaining agreement of other administrations1, 2, 3, 4, 5, 6, 7, 8

Section I  –  Advance publication of information on satellite
networks or satellite systems

General

MOD		EUR/5A28/93

9.1		Before initiating any action under this Article or under Article 11 in respect of frequency assignments for a satellite network or a satellite system, an administration, or one9 acting on behalf of a group of named administrations, shall, prior to the coordination procedure described in Section II of Article 9 below, where applicable, send to the Bureau a general description of the network or system for advance publication in the International Frequency Information Circular (BR IFIC) not earlier than seven years and preferably not later than two years before the planned date of bringing into use of the network or system (see also No. 11.44). The characteristics to be provided for this purpose are listed in Appendix 4. The coordination or notification information may also be communicated to the Bureau at the same time. Where coordination is not required by Section II, notification shall be considered as having been received by the Bureau not earlier than six months after the date of publication of the advance publication information.

Reasons: To suppress the six-month period between the receipt of an advance publication and the receipt of the associated coordination request under Section II of Article 9, in order not to reduce the part dedicated to publication of special sections within the coordination process.

Sub-Section IB  –  Advance publication of information on satellite networks or satellite
systems that are subject to coordination procedure under Section II

MOD		EUR/5A28/94

9.5B		If, upon receipt of the BR IFIC containing information published under No. 9.2B, any administration considers its existing or planned satellite systems or networks or terrestrial stations11 to be affected, it may send its comments to the publishing administration, so that the latter may take those comments into consideration during the coordination procedure. A copy of these comments may also be sent to the Bureau. Thereafter, both administrations shall endeavour to cooperate in joint efforts to resolve any difficulties, with the assistance of the Bureau, if so requested by either of the parties, and shall exchange any additional relevant information that may be available.     (WRC-2000)

Reasons: Consequential to the suppression of the six-month period because the coordination request can be sent before the publication of the advance publication.

Facilitating the conclusion of interim agreements in the bands governed by Appendices 30, 30A and 30B.

MOD		EUR/5A28/95

APPENDIX  30  (Rev.WRC-12)*

Provisions for all services and associated Plans and List1 for the broadcasting-satellite service in the frequency bands 11.7-12.2 GHz (in Region 3), 11.7-12.5 GHz (in Region 1) and 12.2-12.7 GHz (in Region 2) (WRC-03)

…

ARTICLE  4     (Rev.WRC-12)

Procedures for modifications to the Region 2 Plan or 
for additional uses in Regions 1 and 33

…

ADD		EUR/5A28/96

4.1.13bis	In the case of an agreement according to § 4.1.13 between an assignment in the Plan and an assignment in the List, both assignments may be paired. During the specific period of the agreement, only the interference contributions from assignments that are not part of the same pair shall be included for the calculation of interference to assignments that are part of a pair; for the calculation of interference from assignments belonging to a pair to assignments that are not part of that same pair, only the worst interference contribution from that pair shall be used. Paired assignments shall not be operated simultaneously.

MOD		EUR/5A28/97

APPENDIX  30A  (Rev.WRC-12)*

Provisions and associated Plans and List1 for feeder links for the broadcasting-satellite service (11.7-12.5 GHz in Region 1, 12.2-12.7 GHz in Region 2 and 11.7-12.2 GHz in Region 3) in the frequency bands 14.5-14.8 GHz2 and 17.3-18.1 GHz in Regions 1 and 3,
and 17.3-17.8 GHz in Region 2     (WRC-03)

…

ARTICLE  4     (Rev.WRC-12)

Procedures for modifications to the Region 2 feeder-link Plan 
or for additional uses in Regions 1 and 3

…

ADD		EUR/5A28/98

4.1.13bis	In the case of an agreement according to § 4.1.13 between an assignment in the Plan and an assignment in the List, both assignments may be paired. During the specific period of the agreement, only the interference contributions from assignments that are not part of the same pair shall be included for the calculation of interference to assignments that are part of a pair; for the calculation of interference from assignments belonging to a pair to assignments that are not part of that same pair, only the worst interference contribution from that pair shall be used. Paired assignments shall not be operated simultaneously.

MOD		EUR/5A28/99

APPENDIX  30B  (Rev.WRC-12)

Provisions and associated Plan for the fixed-satellite service in the frequency bands 4 500-4 800 MHz, 6 725-7 025 MHz, 10.70-10.95 GHz,
11.20-11.45 GHz and 12.75-13.25 GHz

…

ARTICLE  6     (Rev.WRC-12)

Procedures for the conversion of an allotment into an assignment, for
the introduction of an additional system or for the modification of
an assignment in the List1, 2

…

ADD		EUR/5A28/100

6.16bis	In cases where agreements are required with respect to allotments in the Plan, the agreement of the administrations affected may also be obtained in accordance with this Article, for a specified period. When this specific period of agreement expires, the assignment shall be maintained in the List until the end of the period referred to in § 6.31. After that date this assignment shall lapse unless the agreement of the administrations affected is renewed.

ADD		EUR/5A28/101

6.16ter	In the case of an agreement according to § 6.16bis between an allotment in the Plan and an assignment in the List, the allotment and the assignment may be paired. During the specific period of the agreement, only the interference contributions from allotments and assignments that are not part of the same pair shall be included for the calculation of interference to the allotments and assignments that are part of a pair; for the calculation of interference from allotments and assignments belonging to a pair to allotments and assignments that are not part of that same pair, only the worst interference contribution from that pair shall be used. An allotment paired with an assignment shall not be converted to an assignment during the period of the temporary agreement.

Use of modern means of electronic communications in the administrative correspondence related to coordination and notification of satellite networks.

ADD	EUR/5A28/102

DRAFT NEW RESOLUTION  [EUR/D7/102]  (WRC-12)

Use of electronic means of communications for administrative correspondence related to advance publication, coordination and notification of satellite networks, earth stations and radio astronomy stations

The World Radiocommunication Conference (Geneva, 2012),

considering

that the use of electronic means of communications for administrative correspondence related to advance publication, coordination and notification of satellite networks, earth stations and radio astronomy stations would facilitate the tasks of the Radiocommunication Bureau and of administrations and has the potential to improve the coordination and notification process by reducing the amount of duplicated correspondence,

recognizing

that administrations could use the time freed by a reduction of administrative correspondence to effect coordination,

resolves

that electronic means of communications shall be used in the administrative correspondence between administrations and the Radiocommunication Bureau related to advance publication, coordination and notification of satellite networks, earth stations and radio astronomy stations,

instructs the Radiocommunication Bureau

1	to provide administrations with the necessary technical means ensuring that the electronic correspondence between administrations and the Radiocommunication Bureau is secured;

2	to inform administrations of the availability of such means and of the associated schedule of implementation;

3	to report to the next World Radiocommunication Conference on the experience gained in the application of this Resolution, with a view to making any necessary consequential amendments to the Radio Regulations,

urges administrations

to use, as far as possible, electronic means of communications in the administrative correspondence between themselves related to advance publication, coordination and notification of satellite networks, earth stations and radio astronomy stations.

_______________

_______________