Draft CEPT Brief on agenda item 1.3

1.3	to consider spectrum requirements and possible regulatory actions, including allocations, in order to support the safe operation of unmanned aircraft systems (UAS), based on the results of ITU-R studies, in accordance with Resolution 421 (WRC-07).

## Issue

This agenda item considers the spectrum requirements to support the safe operation of unmanned aircraft systems in non-segregated airspace.

The safe flight operation of UAS needs reliable communication links and associated spectrum, especially for the remote pilot to command and control the flight and to relay the air traffic control communications. In addition the safe flight operation of UAS necessitates advanced techniques to detect and track nearby aircraft, terrain and obstacles to navigation in order to ensure the UAS avoids these objects in a manner equivalent to that achieved by manned aircraft.

Preliminary CEPT position

CEPT supports the spectrum demands of 34 MHz for terrestrial and 56 MHz for satellite spectrum as determined by ITU-R Report M.2171 for the provisioning of unmanned aircraft Control and Non-Payload Communications (CNPC) in non-segregated airspace, comprising ATC relay, command and control (C²) and Sense-and-Avoid data(SAA).

The communication for UAS should be considered as an application of a safety service as defined in RR 1.59

CEPT is of the view that the use of any allocations for the purpose of control and non-payload communication of UAS must be in accordance with international aeronautical standards.

- Satellite component:

.

CEPT is of the view that no change is required to the Radio Regulations for the satellite component. Indeed, no new allocations are required as there are enough frequency bands to accommodate the spectrum requirement for the satellite component and no new or modified procedures/provisions are needed for the existing satellite allocations therefore CEPT supports NOC for the satellite component

- Terrestrial component:

CEPT is of the view that bands allocated to AM(R)S should be used.

Due to the intensive usage of the band 960-1164 MHz in Europe, CEPT does not support the usage of this frequency band for the terrestrial component of UAS for CNPC.

CEPT will be studying until WRC-12 the bands 5 000-5 010 MHz, 5 030-5 091 MHz and 15.4 – 15.5 GHz to satisfy the Agenda Item, along with consequential regulatory and technical considerations to protect the existing services.

Background

Already during the preparation of WRC-07 CEPT/CPG/PT3 discussed the issue of the operation of unmanned aircraft under AI 1.5 and 1.6 of WRC-07.  It was felt that the implications of the operation of unmanned aircraft were not completely visible in time for WRC07. Hence CEPT proposed a separate agenda item for UAS for WRC-12. This proposal was supported by similar proposals from other regions.

Remotely piloted air vehicles have been used by the military for some considerable time and this technology is now finding applications within the civil environment. In a large number of countries there are now significant civil development programmes for these UAS with the intention of development systems that operate seamlessly with existing piloted aircraft in non-segregated airspace.

To date safety of other aircraft has been mostly ensured by providing suitable segregated airspace within which development and operation of unmanned aircraft can take place.  Initially these areas of segregated airspace were fixed, normally within a single Administration. However as products have matured, there have been requirements for international and even transcontinental flights.  This has been temporarily accommodated, whilst the requests are limited for specific tests, by the use of a “bubble” of segregated airspace which moves with the aircraft.  However as the number of requests increase and permanent operations are required these means will no longer be sufficient. The aeronautical technology is now mature to  insert unmanned aircraft used for many different purposes in non segregated airpace. Resolves 1 of Resolution 421 asks the WRC-12 to consider the spectrum requirements and possible regulatory actions, including additional allocations, to support the remote pilot in commanding and controlling the unmanned aircraft systems and in relaying the air traffic control communications. This addresses directly those radio links which connects the remote pilot to the unmanned aircraft. These radio links are replacing partially functionalities which are in manned aircraft completely inside the aircraft, and therefore these links will be subject to the process of airworthiness certification. The implication for the regulatory treatment of these radio links has carefully to be studied.

Resolves 2 of Resolution 421 addresses techniques to detect and track nearby aircraft, terrain and obstacles to navigation (Sense And Avoid/SAA-Systems) in order to ensure the UAS avoids these objects in a manner equivalent to that achieved by manned aircraft (“see and avoid” principle). There are already systems in operation in manned aircraft which help to avoid incidents with terrain and nearby aircraft. It has to be studied if these systems are suitable for use in unmanned aircraft.

Resolution 421 keeps the radiocommunication requirement of payload outside any possible WRC-12 decision. ITU-R is invited to produce a report or recommendation, as appropriate on how the communications requirements for UAS payload could be satisfied.

The range of systems is expected to be considerable, extending  from very light devices to those that equate to current civil air transport aircraft and it is expected that these systems will operate in all types of airspace.

UAS market

It is expected that sales will increase significantly over the next 10 years. In the short-term, it is considered that the majority of systems used will be small or mini-UAS, as these will prove easier to be used within the existing safety regulatory framework and will require less investment. Then, by the middle of the next decade, European government use of UAS will grow consistently and start use of UAS in non-segregated airspace for non-military applications, as reflected below:

Figure 1: Market developement for UAS

For now, the UAS industry is on a stand-by mode as far as the civil market is concerned, waiting for regulations and policies to be defined.

European government use of UAS - embracing applications such as law enforcement, border security and coastguard - is expected to take 45% of the civil and commercial market until 2017.

Spectrum requirements

Studies performed lead to the conclusion that 34 MHz of terrestrial and 56 MHz of satellite safety spectrum are required to provide ATC relay, C² and SAA services.

The details of those requirements are shown in the figures below:

Figure 2: UA in line of sight (LOS) to its Control Station (UASC)

Forward Link

Uplink: UA CS to UAV

Return Link

Downlink: UAV to UA CS

1

2

UACS

ATC

UAV

1+2    34 MHz

Forward Link

Uplink: UACS to satellite

Downlink: satellite to UA

Return Link

Uplink: UA to satellite

Downlink: satellite to UACS

3

2

1

4

UACS

ATC

UAV

Satellite

1+4  	  28 MHz

2+3  	  28MHz

TOTAL 	  56MHz

Figure 3: UA beyond line of sight of its UACS

When using other mechanisms than satellite communication (e.g. an intermediate airplane) for relaying radiocommunication links between the UA control station and an unmanned aircraft, the radiocommunication service is considered as terrestrial radiocommunication (AM(R)S).

The impact of latency on UAS command and control systems is a prime factor when considering the safety of operations. Latency will be of the utmost importance when establishing a safety case for the operation of UAs, particularly in non-segregated airspace. Current air traffic management relies heavily on voice communications although information via data links is being progressively implemented. Hence new operational requirements for the future data link environment will also need to be developed.

Radiocommunication Services allocation

Principles

The following principles should be taken into account for any discussion on frequency bands for Agenda Item 1.3.

1)	Under No. 191 of the ITU Constitution, international telecommunications must give absolute priority to all telecommunications concerning safety of life.

2)	All UAS radiocommunications within the scope of WRC-12 Agenda item 1.3 are radiocommunications concerning safety of life.

3)	No. 1.59 of the RR states that any radiocommunication service used permanently or temporarily for the safeguarding of human life and property is a safety service.

4)	No. 4.10 of the RR states that Member States recognize that the safety aspects of radionavigation and other safety services require special measures to ensure their freedom from harmful interference and that it is necessary therefore to take this factor into account in the assignment and use of frequencies.

5)	It is recognized that the aeronautical safety of life aspects need to be treated within ICAO through the development of new standards and recommended practices (SARPs)

6)	Any special measures as referred to above must be clear, implementable in practice and designed to avoid creating difficult regulatory situations.

7)	Existing allocations should be considered before considering the need for new allocations.

Terrestrial scenario

The terrestrial communication radio links between an unmanned aircraft and the UACS have to be considered as an application of AM(R)S and should be operated in frequency band(s) allocated to this Service.

Satellite scenario

UA/SAT link:For satellite component (UA/SAT link) two cases should be considered.

- Case 1a: spectrum that, from a “mobile-satellite” point of view, should be explicitly  allocated to AMS(R)S and shared only with other “aviation services” (e.g. ARNS, ARNSS, AM(R)S). In other words, an explicit allocation to AMS(R)S

- Case 1b: spectrum that can or could be used by AMS(R)S applications through an MSS allocation1, an AMSS allocation or an AMS(R)S allocation.

It is worthwhile noting that mobile satellite systems currently providing AMS(R)S applications for manned aircraft are belonging to case 1b. However, such systems are currently used only above low density areas (e.g. oceans) and they are not the primary means of communication for applications related to the safety and regularity of flight.

While recognizing that, from a civil aviation radio perspective, case 1a seems to be the most appropriate case from a regulatory point of view, the airworthiness certification process of UAS is still at a very early stage of its development and will not be completed by WRC-12. As a consequence, WRC-12 should not take decisions that could, de facto, prevent some options within the airworthiness certification process of UAS (some existing or future satellite systems covered by case 1b might satisfy in due time appropriate international aeronautical standards that will be required within the airworthiness certification process of UAS).

Therefore, WRC-12 should allow the two cases, considering that any use of a satellite allocation for the purpose of control and non-payload communication of UAS shall be in accordance with international aeronautical regulations

If it is not possible to conclude that a satellite system operating under AMSS or MSS allocations (case 1b) could meet in due time the appropriate international aeronautical standards, the logical consequence of the above would be that “explicit” allocations to AMS(R)S (opposed to AMS(R)S application within AMSS or MSS allocations) should fulfil the spectrum requirement of case 1a.

SAT/UACS link

For satellite component (SAT/CS link) two cases should be considered.

- Case 2a: spectrum that, from a “mobile-satellite” point of view, should be explicitly allocated to AMS(R)S shared only with other “aviation services” (e.g. ARNS, ARNSS, AM(R)S). In other words, an explicit allocation to AMS(R)S

- Case 2b: spectrum that can or could be used by FSS and/or by AMS(R)S related applications through an MSS allocation3, an AMSS allocation3 or an AMS(R)S allocation.

It is worthwhile noting that mobile satellite systems currently providing AMS(R)S applications for manned aircraft are using FSS allocations for the links between the satellite and the earth stations (few large earth stations satisfying the required regulations), therefore are belonging to case 2b. However, such systems are currently used only above low density areas (e.g. oceans) and that they are not the primary means of communication for applications related to the safety of and regularity flight.

While recognizing that, from a civil aviation radio perspective, case 2a seems to be the most appropriate case from a regulatory point of view (and likely the simplest one), the airworthiness certification process of UAS is still at a very early stage of its development and will not be completed by WRC-12, except may be in some countries. As a consequence, WRC-12 should not take decisions that could, de facto, prevent some options within the airworthiness certification process of UAS (some existing or future satellite systems covered by case 2b might satisfy in due time appropriate international aeronautical standards that will be required within the airworthiness certification process of UAS).

Therefore, WRC-12 should allow the two cases, considering that any use of a satellite allocation for the purpose of control and non-payload communication of UAS shall be in accordance with international aeronautical regulations[and if needed through new allocations].

If it is not possible to conclude that a satellite system operating under FSS, AMSS or MSS allocations (case 2b) could meet in due time the appropriate international aeronautical standards, the logical consequence of the above would be that,  “explicit” allocations to AMS(R)S (opposed to AMS(R)S application within AMSS or MSS allocations) should fulfil the spectrum requirement of case 2a.

Existing allocation

Terrestrial component:

DNR ITU-R M.[UAS-BANDS-EXISTING] show that portion(s) of the existing AM(R)S allocation in the band 960-1 164 MHz can be used in some countries to support some UAS terrestrial spectrum requirements under the conditions used in this Report. The band cannot be used to meet the entire 34 MHz terrestrial spectrum requirement for UAS operations, but 10.4 MHz of spectrum within this band would suffice to meet all UAS CNPC requirements except for backup links, video, and downlinking of airborne weather-radar data in some countries.

Due to the intensive use of the band 960 – 1 164 MHz in Europe it is understood that the countries are mentioned above are not in Europe.

Satellite component:

In November 2010, ITU-R has concluded that within the existing AMS(R)S allocation in the band 5000-5150 MHz, the band 5030-5091 MHz can be used to support UAS spectrum requirements. It is possible to design an AMS(R)S system ensuring both that MLS is protected (on the basis of a very large deployment scenario) and that satellite spectrum requirements specified in Report ITU-R M.2171 are accommodated. DNR ITU-R M.[UAS-BANDS-EXISTING] concludes that this safeguards the long-term access to the band for MLS, while enabling an additional aeronautical use of the band, which will increase spectrum efficiency.

[Existing MSS/AMSS and other existing AMS(R)S allocations (UA-Satellite links) and existing FSS/MSS/AMSS and other existing AMS(R)S allocations (UACS-Satellite links) can be used under the current regulation conditions, and therefore no technical studies were required. ]

Additional allocation required

Terrestrial component

Frequency band 15.4-15.5 GHz:

Compatibility studies between AM(R)S and RLS (see WRC-12 AI 1.21)

Studies in the band 15.4-15.5 GHz showed that compatibility of UACS with Radiolocation service receivers would be feasible at separation distances exceeding 400 km but airborne radars would cause interference to UAS airborne receivers at line-of-sight distances over 827 km. Thus sharing the band 15.4-15.5 GHz between UAS and radiolocation radars would be unfeasible (see Report ITU-R M.[UAS-BANDS-NEW-ALLOC]).

Compatibility studies between AM(R)S and FSS

[TBD]

Compatibility studies between AM(R)S and ARNS

Studies conducted in the band 15.4-15.5 GHz show that a separation distance exceeding that of line-of-sight (more than 903 km) is required to provide protection of ARNS systems from interference caused by UAS airborne transmitters. The band sharing would require application of frequency-geographical planning that could be rather difficult for implementation due to a large number of UAS systems envisioned to operate in non-segregated airspace.

Compatibility studies between AM(R)S and radioastronomy service in the adjacent frequency band (15.35-15.4  GHz)

Studies conducted show that compatibility of UAS airborne transmitters operating in the band 15.4-15.5 GHz and RAS station receivers operating in the adjacent band 15.35-15.4 GHz could be unfeasible within line-of-sight distances (464 km) in any azimuth of maximum RAS transmitter antenna pattern.

Portion of the band 5 000-5 150 MHz:

5 000-5 010 MHz:

One administration is currently studying this portion for the UA to the UACS link in particular for small UAS.

Sharing studies show the protection of Radioastronomy below 5 000 MHz is feasible by limiting the out-of-band e.i.r.p. of UAS to -96dBW/10MHz. Using narrow band channel (37.5 kHZ is expected for UAS) will enable the design of selective filter in order to operate small UAS (a small guard band may be necessary)

With regards to protection of RNSS (earth-to-space) in 5 000-5 010 MHz, and of RNSS (space-to-earth) in the adjacent band 5010-5030 MHz, studies are ongoing and are expected to be finalized for ITU-R WG 5B.

5 010-5 030 MHz

Sharing is not possible with the RNSS service link due to the important separation distance which would be required between equipments.

5 030-5 091 MHz

This portion can be considered for both links of the terrestrial component.

Studies performed by the US and presented at the ITU-R WP5B (Annex 7 to Document 5B/617)show that the sharing with MLS is feasible. With regards to AMS(R)S, sharing is more difficult and a segmentation of the band is proposed considering that an AMS(R)S system used for the operation of UAVs will only need 2x20 MHz. The gap of 21 MHz can therefore be used for AM(R)S. However, it will not be sufficient to cover the entire 34 MHz spectrum needed. It should be noted that the US propose to use the band 960-977 MHz in addition to the 5 GHz band to cover the entire demand. However, firstly this band is fully used over Europe (or planned to be by LDACS systems) and secondly accommodating the demand in two different bands will increase the number of equipment to be developed and installed onboard.

Another study confirms that the sharing is possible with both ARNS and AMS(R)S providing that there is a frequency planning between the different equipments. In particular,

- the uplink of the UA CNPC terrestrial link can be in the same subband as the space to earth link of the UA CNPC satellite link, and the downlink of the UA CNPC terrestrial link can be in the same sub-band as the earth to space link of the UA CNPC satellite link.

- In addition, the UA CNPC terrestrial link can also be in any sub-band not used by the UA CNPC satellite link.

5 091-5 150 MHz

The sharing is possible with ARNS and with other AM(R)S equipements (i.e. the ANLE) thanks to appropriate coordination, but will not be possible with AMS allocation used by aeronautical telemetry systems. This band is therefore not proposed to be used by UAS.

Satellite Component

The studies showed that the consideration of the frequency bands 13.25-13.4 GHz as the candidate bands is not reasonable due to unacceptable interference to ARNS systems (airborne Doppler navigation system (ADNS) receivers) and consideration of 15.4–15.7 GHz, 22.5-22.55 GHz, 23.55–23.6 GHz as the candidate bands for the satellite segment of UAS control links as additional allocations to AMS(R )S is possible only after the sharing studies with the existing services are completed.

List of relevant documents

Report ITU-R M.2171 – Characteristics of unmanned aircraft systems (UAS) and spectrum requirements to support their safe operation in non-segregated airspace

Document 5/237 – Draft New Report ITU-R M.[UAS-SENSE AND AVOID] - Characteristics and spectrum considerations for sense and avoid systems use on Unmanned Aircraft Systems (UAS)

Document 5/240 – Draft New Report ITU-R M.[UAS-BANDS-EXIST-ALLOC] - Results of studies of the AM(R)S allocation in the band 960-1 164 MHz and of the AMS(R)S allocation in the band 5 030-5 091 MHz to support control and non-payload communications (CNPC) links for unmanned aircraft systems (UAS)

Document 5B/617 (Annex 7): PDN Report ITU R M.[UAS-BANDS-NEW-ALLOC

Document 5B/617 (Annex 8): PDN Report ITU-R M.[UAS-PERF-AND-REQ]

## Actions to be taken

-

Completing sharing studies for new allocations of terrestrial component

## Relevant information from outside CEPT

European Union

[EC & EDA (date of proposal)]

Regional telecommunication organisations

APT (June 2011)

APT Members are of the view that:

  - The current studies being carried out by ITU-R WP 5B are supported and WP 5B is encouraged to complete the studies during 2011, particularly regarding the possibility of finding additional terrestrial UAS spectrum.

  - The results of studies performed by ITU-R, indicating that 34 MHz of terrestrial and 56 MHz of satellite spectrum are required to provide and ensure safe operation of unmanned aircraft, are supported.

  - The use of existing allocations to AM(R)S, AMS(R)S and/or ARNS, in order to satisfy spectrum needs for UAS safety applications, is supported.  New allocations, including MSS, FSS and AMSS, are therefore not supported at this stage.

  - For satellite UAS the 5030-5091 MHz band is supported.  Method A1 is also supported.

  - For terrestrial UAS the 960-1164 MHz band is supported where conditions allow since the band is heavily used by some APT Members and there are compatibility concerns with existing ARNS systems in the band.  It is therefore felt that additional terrestrial UAS spectrum will be needed.  Method B2 is also supported.

  - Compatibility with existing services, in particular ICAO standards based systems, such as MLS in the 5 GHz band, needs to be ensured.

  - Due to the safety nature of UAS, regulatory provisions associated with the allocation need to be clear, unconditional and implementable in practice.

  - AI 1.3 should be confined to safety communications and not payload.

  - In addition, some APT Members oppose/do not support the use of the following bands for either satellite or terrestrial UAS:  5000-5030 MHz, 5091-5150 MHz, 15.4-15.7 GHz

Arab Group (March 2010)

- The study for these applications shall be limited to frequency bands allocated for aeronautical mobile service (R) and aeronautical mobile-satellite (R) service, providing that the studies prove the possibility of sharing with other existing services in these bands.

the type of Routes to be used for these systems must be determined, and should be provided in a specific Appendix as part of the RR, taking into account the safety and security requirements of other flights as determined by ICAO

CITEL (June 2011)

DIAP:

- ARGENTINA, BRAZIL, MEXICO:

  - NOC for satellite UAS

- ARGENTINA, BRAZIL, CANADA, USA, URUGUAY:

  - MOD Article 5 in the band 5030-5091 MHz to add an AM(R)S allocation

- ARGENTINA, BRAZIL, CANADA:

  - MOD Article 5 in the band 5030-5091 MHz to add an AM(R)S allocation

  - ADD footnote 5.D103 to restrict usage of band by AM(R)S

- CANADA, USA, MEXICO:

  - NOC Article 5 in the band 5091-5150 MHz

Preliminary Proposal:

- CANADA

  - MOD Article 5 to add AM(R)S allocation in 5030-5091 MHz band

  - ADD footnote 5.D103 limiting use by AM(R)S

- MEXICO

  - NOC Satellite component

- CANADA

  - Supports the consideration of a new method to be developed combining Methods A3 and A4 of the CPM

- USA

  - ADD footnotes 5.YYY and 5.ZZZ to allow satellite component

  - ADD Resolution [SAT-UAS-FSS] to support satellite component

RCC (November 2010)

- The allocation in the AM(R)S, AMS(R)S and ARNS bands for UAS safe operation in non-segregated air space is possible on the condition of ensuring compatibility with the existing services.

- 2. The development of appropriate ICAO international standards for the AMS(R)S, AM(R)S and ARNS systems is obligatory since these services are intended for ensuring safety and regularity of flights including on international civil air routes.

- 3. It is not supported the consideration of the following bands as candidate frequency bands for UAS safe operation: 960-1164 MHz, 1525-1559 MHz, 1610-1660.5 MHz, [5030-5091 MHz] and 15.4-15.7 GHz.

International organisations

ITU (January 2011)

The following table presents an analysis of the draft CPM text methods for the satellite component

There are five methods identified in the draft CPM report. Any combination of these methods may be used and the following table summarizes them.

[NOTE: the following table was proposed to be added during PTC-9, it is not shown with revision marks to preserve the colour shading.]

| CPM text Methods | A1 |  | A2 |  | A3 | A4 |  | A5 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Use of the current AMS(R)S allocations for both links (UA to satellite and UACS (mobile and fixed) to satellite) |  | Use of the current MSS, AMSS and AMS(R)S allocations for both links (UA to satellite and UACS (mobile and fixed) to satellite) and the FSS (only for the fixed UACS to satellite link) allocations in accordance with the Radio Regulations |  | Use of the current FSS allocations (except frequency bands covered by RR Appendices 30, 30A and 30B) | Development of a WRC Resolution: to restrict the communication link between UA and satellite to AMS(R)S to confirm the use of AMS(R)S for the radio communication link between UACS and the satellite and to allow the use of the FSS for this link (except frequency bands covered by RR Appendices 30, 30A and 30B) |  | New AMS(R)S allocations in the portions of the [13.25-13.4 GHz], [15.4-15.7 GHz], [22.50-22.55 GHz], and [23.55-23.6 GHz] |  |
| Proposed Radio Regulation (RR) amendments | NOC to the RR |  | NOC to the RR |  | New resolution or recommendation | New resolution |  | New AMS(R)S allocation |  |
| Communication Links | UA-Sat | UACS-Sat | UA-Sat | UACS-Sat | UA-Sat | UA-Sat | UACS-Sat | UA-Sat | UACS-Sat |
| Existing AMS(R)S |  |  |  |  |  |  |  |  |  |
| Existing AMSS/MSS |  |  |  |  |  | Restrict to AMS(R)S |  |  |  |
| Existing FSS bands |  |  |  |  |  | Restrict to AMS(R)S |  |  |  |
| New AMS(R)S |  |  |  |  |  |  |  |  |  |

| Proposed in the method | Not compatible with the method | Not proposed in the method | Not clear |
| --- | --- | --- | --- |

ICAO (Sept 2009)

To support, based on the results of studies identified in Resolution 421, any modification to existing allocations, or new allocations required to accommodate UAS operations in non-segregated airspace while maintaining the safety and regularity of flight of all types of aircraft.

Accordingly, to ensure that allocations used, in particular, for UAS command and control, ATC relay and sense and avoid in non-segregated airspace are in the AM(R)S, AMS(R)S and/or ARNS and do not adversely affect existing aeronautical systems

To oppose the use of this agenda item to seek new spectrum allocations to meet payload requirements.

IATA (21 Sept. 2009)

To support, based on the results of studies, any modification to existing allocations or new allocations required to accommodate UAS operations in non-segregated airspace while maintaining the safety and regularity of flight of all types of aircraft.

Accordingly, to ensure that any AM(R)S, AMS(R)S and ARNS allocations made for UAS command and control, ATC relay and sense and avoid in non-segregated airspace do not adversely affect existing aeronautical systems

To ensure safe and seamless integration of UA in non-segregated airspace, support the agreed requirement for terrestrial and satellite safety spectrum for the provision of ATC relay, command and control and sense and avoid.

To oppose any use of this agenda item to seek new spectrum allocations to meet payload requirements.

IMO (14 Sept 2009)

To oppose any allocation that would affect the interests of the maritime services.

NATO (04 May 2010)

Preliminary NATO Military Position

NATO supports the consideration of spectrum requirements and possible regulatory actions, including allocations, in order to support the safe operation of unmanned aircraft systems (UAS) in the general air traffic (GAT).

For BLOS requirements, NATO is of the view that satellite spectrum for C2, ATC relay and Sense and Avoid should first be accommodated in existing spectrum allocated to the satellite services fulfilling the requirements.

For LOS requirements, NATO supports studies on the 5 GHz and 15.4 GHz bands to accommodate these transmissions.
NATO does not support use of 960-1164 MHz for UAS since this band is already heavily used.
[Some Nations also have concerns on bands 4200-4400 MHz]

It has to be noted that technical specifications mentioning any particular frequency band for insertion into GAT will have to be validated by certification aviation authorities.

If additional bands are required, protection of existing services in those bands must be ensured.

Access to existing frequency bands should not be precluded by any regulatory provision from WRC-12 (always taking into account the protection of existing services and applications).

Further developments are to be monitored in order to promote military use of UAS and protect existing military applications.

Military Importance:  High

SFCG (05 October 09)

SFCG supports the protection of existing space science service allocations.  No allocations of spectrum to support UAS systems should be made in space science service bands unless acceptable sharing criteria are developed.  Airborne system downlinks can be particularly troublesome for space research and EESS receive ground stations.  Airborne system uplinks have the potential to interfere with space-based receivers, especially passive remote sensors.

WMO (12 October 09)

Although no specific frequency bands are mentioned, this agenda item will probably consider spectrum in the range between 2 and 10 GHz in which a number of bands are allocated and used by various meteorological applications.

WMO notes that UAS applications are currently not considered in frequency bands used for meteorological applications.

WMO recognises that UAS have already been operated in the past by some meteorological services and that such vehicles are of interest for future climatological, meteorological and environmental activities, either for research or operational use. It is assumed that these UAS requirements would fit in the current general requirements made for other purposes.

Regional organisations

ASFCG (05 October 2009)

To support, based on the results of studies, any modification to existing allocations or new allocations required to accommodate UAS operations in non-segregated airspace while maintaining the safety and regularity of flight of all types of aircraft.

Accordingly, to ensure that any AM(R)S, AMS(R)S and ARNS allocations made for UAS command and control, ATC relay and sense and avoid in non-segregated airspace do not adversely affect existing aeronautical systems

To ensure safe and seamless integration of UA in the European airspace, support the agreed requirement for terrestrial and satellite safety spectrum for the provision of ATC relay, C2 and S&A.

To oppose any use of this agenda item to seek new spectrum allocations to meet payload requirements.

ESA (November 08)

Same as SFCG position

EUMETNET (12 October 2009)

Same as WMO position

[Eurocontrol (date of proposal)]

[Other relevant information]

____________________

sharing the band with other “mobile-satellite” related allocations.

provided that such an allocation can also be used by satellite links connecting land or ship stations when used in conjunction with this allocation, as there is currently no clear understanding whether the link between the satellite and the UACS can be considered or not as an AMS(R)S feeder-link. An example for such a provision can be found in RR 5.554.

sharing the band with other “mobile-satellite” related allocations.

pending PT-C and ITU developments