| World Radiocommunication Conference (WRC-12) Geneva, 23 January - 17 February 2012 |  |
| --- | --- |
| PLENARY MEETING | Addendum 1 to Document 5(Add.17)-E |
|  | 2011 |
|  | Original: English |
| EUROPEAN COMMON PROPOSALS FOR THE WORK OF THE CONFERENCE |  |
| Agenda item 1.17 (WRC-12): issue A (Broadcasting service) and issue C (fixed service) |  |

- Introduction: methods supported by Europe

1.1 Issue A

1.1.1	Situation internal to the GE-06 area:

For issue A, the GE-06 Agreement is a key regulatory element in the course of this agenda item, for the countries that are party to the agreement. Europe supports “No change” for countries that are party to this agreement (method A1). Three options are associated to method A1 in the draft CPM text. These are in relation with the cumulative effect of the base stations of the mobile service to the broadcasting service, as follows:

Option I: No additional arrangements;

Option II: Optional arrangements to take account of a potential impact of the cumulative effect of interference from the mobile service (MS) to the broadcasting service (BS).  The cumulative effect of interference to the broadcasting service from the identified mobile service could be addressed in a draft Resolution 749 (Rev. WRC-12);

Option III: Mandatory arrangements to take account of a potential impact of the cumulative effect of interference from the MS to the BS. The cumulative effect of interference to the broadcasting service from the identified mobile service is addressed in draft Resolution 749 (Rev. WRC-12) (see § 3/1.17/6).

Europe was strongly involved in carrying out the necessary technical studies about the potential impact of the cumulative effect of interference from base stations, which individually did not trigger the need for coordination with broadcasting. According to the CPM Report to WRC-12, “the studies showed that the potential impact of the cumulative effect of interference from base stations, which individually did not trigger the need for coordination with broadcasting, could be significant. On the other hand, taking into account the elements previously mentioned, the potential impact of cumulative interference might be less significant in practice” (For complete information, see section 3/1.17/4.1 of the CPM Report: http://www.itu.int/md/R07-CPM11.02-R-0001/en).

Therefore, JTG5-6 drew the attention of administrations on this issue and offered the choice between 3 options.

Independently from the options, Europe is of the view that the method to satisfy the agenda item/Issue A within the GE-06 area should comply with the following principles:

- No addition should be made to the GE06 Agreement or the Articles of the Radio Regulations (RR)

- Interference issues should be dealt with on  bilateral and multilateral levels

- In the preparation of bilateral meetings, the results of studies on the impact of cumulative interference carried out in JTG5/6 are available to assist Administrations.

It should also be noted that within the European Union (and in additional European countries), the band 790-862 MHz shall be made available for the mobile service by 2013.

To conclude, Europe supports method A1 option I.

Regarding the protection of the mobile service from the broadcasting service (last Recommends of draft Recommendation [JTG 5-6] (WRC-12) – see final CPM text), Europe remains neutral with respect to adding such a provision in draft Recommendation [JTG 5-6] (WRC-12).Therefore, the draft ECP does not contain it.

1.1.2	Situation at the boundary of the GE-06 area:

Only one method (method A3 = No change) is part of the CPM Report with 2 options:

Option I: No additional arrangements;

Option II: See also draft Recommendation [JTG 5-6] (WRC-12)

Europe acknowledges that option II may ease coordination between countries at the boundary of the GE-06 area. Therefore, the draft ECP contains a recommends (recommends 1) in draft Recommendation [EUR/A117/JTG 5-6] (WRC-12) on that matter.

1.2 Issue C

Only one method (method C = No change) is part of the CPM Report with 2 options:

Option I: No additional arrangements;

Option II: See also draft Recommendation [JTG 5-6] (WRC-12)

1.2.1	Situation internal to Region 1

Europe supports methods C option I.

1.2.2	Situation at the boundary of Region 1 and Region 3

Europe acknowledges that option II may ease coordination between countries of different ITU-R Regions. Therefore, the draft ECP contains a recommends (recommends 3) in draft Recommendation [EUR/A117/JTG 5-6] (WRC-12) on that matter.

ARTICLE  5

Frequency allocations

Section IV  –  Table of Frequency Allocations
(See No. 2.1)

MOD	EUR/XX A17/1

| 460-890 MHz |  |  |  |
| --- | --- | --- | --- |
| Allocation to services |  |  |  |
| Region 1 |  | Region 2 | Region 3 |
| 460-470 FIXED MOBILE 5.286AA Meteorological-satellite (space-to-Earth) 5.287 5.288 5.289 5.290 |  |  |  |
| 470-790 BROADCASTING 5.149 5.291A 5.294 5.296 5.300 5.302 5.304 5.306 5.311A 5.312 |  | 470-512 BROADCASTING Fixed Mobile 5.292 5.293 512-608 BROADCASTING 5.297 608-614 RADIO ASTRONOMY Mobile-satellite except aeronautical mobile-satellite (Earth-to-space) 614-698 BROADCASTING Fixed Mobile 5.293 5.309 5.311A | 470-585 FIXED MOBILE BROADCASTING 5.291 5.298 585-610 FIXED MOBILE BROADCASTING RADIONAVIGATION 5.149 5.305 5.306 5.307 610-890 FIXED MOBILE 5.313A 5.317A BROADCASTING |
|  |  | 698-806 BROADCASTING Fixed MOBILE 5.313B 5.317A |  |
| 790-862 FIXED BROADCASTING MOBILE except aeronautical mobile 5.316B 5.317A 5.312 5.314 5.315 5.316 MOD 5.316A 5.319 ADD 5.A117 |  | 5.293 5.309 5.311A |  |
|  |  | 806-890 FIXED MOBILE 5.317A BROADCASTING |  |
| 862-890 FIXED MOBILE except aeronautical mobile 5.317A BROADCASTING 5.322 |  |  | 5.149 5.305 5.306 5.307 5.311A 5.320 ADD 5.A117 |
| 5.319 5.323 |  | 5.317 5.318 |  |

ADD 	EUR/XX A17/2

5.A117	See Recommendation [EUR/A117/JTG 5-6] (WRC-12), regarding the coordination between the concerned services in the band 790-862 MHz. (WRC-12)

MOD	EUR/XX A17/3

5.316A	Additional allocation:  in Spain, France, Gabon and Malta, the band 790-830 MHz, in Angola, Bahrain, Benin, Botswana, Congo (Rep. of the), French overseas departments and communities of Region 1, Gambia, Ghana, Guinea, Kuwait, Lesotho, Lebanon, Malawi, Morocco, Mauritania, Mozambique, Namibia, Niger, Oman, Uganda, Poland, Qatar, Rwanda, Senegal, Sudan, South Africa, Swaziland, Tanzania, Chad, Togo, Yemen, Zambia and Zimbabwe, the band 790-862 MHz, in Georgia, the band 806-862 MHz, and in Lithuania, the band 830-862 MHz is also allocated to the mobile, except aeronautical mobile, service on a primary basis subject to the agreement by the administrations concerned obtained under No. 9.21 and under the GE06 Agreement, as appropriate, including those administrations mentioned in No. 5.312 where appropriate. See Resolution 749 (Rev.WRC-12). However, stations of the mobile service in the countries mentioned in connection with each band referred to in this footnote shall not cause unacceptable interference to, nor claim protection from, stations of services operating in accordance with the Table in countries other than those mentioned in connection with the band. Frequency assignments to the mobile service under this allocation in Lithuania and Poland shall not be used without the agreement of the Russian Federation and Belarus. This allocation is effective until 16 June 2015. (WRC-12)

ADD	EUR/XX A17/4

Recommendation [EUR/A117/JTG 5-6] (WRC-12)

Use of the band 790-862 MHz in Region 1 and Region 3

The World Radiocommunication Conference (Geneva, 2012),

considering

a)	that the GE06 Agreement covers Region 1 (except Mongolia) and the Islamic Republic of Iran in Region 3;

b)	that in Article 5 of the Radio Regulations the band 790-862 MHz is allocated to several services such as fixed, mobile, broadcasting and aeronautical radionavigation in Regions 1 and 3;

c)	that Resolution 749 (WRC-07) invited ITU-R to conduct sharing studies for Regions 1 and 3 in the band 790-862 MHz between mobile service and other services in order to protect services to which the frequency band is currently allocated;

d)	that this Conference has considered, inter alia, the proposals of administrations and the Report of the Conference Preparatory Meeting CPM to WRC-12 with respect to the results of sharing studies referred to in considering c) above;

e)	that the operation of broadcasting stations and other services, to which the above-mentioned frequency band is allocated in the same geographical area and under certain circumstances may create incompatibility issues,

f)	that the mechanisms of the GE06 Agreement may not adequately protect IMT systems from the future modifications to the Plan for digital television;

g)	that the band 790-862 MHz is used in a number of countries of Region 1 and Region 3 by mobile service, including IMT;

recognizing

a)	that for countries Contracting Members of the GE06 Agreement, the coordination of the services in the band 790-862 MHz is covered by the procedure contained in that Agreement;

b)	that for terrestrial services in frequency bands above 28 MHz, including the band 790-862 MHz, apart from examination under No. 11.31 of conformity with the Table of Frequency Allocations and the other provisions of the Radio Regulations, no other technical and/or regulatory examinations are performed by the Bureau, except for frequency bands which are subject to Regional or World Plan(s) and the bands shared on an equal basis with space services,

recommends

1) to invite administrations, for the coordination between the broadcasting and the mobile services involving on one hand an administration which is a Contracting Member of the GE06 Agreement and on the other hand an administration which is a Non-Contracting Member of the same Agreement, to consider, inter alia, the results of the sharing studies referred to in considerings c) and d) above, on an optional basis and, with mutually agreed criteria, in their bilateral and/or multilateral negotiations/coordination with a view to facilitate the use of the above mentioned band for these services.

2)	intentionally left empty.

3)	to invite administrations for the coordination between the fixed and the mobile service involving one hand an administration of Region 1 and on the other hand an administration of Region 3, to consider, inter alia, the results of the sharing studies referred to in considerings c) and d) above, on an optional basis and, with mutually agreed criteria, in their bilateral and/or multilateral negotiations/coordination3 with a view to facilitate the use of the above mentioned band for these services.

MOD	EUR/XX A17/5

resolution  749  (Rev.wrc-12)

Use of the band 790-862 MHz in countries of Region 1 and the Islamic Republic of Iran by mobile applications
and by other services

The World Radiocommunication Conference (Geneva, 2012),

considering

a)	that the favourable propagation characteristics of the band 470-806/862 MHz are beneficial to provide cost-effective solutions for coverage, including large areas of low population density;

b)	that the operation of broadcasting stations and base stations of the mobile service in the same geographical area may create incompatibility issues;

c)	that many communities are particularly underserved compared to urban centres;

d)	that applications ancillary to broadcasting are sharing the band 470-862 MHz with the broadcasting service in all three Regions, and are expected to continue their operations in this band;

e)	that it is necessary to adequately protect, inter alia, terrestrial television broadcasting and other systems in this band,

recognizing

a)	that, in Article 5 of the Radio Regulations, the band 790-862 MHz, or parts of that band, is allocated, and is used on a primary basis for services including broadcasting;

b)	that the GE06 Agreement applies in Region 1 countries except Mongolia and the Islamic Republic of Iran in the frequency band 174-230/470-862 MHz;

c)	that the transition from analogue to digital television is expected to result in situations where the band 790-862 MHz will be used for both analogue and digital terrestrial transmission; and the demand for spectrum during the transition period may be even greater than the stand-alone usage of analogue broadcasting systems;

d)	the switch-over to digital may result in spectrum opportunities for new applications;

e)	the timing of the switch-over to digital is likely to vary from country to country;

f)	that the use of spectrum for different services should take into account the need for sharing studies;

g)	that the Radio Regulations provide that the identification of a given band for IMT does not preclude the use of that band by any application of the services to which it is allocated and does not establish priority in the Radio Regulations;

h)	that the GE06 Agreement contains provisions for the terrestrial broadcasting service and other terrestrial services, a Plan for digital TV, and the List of other primary terrestrial services,

further recognizing

a)	that the frequency band 790-862 MHz, as part of a wider frequency band, was allocated to mobile service in Region 3 since 1971 (prior to WRC-07);

b)	that the use of the above-mentioned frequency band in Region 3 (including the Islamic Republic of Iran) is only subject to the conformity examination with respect to the Table of Frequency Allocations (No. 11.31 examination) by the Bureau;

c)	intentionally left empty5;

d)	that the GE06 Agreement, in its relevant Annexes, establishes the relation between the digital terrestrial broadcasting, on the one hand, and other primary terrestrial services, including the aeronautical radionavigation service in the countries mentioned in No. 5.312), on the other hand;

e)	intentionally left empty5;

f)	that the band 790-862 MHz in Region 1 and the band 790-806 MHz in Region 3 were identified by the WRC-07 for use by administrations wishing to implement International Mobile Telecommunications (IMT), whereas the band 806-960 MHz in Region 3 was identified for IMT in the WRC-2000;

g)	that for Contracting Members to the GE06 Agreement, the use of stations of the mobile service in relation to the broadcasting services is also subject to the successful application of the procedures of the GE06 Agreement;

h)	that Resolution 749 (WRC-07) resolved to invite ITU-R to conduct sharing studies for Regions 1 and 3 in the band 790-862 MHz between the mobile service and other services in order to protect the services to which the frequency band is currently allocated and to report the results of the studies for consideration by WRC-12 to take appropriate action;

i)	that for countries which are Contracting Members of the GE06 Agreement, the coordination of the mobile service with respect to the digital terrestrial broadcasting service is covered by the procedure contained in that Agreement;

j)	that the coordination between the digital terrestrial broadcasting service in the Islamic Republic of Iran and the primary mobile service in countries that are Contracting Members to the GE06 Agreement is covered by the procedure contained in the GE06 Agreement;

k)	that the coordination between terrestrial services (fixed, mobile and broadcasting) in the frequency band 790-862 MHz between the Islamic Republic of Iran, on the one hand, and the other countries of Region 3, on the other hand, is a matter to be left to the administrations concerned, based on bilateral or multilateral negotiations, if it is mutually agreed by the administrations concerned,

noting

a)	that Resolution ITU-R 57 provides principles for the process of development of IMT-Advanced and this process had already started after WRC-07,

b)	that resolves 4 of Resolution 224 (WRC-07) emphasizes the coordination needed with all neighbouring administrations prior to implementing IMT in the bands 790-862 MHz;

c)	that some countries mentioned in Nos. 5.316 and 5.316A use parts of the band 790- 862 MHz, for public safety services; see recognizing b) of Resolution 224 (WRC-07),

emphasizing

a)	that the use of the band 470-862 MHz by broadcasting and other primary services is also covered by the GE06 Agreement;

b)	that the requirements of the different services to which the band is allocated, including mobile and broadcasting services, shall be taken into account,

taking into account

a)	that the results of the studies carried out by ITU-R pursuant to Resolution 749 (WRC-07) indicates that there is a need to protect certain other primary terrestrial services from the newly allocated mobile service in Region 1,

resolves

1	that in Region 1;

1.1	Intentionally left empty5;

1.2	Intentionally left empty5;

1.3	when the coordination between administrations is being effected, the protection ratios applicable to the generic case NB contained in the GE06 Agreement for the broadcasting service shall be used only for mobile systems with a  bandwidth of 25 kHz. If another bandwidth is used, the relevant protection ratios are to be found in Recommendation ITU-R BT. 1368;

2	that for the Islamic Republic of Iran;

2.1	when the coordination between administrations is being effected, the protection ratios applicable to the generic case NB contained in the GE06 Agreement for the broadcasting service shall be used only for mobile systems with a  bandwidth of 25 kHz. If another bandwidth is used, the relevant protection ratios are to be found in Recommendation ITU-R BT.1368;

3	that with respect to adjacent channel interference;

3.1	that in the band 790-862 MHz, adjacent channel interference within a given country is a national matter and needs to be dealt with by each administration as a national matter;

3.2	that adjacent band interference (below 790 MHz and above 862 MHz) should be treated by administrations concerned, using mutually agreed criteria or those contained in relevant ITU-R Recommendations,

--

instructs the Director of Radiocommunication Bureau

to implement this Resolution and report the results of implementation to WRC-[15/16]

-

Europe acknowledges the situation that there are no existing studies dealing with the protection of Mobile service from broadcasting

Additional modifications may be proposed at a later stage with the consideration of issue B.

This is not coordination referred to in the RR.

Additional modifications may be proposed at a later stage with the consideration of issue B

Additional modifications may be proposed at a later stage with the consideration of issue B.

See also draft Recommendation [EUR/A117/JTG5-6] (WRC-12)