| World Radiocommunication Conference (WRC-12) Geneva, 23 January - 17 February 2012 |  |
| --- | --- |
| PLENARY MEETING | Addendum 10 to Document 5-E |
|  | 2011 |
|  | Original: English |
| EUROPEAN COMMON PROPOSALS FOR THE WORK OF THE CONFERENCE |  |
| PART 10 |  |

Agenda item 1.10

1.10	to examine the frequency allocation requirements with regard to operation of safety systems for ships and ports and associated regulatory provisions, in accordance with Resolution 357 (WRC-07);

Introduction

Taking into account the studies performed during this study period, this ECP proposes the following:

- The regulatory status of the frequencies AIS 1 and AIS 2 is reinforced through the addition of a new provision in Article 5 while maintaining the  existing allocation to the fixed and land  mobile service and; recognizing the use by search and rescue aircraft of the AIS frequencies, as authorized by RR Appendix 18,

- Taking into account Report ITU-R M.2169 and Recommendation ITU-R M.1371-4, it is proposed to identify channels 75 and 76 of RR Appendix 18 for the AIS in order to improve the satellite detection of AIS Message 27.

- It is proposed to make an exclusive primary allocation to the maritime mobile service in the band 495-505 kHz in all three regions and a co-primary allocation in the band 510-525 kHz in Region 2. This will allow the broadcast of safety and security information for ships and ports and harmonized the spectrum for the maritime mobile service in the three Regions,

- The bands 157.025 - 157.325 MHz et 161.625 - 161.925 MHz are identified for the use of new digital technologies,

- The duplex channels 18, 19, 20, 78 and 79 are proposed to be used in simplex. This will offer more availability of channels for the port operation and more flexibility to conduct the day by day operation,

- The frequency 160.900 MHz is identified and reserved for the man over board systems.

ARTICLE 5

Frequency allocations

Section IV  –  Table of Frequency Allocations
(See No. 2.1)

MOD	EUR/XXA10/1

495-1 800 kHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 495-505 MARITIME MOBILE |  |  |
| 505-526.5 MARITIME MOBILE 5.79 5.79A 5.84 AERONAUTICAL RADIONAVIGATION | 505-510 MARITIME MOBILE 5.79 | 505-526.5 MARITIME MOBILE 5.79 5.79A 5.84 AERONAUTICAL RADIONAVIGATION Aeronautical mobile |
|  | 510-525 MARITIME MOBILE 5.79A 5.84 AERONAUTICAL RADIONAVIGATION | Land mobile |
| 5.72 | … |  |
| … |  | … |

Reasons:	Due to further requirements for spectrum to accommodate existing and new maritime systems, which will require more capacity than NAVTEX, SafetyNET or a voice announcement can provide, it is appropriate to make an exclusive primary allocation to the maritime mobile service in the band 495–505 kHz.  A co-primary allocation to the maritime mobile service in the band 510-525 kHz in Region 2 would provide a global harmonized frequency for those new systems.

SUP		EUR/XXA10/2

5.82A

Reasons:	Subsequent suppression due to the exclusive primary allocation to the maritime mobile service

SUP		EUR/XXA10/3

5.82B

Reasons:	Subsequent suppression due to the exclusive primary allocation to the maritime mobile service

MOD		EUR/XXA10/4

148-223 MHz

| Allocation to services |  |  |  |
| --- | --- | --- | --- |
| Region 1 | Region 2 |  | Region 3 |
| … | … |  |  |
| 156.7625-156.8375 MARITIME MOBILE (distress and calling) 5.111 5.226 ADD 5.A110 |  |  |  |
| 156.8375-161.9625 FIXED MOBILE except aeronautical mobile |  | 156.8375-161.9625 FIXED MOBILE |  |
| 5.226 |  | 5.226 |  |
| 161.9625-161.9875 MARITIME MOBILE 5.226 5.227A ADD 5.B110 ADD 5.C110 |  | 161.9625-161.9875 MARITIME MOBILE 5.226 5.227A ADD 5.B110 ADD 5.C110 |  |
| 161.9875-162.0125 FIXED MOBILE except aeronautical mobile 5.226 5.229 |  | 161.9875-162.0125 FIXED MOBILE 5.226 |  |
| 162.0125-162.0375 MARITIME MOBILE 5.226 5.227A 5.229 ADD 5.B110 ADD 5.C110 |  | 162.0125-162.0375 MARITIME MOBILE 5.226 5.227A ADD 5.229 ADD 5.B110 ADD 5.C110 |  |
| 162.0375-174 FIXED MOBILE except aeronautical mobile 5.226 5.227A 5.229 |  | 162.0375-174 FIXED MOBILE 5.226 5.227A 5.230 5.231 5.232 |  |

Reasons:	Exclusive allocation is proposed to improve the regulatory status of AIS1 and AIS 2, taking into accounts the possibility for fixed and mobile allocations to continue to operate under 5.C110.

ADD		EUR/XXA10/5

5.A110		Additional allocation:  the bands 156.7625-156.7875 MHz and 156.8125-156.8375 MHz are also allocated to the mobile-satellite service (Earth-to-space) on a secondary basis for the reception of automatic identification system (AIS) emissions, broadcasting long range AIS messages (Message 27, see the most recent version of Recommendation  ITU-R M.1371), from stations operating in the maritime-mobile service.    (WRC-12)

Reasons:	The bands 156.7625-156.7875 MHz and 156.8125-156.8375 MHz are allocated additionally on a secondary basis to the mobile-satellite service (Earth-to-space) in order to improve the satellite detection of AIS message 27, which is described in Recommendation ITU-R M.1371. This message, which has been developed for this purpose, insures a good detection from the satellite on these two exclusive bands of the maritime mobile service. Report ITU-R M. 2169 shows that the transmission of the message 27 on these bands will not interfere with channel 16. See method B1 of the CPM Report.

ADD		EUR/XXA10/6

5.B110		The bands 161.9625-161.9875 MHz and 162.0125-162.0375MHz may be used by aircraft stations for the purpose of search and rescue operations and other safety related communications.		(WRC-12)

Reasons:	Recognize the use by search and rescue aircraft of the AIS frequencies, as authorized by RR Appendix 18.

ADD		EUR/XXA10/7

5.C110		Additional allocation: In Region 1 the bands 161.9625-161.9875 MHz and 162.0125-162.0375MHz are also allocated to the fixed and land mobile services on a primary basis. In region 2 and 3 the bands 161.9625-161.9875 MHz and 162.0125-162.0375MHz are also allocated to the fixed, land mobile and aeronautical services on a primary basis. The use of these bands by the fixed, land mobile and aeronautical services shall not cause harmful interference to nor claim protection from the maritime mobile VHF radiocommunication service.	(WRC-12)

Reasons:	Radio Regulations recognize the AIS-SART operation as having a safety function on the two AIS frequencies; see RR Appendix 15 (Rev.WRC-07)).

Additional protection, similar to that in No.5.227, is given to AIS frequencies which are used for search and rescue, safety of navigation, ship movement and tracking of vessels, as well as used by search and rescue aircraft authorized by RR Appendix 18 and the latest version of Recommendation ITU-R M.1371.

The proposed provision maintains the existing frequency assignments for fixed and land mobile stations.

MOD		EUR/XXA10/8

APPENDIX  18  (Rev.WRC-12)

Tables of transmitting frequencies in the
VHF maritime mobile band

(See Article 52)

NOTE A – For assistance in understanding the Tables, see Notes a) toHHH) below.	(WRC-12)

NOTE B – Table 1 below defines the channel numbering for maritime VHF communications based on 25 kHz channel spacing and use of several duplex channels. The channel numbering and the conversion of two-frequency channels for single-frequency operation shall be in accordance with Recommendation ITU-R M.1084-4 Annex 4, Tables 1 and 3.  The table below describes also the harmonized band where the digital technologies defined in Rec. ITU-R M.1842 could be deployed.		(WRC-12)

NOTE C – Table 2 defines the channel numbering based on 25 kHz channel spacing for the digital band or for analogue usage, see footnote EEE) and HHH).	(WRC-12)

TABLE 1

| Channel designator | Notes | Transmitting frequencies (MHz) |  | Inter-ship | Port operations and ship movement |  | Public correspondence |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | From ship stations | From coast stations |  | Single frequency | Two frequency |  |
| 60 | m) | 156.025 | 160.625 |  | x [*] | x | x |
| 01 | m) | 156.050 | 160.650 |  | x [*] | x | x |
| 61 | m) | 156.075 | 160.675 |  | x | x | x |
| 02 | m) | 156.100 | 160.700 |  | x | x | x |
| 62 | m) | 156.125 | 160.725 |  | x | x | x |
| 03 | m) | 156.150 | 160.750 |  | x | x | x |
| 63 | m) | 156.175 | 160.775 |  | x | x | x |
| 04 | m) | 156.200 | 160.800 |  | x | x | x |

| Channel designator | Notes | Transmitting frequencies (MHz) |  | Inter-ship | Port operations and ship movement |  | Public correspondence |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | From ship stations | From coast stations |  | Single frequency | Two frequency |  |
| 64 | m) | 156.225 | 160.825 |  | x | x | x |
| 05 | m) | 156.250 | 160.850 |  | x | x | x |
| 65 | m) | 156.275 | 160.875 |  | x | x | x |
| 06 | f) | 156.300 |  | x |  |  |  |
| MOB | AAA) | 160.900 |  | x | x |  |  |
| 66 | m) | 156.325 | 160.925 |  | x [*] | x | x |
| 07 | m) | 156.350 | 160.950 |  | x [*] | x | x |
| 67 | h) | 156.375 | 156.375 | x | x |  |  |
| 08 |  | 156.400 |  | x |  |  |  |
| 68 |  | 156.425 | 156.425 |  | x |  |  |
| 09 | i) | 156.450 | 156.450 | x | x |  |  |
| 69 |  | 156.475 | 156.475 | x | x |  |  |
| 10 | h), q) | 156.500 | 156.500 | x | x |  |  |
| 70 | f), j) | 156.525 | 156.525 | Digital selective calling for distress, safety and calling |  |  |  |
| 11 | q) | 156.550 | 156.550 |  | x |  |  |
| 71 |  | 156.575 | 156.575 |  | x |  |  |
| 12 |  | 156.600 | 156.600 |  | x |  |  |
| 72 | i) | 156.625 |  | x |  |  |  |
| 13 | k) | 156.650 | 156.650 | x | x |  |  |
| 73 | h), i) | 156.675 | 156.675 | x | x |  |  |
| 14 |  | 156.700 | 156.700 |  | x |  |  |
| 74 |  | 156.725 | 156.725 |  | x |  |  |
| 15 | g) | 156.750 | 156.750 | x | x |  |  |
| 75 | n), BBB) | 156.775 | 156.775 |  | x |  |  |
| 16 | f ) | 156.800 | 156.800 | DISTRESS, SAFETY AND CALLING |  |  |  |
| 76 | n), BBB) | 156.825 | 156.825 |  | x |  |  |
| 17 | g) | 156.850 | 156.850 | x | x |  |  |
| 77 |  | 156.875 |  | x |  |  |  |
| 18 | CCC) | 156.900 | 161.500 |  | x | x | x |
| 1018 |  | 156.900 | 156.900 |  | x |  |  |
| 2018 |  | 161.500 | 161.500 |  | x |  |  |
| 78 | CCC) | 156.925 | 161.525 |  | x [*] | x | x |
| 1078 |  | 156.925 | 156.925 |  | x |  |  |
| 2078 |  | 161.525 | 161.525 |  | x |  |  |
| 19 | CCC) | 156.950 | 161.550 |  | x [*] | x | x |
| 1019 |  | 156.950 | 156.950 |  | x |  |  |
| 2019 |  | 161.550 | 161.550 |  | x |  |  |
| 79 | CCC) | 156.975 | 161.575 |  | x [*] | x | x |
| 1079 |  | 156.975 | 156.975 |  | x |  |  |
| 2079 |  | 161.575 | 161.575 |  | x |  |  |
| 20 | CCC) | 157.000 | 161.600 |  | x [*] | x | x |
| 1020 |  | 157.000 | 157.000 |  | x |  |  |
| 2020 |  | 161.600 | 161.600 |  | x |  |  |

| Channel designator | Notes | Transmitting frequencies (MHz) |  | Inter-ship | Port operations and ship movement |  | Public correspondence |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | From ship stations | From coast stations |  | Single frequency | Two frequency |  |
| Digital Band | DDD) EEE) FFF) GGG) | 157.025 to 157.325 | 161.625 to 161.925 |  | x | x |  |
| 27 |  | 157.350 | 161.950 |  |  | x | x |
| 87 |  | 157.375 | 157.375 |  | x |  |  |
| 28 |  | 157.400 | 162.000 |  |  | x | x |
| 88 |  | 157.425 | 157.425 |  | x |  |  |
| AIS 1 | f ), l), p) | 161.975 | 161.975 |  |  |  |  |
| AIS 2 | f ), l), p) | 162.025 | 162.025 |  |  |  |  |

Reasons:

The proposed changes in Appendix 18 are in line with method B1 of the CPM Report: identification of channels 75 and 76 to be used by the mobile –satellite service (Earth-to-space); method C: broadcasts of safety and security information for ships and ports; method D1and D2: introduction of a duplex digital band and identification of additional simplex channels.

* "The Radio Regulations Board approved a Rule of Procedure after WRC-07 regarding simplex use in Appendix 18 (re. RRB08/49, see Circular Letter CR/296), effectively implementing this part of the proposal. WRC-07 revised Appendix 18 to allow simplex use of channels 01, 07, 19, 20, 21, 60, 66, 78, 79, 80, and 81 subject to coordination with affected administrations (Note m)). However, WRC-07 omitted placing an "x" in the "Single frequency" column against affected channels in Appendix 18, thereby unintentionally omitting this from the Radio Regulations. Therefore this method included this Rule of Procedure."

Notes referring to the Table

General notes

NOC		a) to e)

Specific notes

NOC		f) to n)

SUP		EUR/XXA10/9

o)

Reasons:	This note is not necessary any more, due to the creation of the new digital band, see also note DDD), EEE), FFF) and GGG)

NOC		p) to q)

ADD		EUR/XXA10/10

AAA)		In the maritime mobile service, the frequency 160.900 MHz is designated solely for use by the man overboard (MOB) systems.	(WRC-12)

Reasons:	This note identifies a worldwide frequency for the man overboard (MOB) systems using RR Appendix 18 frequencies.

ADD		EUR/XXA10/11

BBB) 	Additionally, these channels (75 and 76) may be used by the mobile-satellite service (Earth-to-space) for the reception of long range AIS broadcast messages from ships (Message 27; see the most recent version of Recommendation ITU-R M.1371).	(WRC-12)

Reasons:	This note is a consequence to the change in article 5 for method B1, see No. 5.A110 above.

ADD		EUR/XXA10/12

CCC)		Until 1 January 2017, the existing duplex channels 18-20 and 78-79 can continue to be assigned. From that date, no new coast station assignments for duplex mode are permitted, and the frequencies associated with channels 18-20 and 78-79 shall only be available as single-frequency channels. Single-frequency channels shall be identified by the channel number designation in accordance with Recommendation ITU-R M.1084.     (WRC-12)

Reasons:	 This provisions set up the conditions of assignment for these channels after the date of implementation.

ADD		EUR/XXA10/13

DDD)	Until 1 January 2017, this duplex band is used for analogue communications using the channelling arrangement described in Table 2. 	(WRC-12)

Reasons: 	This note will protect the existing usage until the date of implementation.

ADD		EUR/XXA10/14

EEE)		From 1 January 2017, the duplex bands (157.025 to 157.325 MHz and 161.625 to 161.925 MHz) are identified for the utilisation of the digital systems described in the most recent version of Recommendation ITU-R M.1842. This band could also be used for analogue modulation described in the most recent version of Recommendation ITU-R M.1084 by the administration that wishes to do so, subject to not claiming protection from other stations in the maritime mobile service using digitally modulated emissions. See Table 2 for the channel numbering based on 25 kHz channel spacing for the digital or for analogue usage. 	(WRC-12)

Reasons: 	This note will authorize the administrations that wish to do so to continue to use analogue technologies

ADD		EUR/XXA10/15

FFF)		In the United States and Canada, only the bands 157. 200 – 157.325 and 161.800 – 161.925 MHz (corresponding to the channels: 24, 84, 25, 85, 26 and 86; see Table 2) are authorized for digitally modulated emissions. 	(WRC-12)

Reasons:	This note is extracted from the CPM text (note DDD)  and is specifically designed for the United States and Canada.

ADD		EUR/XXA10/16

GGG)	In China, only the bands 157.150-157.325 and 161.750-161.925 MHz (corresponding to the channels: 23, 83, 24, 84, 25, 85, 26 and 86; see Table 2) are authorized for digitally modulated emissions.    (WRC-12)

Reasons:	This note is extracted from the CPM text (note FFF) and is specifically designed for China.
ADD		EUR/XXA10/17

Table 2

Channel numbering based on 25 kHz channel spacing for
the digital band (narrow band usage) or
for analogue usage

| Channel designator | Notes | Transmitting frequencies (MHz) |  | Inter-ship | Port operations and ship movement |  | Public correspondence |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | From ship stations | From coast stations |  | Single frequency | Two frequency |  |
| 80 | HHH) | 157.025 | 161.625 |  | x | x |  |
| 21 | HHH) | 157.050 | 161.650 |  | x | x |  |
| 81 | HHH) | 157.075 | 161.675 |  | x | x |  |
| 22 | HHH) | 157.100 | 161.700 |  | x | x |  |
| 82 | HHH) | 157.125 | 161.725 |  | x | x |  |
| 23 | HHH) | 157.150 | 161.750 |  | x | x |  |
| 83 | HHH) | 157.175 | 161.775 |  | x | x |  |
| 24 | HHH) | 157.200 | 161.800 |  | x | x |  |
| 84 | HHH) | 157.225 | 161.825 |  | x | x |  |
| 25 | HHH) | 157.250 | 161.850 |  | x | x |  |
| 85 | HHH) | 157.275 | 161.875 |  | x | x |  |
| 26 | HHH) | 157.300 | 161.900 |  | x | x |  |
| 86 | HHH) | 157.325 | 161.925 |  | x | x |  |

ADD		EUR/XXA10/18

HHH) 	These channels may be operated as single and duplex frequency channels, subject to coordination with affected administrations.	(WRC-12)

Reasons:	This table allows the administrations to continue to use those duplex channels as they are for radio telephony analogue communications or for digitally modulated emissions using narrow band technologies.

______________________