| World Radiocommunication Conference (WRC-12) Geneva, 23 January - 17 February 2012 |  |
| --- | --- |
| PLENARY MEETING | Addendum 12 to Document 5-E |
|  | 2011 |
|  | Original: English |
| EUROPEAN COMMON PROPOSALS FOR THE WORK OF THE CONFERENCE |  |
| part 12 |  |

Agenda item 1.12

1.12	to protect the primary services in the band 37-38 GHz from interference resulting from aeronautical mobile service operations, taking into account the results of ITU-R studies, in accordance with Resolution 754 (WRC-07);

Introduction

The band 37-38 GHz is allocated on a primary basis to the fixed and mobile services, and the 37.5-38 GHz portion of this band is also allocated on a primary basis to the fixed-satellite service (space-to-Earth). The band 37-38 GHz is also allocated on a primary basis to space research service (space-to-Earth) and is required to support the increased data requirements of planned manned and scientific missions.

Resolution 754 (WRC-07) resolves:

1	to invite ITU-R to conduct appropriate studies involving the aeronautical mobile service and the affected primary services in the band 37-38 GHz in order to determine the compatibility of the aeronautical mobile service with these other services;

2	to invite WRC-11 to review the results of the studies under resolves 1 and consider the inclusion of any appropriate compatibility criteria within the Radio Regulations or appropriate modifications to the Table of Frequency Allocations,

Studies have shown that in order to protect all other primary services allocated in the band 37-38 GHz from harmful interference from the aeronautical mobile service, any aircraft station would have to meet the following pfd mask:

–180 – 9.4 · θ 		dB(W/(m2 · Hz))		for  0˚ ≤ θ ≤ 5˚

–227 			dB(W/(m2 · Hz))		for  5˚ < θ

in any 1 Hz bandwidth, where θ is the angle of arrival of the incident wave above the horizontal plane at the space research service earth station, in degrees. These limits relate to the power flux-density that would be obtained assuming free-space loss.

This pfd mask is very stringent and will even prevent low power applications such as WAIC, with a power spectral density as low as -16 dBm/MHz to operate in the band.

Studies have also shown that aeronautical mobile receivers will suffer from very high levels of interference (I/N of -10 dB exceeded for more than 40% of the time) from high density of fixed service (HDFS) links deployed worldwide in the band 37-38 GHz, unless the AMS use is limited to intra-aircraft transmissions. However, WAIC are also planned to operate outside the aircraft and such application would therefore be subject to the same interference level.

Due to the fact that no AMS application would be able to meet the pfd mask defined for the protection of other primary services and that AMS receivers would suffer from harmful interference from HDFS, CEPT is of the opinion that the allocation to the mobile service cannot be used by the aeronautical mobile service. CEPT therefore proposes to restrict this allocation to land and maritime mobile.

ARTICLE 5

Frequency allocations

Section IV  –  Table of Frequency Allocations
(See No. 2.1)

MOD 	EUR/XXA12/1

34.2-40 GHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 37-37.5 FIXED MOBILE except aeronautical mobile SPACE RESEARCH (space-to-Earth) 5.547 |  |  |
| 37.5-38 FIXED FIXED-SATELLITE (space-to-Earth) MOBILE except aeronautical mobile SPACE RESEARCH (space-to-Earth) Earth exploration-satellite (space-to-Earth) 5.547 |  |  |

Reasons:	To limit the allocation to the mobile service to land and maritime mobile.

SUP 	EUR/XXA12/2

RESOLUTION  754  (WRC-07)

Consideration of modification of the aeronautical component of
the mobile service allocation in the 37-38 GHz band for
protection of other primary services in the band

Reasons:	Completion of studies required under this resolution.