| World Radiocommunication Conference (WRC-12) Geneva, 23 January - 17 February 2012 |  |
| --- | --- |
| PLENARY MEETING | Addendum 16 to Document 5-E |
|  | 2011 |
|  | Original: English |
| EUROPEAN COMMON PROPOSALS FOR THE WORK OF THE CONFERENCE |  |
| part 16 |  |

Agenda Item 1.16

1.16	to consider the needs of passive systems for lightning detection in the meteorological aids service, including the possibility of an allocation in the frequency range below 20 kHz, and to take appropriate action, in accordance with Resolution 671 (WRC-07);

Introduction

Resolution 671 (WRC 07), Recognition of systems in the meteorological aids service

in the frequency range below 20 kHz, invites ITU-R to conduct, and complete in time for WRC-12, the required studies leading to technical and procedural recommendations to the Conference enabling it to decide on an appropriate method of providing recognition to long-established systems, including the possibility of making an allocation to the meteorological aids service in the frequency range below 20 kHz;

The studies shall include sharing and compatibility studies with services already having allocations in potential spectrum for systems in the meteorological aids service taking into account the needs of other services, without placing constraints on existing services operating in accordance with the Radio Regulations.

Background

Long-range lightning detection, using observations near 10 kHz, has been performed since 1939.  The measurement system has evolved to become a distributed network of ground-based sensors using that can locate the origin of the lightning stroke, using Arrival Time Difference (ATD) of the lightning emission received at the individual sensor sites.

However, as the system relies on naturally occurring emissions it can be badly compromised by a few interference sources, which can result in the total loss of data to the system.

The lightning detection systems have co-existed with radio navigation service for many years. However, the occurrence of interference from other applications has recently become more evident, increasing concerns that lightning detection systems may not be able to maintain the current quality of service or to provide global coverage in the absence of recognition afforded to these systems within the Radio Regulations.

Given the nature of Radionavigation Service and the Meteorological Aids Service, using ATD sensors, operating in this frequency band, the low density levels and the static nature of station deployments, and that this environment were to continue, it can be concluded that sharing between these two services is feasible and practical at large separation distances between the MetAids stations and RNS stations or with mitigation technique used by MetAids stations. Based on the above-mentioned, in case the frequency band 9–11.3 kHz is allocated to the MetAids on a co-primary basis with the existing allocation to the RNS, MetAids shall not constrain the operation of RNS systems submitted for notification prior to the date of entry into force of the provisions contained in the WRC-12 Final Acts.

Moreover while extending the MetAids allocation in the frequency band 8.3–9 kHz it should be noted that other services have already operated in this frequency band on the national basis. A national footnote for additional allocations to other services between 8.3 and 9 kHz is required to ensure protection of these services.

ARTICLE 5

Frequency allocations

Section IV  –  Table of Frequency Allocations

(See No. 2.1)

MOD		EUR/XXA16/1

9-110 kHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| Below 8.3 (Not allocated) MOD 5.53 MOD 5.54 |  |  |
| 8.3 – 9 METEOROLOGICAL AIDS ADD 5.А116 ADD 5.B116 |  |  |
| 9-11.3 METEOROLOGICAL AIDS ADD 5.А116 RADIONAVIGATION |  |  |
| 11.3-14 RADIONAVIGATION |  |  |
| 14-19.95 FIXED MARITIME MOBILE 5.57 5.55 5.56 |  |  |
| . . . |  |  |

MOD		EUR/XXA16/2

5.53 	Administrations authorizing the use of frequencies below 8.3 kHz shall ensure that no harmful interference is caused thereby to the services to which the bands above 8.3 kHz are allocated.

MOD		EUR/XXA16/3

5.54	Administrations conducting scientific research using frequencies below 8.3 kHz are urged to advise other administrations that may be concerned in order that such research may be afforded all practicable protection from harmful interference.

ADD		EUR/XXA16/4

5.А116	 	Use of the 8.3-11.3 kHz frequency range by MetAids stations is limited to passive use only. In the band 9-11.3 kHz, the MetAids stations shall not claim protection from stations of the radionavigation service submitted for notification to the Bureau prior to [the date of entry into force of the provisions contained in the WRC-12 Final Acts].For sharing between stations of the meteorological aids service and stations in the radionavigation service submitted after this date the most recent version of Recommendation ITU-R RS.1881 should be applied.

Reasons:  	The results of the studies indicate that the co-existence between the radionavigation service and ATD sensors is possible, not only from a theoretical basis as shown in the studies but also by the successful operation of the ATD systems over many years.

ADD		EUR/XXA16/5

5.B116 	Additional allocation: in the Russian Federation, the frequency band 8.3-9 kHz is also allocated to the radionavigation, fixed and mobile services on a primary basis.

SUP		EUR/XXA16/6

RESOLUTION 671 (WRC-07)

Recognition of systems in the meteorological aids service

in the frequency range below 20 kHz

Reasons: 	Completion of studies required under this resolution.