| World Radiocommunication Conference (WRC-12) Geneva, 23 January - 17 February 2012 |  |
| --- | --- |
| PLENARY MEETING | Addendum 2 to Document 5-E |
|  | 2011 |
|  | Original: English |
| EUROPEAN COMMON PROPOSALS FOR THE WORK OF THE CONFERENCE |  |
| part 2 |  |
| Agenda item 1.2 1.2 taking into account the ITU-R studies carried out in accordance with Resolution 951 (Rev.WRC-07), to take appropriate action with a view to enhancing the international regulatory framework; |  |

Introduction

This ECP on agenda item 1.2 contains four issues:

- Issue A – General allocation issues

- Issue B – Terrestrial in particular fixed and mobile convergence

- Issue C – Satellite convergence

- Issue D – Suppression of Resolution 951 (Rev. WRC-07)

Issue A – General allocation issues

Introduction

Issue A consists of proposals with regard to principles of allocating spectrum, which are intended to enhance the Radio Regulations.

ADD	EUR/XXA2/1

DRAFT RESOLUTION  [EUR/A12]  (WRC-12)

Principles for the allocation of frequency bands

The World Radiocommunication Conference (Geneva, 2012),

considering

a)	that ITU should maintain an international Table of Frequency Allocations covering the usable radio-frequency spectrum;

b)	that it may be desirable, in certain cases, to allocate frequency bands to the most broadly defined services of Article 1 in order to improve flexibility of use but without detriment to other services;

c)	that the development of common worldwide allocations is desirable in order to improve and harmonize utilization of the radio-frequency spectrum;

d)	that adherence to these principles for the allocation of spectrum will allow the Table of Frequency Allocations to focus on matters of regulatory significance while enabling greater flexibility in national spectrum use;

e) 	that ITU should promote the introduction of  new applications to address issues such as emerging technologies, climate change (e.g. collection of Earth observation data), disaster managementand other socio-economic matters;

f) 	that flexibility and harmonization of spectrum usage are important factors to support bridging the digital divide and bringing the benefits of ICT (Information and Communication Technologies)to all citizens;

g) 	that ensuring connectivity to schools, rural communities and health facilities is vital to economic development and to making effective use of ICTs and that increased deployment of affordable broadband will help facilitate this;

h)	that wireless applications, either terrestrial or satellite, are often the most cost-effective and practicable means of delivering advanced ICT in many countries;

i)	that there is a need to continually take advantage of technological developments in order to increase the efficient use of spectrum and facilitate spectrum access;

j)	that Recommendation ITU-R SM.1133 provides a guide to the use of broadly defined services,

recognizing

a)	that in adjacent Regions, sub-Regions or countries, when a frequency band is allocated to different services of the same category, the basic principle is the equality of right to operate among different services of the same category (see No. 4.8);

- that Resolution 26 (Rev.WRC-07) provides guidelines for the use of footnotes, including additions, modifications or deletions;

resolves to invite future World Radiocommunication Conferences

1	to allocate frequency bands to the most broadly defined services and adopt associated regulatory provisions, if needed, with a view to providing the maximum flexibility to administrations in spectrum use, taking into account all relevant factors, including safety, technical (including compatibility and sharing studies), operational, and economic elements;

2	to allocate frequency bands on a worldwide basis (aligned services, categories of service and frequency band limits) taking into account all relevant factors including, safety, technical (including compatibility and sharing studies), operational, and economic elements;

3	to allocate frequencies by including the name of the service explicitly in the table of frequency allocations of Article 5 and to the extent possible not by footnotes;

4	to take into account relevant studies by the Radiocommunication Sector, including report(s) of the relevant Conference Preparatory Meeting(s) (CPM) and technical and operational developments, forecasts and usages contributed by the members in applying resolves 1, 2 and 3;

5	to take into account the use of radiocommunications in achieving national, regional and global priorities, including relevant ITU Plenipotentiary Conference and WRC Resolutions on addressing the digital divide, emerging technologies, climate change (e.g. collection of Earth observation data), disaster management and other socio-economic issues,

resolves to invite administrations

to take account of resolves 1 to 5 and considering a) to j) when making proposals to World Radiocommunication Conferences,

invites ITU-R

1	 to identify areas for study and to undertake the studies necessary to determine the impact on existing services of those agenda items of future world radiocommunication conferences which involve broadening the scope of existing service allocations;

2	when carrying out technical studies relating to a frequency band, to examine the compatibility of a broadly defined service with the existing utilizations and the possibility of aligning allocations on a worldwide basis, having regard to considerings a) to j) and resolves 1 to 5 above;

3	to report the result of these studies through the Director of the Bureau to future World Radiocommunication Conferences,

instruct the Secretary-General

to communicate this Resolution to ICAO, IMO, WMO and other international organizations concerned,

resolves to invite the Director of the Radiocommunication Bureau

to bring this Resolution to the attention of ITU-T and ITU-D.

Reasons: 	Europe notes the principles for the allocation of frequency bands contained in Recommendation 34 (WRC-95).  This recommendation supports the harmonisation of the spectrum use, flexibility to allow new services into bands already allocated to other services and to allocate frequency bands to the broadest defined services.

Europe is of the view that these principles should be further enhanced by amending the recommendation and “upgrade” it to a resolution.

Europe proposes the adoption of a new Resolution in order to strengthen the principles for the allocation of frequency bands. This should be seen as one way to satisfy the agenda item and it has to be combined with other methods in order to be efficient.

SUP	EUR/XXA2/2

RECOMMENDATION  34  (WRC-95)

Principles for the allocation of frequency bands

Reasons: 	Consequential change.

Issue B – Terrestrial in particular fixed and mobile convergence

Introduction

Issue B deals with convergence of fixed and mobile (except aeronautical and maritime mobile) services. Convergence, in this context, refers to the way in which the same radio applications may operate under the fixed and mobile services. Allocations in the Radio Regulations are generally made based on successful results of technical sharing or compatibility studies based on the assumption of some form of operational environment. Each station needs to be classified under a specific radiocommunication service and to undergo the relevant processes for coordination and notification.

However, there is overlap in classifying under which radio service a radio station should be notified. There are also differences in the notification process especially for radio applications using a point-multipoint (P-MP) or point-area (P-Area) network topology.

As a result, fixed and mobile services are often allocated in the same frequency band, thus giving administrations flexibility to allow operators to deploy fixed or mobile stations, as appropriate. An analysis of the frequency bands between 30 MHz and 5 GHz shows that this joint allocation is already in place in practice in most cases.

For fixed wireless access, which is one subset of a convergent network, as demonstrated by the review of the procedures within ITU-R, it is possible to notify and coordinate the corresponding stations.

Therefore, the issue is how convergence between fixed and mobile services (i.e. frequency band allocated on an equal basis to both services and procedures apply in a similar way) can be accommodated while keeping the sharing environment with other radiocommunication services (e.g. space services) unchanged.

In that respect, Europe proposes to modify the current definition of the fixed service (No. 1.20)  to state more explicitly that between two fixed stations, one of the two  could be at a specified fixed point or any fixed point in a specified area  In any case, the other one remains at a specified fixed point. A consequential change to the fixed station definition (No. 1.66) is also proposed. This method proposes also to modify accordingly different provisions of the Radio Regulations (No.11.9) and Table 1 of Appendix 4.

To keep the current balance between terrestrial (FS) and satellite services (e.g. FSS) while accommodating convergence in terms of notification, the following actions are proposed:

- Data items of transmitting stations for the application of No. 11.2 are harmonised to the extent possible (e.g. description of a receiving area is enhanced by defining a receiving area as a discus in addition to a polygon)

- In bands shared with satellite services, data items related to the notification of fixed station shall remain on a known location basis (i.e. with individual point-point (P-P) transmission links).

- In bands not shared with satellite services, data items of receiving land station on an area basis for the application of No. 11.9 are extended to its counterpart in the fixed service (e.g. multiple fixed stations transmitting towards the same fixed station (central station) can be notified on an area basis).

For broadband wireless access, which is a convergent network encompassing fixed, nomadic and mobile terminals, it is possible to notify under the mobile service.

MOD		EUR/XXA2/3

1.20		fixed service:  A radiocommunication service between a fixed station at a specified fixed point and one or more fixed stations at given locations; a given location may be a specified fixed point or any fixed point within a specified area.

Reasons: 	This definition clarifies the scope of applications allowed with the current definition. This change does not narrow the scope of applications currently eligible to the fixed radiocommunication service.

MOD		EUR/XXA2/4

1.66		fixed station:  A station that operates at a fixed point in the fixed service.

Reasons: 	This proposed change is consequential to the one proposed on No. 1.20.

MOD		EUR/XXA2/5

11.9		Similar notification shall be made for a frequency assignment to a receiving earth station or space station, or to a receiving high altitude platform station in the fixed service using the bands mentioned in Nos. 5.543A and 5.552Aor to a land station for reception from mobile stations or, for bands not allocated in Article 5 also to space services, to a fixed station for reception from multiple fixed stations1, when:     (WRC-12)

ADD		EUR/XXA2/6

——————————————

1	11.A12		For the conditions mentioned in No. 21.2 or No. 21.4 or in frequency bands allocated to space services, individual notices of frequency assignments of fixed stations are necessary.

Reasons: It is proposed to extend the possibility to notify Area-P transmission links of the fixed service (with the understanding that all subscribers in the specified area are all fixed but at unspecified/unknown locations). This extension will be limited to the bands not allocated to space services. For this case (i.e. band allocated to fixed and space services, frequency assignments of a receiving fixed station shall be done from specified fixed points in accordance with No. 11.2 (i.e. MP-P transmission links are notified with as many individual P-P links as fixed MP-P system has got).

MOD		EUR/XXA2/7

The proposed changes concerned sections 4, 5, 8 and 12 of Appendix 4. The proposed changes are provided in the extract of Appendix 4 below.

APPENDIX 4 (REV.WRC-07)

Consolidated list and tables of characteristics for use in the
application of the procedures of Chapter II

ANNEX 1

Characteristics of stations in the terrestrial services TABLE 1

Characteristics for terrestrial services

| Column No. | Item identifier | Notice related to Description of data items and requirements | Broadcasting (sound and television) stations in the VHF/UHF bands up to 960 MHz, for the application of No. 11.2 and No. 9.21 | Broadcasting (sound) stations in the LF/MF bands, for the application of No. 11.2 | Transmitting stations (except broadcasting stations in the planned LF/MF bands, in the HF bands governed by Article 12, and in the VHF/UHF bands up to 960 MHz), for the application of No. 11.2 and No. 9.21 | Receiving land stations or receiving fixed stations, for the application of No. 11.9 and No. 9.21 | Typical transmitting stations, for the application of No. 11.17 | Maritime mobile frequency allotment, for the application of plan modification under Appendix 25 (Nos. 25/1.1.1, 25/1.1.2, 25/1.25) | Broadcasting stations in the HF bands, for the application of No. 12.16 | Item identifier |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 4 |  | LOCATION OF THE TRANSMITTING ANTENNA(S) |  |  |  |  |  |  |  |  |
| 4.7 |  | For an area in which transmitting stations operate: |  |  |  |  |  |  |  |  |
| 4.7.1 | 4CC | the geographical coordinates of the centre of the circular zone, in which mobile transmitting stations associated with a receiving land stationor transmitting fixed stations at an unspecified location associated with the same receiving fixed station, or a typical transmitting station are operating Latitude and longitude are provided in degrees, minutes and seconds In the case of a receiving land station, or receiving fixed station from stations at fixed point within a specified area,required: – for the maritime radionavigation service; and – for other services if neither the code of a geographical area or standard defined area (4E) nor the geographical coordinates of a given emission zone (4F) is provided In the case of a typical transmitting station, required if neither a geographical area or standard defined area (4E) nor the geographical coordinates of a given emission zone (4F) is provided |  |  |  | + | + |  |  | 4CC |
| 4.7.2 | 4D | the nominal radius, in km, of the circular zone, in which mobile transmitting stations associated with a receiving land station, or transmitting fixed stations at an unspecified location associated with the same receiving fixed station,or a typical transmitting station are operating In the case of a receiving land station, or receiving fixed station from stations at fixed point within a specified area,required: – for the maritime radionavigation service; and – for other services if neither the code of a geographical area or standard defined area (4E) nor the geographical coordinates of a given emission zone (4F) is provided In the case of a typical transmitting station, required if neither a geographical area or standard defined area (4E) nor the geographical coordinates of a given emission zone (4F) is provided |  |  |  | + | + |  |  | 4D |
| 4.7.3 | 4E | the code of the geographical area or standard defined area (see the Preface) Note – The standard defined area for a receiving land station in the maritime mobile service may be a maritime zone. The standard defined area for a maritime mobile frequency allotment is the allotment area In the case of a receiving land station or receiving fixed station from stations at a fixed point within a specified area, for all services, except the maritime radionavigation service, required if neither a circular zone (4CC and 4D) nor the geographical coordinates of a given emission zone (4F) is provided In the case of a typical transmitting station, required if neither a circular zone (4CC and 4D)nor the geographical coordinates of a given emission zone (4F)is provided |  |  |  | + | + | X |  | 4E |
| 4.7.4 | 4F | the geographical coordinates of a given emission zone, in which mobile transmitting stations associated with a receiving land station, or transmitting fixed stations at an unspecified location associated with a receiving fixed station or a typical transmitting station are operating. A minimum of 3 geographical coordinates are to be provided. All geographical coordinates (latitude and longitude) are provided in degrees, minutes and seconds In the case of a receivingtransmitting mobileland station, or receiving transmitting fixed station from stations at an unspecified point within a specified area, required: – for fixed and land mobileall services other than the maritime radionavigation service if neither the code of a geographical area or standard defined area (4E) nor a circular zone (4CC and 4D) is provided In the case of a typical transmitting station, required if neither a geographical area or standard defined area (4E) nor a circular zone (4CC and 4D) is provided |  |  |  | + | + |  |  | 4F |
| 4.8 | 4G | the ground conductivity Required for an assignment subject to the GE75 Regional Agreement |  | + |  |  |  |  |  | 4G |
| 5 |  | LOCATION OF THE RECEIVING ANTENNA(S) |  |  |  |  |  |  |  |  |
| 5.1 | 5A | the name of the locality by which the receiving station is known or in which it is situated In the case of a transmitting station, required for an associated receiving station in the fixed service if neither the geographical coordinates of a given reception zone (5CA) nor a circular receiving area (5E and 5F) nor the code of the geographical area or standard defined area of reception (5D) are provided |  |  | + | X |  |  |  | 5A |
| 5.2 | 5B | the code of the geographical area in which the receiving station(s) is located (see the Preface) In the case of a transmitting station, required for an associated receiving station in the fixed service if neither the geographical coordinates of a given reception zone (5CA) nor a circular receiving area (5E and 5F) nor the code of the geographical area or standard defined area of reception (5D) are provided |  |  | + | X |  |  |  | 5B |
| 5.3 | 5C | the geographical coordinates of the site of the receiving station Latitude and longitude are provided in degrees, minutes and seconds In the case of a transmitting station, required for an associated receiving station in the fixed service if neither the geographical coordinates of a given reception zone (5CA) nor a circular receiving area (5E and 5F) nor the code of the geographical area or standard defined area of reception (5D) are provided |  |  | + | X |  |  |  | 5C |
| 5.4 |  | For an area in which receiving stations operate: |  |  |  |  |  |  |  |  |
| 5.4.1 | 5CA | the geographical coordinates of a given reception zone A minimum of 3 geographical coordinates are to be provided. All geographical coordinates (latitude and longitude) are provided in degrees, minutes and seconds For an associated receiving station in the fixed service, required if neither the name of the locality (5A), geographical area (5B) and geographical coordinates (5C) nor a circular receiving area (5E and 5F) nor a geographical area or standard defined area of reception (5D) are provided For all other services, except where the assignment is subject to the GE06 Agreement, required if neither a circular area (5E and 5F) nor a geographical area or standard defined area of reception (5D) is provided |  |  | + |  |  |  |  | 5CA |
| 5.4.2 | 5D | the code of the geographical area or standard defined area of reception (see the Preface) Note – The standard defined area of a transmitting station may be represented by a maritime zone or aeronautical zone. The standard defined area of a maritime mobile frequency allotment is a maritime zone. The standard defined area of an HF broadcasting station subject to Article 12 is represented by a CIRAF zone In the case of a transmitting station, except transmitting stations in the maritime radionavigation service, aeronautical radionavigation service subject to the GE85-MM-R1 Regional Agreement or the maritime mobile service subject to the GE85-MM-R1 Regional Agreement, required if neither the name of the locality (5A), geographical area (5B) and geographical coordinates (5C) for fixed stations nor a circular receiving area (5E and 5F) nor geographical coordinates of a given reception zone (5CA) is provided |  |  | + |  |  | X | X | 5D |
| 5.4.3 | 5E | the geographical coordinates of the centre of the circular receiving area Latitude and longitude are provided in degrees, minutes and seconds Required: – for the maritime radionavigation service, aeronautical radionavigation service subject to the GE85-MM-R1 Regional Agreement or the maritime mobile service subject to the GE85-MM-R1 Regional Agreement; and – for all other services if neither the name of the locality (5A), geographical area (5B) and geographical coordinates (5C) for fixed stations nor a geographical area orstandard defined area of reception (5D) nor the geographical coordinates of a given reception zone (5CA) is provided |  |  | + |  |  |  |  | 5E |
| 5.4.4 | 5F | the radius, in km, of the circular receiving area Required: – for the maritime radionavigation service, aeronautical radionavigation service subject to the GE85-MM-R1 Regional Agreement or the maritime mobile service subject to the GE85-MM-R1 Regional Agreement; and – for all other services if neither the name of the locality (5A), geographical area (5B) and geographical coordinates (5C) for fixed stations nor the geographical area or standard defined area of reception (5D) nor the geographical coordinates of a given reception zone (5CA) is provided |  |  | + |  |  |  |  | 5F |
| 5.5 | 5G | the maximum length of the circuit, in km, for non-circular receiving areas Stations in the HF bands only |  |  | O |  |  | O |  | 5G |
| 8 |  | POWER CHARACTERISTICS |  |  |  |  |  |  |  |  |
| 8.3 | 8AA | the power delivered to the antenna, in dBW In the case of a transmitting station, required for an assignment: – in the bands below 28 MHz, in all services except the radionavigation service; or – in the bands above 28 MHz shared with space services; or – in the bands above 28 MHz not shared with space services: – in the aeronautical mobile service, meteorological aids service; or – in all other services, if the radiated power is not supplied In the case of a receiving land station or receiving fixed station, required if the associated transmitting station’s radiated power is not supplied In the case of a typical transmitting station, required if the radiated power is not supplied |  |  | + | + | + | X |  | 8AA |
| 8.4 | 8AB | the maximum power density1 (dB(W/Hz)) for each carrier type averaged over the worst 4 kHz band for carriers below 15 GHz, or averaged over the worst 1 MHz band for carriers above 15 GHz, supplied to the antenna transmission line For the fixed service in the bands shared with space services |  |  | C |  |  |  |  | 8AB |
| 8.5 | 8AC | the maximum power density (dB(W/Hz)) averaged over the worst 4 kHz band, calculated for the maximum effective radiated power Note – For a receiving land stationor receiving fixed station, the maximum power density refers to the associated transmitting station In the case of a VHF/UHF broadcasting station, required for assignments subject to § 5.1.3 of the GE06 Regional Agreement In the case of a transmitting station, a receiving land station, or a typical transmitting station, required for assignments subject to the GE06 Regional Agreement | + |  | + | + | + |  |  | 8AC |
| 12 |  | OPERATING ADMINISTRATION OR AGENCY |  |  |  |  |  |  |  |  |
| 12.1 | 12A | the symbol for the operating agency | O | O | O | O | O |  | O | 12ª |
| 12.2 | 12B | the symbol for the address of the administration responsible for the station and to which communication should be sent on urgent matters regarding interference, quality of emissions and questions referring to the technical operation of the circuit (see Article 15, also the Preface) In the case of a VHF/UHF broadcasting station, transmitting station, or a receiving land station or receiving fixed station, required for application of Article 11 | + | X | + | + | X |  | X | 12B |
| 13 |  | REMARKS |  |  |  |  |  |  |  |  |
| 13.1 | 13C | Remarks for assisting the Bureau in processing the notice | O | O | O | O | O | O | O | 13C |

Reasons: Modify the characteristics to be provided for:

- transmitting stations (except broadcasting stations in the planned LF/MF bands, in the HF bands governed by Article 12, and in the VHF/UHF bands up to 960 MHz), for the application of Nos. 11.2 and 9.21 (i.e. previous notice forms T11 and T12)

- receiving land stations, for the application of Nos. 11.9 and 9.21 (i.e. previous notice form T13)

The proposed changes concerned sections 4, 5, 8 and 12 of Appendix 4.

- Items and sub-items 5.xx are related to the location of receiving antennas. The change consists only in allowing notification of the circular receiving area in addition to receiving area defined as a polygon for the fixed service. It should also be noted that, addition of two data items 5E and 5F to transmitting stations of the fixed service will completely resolve the problem of the notification of point-to-area fixed systems. Such extension can also be implemented for typical stations in application of No.11.17. This course of action is the preferred option due to the fact that there are several other forms of notice types for terrestrial services. Some of them are exclusively for terrestrial broadcasting plans and some others are for terrestrial non broadcasting plans. The creation of additional notice types may confuse administrations.

Consequently, stations of P-MP applications would be notified using the same notice types (fixed or mobile service) irrespective of the radiocommunication service they refer to.

- Items and sub-items 4.xx are related to the location of transmitting antennas. It is necessary to extend the possibilities of notifying receiving land stations to receiving fixed station from fixed points in a specified area, for the application of No. 11.9 (i.e. only in bands not shared with space services). This allows the notification of an area in which transmitting stations operate in a discus (existing items 4CC and 4D), as geographical or standard defined area (existing item 4E) and also with the geographical coordinates (minimum of 3) of a given emission zone (new item 4F). The items 4A, 4AA, 4B and 4C still remain to notify transmitting station (FS or MS) on a known location basis (e.g. P-P or P-MP links) or for all cases in bands shared with satellite services.

- Item 12B to ensure that the administration responsible for the station and to which communication should be sent on urgent matters regarding interference, quality of emissions and questions referring to the technical operation of the circuit is known similarly for receiving land or receiving fixed station

- Items 8.3 and 8.5 to ensure that emission power is provided in the case of a receiving land station or receiving fixed station if the associated transmitting station’s radiated power is not supplied

Issue C – Satellite convergence

Introduction

Issue C deals with convergence between satellite services.

Taking into account the way in which the Radio Regulations have constantly changed to adapt to changing environments for satellite networks, it is pertinent to ask whether there are any problems with the current satellite service definitions and allocations that requires fundamental changes which cannot be addressed through the regular revisions of the Radio Regulations by the WRCs.

Introduction of several satellite services within the same band should only be considered if this does not unduly constrain the current and future satellite services operating in these bands.

Coordination procedures and types of protection criteria will have a strong impact on the effect of having satellite networks with significantly different technical characteristics operating in the same frequency band. Use of different satellite services within a frequency band or satellite services with significantly different technical parameters from current utilization may require development of new coordination procedures and types of protection criteria.

NOC	EUR/XXA2/8

No change to the Radio Regulations under this issue.

Reasons: 	It is considered that there is sufficient flexibility within the present Radio Regulations and the WRC process to accommodate current or future requirements within the time-frame typically set forth for WRCs for satellite services.

Issue D – Suppression of Resolution 951

Introduction

Resolution 951 was adopted at WRC-03 and was revised at WRC-07. During both study periods the Resolution has given rise to extensive studies within both ITU-R and administrations.

Europe is therefore of the opinion that, with the outcome of Agenda item 1.2 (see issue A, B and C) the work is considered completed. If further activities are required they could be undertaken on the basis of specific agenda items with a more limited scope.

SUP	EUR/XXA2/9

RESOLUTION 951 (Rev. WRC-07)

Enhancing the international spectrum regulatory framework

Reasons: 	The work is considered completed. If further activities are required they could be undertaken on the basis of specific agenda items with a more limited scope.