| World Radiocommunication Conference (WRC-12) Geneva, 23 January - 17 February 2012 |  |
| --- | --- |
| PLENARY MEETING | Addendum 24 to Document 5-E |
|  | 2011 |
|  | Original: English |
| EUROPEAN COMMON PROPOSALS FOR THE WORK OF THE CONFERENCE |  |
| part 24 |  |

Agenda item 1.24

1.24	to consider the existing allocation to the meteorological-satellite service in the band 7 750-7 850 MHz with a view to extending this allocation to the band 7 850-7 900 MHz, limited to non-geostationary meteorological satellites in the space-to-Earth direction, in accordance with Resolution 672 (WRC-07);

Introduction

This ECP on Agenda Item 1.24 proposes the extension of the existing primary allocation to the meteorological-satellite service in the band 7750 – 7 850 MHz to the band 7850 – 7900 MHz, limited to non-geostationary meteorological satellites in the space-to-Earth direction in order to satisfy the additional spectrum requirements within Europe for MetSat in the band     7850 – 7900 MHz enabling the next generation MetSat systems to transmit higher data rates.

The extension of the MetSat allocation into the frequency band 7850 – 7900 MHz is possible due to the fact that compatibility assessment has shown that sharing with the FS (including ENG/OB) is feasible under the same regulatory conditions as existing in the currently allocated band 7750 - 7850 MHz, applying the pfd limits already applicable to the band 7750 – 7850 MHz, as contained in Table 21-4 of Article 21, and by applying Table 8c of Appendix 7 for this band.

ARTICLE 5

Frequency allocations

Section IV – Table of Frequency Allocations

(See No. 2.1)

MOD		EUR/XXA24/1

7 250-8 500 MHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| . . . |  |  |
| 7 750-7 900 FIXED METEOROLOGICAL-SATELLITE (space-to-Earth) MOD 5.461B MOBILE except aeronautical mobile |  |  |
| . . . |  |  |

Reasons:	In order to provide the required spectrum for the next generation meteorological satellite (MetSat) systems, an extension on a world-wide basis of the existing allocation to the MetSat Service in the band 7 750 – 7 850 MHz by 50 MHz into the band 7 850 – 7 900 MHz is proposed, applying the same regulatory conditions as in the already allocated frequency band 7 750 – 7 850 MHz. Studies for the band 7 850 – 7 900 MHz conclude that sharing between the MetSat Service and the other allocated services is feasible.

MOD		EUR/XXA24/2

5.461B		The use of the band 7 750-7 900 MHz by the meteorological-satellite service (space-to-Earth) is limited to non-geostationary satellite systems.     (WRC-12)

Reasons:	As the band 7 850 – 7 900 MHz will be used in conjunction with the already allocated band 7 750 – 7 850 MHz by non-geostationary MetSat systems only, consequentially the same restriction should be applied in both bands.

APPENDIX 7

Methods for the determination of the coordination area around an earth

station in frequency bands between 100 MHz and 105 GHz

MOD		EUR/XXA24/3

TABLE  8c

Parameters required for the determination of coordination distance for a receiving earth station

| Receiving space radiocommunication service designation |  |  | Fixed-satellite |  | Fixed-satellite, radio- determination satellite | Fixed-satellite | Fixed- satellite |  | Meteorological-satellite 7, 8 | Meteorological-satellite 9 | Earth exploration- satellite 7 | Earth exploration- satellite 9 | Space research 10 |  | Fixed-satellite |  | Broadcasting-satellite |  | Fixed- satellite 9 | Broadcasting-satellite | Fixed-satellite 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  |  |  |  | Deep space |  |  |  |  |  |  |  |  |
| Frequency bands (GHz) |  |  | 4.500-4.800 |  | 5.150-5.216 | 6.700-7.075 | 7.250-7.750 |  | 7.450-7.550 | 7.750-7.900 | 8.025-8.400 | 8.025-8.400 | 8.400-8.450 | 8.450-8.500 | 10.7-12.75 |  | 12.5-12.75 12 |  | 15.4-15.7 | 17.7-17.8 | 17.7-18.8 19.3-19.7 |
| Transmitting terrestrial service designations |  |  | Fixed, mobile |  | Aeronautical radionavigation | Fixed, mobile | Fixed, mobile |  | Fixed, mobile | Fixed, mobile | Fixed, mobile | Fixed, mobile | Fixed, mobile |  | Fixed, mobile |  | Fixed, mobile |  | Aeronau-tical radio-navigation | Fixed | Fixed, mobile |
| Method to be used |  |  | § 2.1 |  | § 2.1 | § 2.2 | § 2.1 |  | § 2.1, § 2.2 | § 2.2 | § 2.1 | § 2.2 | § 2.2 |  | § 2.1, § 2.2 |  | § 1.4.5 |  |  | § 1.4.5 | § 2.1 |
| Modulation at earth station 1 |  |  | A | N |  | N | A | N | N | N | N | N | N | N | A | N | A | N | – |  | N |
| Earth station interference parameters and criteria | p0 (%) |  | 0.03 | 0.005 |  | 0.005 | 0.03 | 0.005 | 0.002 | 0.001 | 0.083 | 0.011 | 0.001 | 0.1 | 0.03 | 0.003 | 0.03 | 0.003 | 0.003 |  | 0.003 |
|  | n |  | 3 | 3 |  | 3 | 3 | 3 | 2 | 2 | 2 | 2 | 1 | 2 | 2 | 2 | 1 | 1 | 2 |  | 2 |
|  | p (%) |  | 0.01 | 0.0017 |  | 0.0017 | 0.01 | 0.0017 | 0.001 | 0.0005 | 0.0415 | 0.0055 | 0.001 | 0.05 | 0.015 | 0.0015 | 0.03 | 0.003 | 0.0015 |  | 0.0015 |
|  | NL (dB) |  | 1 | 1 |  | 1 | 1 | 1 | – | – | 1 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 1 |  | 1 |
|  | Ms (dB) |  | 7 | 2 |  | 2 | 7 | 2 | – | – | 2 | 4.7 | 0.5 | 1 | 7 | 4 | 7 | 4 | 4 |  | 6 |
|  | W (dB) |  | 4 | 0 |  | 0 | 4 | 0 | – | – | 0 | 0 | 0 | 0 | 4 | 0 | 4 | 0 | 0 |  | 0 |
| Terrestrial station parameters | E (dBW) in B 2 | A | 92 3 | 92 3 |  | 55 | 55 | 55 | 55 | 55 | 55 | 55 | 25 5 | 25 5 | 40 | 40 | 55 | 55 |  |  | 35 |
|  |  | N | 42 4 | 42 4 |  | 42 | 42 | 42 | 42 | 42 | 42 | 42 | –18 | –18 | 43 | 43 | 42 | 42 |  | 40 | 40 |
|  | Pt (dBW) in B | A | 40 3 | 40 3 |  | 13 | 13 | 13 | 13 | 13 | 13 | 13 | –17 5 | –17 5 | –5 | –5 | 10 | 10 |  |  | –10 |
|  |  | N | 0 | 0 |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | –60 | –60 | –2 | –2 | –3 | –3 |  | –7 | –5 |
|  | Gx (dBi) |  | 52 3, 4 | 52 3, 4 |  | 42 | 42 | 42 | 42 | 42 | 42 | 42 | 42 | 42 | 45 | 45 | 45 | 45 |  | 47 | 45 |
| Reference band- width 6 | B (Hz) |  | 106 | 106 |  | 106 | 106 | 106 | 107 | 107 | 106 | 106 | 1 | 1 | 106 | 106 | 27  106 | 27  106 |  |  | 106 |
| Permissible interference power | Pr( p) (dBW) in B |  |  |  |  | –151.2 |  |  | –125 | –125 | –154 11 | –142 | –220 | –216 |  |  | –131 | –131 |  |  |  |

Notes to Table 8c:

1	A: analogue modulation; N: digital modulation.

2	E is defined as the equivalent isotropically radiated power of the interfering terrestrial station in the reference bandwidth.

3	In this band, the parameters for the terrestrial stations associated with transhorizon systems have been used. If an administration believes that transhorizon systems do not need to be considered, the line-of-sight radio-relay parameters associated with the frequency band 3.4-4.2 GHz may be used to determine the coordination area.

4	Digital systems assumed to be non-transhorizon. Therefore Gx  42.0 dBi. For digital transhorizon systems, parameters for analogue transhorizon systems above have been used.

5	These values are estimated for 1 Hz bandwidth and are 30 dB below the total power assumed for emission.

6	In certain systems in the fixed-satellite service it may be desirable to choose a greater reference bandwidth B. However, a greater bandwidth will result in smaller coordination distances and a later decision to reduce the reference bandwidth may require recoordination of the earth station.

7	Geostationary-satellite systems.

8	Non-geostationary satellites in the meteorological-satellite service notified in accordance with No. 5.461A may use the same coordination parameters.

9	Non-geostationary-satellite systems.

10	Space research earth stations in the band 8.4-8.5 GHz operate with non-geostationary satellites.

11	For large earth stations:	Pr( p )  (G – 180) 	dBW

For small earth stations:	Pr(20%)  2 (G – 26) – 140	dBW 	for   26  G  29 dBi

Pr(20%)  G – 163	dBW 	for           G  29 dBi

Pr( p )%  G – 163 	dBW 	for           G  26 dBi

12	Applies to the broadcasting-satellite service in unplanned bands in Region 3.

Reasons:	Consequential modifications to Table 8c of Appendix 7 in order to apply the same parameters required for the determination of coordination distance for a receiving earth station in the extension band 7850 – 7900 MHz as in the already allocated band 7750 – 7850 MHz.

MOD		EUR/XXA24/4

TABLE  21-4  (continued )     (Rev.WRC-07)

| Frequency band | Service* | Limit in dB(W/m2) for angles of arrival () above the horizontal plane |  |  | Reference bandwidth |
| --- | --- | --- | --- | --- | --- |
|  |  | 0-5 | 5-25 | 25-90 |  |
| … | … | … | … | … | … |
| 4 500-4 800 MHz 5 670-5 725 MHz (Nos. 5.453 and 5.455) 7 250-7 900 MHz | Fixed-satellite (space-to-Earth) Meteorological-satellite (space-to-Earth) Mobile-satellite Space research | –152 | –152  0.5( – 5) | –142 | 4 kHz |
| … | … | … | … | … | … |

Reasons:	Consequential modifications to Table 21-4 in order to apply the same pfd limits in the extension band 7850 – 7900 MHz as in the already allocated band 7750 – 7850 MHz.

SUP		EUR/XXA24/5

RESOLUTION 672 (WRC-07)

Extension of the allocation to the meteorological satellite service

in the band 7 750 – 7 850 MHz

Reasons: 	Completion of studies required under this resolution.