| World Radiocommunication Conference (WRC-12) Geneva, 23 January - 17 February 2012 |  |
| --- | --- |
| PLENARY MEETING | Addendum 4 to Document 5(Add.28)-E |
|  | 2011 |
|  | Original: English |
| EUROPEAN COMMON PROPOSALS FOR THE WORK OF THE CONFERENCE |  |
| part 28 Addendum 4 |  |
| Agenda Item 7 Sub part D |  |

Miscellaneous issues

Introduction

Proposals within this sub-part are related to issues 1C, 1E and 2D of the CPM report on WRC-12 Agenda Item 7 as well as a new issue not contained in the CPM report. Even if they are largely independent, they are grouped altogether for convenience.

Review of the bands listed in Table 5-1 of Appendix 5 for Nos. 9.11 and 9.19

Because inaccuracies in Table 5-1 of Appendix 5 lead to confusion in applying the provisions of Section II of Article 9 and may cause difficulties for both administrations and the Bureau, Europe proposes to update the list of frequency bands indicated in the row corresponding to No. 9.11, on the basis of the current Radio Regulations. Europe also proposes to explicitly list the bands where No. 9.19 applies instead of referring to the row dealing with No. 9.11.

Footnote 2 to Tables A, B, C and D of Annex 2 of Appendix 4

An ambiguity in the text of Footnote 2 to Tables A, B, C and D of Annex 2 of Appendix 4 could lead to either under estimation, or over-estimation, of the maximum power density of carriers. Europe proposes to clarify this footnote by pointing to the most recent version of Recommendation ITU-R SF.675.

Addition of a data item in Appendix 4 for submitting the antenna dimension aligned with the geostationary arc

Europe proposes to add a new data item in Appendix 4 in order to allow administrations to implement Recommendation ITU-R S.1855 in their satellite coordination requests. This addition will permit the use of an alternative reference radiation pattern for earth station antennas.

Modification to No. 9.1 to prevent sending notification of satellite networks subject to Section II of Article 9 at the same time as request for coordination

Currently satellite networks or systems subject to Section II of Article 9 are able to be notified on the same day where the associated coordination request is sent. This is in contrast with satellite networks or systems not subject to coordination where notification has to be delayed at least six months after the API publication. Sending both a coordination request and its associated notification on the same day undermines the coordination procedure set forth by Article 9 and should therefore be prohibited. The modification of No. 9.1 could be considered to implement such delaying of the notification under Article 11 for satellite networks or systems subject to Section II of Article 9.

The table below links the European proposals with the issues and methods as numbered in the CPM report.

| Topic | Related issue in the CPM text on WRC-12 agenda item 7 | Method from the CPM text that Europe supports, if any | List of European proposals |  |
| --- | --- | --- | --- | --- |
|  |  |  | Intent of the proposal | Proposal number (EUR/XXA28/) |
| Review of the bands listed in Table 5-1 of Appendix 5 for Nos. 9.11 and 9.19 | Issue 2D | Sole Method of the CPM report | Updating the frequency bands listed in the row corresponding to No. 9.11, based on the current Radio Regulations. Listing the bands where No. 9.19 applies instead of referring to the row dealing with No. 9.11. | 64 |
| Footnote 2 to Tables A, B, C and D of Annex 2 of Appendix 4 | Issue 1C | Method A | Clarifying the footnote by pointing to the most recent version of Recommendation ITU-R SF.675 for calculating the maximum power density of carriers. | 61,62 |
| Addition of a data item in Appendix 4 for submitting the antenna dimension aligned with the geostationary arc | Issue 1E | Sole Method of the CPM report | Adding a new data item in Appendix 4 to ease the implementation of Recommendation ITU-R S.1855 in satellite coordination. | 63 |
| Modification to No. 9.1 to prevent sending notification of satellite networks subject to Section II of Article 9 at the same time as request for coordination | - | - | Modifying No. 9.1 to delay sending the notification of satellite networks subject to Section II of Article 9 not earlier than six months after the date of publication of the request for coordination. | 60 |

ARTICLE  9

Procedure for effecting coordination with or 
            obtaining agreement of other administrations1, 2, 3, 4, 5, 6, 7, 8

Section I  –  Advance publication of information on satellite
networks or satellite systems

General

MOD		EUR/XXA28/60

9.1		Before initiating any action under this Article or under Article 11 in respect of frequency assignments for a satellite network or a satellite system, an administration, or one9 acting on behalf of a group of named administrations, shall, prior to the coordination procedure described in Section II of Article 9 below, where applicable, send to the Bureau a general description of the network or system for advance publication in the International Frequency Information Circular (BR IFIC) not earlier than seven years and preferably not later than two years before the planned date of bringing into use of the network or system (see also No. 11.44). The characteristics to be provided for this purpose are listed in Appendix 4. The coordination or notification information may also be communicated to the Bureau at the same time; it shall be considered as having been received by the Bureau not earlier than six months after the date of receipt of the information for advance publication where coordination is required by Section II of Article 9. Where coordination is not required by Section II, notification shall be considered as having been received by the Bureau not earlier than six months after the date of publication of the advance publication information. Where coordination is required by Section II, notification shall be considered as having been received by the Bureau not earlier than six months after the date of publication of the request for coordination. .     (WRC-12)

Reasons: To align the notification conditions for all cases and making sure it cannot happen earlier than the publication of the complete list of required coordination (for both geostationary and non-geostationary satellite networks) has been published.

APPENDIX  4  (Rev.WRC-12)

Consolidated list and tables of characteristics for use in the
application of the procedures of Chapter III

ANNEX  2

Characteristics of satellite networks, earth stations
or radio astronomy stations2

MOD	EUR/XXA28/61

Table of characteristics to be submitted for space and radio astronomy services     (WRC-12)

| Items in Appendix | C _ CHARACTERISTICS TO BE PROVIDED FOR EACH GROUP OF FREQUENCY ASSIGNMENTS FOR A SATELLITE ANTENNA BEAM OR AN EARTH STATION OR RADIO ASTRONOMY ANTENNA | Advance publication of a geostationary-satellite network | Advance publication of a non- geostationary-satellite network subject to coordination under Section II of Article 9 | Advance publication of a non- geostationary-satellite network not subject to coordination under Section II of Article 9 | Notification or coordination of a geostationary-satellite network (including space operation functions under Article 2A of Appendices 30 or 30A) | Notification or coordination of a non- geostationary-satellite network | Notification or coordination of an earth station (including notification under Appendices 30A or 30B) | Notice for a satellite network in the broadcasting-satellite service under Appendix 30 (Articles 4 and 5) | Notice for a satellite network (feeder- link) under Appendix 30A (Articles 4 and 5) | Notice for a satellite network in the fixed-satellite service under Appendix 30B (Articles 6 and 8) | Items in Appendix | Radio astronomy |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| C.8 | POWER CHARACTERISTICS OF THE TRANSMISSION |  |  |  |  |  |  |  |  |  | C.8 |  |
|  | Not required for passive sensors |  |  |  |  |  |  |  |  |  |  |  |
| C.8.a | For the case where individual carriers can be identified: |  |  |  |  |  |  |  |  |  | C.8.a |  |
| C.8.a.1 | the maximum value of the peak envelope power, in dBW, supplied to the input of the antenna for each carrier type |  |  | + | + | + | C |  |  |  | C.8.a.1 |  |
|  | Required if neither C.8.b.1 nor C.8.b.3.a is provided |  |  |  |  |  |  |  |  |  |  |  |
| C.8.a.2 | the maximum power density, in dB(W/Hz), supplied to the input of the antenna for each carrier typeMOD 2 |  |  | + | + | + | O |  |  |  | C.8.a.2 |  |
|  | Required if neither C.8.b.2 nor C.8.b.3.b is provided |  |  |  |  |  |  |  |  |  |  |  |
| C.8.b | For the case where it is not appropriate to identify individual carriers: |  |  |  |  |  |  |  |  |  | C.8.b |  |
| C.8.b.1 | the total peak envelope power, in dBW, supplied to the input of the antenna |  |  | + | + | + | + 1 | X | X |  | C.8.b.1 |  |
|  | For coordination or notification of an Appendix 30A earth station the values shall include the maximum range of power control |  |  |  |  |  |  |  |  |  |  |  |
|  | Required if neither C.8.a.1 nor C.8.b.3.a is provided |  |  |  |  |  |  |  |  |  |  |  |
| C.8.b.2 | the maximum power density, in dB(W/Hz), supplied to the input of the antennaMOD 2 |  |  | + | + | + | + 1 | X | X | X | C.8.b.2 |  |
|  | For coordination or notification of an Appendix 30A earth station the values shall include the maximum range of power control |  |  |  |  |  |  |  |  |  |  |  |
|  | Required if neither C.8.a.2 nor C.8.b.3.b is provided |  |  |  |  |  |  |  |  |  |  |  |
| C.8.b.3 | For the case of active sensors: |  |  |  |  |  |  |  |  |  | C.8.b.3 |  |
| C.8.b.3.a | the mean peak envelope power, in dBW, supplied to the input of the antenna |  |  | + | + | + |  |  |  |  | C.8.b.3.a |  |
|  | Required if neither C.8.a.1 nor C.8.b.1 is provided |  |  |  |  |  |  |  |  |  |  |  |
| C.8.b.3.b | the mean power density, in dB(W/Hz), supplied to the input of the antenna |  |  | + | + | + |  |  |  |  | C.8.b.3.b |  |
|  | Required if neither C.8.a.2 nor C.8.b.2 is provided |  |  |  |  |  |  |  |  |  |  |  |
| C.8.c | For all space applications, except active or passive sensors: |  |  |  |  |  |  |  |  |  | C.8.c |  |
| C.8.c.1 | the minimum value of the peak envelope power, in dBW, supplied to the input of the antenna for each carrier type |  |  | + | + | + | + 1 |  |  |  | C.8.c.1 |  |
|  | If not provided, the reason for absence under C.8.c.2 |  |  |  |  |  |  |  |  |  |  |  |
| C.8.c.2 | if C.8.c.1 is not provided, the reason for absence of the minimum value of the peak envelope power |  |  | + | + | + | + 1 |  |  |  | C.8.c.2 |  |
| C.8.c.3 | the minimum power density, in dB(W/Hz), supplied to the input of the antenna for each carrier typeMOD 2 |  |  | + | + | + | + 1 |  |  |  | C.8.c.3 |  |
|  | If not provided, the reason for absence under C.8.c.4 |  |  |  |  |  |  |  |  |  |  |  |
|  | For the maximum saturated peak envelope power of the satellite transponder, this corresponds to the bandwidth of each transponder |  |  |  |  |  |  |  |  |  |  |  |
|  | Required only for a space-to-Earth or space-to-space link, if different from item C.3.a |  |  |  |  |  |  |  |  |  |  |  |

MOD	EUR/XXA28/62

Footnotes to Tables A, B, C and D

1	Not required for coordination under No. 9.7A.

2	The most recent version of Recommendation ITU-R SF.675 should be used in calculating the maximum power density per Hz. For carriers below 15 GHz, the power density is averaged over the worst 4 kHz band. For carriers at or above 15 GHz, the power density is averaged over the worst 1 MHz band.

Reasons: to clarify the footnote by pointing to the most recent version of Recommendation ITU-R SF.675 for calculating the maximum power density of carriers.

APPENDIX  4  (Rev.WRC-12)

Consolidated list and tables of characteristics for use in the
application of the procedures of Chapter III

ANNEX  2

Characteristics of satellite networks, earth stations
or radio astronomy stations2

MOD	EUR/XXA28/63

Table of characteristics to be submitted for space and radio astronomy services     (WRC-12)

| Items in Appendix | A _ GENERAL CHARACTERISTICS OF THE SATELLITE NETWORK, EARTH STATION OR RADIO ASTRONOMY STATION | Advance publication of a geostationary-satellite network | Advance publication of a non- geostationary-satellite network subject to coordination under Section II of Article 9 | Advance publication of a non- geostationary-satellite network not subject to coordination under Section II of Article 9 | Notification or coordination of a geostationary-satellite network (including space operation functions under Article 2A of Appendices 30 or 30A) | Notification or coordination of a non- geostationary-satellite network | Notification or coordination of an earth station (including notification under Appendices 30A or 30B) | Notice for a satellite network in the broadcasting-satellite service under Appendix 30 (Articles 4 and 5) | Notice for a satellite network (feeder- link) under Appendix 30A (Articles 4 and 5) | Notice for a satellite network in the fixed-satellite service under Appendix 30B (Articles 6 and 8) | Items in Appendix | Radio astronomy |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A.7 | SPECIFIC EARTH STATION OR RADIO ASTRONOMY STATION SITE CHARACTERISTICS |  |  |  |  |  |  |  |  |  | A.7 |  |
| ... |  |  |  |  |  |  |  |  |  |  |  |  |
| A.7.f | the antenna diameter, in metres |  |  |  |  |  |  |  |  |  | A.7.f |  |
|  | Required only for fixed-satellite service earth stations operating in the frequency band 13.75-14 GHz |  |  |  |  |  | + 1 |  |  |  |  |  |
| A.7.g | the antenna dimension aligned with the geostationary arc, in metres (see the most recent version of Recommendation ITU-R S.1855) |  |  |  |  |  | O |  |  |  | A.7.g |  |
| ... |  |  |  |  |  |  |  |  |  |  |  |  |

| Items in Appendix | C _ CHARACTERISTICS TO BE PROVIDED FOR EACH GROUP OF FREQUENCY ASSIGNMENTS FOR A SATELLITE ANTENNA BEAM OR AN EARTH STATION OR RADIO ASTRONOMY ANTENNA | Advance publication of a geostationary-satellite network | Advance publication of a non- geostationary-satellite network subject to coordination under Section II of Article 9 | Advance publication of a non- geostationary-satellite network not subject to coordination under Section II of Article 9 | Notification or coordination of a geostationary-satellite network (including space operation functions under Article 2A of Appendices 30 or 30A) | Notification or coordination of a non- geostationary-satellite network | Notification or coordination of an earth station (including notification under Appendices 30A or 30B) | Notice for a satellite network in the broadcasting-satellite service under Appendix 30 (Articles 4 and 5) | Notice for a satellite network (feeder- link) under Appendix 30A (Articles 4 and 5) | Notice for a satellite network in the fixed-satellite service under Appendix 30B (Articles 6 and 8) | Items in Appendix | Radio astronomy |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| C.10 | TYPE AND IDENTITY OF THE ASSOCIATED STATION(S) |  |  |  |  |  |  |  |  |  | C.10 |  |
|  | (the associated station may be another space station, a typical earth station of the network or a specific earth station) |  |  |  |  |  |  |  |  |  |  |  |
|  | For all space applications except active or passive sensors |  |  |  |  |  |  |  |  |  |  |  |
| ... |  |  |  |  |  |  |  |  |  |  |  |  |
| C.10.d | For an associated earth station (whether specific or typical): |  |  |  |  |  |  |  |  |  | C.10.d |  |
| ... |  |  |  |  |  |  |  |  |  |  |  |  |
| C.10.d.7 | the antenna diameter, in metres |  |  |  |  |  |  |  |  |  | C.10.d.7 |  |
|  | In cases other than Appendix 30A, required for fixed-satellite service networks operating in the frequency band 13.75-14 GHz and for maritime mobile-satellite service networks operating in the frequency band 14-14.5 GHz |  |  |  | + | + |  |  | X |  |  |  |
| C.10.d.8 | the equivalent antenna diameter (i.e. the diameter, in metres, of a parabolic antenna with the same off-axis performance as the receiving associated earth station antenna) |  |  |  |  |  |  | X |  |  | C.10.d.8 |  |
| C.10.d.9 | the antenna dimension aligned with the geostationary arc, in metres (see the most recent version of Recommendation ITU-R S.1855) |  |  |  | O |  | O |  |  |  | C.10.d.9 |  |
| ... |  |  |  |  |  |  |  |  |  |  |  |  |

Reasons: to add a new data item in Appendix 4 to ease the implementation of Recommendation ITU-R S.1855 in satellite coordination (and to correct the alignment of item C.10.d.8).

APPENDIX 5 (Rev.WRC-12)

Identification of administrations with which coordination is to be effected or
agreement sought under the provisions of Article 9

TABLE 5-1

Technical conditions for coordination (see Article 9)

MOD	EUR/XXA28/64

(…)

TABLE 5-1 (continued)     (WRC-07)

| Reference of Article 9 | Case | Frequency bands (and Region) of the service for which coordination is sought | Threshold/condition | Calculation method | Remarks |
| --- | --- | --- | --- | --- | --- |
| No. 9.11 GSO, non-GSO/ terrestrial | A space station in the BSS in any band shared on an equal primary basis with terrestrial services and where the BSS is not subject to a Plan, in respect of terrestrial services | 620-790 MHz (see Resolution 549 (WRC-07)) 1 452-1 492 MHz 2 310-2 360 MHz (No. 5.393) 2 535-2 655 MHz (Nos. 5.417A and 5.418) 17.7-17.8 GHz (Region 2) [21.4-22 GHz (Regions 1 and 3)]* 74-76 GHz | Bandwidths overlap: The detailed conditions for the application of No. 9.11 in the bands 2 630-2 655 MHz and 2 605-2 630 MHz are provided in Resolution 539 (Rev.WRC-03) for non-GSO BSS (sound) systems pursuant to Nos. 5.417A and 5.418, and in Nos. 5.417A and 5.418 for GSO BSS (sound) networks pursuant to those provisions. - | Check by using the assigned frequencies and bandwidths |  |

* Editor’s note: inclusion or not of the band 21.4-22 GHz in this row of Table 5-1 will be decided by WRC-12 under agenda item 1.13.

(…)

TABLE 5-1 (end)

| Reference of Article 9 | Case | Frequency bands (and Region) of the service for which coordination is sought | Threshold/condition | Calculation method | Remarks |
| --- | --- | --- | --- | --- | --- |
| No. 9.19 Terrestrial, GSO, non-GSO/ GSO, non-GSO | Any transmitting station of a terrestrial service or a transmitting earth station in the FSS (Earth-to-space) in a frequency band shared on an equal primary basis with the BSS, with respect to typical earth stations included in the service area of a space station in the BSS | 620-790 MHz (see Resolution 549 (WRC-07)) 1 452-1 492 MHz 2 310-2 360 MHz (terrestrial services in all three Regions in respect of BSS allocation in No. 5.393) 2 520-2 670 MHz (see No. 5.416)11.7-12.7 GHz (see Article 6 of Appendix 30) 12.5-12.7 GHz (terrestrial services in Nos. 5.494 and 5.496 as well as in Regions 2 and 3, or transmitting earth station in the FSS (Earth-to-space) in Region 1, in respect of BSS allocation in Region 3) 12.7-12.75 GHz (terrestrial services in Nos. 5.494 and 5.496 as well as in Regions 2 and 3, or transmitting earth station in the FSS (Earth-to-space) in Regions 1 and 2, in respect of BSS allocation in Region 3) 17.7-17.8 GHz (terrestrial services in all three Regions in respect of BSS allocation in Region 2) 17.3-17.8 GHz (transmitting earth stations in the FSS (Earth-to-space) in respect of BSS allocation in Region 2) (see Article 4 of Appendix 30A) [21.4-22 GHz (Regions 1 and 3)]* 40.5-42.5 GHz 74-76 GHz | i) Necessary bandwidths overlap; and ii) the power flux-density (pfd) of the interfering station at the edge of the BSS service area exceeds the permissible level | Check by using the assigned frequencies and bandwidths | See also Article 6 of Appendix 30 |

* Editor’s note: inclusion or not of the band 21.4-22 GHz in this row of Table 5-1 will be decided by WRC-12 under agenda item 1.13.

Reasons: to update the list of frequency bands indicated in the row corresponding to No. 9.11, on the basis of the current Radio Regulations and to explicitly list the bands where No. 9.19 applies instead of referring to the row dealing with RR No. 9.11.