| World Radiocommunication Conference (WRC-12) Geneva, 23 January - 17 February 2012 |  |
| --- | --- |
| PLENARY MEETING | Addendum 6 to Document 5-E |
|  | 2011 |
|  | Original: English |
| EUROPEAN COMMON PROPOSALS FOR THE WORK OF THE CONFERENCE |  |
| part 6 |  |

Agenda item 1.6

1.6	to review No. 5.565 of the Radio Regulations in order to update the spectrum use by the passive services between 275 GHz and 3 000 GHz, in accordance with Resolution 950 (Rev.WRC-07), and to consider possible procedures for free-space optical-links, taking into account the results of ITU-R studies, in accordance with Resolution 955 (WRC-07);

Introduction

This ECP on Agenda Item 1.6 has been split into two parts:

Sub-Part 6A - Passive services (i.e. Earth exploration satellite, radioastronomy and space research services) between 275 and 3000 GHz

Sub-Part 6B - Free-space optical-links above 3000 GHz

Sub-Part 6A

Passive services (i.e. Earth Exploration Satellite, Radioastronomy and Space Research services) between 275 and 3000 GHz

Introduction

Resolution 950 (Rev.WRC-07) calls for studies with a view of updating passive services use (i.e. Earth exploration satellite, radioastronomy and space research services) between 275 and 3000 GHz and consistently review No. 5.565.

MOD 	EUR/XXA6/1

248-3 000 GHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| ... |  |  |
| 275-3 000 (Not allocated) MOD 5.565 |  |  |

MOD 		EUR/XXA6/2

5.565	The following frequency bands in the range 275-1 000 GHz  are identified for use by administrations for passive service applications:

- radio astronomy service: 275-323 GHz, 327-371 GHz, 388-424 GHz, 426-442 GHz, 453-510 GHz, 623-711 GHz, 795-909 GHz and 926-945 GHz;

- Earth exploration-satellite service (passive) and space research service (passive): 275-286 GHz, 296-306 GHz, 313-356 GHz, 361-365 GHz, 369-392 GHz, 397-399 GHz, 409-411 GHz, 416-434 GHz, 439-467  GHz, 477-502 GHz, 523-527 GHz, 538-581 GHz, 611-630 GHz, 634-654 GHz, 657-692 GHz 713-718 GHz, 729-733 GHz, 750-754 GHz, 771-776 GHz, 823-846 GHz, 850-854 GHz, 857-862 GHz, 866-882 GHz, 902-928 GHz, 951-956 GHz, 968-973 GHz and 985-990 GHz.

The use of the range 275-1 000 GHz by the passive services does not preclude use of this range by active services.

Administrations wishing to make frequencies in the 275-1 000 GHz range available for active service applications are urged to take all practicable steps to protect these passive services from harmful interference until the date when the Table of Frequency Allocation is established in the above-mentioned 275-1 000 GHz frequency range.

All frequencies in the range 1 000-3 000 GHz may be used by both active and passive services.     (WRC-2012)

Reasons:	The revision of No. 5.565 is consequential to the agenda item in order to consider the results of ITU-R technical studies captured in various ITU-R Recommendations and Reports providing up-to-date detailed requirements of passive services/applications.

SUP 		EUR/XXA6/3

RESOLUTION 950 (Rev.WRC-07)

Consideration of the use of the frequencies
between 275 and 3 000 GHz

Reasons: 	Consequential. This Resolution is not necessary anymore

Sub-Part 6B

Free-space optical-links above 3000 GHz

Introduction

Resolution 955 (WRC-07) asks to consider possible procedures if allocations to various services related to free-space optical links in the Radio Regulations above 3 000 GHz are considered feasible.

NOC 		EUR/XXA6/4

No change to the Radio regulations under this issue (except suppression of Resolution 955).

Reasons: It is not considered necessary to introduce in the Radio Regulations any procedures related to free-space optical links.

SUP		EUR/XXA6/5

RESOLUTION 955 (WRC-07)

Consideration of procedures for free-space optical links

Reasons: 	Consequential. This Resolution is not necessary anymore