| World Radiocommunication Conference (WRC-12) Geneva, 23 January - 17 February 2012 |  |
| --- | --- |
| PLENARY MEETING | Addendum 8 to Document 5-E |
|  | 2011 |
|  | Original: English |
| EUROPEAN COMMON PROPOSALS FOR THE WORK OF THE CONFERENCE |  |
| part 8 |  |

Agenda item 1.8

1.8	to consider the progress of ITU-R studies concerning the technical and regulatory issues relative to the fixed service in the bands between 71 GHz and 238 GHz, taking into account Resolutions 731 (WRC-2000) and 732 (WRC-2000).

Introduction

Studies have been conducted within ITU-R to gather the characteristics/deployment scenarios and future trends and requirements of FWS systems in bands above 71 GHz and ITU-R Report F.2107 has been revised.These studies have demonstrated increasing interest and developments in active service technology (e.g. wide band high capacity FWS systems > 10Gbit/s) in these bands. However, it is recognised that the technology development/deployment in the bands above 94GHz is still at early stage and that further studies would be required to better understand how the FWS will evolve.

In addition, Resolutions 731 (WRC-2000) and 732 (WRC-2000) are not limited to the fixed service but were adopted to address future active service technology requirements, sharing and allocation issues in bands above 71GHz. Therefore, Resolutions 731 (WRC-2000) and 732 (WRC-2000) are proposed to be retained unchanged.

With regard to bands below 94GHz, in band sharing and adjacent band compatibility issues between fixed services in the 71-76/81-86 GHz band and radio astronomy in the 81-86 GHz, 76-77.5 GHz, 79-81 GHz and 86-92 GHz bands and Earth exploration-satellite service in the 86–92 GHz band have been studied. The results of these studies are contained in ECC Report 124.

It appears from the studies that the protection of the radio astronomy service from interference in bands adjacent to the fixed service operating in the bands 71-76 and 81-86 GHz is a national issue which does not require any additional regulatory provision in Article 5.

The protection of radio astronomy stations operating in the band 81-86 GHz from interference from FS stations in the same band may require the establishment of exclusion or coordination area around a radio astronomy station, to be determined on a case-by-case basis taking into account specificities such as terrain elevation and additional clutter. Recommendation ITU-R RA.1031 may be used to calculate those distances. The same would apply for sharing in the bands 92-94 GHz, 94.1-95 GHz and 111.8-114.25 GHz. The regulatory provisions are already contained in No. 29.9.

The protection of the Earth exploration-satellite service operating in the band 86-92 GHz from interference from unwanted emissions from FS stations operating in the adjacent band 81-86 GHz would be achieved by an unwanted emission power mask in the band 86-92 GHz starting with –41dBW/100 MHz at 86 GHz and decaying to –55 dBW/100MHz at 87 GHz. The same kind of mask would apply for the protection of EESS(passive) in the band 86-92 GHz from interference from the fixed service in the band 92-94 GHz, starting with -41dBW/100 MHz at 92 GHz and decaying to -55 dBW/100 MHz at 91 GHz.

ARTICLE 5

Frequency allocations

Section IV  –  Table of Frequency Allocations
(See No. 2.1)

MOD	EUR/XXA8/1

81-86 GHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 81-84 FIXED ADD 5.A18 FIXED-SATELLITE (Earth-to-space) MOBILE MOBILE-SATELLITE (Earth-to-space) RADIO ASTRONOMY Space research (space-to-Earth) 5.1495.561A |  |  |
| 84-86 FIXED ADD 5.A18 FIXED-SATELLITE (Earth-to-space) 5.561B MOBILE RADIO ASTRONOMY 5.149 |  |  |

ADD	EUR/XXA8/2

5.A18	Stations in the fixed service operating in the band 81 – 86 GHz shall not exceed the following unwanted emission power limits at the antenna port, where f is the centre frequency of the 100 MHz reference bandwidth expressed in GHz :

-41 – 14(f-86)	dBW/100 MHz 	for 86.05  f  87 GHz

-55			dBW/100 MHz	for 87  f  91.95 GHz

MOD	EUR/XXA8/3

86-111.8 GHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 86-92 EARTH EXPLORATION-SATELLITE (passive) RADIO ASTRONOMY SPACE RESEARCH (passive) 5.340 |  |  |
| 92-94 FIXED ADD 5.B18 MOBILE RADIO ASTRONOMY RADIOLOCATION 5.149 |  |  |

ADD	EUR/XXA8/4

5.B18	Stations in the fixed service operating in the band 92 – 94 GHz shall not exceed the following unwanted emission power limits at the antenna port, where f is the centre frequency of the 100 MHz reference bandwidth expressed in GHz :

-41 – 14(92-f)	dBW/100 MHz 	for 91  f  91.95 GHz

-55			dBW/100 MHz	for 86.05  f  91 GHz

Reasons:		The limitation of the unwanted emission power radiated by stations of the fixed service in the bands 81-86 GHz and 92-94 GHz is needed in order to protect the Earth exploration-satellite service operating in the band 86-92 GHz from harmful interference.

NOC	EUR/XXA8/5

No change to the Table of frequency allocations between 94 and 238 GHz

Reasons:		Further studies are required to better understand how the fixed service systems will evolve in the future. It is therefore considered premature to take any regulatory action at this time for these bands.

NOC	EUR/XXA8/6

RESOLUTION 731 (WRC-2000)

Consideration by a future competent world radiocommunication conference of issues dealing with sharing and adjacent-band compatibility between
passive and active services above 71 GHz

NOC	EUR/XXA8/7

RESOLUTION 732 (WRC-2000)

Consideration by a future competent world radiocommunication
conference of issues dealing with sharing between 
active services above 71 GHz

Reasons:		Resolutions 731 (WRC-2000) and 732 (WRC-2000) were adopted at WRC-2000 to address future active service technology requirements, sharing and allocation issues in bands above 71GHz ‘at a future competent conference’. In order to address future requirements of the active services above 71GHz, Resolutions 731 (WRC-2000) and 732 (WRC-2000) need to be maintained.