| World Radiocommunication Conference (WRC-12) Geneva, 23 January - 17 February 2012 |  |
| --- | --- |
| PLENARY MEETING | Addendum 9 to Document 5-E |
|  | 2011 |
|  | Original: English |
| EUROPEAN COMMON PROPOSALS FOR THE WORK OF THE CONFERENCE |  |
| part 9 |  |

Agenda item 1.9

1.9	to revise frequencies and channelling arrangements of Appendix 17 to the Radio Regulations, in accordance with Resolution 351 (Rev.WRC-07), in order to implement new digital technologies for the maritime mobile service;

Introduction

The future spectrum needs of the maritime mobile service in the HF bands are closely related to the introduction of new data exchange technologies into the maritime mobile service as an alternative standard for narrow-band direct printing (NBDP). The use of NBDP is in decline. However, IMO has noted that NBDP is currently still used for broadcasting of MSI, ship reporting, weather forecasts and for business communications, e.g. by fishing fleets. All these functions could however be provided by alternative data communications technology.

There is scope in the maritime mobile service for improving the utility of the present spectrum by allowing data transmissions to use certain parts of Appendix 17 currently designated for use by voice channels, in order to provide additional flexibility within the maritime mobile spectrum for data exchange services.

GMDSS compliance needs of course to be addressed before the NBDP requirement could be removed completely. NBDP continues to remain useful for distress communications in the polar regions (sea area A4) when other terrestrial means of communication are no longer reliable, and as there is no coverage from geostationary satellites. This functionality could however be preserved using the HF distress and safety frequencies in Appendix 15.

APPENDIX 17 (Rev.WRC-12)

Frequencies and channelling arrangements in the high-frequency
bands for the maritime mobile service

(See Article 52)

PART  A  –  Table of subdivided bands     (WRC-12)

In the Table, where appropriate1, the assignable frequencies in a given band for each usage are:

–	indicated by the lowest and highest frequency, in heavy type, assigned in that band;

–	regularly spaced, the number of assignable frequencies ( f.) and the spacing in kHz being indicated in italics.

MOD  	EUR/XXA9/1

Table of frequencies (kHz) to be used in the band between 4 000 kHz and 27 500 kHz
allocated exclusively to the maritime mobile service

| Band (MHz) | 4 | 6 | 8 | 12 | 16 | 18/19 | 22 | 25/26 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Limits (kHz) | 4 063 | 6 200 | 8 195 | 12 230 | 16 360 | 18 780 | 22 000 | 25 070 |
| Frequencies assignable to ship stations for oceanographic data transmission c) | 4 063.3 to 4 064.8 6 f. 0.3 kHz |  |  |  |  |  |  |  |
| Limits (kHz) | 4 065 | 6 200 | 8 195 | 12 230 | 16 360 | 18 780 | 22 000 | 25 070 |
| Frequencies assignable to ship stations for telephony, duplex operation a) i ) hh) | 4 066.4 to 4 144.4 27 f. 3 kHz | 6 201.4 to 6 222.4 8 f. 3 kHz | 8 196.4 to 8 292.4 33 f. 3 kHz | 12 231.4 to 12 351.4 41 f. 3 kHz | 16 361.4 to 16 526.4 56 f. 3 kHz | 18 781.4 to 18 823.4 15 f. 3 kHz | 22 001.4 to 22 157.4 53 f. 3 kHz | 25 071.4 to 25 098.4 10 f. 3 kHz |
| Limits (kHz) | 4 146 | 6 224 | 8 294 | 12 353 | 16 528 | 18 825 | 22 159 | 25 100 |

Table of frequencies (kHz) to be used in the band between 4 000 kHz and 27 500 kHz
allocated exclusively to the maritime mobile service (continued )

| Band (MHz) | 4 | 6 | 8 | 12 | 16 | 18/19 | 22 | 25/26 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Limits (kHz) | 4 146 | 6 224 | 8 294 | 12 353 | 16 528 | 18 825 | 22 159 | 25 100 |
| Frequencies assignable to ship stations and coast stations for telephony, simplex operation - hh) | 4 147.4 to 4 150.4 2 f. 3 kHz | 6 225.4 to 6 231.4 3 f. 3 kHz | 8 295.4 to 8 298.4 2 f. 3 kHz | 12 354.4 to 12 366.4 5 f. 3 kHz | 16 529.4 to 16 547.4 7 f. 3 kHz | 18 826.4 to 18 844.4 7 f. 3 kHz | 22 160.4 to 22 178.4 7 f. 3 kHz | 25 101.4 to 25 119.4 7 f. 3 kHz |
| Limits (kHz) | 4 152 | 6 233 | 8 300 | 12 368 | 16 549 | 18 846 | 22 180 | 25 121 |
| Frequencies assignable to ship stations for -data transmissions p) ee) |  |  |  |  |  |  |  | z |
| Limits (kHz) | 4 172 | 6 261 | 8 340 | 12 420 | 16 617 | 18 870 | 22 240 | 25 161.25 |
| Frequencies assignable to ship stations for oceanographic data transmission c) p) |  |  |  |  |  |  |  |  |
| Limits (kHz) | 4 172 | 6 262.75 | 8 341.75 | 12 421.75 | 16 618.75 | 18 870 | 22 241.75 | 25 161.25 |
| Frequencies assignable to ship stations for data transmission d) p) aa) bb) cc) |  |  |  |  |  |  |  |  |
| Limits (kHz) | 4 175.25 | 6 266.25 | 8 341.75 | 12 421.75 | 16 618.75 | 18 870 | 22 241.75 | 25 161.25 |
| Frequencies (paired) assignable to ship stations for narrow-band direct-printing (NBDP) telegraphy and data transmission systems at speeds not exceeding 100 Bd for FSK and 200 Bd for PSK d) j) | 4 176 to 4 178 5 f. 0.5 kHz | 6 266.5 to 6 268.5 5 f. 0.5 kHz |  |  |  |  |  |  |
| Limits (kHz) | 4 178.25 | 6 268.75 | 8 341.75 | 12 421.75 | 16 618.75 | 18 870 | 22 241.75 | 25 161.25 |
| Frequencies assignable to ship stations for data transmission d) p) aa) bb) cc) |  |  |  |  |  |  |  |  |
| Limits (kHz) | 4 181.75 | 6 275.75 | 8 341.75 | 12 421.75 | 16 618.75 | 18 870 | 22 241.75 | 25 161.25 |

Table of frequencies (kHz) to be used in the band between 4 000 kHz and 27 500 kHz
allocated exclusively to the maritime mobile service (continued )

| Band (MHz) | 4 | 6 |  | 8 | 12 | 16 | 18/19 |  | 22 |  | 25/26 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Limits (kHz) | 4 181.75 | 6 275.75 |  | 8 341.75 | 12 421.75 | 16 618.75 | 18 870 |  | 22 241.75 |  | 25 161.25 |
| Frequencies assignable to ship stations for data transmission m) p) |  |  |  |  |  |  |  |  |  |  |  |
| Limits (kHz) | 4 186.75 | 6 280.75 |  | 8 341.75 | 12 421.75 | 16 618.75 | 18 870 |  | 22 241.75 |  | 25 161.25 |
| Frequencies assignable to ship stations for data transmission d) p) aa) bb) cc) |  |  |  |  |  |  |  |  |  |  |  |
| Limits (kHz) | 4 186.75 | 6 284.75 |  | 8 341.75 | 12 421.75 | 16 618.75 | 18 870 |  | 22 241.75 |  | 25 161.25 |
| Frequencies assignable to ship stations for data transmission m) p) |  |  |  |  |  |  |  |  |  |  |  |
| Limits (kHz) | 4 202.25 | 6 300.25 |  | 8 365.75 | 12 476.75 | 16 683.25 | 18 870 |  | 22 279.25 |  | 25 171.25 |
| Frequencies assignable to ship stations for data transmission m) p) |  |  |  |  |  |  |  |  |  |  |  |
| Limits (kHz) | 4 202.25 | 6 300.25 |  | 8 370.75 | 12 476.75 | 16 683.25 |  | 18 870 |  | 22 284.25 | 25 172.75 |
| Frequencies assignable to ship stations for data transmission m) p) |  |  |  |  |  |  |  |  |  |  |  |
| Limits (kHz) | 4 202.25 | 6 300.25 |  | 8 376.25 | 12 476.75 | 16 683.25 |  | 18 870 |  | 22 284.25 | 25 172.75 |

Table of frequencies (kHz) to be used in the band between 4 000 kHz and 27 500 kHz
allocated exclusively to the maritime mobile service (continued))

| Band (MHz) | 4 | 6 | 8 | 12 | 16 | 18/19 | 22 | 25/26 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Limits (kHz) | 4 202.25 | 6 300.25 | 8 376.25 | 12 476.75 | 16 683.25 | 18 870 | 22 284.25 | 25 172.75 |
| Frequencies (paired) assignable to ship stations for NBDP telegraphy and data transmission systems at speeds not exceeding 100 bauds for FSK and 200 bauds for PSK d) j) |  |  | 8 376.5 to 8 378.5 5 f. 0.5 kHz |  |  |  |  |  |
| Limits (kHz) | 4 202.25 | 6 300.25 | 8 378.75 | 12 476.75 | 16 683.25 | 18 870 | 22 284.25 | 25 172.75 |
| Frequencies assignable to ship stations for data transmission d) p) aa) bb) cc) |  |  |  |  |  |  |  |  |
| Limits (kHz) | 4 202.25 | 6 300.25 | 8 396.25 | 12517.25 | 16 693.25 | 18 892.75 | 22 351.75 | 25 192.75 |
| Frequencies (paired) assignable to ship stations for NBDP telegraphy and data transmission systems at speeds not exceeding 100 bauds for FSK and 200 bauds for PSK d) j) |  |  |  | 12 517.5 to 12 522 10 f. 0.5 kHz | 16 693.5 to 16 696.5 7 f. 0.5 kHz |  |  |  |
| Limits (kHz) | 4 202.25 | 6 300.25 | 8 396.25 | 12 522.25 | 16 696.75 | 18 892.75 | 22 351.75 | 25 192.75 |
| Frequencies assignable to ship stations for data transmission d) p) aa) bb) cc) |  |  |  |  |  |  |  |  |
| Limits (kHz) | 4 202.25 | 6 300.25 | 8 396.25 | 12 549.75 | 16 733.75 | 18 892.75 | 22 351.75 | 25 192.75 |
| Frequencies assignable to ship stations for data transmission m) p) |  |  |  |  |  |  |  |  |
| Limits (kHz) | 4 202.25 | 6 300.25 | 8 396.25 | 12 554.75 | 16 738.75 | 18 892.75 | 22 351.75 | 25 192.75 |
| Frequencies assignable to ship stations for data transmission aa) bb) cc) d) p) |  |  |  |  |  |  |  |  |
| Limits (kHz) | 4 202.25 | 6 300.25 | 8 396.25 | 12 559.75 | 16 784.75 | 18 892.75 | 22 351.75 | 25 192.75 |

Table of frequencies (kHz) to be used in the band between 4 000 kHz and 27 500 kHz
allocated exclusively to the maritime mobile service (continued)

| Band (MHz) | 4 |  | 6 |  | 8 | 12 | 16 |  |  | 18/19 |  | 22 | 25/26 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Limits (kHz) | 4 202.25 |  | 6 300.25 |  | 8 396.25 | 12 559.75 | 16 784.75 |  |  | 18 892.75 |  | 22 351.75 | 25 192.75 |
| Frequencies (non paired) assignable to ship stations for data transmission systems b) p) dd) m) gg) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Limits (kHz) |  | 4 207.25 |  | 6 311.75 | 8 414.25 | 12 576.75 |  | 16 804.25 | 18 898.25 |  | 22 374.25 |  | 25 208.25 |
| Frequencies assignable to ship stations for digital selective calling k) l) |  | 4 207.5 to 4 209 4 f. 0.5 kHz |  | 6 312 to 6 313.5 4 f. 0.5 kHz | 8 414.5 to 8 416 4 f. 0.5 kHz | 12 577 to 12 578.5 4 f. 0.5 kHz |  | 16 804.5 to 16 806 4 f. 0.5 kHz | 18 898.5 to 18 899.5 3 f. 0.5 kHz |  | 22 374.5 to 22 375.5 3 f. 0.5 kHz |  | 25 208.5 to 25 209.5 3 f. 0.5 kHz |
| Limits (kHz) |  | 4 209.25 |  | 6 313.75 | 8 416.25 | 12 578.75 |  | 16 806.25 | 18 899.75 |  | 22 375.75 |  | 25 210 |
| Limits (kHz) |  | 4 209.25 |  | 6 313.75 | 8 416.25 | 12 578.75 |  | 16 806.25 | 19 680.25 |  | 22 375.75 |  | 26 100.25 |
| Frequencies assignable to coast stations for data transmission n) o) p) aa) bb) cc) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Limits (kHz) |  | 4 213.75 |  | 6 317.75 | 8 416.25 | 12 619.75 |  | 16 816.75 | 19 703.25 |  | 22 443.75 |  | 26 120.75 |
| Frequencies (paired) assignable to coast stations for NBDP and data transmission systems, at speeds not exceeding 100 Bd for FSK and 200 Bd for PSK d) |  | 4 214 to 4 215.5 4 f. 0.5 kHz |  | 6 318 to 6 319.5 4 f. 0.5 kHz | 8 416.5 to 8 418.5 5 f. 0.5 kHz | 12 620 to 12 624 9 f. 0.5 kHz |  | 16 817 to 16 819.5 6 f. 0.5 kHz |  |  |  |  |  |
| Limits (kHz) |  | 4 215.75 |  | 6 319.75 | 8 418.75 | 12 624.25 |  | 16 819.75 | 19 703.25 |  | 22 443.75 |  | 26 120.75 |
| Frequencies assignable to coast stations for data transmission d) p) aa) bb) cc) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Limits (kHz) |  | 4 219.25 |  | 6 330.75 | 8 436.25 | 12 656.75 |  | 16 902.75 | 19 703.25 |  | 22 443.75 |  | 26 120.75 |
| Frequencies assignable to coast stations for digital selective calling l) |  | 4 219.5 to 4 220.5 3 f. 0.5 kHz |  | 6 331 to 6 332 3 f. 0.5 kHz | 8 436.5 to 8 437.5 3 f. 0.5 kHz | 12 657 to 12 658 3 f. 0.5 kHz |  | 16 903 to 16 904 3 f. 0.5 kHz | 19 703.5 to 19 704.5 3 f. 0.5 kHz |  | 22 444 to 22 445 3 f. 0.5 kHz |  | 26 121 to 26 122 3 f. 0.5 kHz |
| Limits (kHz) |  | 4 221 |  | 6 332.5 | 8 438 | 12 658.5 |  | 16 904.5 | 19 705 |  | 22 445.5 |  | 26 122.5 |

Table of frequencies (kHz) to be used in the band between 4 000 kHz and 27 500 kHz
allocated exclusively to the maritime mobile service (end)

| Band (MHz) | 4 | 6 | 8 | 12 | 16 | 18/19 | 22 | 25/26 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Limits (kHz) | 4 221 | 6 332.5 | 8 438 | 12 658.5 | 16 904.5 | 19 705 | 22 445.5 | 26 122.5 |
| Frequencies assignable to coast stations for wide-band, facsimile, special and data transmission systems and direct-printing telegraphy systems m) p) ee) ff) |  |  |  |  |  |  |  |  |
| Limits (kHz) | 4 351 | 6 501 | 8 707 | 13 077 | 17 242 | 19 755 | 22 696 | 26 145 |
| Frequencies assignable to coast stations for telephony, duplex operation a) hh) | 4 352.4 to 4 436.4 29 f. 3 kHz | 6 502.4 to 6 523.4 8 f. 3 kHz | 8 708.4 to 8 813.4 36 f. 3 kHz | 13 078.4 to 13 198.4 41 f. 3 kHz | 17 243.4 to 17 408.4 56 f. 3 kHz | 19 756.4 to 19 798.4 15 f. 3 kHz | 22 697.4 to 22 853.4 53 f. 3 kHz | 26 146.4 to 26 173.4 10 f. 3 kHz |
| Limits (kHz) | 4 438 | 6 525 | 8 815 | 13 200 | 17 410 | 19 800 | 22 855 | 26 175 |

Reasons:

–	to reduce the current frequencies for NBDP to the GMDSS requirement (see Appendix 15), with the addition of some channels, in order to maintain the current usage;

–	to prohibit the usage of new digital technologies in these GMDSS/NBDP core bands;

–	to release after a transition period the rest of the frequency bands of NBDP for a possible use for data transmission with digital technologies (e.g. see Recommendation ITU-R M.1798) in the maritime mobile service. At the same time it offers the possibility for the administrations, who wish to do so, to continue to use (by footnotes) these frequencies bands for NBDP, after the transition period without claiming protection or causing interference;

–	to release the frequency bands designated for facsimile, wide-band telegraphy, Morse telegraphy A1A/A1B for new digital technologies within the maritime community. While enabling at the same time the possibility (by footnotes), to use facsimile, wide-band telegraphy or Morse telegraphy A1A/A1B, for administration who wish to continue to do so without claiming protection or causing interference;

–	to not specify any bandwidth in the new created bands dedicated for digital technologies;

–	to keep the frequency bands designated for duplex radiotelephony untouched (linked with Appendix 25).

NOC

Note a)

MOD	EUR/XXA9/2

Note b)	Until the 1st January 2015, see Part B, Section III. After this date the Section III will no longer apply.

Reasons:

Introduction of new technologies is possible in proposed frequency bands after a proper transition period.  Europe proposes to set the end date of the transition period on 1 January 2015 in order to safeguard the simultaneous use of current systems and new digital technology.

Requirements for the proposed transition period are:

–	a transition period needs to be established during which the current usage for NBDP (general communication) is unchanged (same channelling arrangement) and the Administration who will introduce digital communications are encouraged to coordinate with affected administrations;

–	at the end of the transition period the analogue NBDP transmission have to stop outside the core band (new channelling arrangement for the remaining NBDP applications); however an Administration, who wishes so, could continue to use the analogue NBDP technology without claiming protection from other stations and without causing any harmful interference to other stations in the maritime mobile service using digital technologies;

–	the date for the cessation of the transition period is set at the 1st January 2015;

–	in the frequency bands designated for wide-band telegraphy, facsimile, Morse telegraphy A1A/A1B new digital technologies can be implemented without taking a transition period into account;

–	after 1st January 2015, the Administrations who will make assignments to stations using digital data transmissions are encouraged to effect coordination with potentially affected Administrations;

NOC

c) and d)

SUP	EUR/XXA9/3

Note e)

Reasons: 	Consequential suppression as result of the proposed deletion of sections IV and V, dealing with Morse telegraphy.

SUP	EUR/XXA9/4

Note f)

Reasons: 	Consequential suppression as result of the proposed deletion of sections IV and V, dealing with Morse telegraphy.

SUP	EUR/XXA9/5

Note g)

Reasons: 	Consequential suppression as result of the proposed deletion of sections IV and V, dealing with Morse telegraphy

NOC

Note h) to l)

MOD	EUR/XXA9/6

Note m)	  Frequencies from these frequency bands may also be used for A1A or A1B Morse telegraphy subject to not claiming protection from other stations in the maritime mobile service using digitally modulated emissions

Reasons: 	This modified Note will authorise administrations who wish to continue to use the Morse telegraphy bands to do so without claiming protection from other stations in the maritime mobile service using digital technologies.

NOC

Note n) and o)

MOD	EUR/XXA9/7

Note p)	These sub-bands, except the frequencies referred to in Notes i), j), n) and o), may be used for digitally modulated emissions for maritime mobile service ( e.g. as described in Recommendation ITU-R 1798). The provision of No. 15.8 applies.

Reasons:	This footnote designates the sub bands where digital technologies, e.g. as  described in Recommendation ITU-R 1798, could be used.

ADD	EUR/XXA9/8

Note aa)	Until 1 January 2015 these bands may be used by narrow-band direct printing applications. The use of these bands by digital data transmission applications is subject to prior agreement between interested and affected administrations to ensure there will be no interference with the narrow-band direct printing applications.

ADD 	EUR/XXA9/9

Note bb)	After 1 January 2015 these bands, except the frequencies referred to in Notes n) and o), may be used by narrow-band direct printing applications by the administrations, subject to not claiming protection from other stations in the maritime mobile service using digital data transmissions.

ADD 	EUR/XXA9/10

Note cc)	After 1 January 2015 the administrations who will make assignment to stations using digital data transmissions are encouraged to effect coordination with potentially affected administrations.

ADD 	EUR/XXA9/11

Note dd)	These bands may be used by narrow-band direct printing applications by the administrations, subject to not claiming protection from other stations in the maritime mobile service using digitally modulated emissions.

ADD 	EUR/XXA9/12

Note ee)	Frequencies from these bands may be used for wide-band telegraphy, facsimile, A1A Morse telegraphy and special data transmission on condition that harmful interference is not caused to and protection is not claimed from stations in the maritime mobile service using digitally modulated emissions.

ADD 	EUR/XXA9/13

Note ff)		The bands 4 345-4 351 kHz, 6 495-6 501 kHz, 8 701-8 707 kHz may be used for simplex (single-sideband) telephone operation (regularly spaced by 3 kHz), in accordance with provision No. 52.177, subject to not claiming protection from other stations in the maritime mobile service using digitally modulated emissions.

ADD 	EUR/XXA9/14

Note gg)	When assigning frequencies on the bands 4 202.25-4 207.25 kHz, 6 300.25-6 311.75 kHz, 8 396.25-8 414.25 kHz, 12 559.75-12 576.75 kHz and 16 784.75-16 804.25 kHz administrations shall take all necessary precautions in order not to cause interference on the DSC distress frequencies 4 207.5 kHz, 6 312 kHz, 8 414.5 kHz, 12 577 kHz and 16 804.5 kHz.

ADD	EUR/XXA9/15

Note hh) 	The bands 4 066.4-4 150.4 kHz, 4 352.4-4 436.4 kHz, 6 201.4-6 231.4 kHz, 6 502.4-6 523.4 kHz, 8 196.4-8 298.4 kHz, 8 708.4-8 813.4 kHz, 12 231.4-12 366.4 kHz, 13 078.4-13 198.4 kHz, 16 361.4-16 574.4 kHz, 17 243.4-17 408.4 kHz, 18 781.4-18 844.4 kHz, 19 756.4-19 798.4 kHz, 22 001.4-22 178.4 kHz, 22 697.4-22 853.4 kHz, 25 071.4-25 119.4 kHz, 26 146.4-26 173.4 kHz may be used, in accordance with the Appendix 25 allotment Plan, for digitally modulated emissions as described e.g. in Recommendation ITU-R M.1798 on condition that it shall not cause harmful interference to, or claim protection from other stations in the maritime mobile service using radiotelephony operations. The digitally modulated emissions may be used provided that their occupied bandwidth does not exceed 2 800 Hz, it is situated wholly within one frequency channel and the peak envelope power of coast stations does not exceed 10 kW and the peak envelope power of ship stations does not exceed 1.5 kW per channel.

PART  B  –  Channelling arrangements     (WRC-12)

NOC     	EUR/XXA9/16

Section I – Radiotelephony

Reasons: 	Frequency bands designated for duplex for radiotelephony linked with Appendix 25 should be kept untouched since any modifications may have a negative impact on the integrity of the Appendix 25 allotment plan

MOD  	EUR/XXA9/17

Section II  –  Narrow-band direct-printing telegraphy (paired frequencies)

Table of frequencies for two-frequency operation by coast stations (kHz) to be used until the 1st  January 2015.

Reasons: 	To indicate clearly that the Table of frequencies for two-frequency operation by coast stations (kHz) does not change during the transition period which is proposed to end on 1 January 2015. The table itself is unchanged.

ADD 	EUR/XXA9/18

Table of frequencies for two-frequency operation by coast stations (kHz)
to be used after the 1st January 2015

| Channel No. | 4 MHz band1 |  | 6 MHz band |  | 8 MHz band |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Transmit | Receive | Transmit | Receive | Transmit | Receive |
| 1 2 3 4 5 |  |  |  |  | 8 376.52 8 417 8 417.5 8 418 8 418.5 | 8 376.52 8 377 8 377.5 8 378 8 378.5 |
| 6 7 8 9 10 | 4 214 4 214.5 4 215 | 4 176 4 176.5 4 177 | 6 318 6 318.5 6 319 | 6 266.5 6 267 6 267.5 |  |  |
| 11 12 13 | 4 177.52 4 215.5 | 4 177.52 4 178 | 6 2682 6 319.5 | 6 2682 6 268.5 |  |  |
| 1 Ship stations may use the coast station receiving frequencies for transmitting A1A or A1B Morse telegraphy (working), with the exception of channel No. 11 (see Appendix 15). 2 For the conditions of use of this frequency, see Article 31. |  |  |  |  |  |  |

Table of frequencies for two-frequency operation
by coast stations (kHz)

| Channel No. | 12 MHz band |  | 16 MHz band |  |
| --- | --- | --- | --- | --- |
|  | Transmit | Receive | Transmit | Receive |
| 21 22 23 24 25 |  |  | 16 817 16 817.5 16 818 16 6952 16 818.5 | 16 693.5 16 694 16 694.5 16 6952 16 695.5 |
| 26 27 |  |  | 16 819 16 819.5 | 16 696 16 696.5 |
| 82 83 84 85 86 | 12 620 12 620.5 12 621 12 621.5 12 622 | 12 517.5 12 518 12 518.5 12 519 12 519.5 |  |  |
| 87 88 89 90 91 | 12 5202 12 622.5 12 623 12 623.5 12 624 | 12 5202 12 520.5 12 521 12 521.5 12 522 |  |  |

Reasons: 	The Table of frequencies for two-frequency operation by coast stations (kHz) will change after the transition period on 1 January 2015.

MOD  	EUR/XXA9/19

Section III  –  Narrow-band direct-printing telegraphy 
(non-paired frequencies) 1

ADD 	EUR/XXA9/20

_______________

1	This section is valid until the 1st January 2015 and will not apply after this date

SUP 	EUR/XXA9/21

Section IV  –  Morse telegraphy (calling)

Reasons: 	Consequential suppression as result of the proposed deletion of the sections dealing with Morse telegraphy

SUP 	EUR/XXA9/22

Section V  –  Morse telegraphy (working)

Reasons:	Consequential suppression as result of the proposed deletion of the sections dealing with Morse telegraphy

SUP          EUR/XXA9/23

Resolution 351 (Rev.WRC-07)

Review of the frequency and channel arrangements in the HF bands allocated to the maritime mobile service contained in Appendix 17 with a view
to improving efficiency through the use of new digital technology
by the maritime mobile service

Reasons:	With the revision of the Appendix 17 which is proposed, it is considered that this Resolution has been completely treated and therefore could be deleted

_________________________________

____________________

1 	Within the non-shaded boxes.