	[image: image1.wmf]ECC

Electronic Communications Committee

CEPT

ECC PT1

	ECC PT1(12)005

	ECC PT1 #40
	

	Wroclaw, 11-13 April 2012
	

	
	

	Date issued: 
	11 October 2011

	Source:
	SE7

	Subject:   
	Protection criteria of mobile systems below 470 MHz

	
Password protection required? (Y/N) 



	



[image: image2.emf]SE7(11)Temp019rev1  LS on the protectional requirements of mobile systems in 450-470 MHz.DOC


N








PAGE  
2

_1378905586.doc


		[image: image1.wmf]ECC


Electronic Communications Committee


CEPT




		Doc. SE7(11)Temp019

29th September 2011



		PT SE7 Meeting


ECO, Copenhagen, 27-29 September 2011





		To: Mr Alexandre Kholod


Chairman SE43 (alexandre.kholod@bakom.admin.ch)


Cc: Mr Bruno Espinosa

Chairman WG SE (espinosa@anfr.fr)


Cc: Mr Didier Chauveau


Chairman ECC PT1 chauveau@anfr.fr

Source: SE7

Subject: Protection criteria of mobile systems below 470 MHz
 





Dear Alexandre, 


SE7 wishes to thank SE43 for their liaison statement regarding the protection criteria of mobile systems below 470 MHz.


The mobile systems in the band 450-470 MHz, which have been considered in the studies done by SE7, are


· Analogue FM mobile radio. The typical bandwidth is 25 kHz, but other bandwidths (20 kHz or 12.5 kHz) are also in use. 

· TETRA


· TETRA TEDS (with different bandwidths of 25, 50, 100 and 150 kHz)


· Tetrapol


· CDMA-PAMR


The technical characteristics of those systems are in ECC Report 99. See Appendix B of the Report for protectional reguirements. They are extracted from the relevant ETSI Standards or other publicly available documents. For the non-CDMA systems, to guarantee interference free situation, the total interfering signal level at the victim receiver should be below the level of (sensitivity – protection ratio). For the CDMA systems, the impact is characterised as the percentage of the capacity loss of the victim cell.

WIth Best Regards,


Petteri Jokela

Chairman SE7


petteri.jokela@ficora.fi 









PAGE  

Page 1/1




