EFISMG(12)10 Rev

ECC Decision (01)03

(2007/344/EC)

ECO Frequency Information System (EFIS)

amended 25 June 2010

amended 15 June 2011

amended tbd

# explanatory memorandum

## INTRODUCTION

Understanding how frequencies are actually utilised is an important step in harmonising spectrum within Europe and beyond. Industry, the European Commission and administrations have expressed a strong interest in having a database containing frequency utilisation information that is comparable across Europe.

Its purpose would be

to give the CEPT a tool to illustrate the extent of harmonisation within Europe,

to allow administrations to quickly search for and compare spectrum utilisation information of other

CEPT countries, and

to meet the European Commission and industry requirements that have been made known to CEPT at many occasions.

The ERO made a proposal to develop the ERO Frequency Information System (EFIS), now ECO Frequency Information System that would fulfil this purpose. The development of EFIS takes place in close collaboration with those that have to input information into EFIS (i.e. administrations) and those that will use the information contained in EFIS (i.e. administrations, industry and other interested parties). Comments from EICTA (European Information and Communications Technology Industry association) and ETSI (European Telecommunications Standards Institute) have been received encouraging CEPT to develop EFIS and signalling that industry is willing to support this process with their expertise.

## background

The issue of frequency databases has been discussed for some time and it is evident that there are many different approaches that have been taken on the national level in presenting frequency data. Discussions have also shown that administrations are reluctant to having all national frequency data collected in one central place and to providing additional resources beyond those needed on the national level.

EFIS can basically be described as a search engine that allows the user to search for a specific utilisation in one or more CEPT countries, thus enabling a comparison between the Radio Regulations, the European table (ERC Report 25) and current national utilisations. The result of the search is a list of frequency bands or a frequency range showing the relevant allocations and applications. Further details are not necessarily contained in EFIS, but can be accessed via a link to the relevant national table or to other important documents. The limitation of the actual database to concise information simplifies the task and the expected workload, while providing a commonly accessible search and comparison tool that complements and adds value to the national initiatives.

One of the main objectives of EFIS is to ensure that it provides good quality information, which is regularly updated and maintained. This ECC Decision is the mechanism for ensuring that this objective is met. Furthermore, this ECC Decision provides two lists of harmonised terms, which are essential for making an efficient and meaningful search for frequency information. Especially the List of Searchable Applications in Annex 2, is an important part of EFIS, because it describes the utilisation of a certain frequency band. This is key information that industry is interested in and the terms provide the starting point for a more detailed search in the national frequency tables, which are established and maintained by administrations.

Just like the List of Radio Services in the ITU RR in Annex 1, the List of Searchable Applications in Annex 2 has several layers of detail that allow administrations to choose the level of detail it would like to indicate within a certain frequency band. When searching for and comparing information EFIS makes use of these layers. For example, a search for a specific term in layer 2 will automatically start a search for all terms in layer 3 under that specific term. This functionality allows for an efficient and meaningful comparison, even though each administration has the flexibility to choose the level of detail it would like to indicate in a specific band. Annex 3 contains the list of parameters for radio interfaces in EFIS, developed on the basis of the template and the guide developed by TCAM RIG II and adopted by TCAM. The use of these parameters in EFIS allows an efficient comparison of interfaces within Europe. In Annex 4 a standard for information on the right of use for frequency bands of high economic interest, where market mechanisms apply, is given.

## requirement for an ecc decision

Administrations have developed different formats for presenting national frequency utilisation information. Furthermore, it is often difficult to compare the information contained in the numerous national tables, because of different expressions and languages being used. Consequently, there is a need for a tool that provides administrations, industry and the interested public with comparable spectrum information. This information will then lead the user to more detailed information on the national level or it will give a reliable picture of the spectrum harmonisation that has been achieved in Europe. For this tool to be successful administrations must agree upon a harmonised list of terms to be used as well as a procedure that will ensure that the information that has been collected is updated regularly and of good quality.

An ECC Decision will ensure that the harmonised terms and procedures, which are essential for the success of the system, are used by administrations and ECO when entering data into EFIS.

# ECC Decision of 15 November 2011 on eco frequency information system (efis) (ECC/DEC/(01)03)

“The European Conference of Postal and Telecommunications Administrations,

Comparable technical specifications to those given in this ECC Decision are given in EC Decision 2007/344/EC. EU Member States and, if so approved by the EEA Joint Committee, Iceland, Liechtenstein and Norway are obliged to implement the EC Decision.

considering

- that administrations, industry, and the European Commission have expressed a strong interest in having a database containing frequency utilisation information that is comparable across Europe;

- that EFIS is designed to fulfil this requirement;

- that the data collected in EFIS is to be used for a meaningful search and comparison of spectrum information available within CEPT member countries;

- that for EFIS to be successful administrations must agree upon a harmonised list of terms to be used as well as a procedure that will ensure that the information that has been collected is updated regularly and of good quality;

- the decision taken at the 12th meeting of the ERO Council to make available the necessary resources within ERO in order to fulfil the tasks required under this Decision;

- that the List of Searchable Applications aims to facilitate an efficient and meaningful search and not a legally binding description of the applications used on the national level;

- there is a need to administer and further develop EFIS;

- that there is a need to establish a contact person within each administration for the maintenance of the national frequency information;

- that the future development of EFIS should take into account the R&TTE Directive 1999/5/EC, Proposal for a Decision of the European Parliament and Council on a regulatory framework for radio spectrum policy in the EC, the ERC/DEC/(97)01 and its future revisions on publication of National Tables of Frequency Allocations (NTFAs) and the publication of national frequency utilisation information;

- that there is considerable difference in national licensing, laws and regulations;

- that the EU Member States and Iceland, Liechtenstein and Norway adopted the EC Decision 2007/344/EC of 16 May 2007 which makes it mandatory for those countries to provide information on the radio interface specifications and rights of use of radio spectrum in accordance with Decides 2, however, the EC Decision does not apply to other CEPT countries which may provide the information on an optional basis;

- that there is a need to limit the amount of resources needed to update and maintain EFIS as far as possible;

- that the duplication of information should be avoided as far as possible;

- that the availability of NTFAs in the English language and in PDF format would be preferable.

DECIDES

Administrations shall enter and maintain the following mandatory data into EFIS:

  - Spectrum allocations on a national level according to the List of Radio Services in the ITU RR in Annex 1;

  - Spectrum applications on a national level according to the List of Searchable Applications in

    - Annex 2;

  - A Contact Person within the Administration who will be responsible for the maintenance of the national frequency information related to EFIS.

Administrations should enter and maintain the following data into EFIS:

- Radio interface specifications on a national level according to the template in Annex 3;

- Right of use information on a national level only for frequency bands for electronic communication services, where spectrum trading is allowed or where comparative or competitive selection procedures are used according to the model in Annex 4.

Administrations may enter and maintain the following optional data into EFIS:

- Short comments related to an allocation or application;

- Documents or hyperlinks that can be filed within EFIS according to a frequency band, an application or both (e.g. related to Activities or R&TTE interface information).

Administrations shall provide ECO with a copy of their most detailed public national frequency table (e.g. NTFA or frequency utilisation table) in a format acceptable to ECO. The table should be sent to ECO no later than one week after publication.

ECO shall1 enter and maintain the data in EFIS related to the Radio Regulations (Region 1), the European Table of Frequency Allocations and Utilisations, and other appropriate tables that are not maintained by an administration.

ECO shall1 administer EFIS and execute further developments of EFIS according to agreements reached in the ECC and the ECO Council.

For uploading or downloading data to or from EFIS by administrations, the Harmonised Interface in Annex 5 shall be used. Administrations with a national frequency database are encouraged to develop a software tool that will allow automatic transfer of relevant data from their database into EFIS. This will allow for easy updating and maintenance of allocations, applications, radio interfaces and right of use information.

The List of Radio Services in the ITU RR, the List of Searchable Applications and the Harmonised Interface are the valid versions when this Decision comes into force. Depending on regulatory and market developments, the ECC or a delegated subgroup may develop new versions of these annexes subject to positive acceptance by administrations that have committed themselves to this Decision without the need for Public Consultation. ECO shall archive all versions and distribute any new versions to all Contact Persons stating when the new version will come into force. The List of Searchable Applications shall be reviewed at least once a year through a procedure initiated by ECO.

that this Decision shall enter into force on 25 June 2010;

that CEPT Member administrations shall communicate the national measures implementing this Decision to the ECC Chairman and the Office when the Decision is nationally implemented.”

Note:

Please check the Office documentation database http://www.ecodocdb.dk for the up to date position on the implementation of this and other ECC Decisions.

list of radio services in the itu radio regulations (rr)

This is the list of services, which have an allocation in Article 5 of the RR.

For the purpose of this decision the List of Radio Services in the ITU RR is divided into three layers of detail in accordance with the definitions given in the RR. When searching for and comparing information EFIS makes use of these layers. For example, a search for a specific term in layer 2 will automatically start a search for all terms in layer 3 under that specific term. If nothing is found in either layer 2 or 3, EFIS also checks layer 1 and informs the user if there is a hit.

Example of tables (style: ECC Table title)

| Layer 1 | Layer 2 | Layer 3 |
| --- | --- | --- |
| Amateur |  |  |
| Amateur-Satellite |  |  |
| Broadcasting |  |  |
| Broadcasting-Satellite |  |  |
| Earth Exploration-Satellite | Earth Exploration-Satellite (active) Earth Exploration-Satellite (passive) Meteorological-Satellite |  |
| Fixed |  |  |
| Fixed-Satellite |  |  |
| Inter-Satellite |  |  |
| Mobile | Aeronautical Mobile | Aeronautical Mobile (R) Aeronautical Mobile (OR) |
|  | Land Mobile |  |
|  | Maritime Mobile |  |
|  |  | Maritime Mobile (distress and safety) |
|  |  | Maritime Mobile (distress and calling) |
|  |  | Maritime Mobile (distress, safety and calling) |
|  |  | Maritime Mobile (distress and calling via DSC) |
|  | Mobile (distress and safety) |  |
|  | Mobile (distress and calling) |  |
|  | Mobile (distress, safety and calling) |  |
|  | Mobile except aeronautical mobile |  |
|  | Mobile except aeronautical mobile (R) |  |
| Mobile-Satellite | Aeronautical Mobile-Satellite | Aeronautical Mobile-Satellite (R) Aeronautical Mobile-Satellite (OR) |
|  | Land Mobile-Satellite |  |
|  | Maritime Mobile-Satellite |  |
|  | Mobile-satellite except aeronautical mobile-satellite |  |
|  | Mobile-satellite except aeronautical mobile-satellite (R) |  |
| Meteorological Aids |  |  |
| Radio Astronomy |  |  |
| Radiodetermination | Radionavigation | Aeronautical Radionavigation Maritime Radionavigation Maritime Radionavigation (radiobeacons) |
|  | Radiolocation |  |
| Radiodetermination-Satellite | Radionavigation-Satellite | Aeronautical Radionavigation-Satellite Maritime Radionavigation-Satellite |
|  | Radiolocation-Satellite |  |
| Space Operation |  | Space Operation (satellite identification) |
| Space Research | Space Research (active) Space Research (deep space) Space Research (passive) |  |
| Standard Frequency and Time Signal |  |  |
| Standard Frequency and Time Signal-Satellite |  |  |

Note:

For bolded services it is possible to give additions/attributes (space-to-Earth, Earth-to-space, space-to-space). 
An alphabetical list of services with all in the Radio Regulations, Edition 2008, and existing combinations of additions/attributes for allocations is provided below.

LIST OF ALL RADIOCOMMUNICATION SERVICES WITH ADDITIONS,
USED IN THE RADIO REGULATIONS, EDITION 2008

| Radiocommunication service: | Addition: |
| --- | --- |
| Aeronautical mobile | (R) |
| Aeronautical mobile | (OR) |
| Amateur-satellite | (Earth-to-space) |
| Amateur-satellite | (space-to-Earth) |
| Earth exploration-satellite | (Earth-to-space) |
| Earth exploration-satellite | (space-to-Earth) |
| Earth exploration-satellite | (Earth-to-space) (space-to-space) |
| Earth exploration-satellite | (space-to-Earth) (space-to-space) |
| Earth exploration-satellite | (active) |
| Earth exploration-satellite | (passive) |
| Fixed-satellite | (Earth-to-space) |
| Fixed-satellite | (space-to-Earth) |
| Fixed-satellite | (Earth-to-space) (space-to-Earth) |
| Fixed-satellite | (space-to-Earth) (Earth-to-space) |
| Maritime mobile | (distress and calling via DSC) |
| Maritime mobile | (distress and calling) |
| Maritime radionavigation | (radiobeacons) |
| Meteorological-satellite | (Earth-to-space) |
| Meteorological-satellite | (space-to-Earth) |
| Mobile | except aeronautical mobile |
| Mobile | except aeronautical mobile (R) |
| Mobile | (distress and calling) |
| Mobile-satellite | (Earth-to-space) |
| Mobile-satellite | (space-to-Earth) |
| Mobile-satellite | except aeronautical mobile-satellite (Earth-to-space) |
| Radiodetermination-satellite | (Earth-to-space) |
| Radiodetermination-satellite | (space-to-Earth) |
| Radiolocation-satellite | (Earth-to-space) |
| Radionavigation-satellite | (Earth-to-space) |
| Radionavigation-satellite | (space-to-Earth) (space-to-space) |
| Space operation | (satellite identification) |
| Space operation | (Earth-to-space) |
| Space operation | (space-to-Earth) |
| Space operation | (Earth-to-space) (space-to-space) |
| Space operation | (space-to-Earth) (space-to-space) |
| Space research | (Earth-to-space) |
| Space research | (space-to-Earth) |
| Space research | (space-to-space) |
| Space research | (deep space) |
| Space research | (Earth-to-space) (space-to-space) |
| Space research | (space-to-Earth) (space-to-space) |
| Space research | (deep space) (Earth-to-space) |
| Space research | (deep space) (space-to-Earth) |
| Space research | (active) |
| Space research | (passive) |
| Standard frequency and time signal | (20 kHz) |
| Standard frequency and time signal | (2 500 kHz) |
| Standard frequency and time signal | (5 000 kHz) |
| Standard frequency and time signal | (10 000 kHz) |
| Standard frequency and time signal | (15 000 kHz) |
| Standard frequency and time signal | (20 000 kHz) |
| Standard frequency and time signal | (25 000 kHz) |
| Standard frequency and time signal-satellite | (400.1 MHz) |
| Standard frequency and time signal-satellite | (Earth-to-space) |
| Standard frequency and time signal-satellite | (space-to-Earth) |

list of searchable applications

Explanatory Note

The list of Searchable Applications has been developed in order to allow an efficient and meaningful search for frequency information within Europe. It is based on the following principles:

- The list should facilitate an efficient and meaningful search and not a legally binding description of the Application terms used.

- The list should only use unambiguous terms, which give clear guidance for data entry and retrieval.

- The List of Searchable Applications is complementary to the List of Radio Services in the ITU RR and it is meant to describe the actual utilisation of the frequency bands. In other words, the List of Radio Services in the ITU RR gives the regulatory framework and the List of Searchable Applications gives the actual use.

- The List of Searchable Applications should allow administrations to associate the terms used on a national level with the terms used in the list.

The List of Searchable Applications is divided into three layers of detail. This allows each administration to choose the level of detail it would like to indicate within a certain frequency band. When searching for and comparing information EFIS makes use of these layers. For example, a search for a specific term in layer 2 will automatically start a search for all terms in layer 3 under that specific term. If nothing is found in either layer 2 or 3, EFIS also checks layer 1 and informs the user if there is a hit. This functionality allows for an efficient and meaningful comparison, even though each administration has the flexibility to choose the level of detail it would like to indicate in a specific band.

Wherever possible administrations should use the highest detail possible (layer 3) when entering data into EFIS. In general, layer 3 only represents a few more specific expressions that do not necessarily cover all possible applications of the relevant term in layer 2. Those applications that are not covered by these more specific expressions are to be associated with the more general term in layer 2 or even in Layer 1, if necessary.

Due to the fact that some detailed applications can belong to 2 or even more general applications, e.g. the detailed application GPS can be regarded as an aeronautical, maritime or military application, they may show up several times in the list when presented in hierarchical mode. In alphabetical presentation mode each term in the list is only mentioned once.

The layer 1 term TRA-ECS is to be used under certain conditions; i.e. notably for EU Member States to be in line with common EU regulatory approaches or by CEPT administrations for frequency bands for which they find the term applicable. In such case and when more detailed information is available, administrations are urged to fill in information in layers 2 and 3 corresponding to the applications currently in use as listed for the layer 1 applications: e. g. Aeronautical, Broadcasting, Fixed, Land Mobile, and Maritime.

The abbreviations used in the list are described at the end of this annex.

| Layer 1 | Layer 2 | Layer 3 |
| --- | --- | --- |
| Aeronautical | Aeronautical communications | Aeronautical satcoms AGA communications (civil) SAR (communications) |
|  | Aeronautical navigation | ASDE Airborne doppler navigation aids Airborne weather radar Altimeters Beacons (aeronautical) DME ILS Loran C MLS SAR (navigation) VOR |
|  | Aeronautical surveillance | ADS ASDE Primary radar SSR |
|  | Aeronautical emergency | ELT |
|  | Aeronautical telemetry |  |
|  | Aeronautical telecommand |  |
|  | Aeronautical telemetry/telecommand |  |
|  | Satellite navigation systems | GALILEO GLONASS GPS |
| Broadcasting | Broadcasting (terrestrial) | AM sound analogue DRM DVB-T FM sound analogue MWS T-DAB TV analogue (terrestrial) |
|  | Broadcasting-satellite receivers | Satellite radio Satellite TV SIT/SUT |
|  | PMSE | In-ear monitor systems Cordless cameras Radio microphones SAP/SAB and ENG/OB SAP/SAB portable audio links SAP/SAB portable video links SAP/SAB airborne video links SAP/SAB engineering links SAP/SAB remote control SAP/SAB telecommand SAP/SAB P to P audio links SAP/SAB P to P video links SAP/SAB vehicular audio links SAP/SAB vehicular video links Talkback |
| Fixed | Point-to-Multipoint | MWS Scanning telemetry Subscriber access excluding MWS Unplanned, uncoordinated fixed links |
|  | Point-to-Point | Private fixed networks Public fixed networks SAP/SAB P to P audio links SAP/SAB P to P video links Unplanned, uncoordinated fixed links |
|  | BWA | BFWA |
|  | MFCN | IMT |
| Defence systems | Aeronautical military systems | AGA communications (military) IFF JTIDS/MIDS TACAN-DME |
|  | Land military systems | Fixed radio relay (military) Tactical mobile Tactical radio relay |
|  | Maritime military systems | Sonobuoy |
|  | Meteorological aids (military) |  |
|  | Radiolocation (military) | Air-defence radar Tactical radar |
|  | Satellite systems (military) | Earth exploration-satellite (military) GLONASS GPS Satellite communications (military) |
|  | Telemetry (military) |  |
|  | Telecommand (military) |  |
|  | Telemetry/Telecommand (military) |  |
| Land mobile | Digital cellular | AES GSM GSM-R IMT MCA MCV |
|  | BWA |  |
|  | ITS |  |
|  | Analogue cellular | NMT |
|  | Cordless telephones | CT0 CT1 CT1+ CT2 DECT |
|  | D-GPS |  |
|  | PPDR | BBDR Emergency services PLB LAES |
|  | Inland waterway communications |  |
|  | MFCN | IMT |
|  | Paging | On-site paging POCSAG Talkback pocket unit Wide area paging |
|  | PMR/PAMR | PAMR PMR PMR 446 TETRA TETRAPOL |
|  | PMSE | In-ear monitor systems Cordless cameras Radio microphones SAP/SAB and ENG/OB SAP/SAB airborne video links SAP/SAB engineering links SAP/SAB portable audio links SAP/SAB portable video links SAP/SAB P to P audio links SAP/SAB P to P video links SAP/SAB remote control SAP/SAB telecommand SAP/SAB vehicular audio links SAP/SAB vehicular video links Talkback |
|  | Telemetry (civil) | Scanning telemetry |
|  | Telemetry/Telecommand (civil) |  |
| Maritime | GMDSS | DSC EPIRBs INMARSAT C MSI NAVTEX SAR (communications) SAR (navigation) |
|  | Satellite navigation systems | GALILEO GLONASS GPS |
|  | Maritime communications | AIS INMARSAT Inland waterway communications On-board communications |
|  | Maritime navigation | Beacons (maritime) Inland waterway radar Loran C Maritime radar RTE SAR (navigation) |
| Meteorology | Oceanographic buoys |  |
|  | Sondes |  |
|  | Weather radar |  |
|  | Weather satellites |  |
|  | Wind profilers |  |
| Satellite systems (civil) | Aeronautical satcoms | INMARSAT |
|  | Amateur-satellite |  |
|  | Broadcasting-satellite receivers | Satellite radio Satellite TV SIT/SUT |
|  | Earth exploration-satellite | Active sensors (satellite) Passive sensors (satellite) Synthetic aperture radar Weather satellites |
|  | Feeder links |  |
|  | FSS Earth stations | AES ESV HEST LEST SIT/SUT SNG VSA |
|  | Inter-satellite links |  |
|  | MSS Earth stations | CGC INMARSAT IMT-2000 satellite component S-PCS |
|  | Satellite navigation systems | GALILEO GLONASS GPS |
|  | Standard frequency and time signal-satellite |  |
|  | Space operations |  |
|  | Space research | Active sensors (satellite) Deep space (satellite) Passive sensors (satellite) |
| Radio astronomy | Continuum measurements Spectral line observations VLBI observations |  |
| Short Range Devices | Alarms | Social alarms |
|  | Railway applications | AVI Eurobalise Euroloop |
|  | Tracking, tracing and data acquisition | Animal tracking Asset tracking and tracing Detection of avalanche victims Meter reading |
|  | Radiodetermination applications | Detection of movement and alert GBSAR LPR TLPR |
|  | Inductive applications |  |
|  | Active medical implants | LP-AMI Medical implants Medical telemetry ULP-AID ULP-AMI ULP-MMI |
|  | Model control | Flying model control |
|  | Non-specific SRDs |  |
|  | Radio microphones and ALD | Aids for hearing impaired Consumer radio microphones Personal hearing aids Public hearing aids Radio microphones |
|  | Wideband data transmission systems | DECT Radio LANs |
|  | RFID |  |
|  | RTTT | SRR Vehicle and infrastructure radar |
|  | UWB applications | Automotive SRR BMA Communication applications GPR/WPR Material Sensing SRR LT2 LAES |
|  | Wireless audio applications | Baby monitoring Band II LPD Cordless headphones and loudspeakers Narrow band analogue voice devices |
| TRA-ECS |  |  |
| Other | Amateur |  |
|  | CB radio | DSB/SSB AM CB / CEPT PR 27 |
|  | GNSS Repeater |  |
|  | GNSS Pseudolites |  |
|  | HAPS |  |
|  | ISM | Microwave ovens |
|  | Meteor scatter communications |  |
|  | Land radionavigation |  |
|  | Radiolocation (civil) |  |
|  | Standard frequency and time signal |  |
|  | Tracking systems |  |

LIST OF SEARCHABLE APPLICATIONS IN ALPHABETIC ORDER

| List of searchable applications: | Layer | Comment: (in case of addition of new term, term deleted, indicating reason for change of term etc.) |
| --- | --- | --- |
| Active medical implants | 2 |  |
| Active sensors (satellite) | 3 |  |
| ADS | 3 |  |
| Aeronautical | 1 |  |
| Aeronautical communications | 2 |  |
| Aeronautical emergency | 2 |  |
| Aeronautical military systems | 2 |  |
| Aeronautical navigation | 2 |  |
| Aeronautical satcoms | 3(2) |  |
| Aeronautical surveillance | 2 |  |
| Aeronautical telecommand | 2 |  |
| Aeronautical telemetry | 2 |  |
| Aeronautical telemetry/telecommand | 2 |  |
| AES | 3 |  |
| AGA communications (civil) | 3 |  |
| AGA communications (military) | 3 |  |
| Aids for hearing impaired | 3 |  |
| Airborne doppler navigation aids | 3 |  |
| Airborne weather radar | 3 |  |
| Air-defence radar | 3 |  |
| AIS | 3 |  |
| Alarms | 2 |  |
| Altimeters | 3 |  |
| AM sound analogue | 1 |  |
| Amateur | 2 |  |
| Amateur-satellite | 2 |  |
| Analogue cellular | 2 |  |
| Animal tracking | 3 |  |
| ASDE | 3 |  |
| Asset tracking and tracing | 3 |  |
| Automotive SRR | 3 |  |
| AVI | 3 |  |
| Baby monitoring | 3 |  |
| Band II LPD | 3 |  |
| BBDR | 3 |  |
| Beacons (aeronautical) | 3 |  |
| Beacons (maritime) | 3 |  |
| BFWA | 3 |  |
| Broadcasting | 1 |  |
| Broadcasting (terrestrial) | 2 |  |
| Broadcasting-satellite receivers | 2 |  |
| BMA | 3 |  |
| BWA | 2 |  |
| CB radio | 2 |  |
| CEPT PR 27 | 3 |  |
| CGC | 3 |  |
| Communication applications | 3 |  |
| Consumer radio microphones | 3 |  |
| Continuum measurements | 2 |  |
| Cordless cameras | 3 |  |
| Cordless headphones and loudspeakers | 3 |  |
| Cordless telephones | 2 |  |
| CT0 | 3 |  |
| CT1 | 3 |  |
| CT1+ | 3 |  |
| CT2 | 3 |  |
| DECT | 3 |  |
| Deep space (satellite) | 3 |  |
| Defence systems | 1 |  |
| Detection of avalanche victims | 3 |  |
| Detection of movement and alert | 3 |  |
| D-GPS | 2 |  |
| Digital cellular | 2 |  |
| DME | 3 |  |
| DRM | 3 |  |
| DSB/SSB AM CB | 3 |  |
| DSC | 3 |  |
| DVB-T | 3 |  |
| Earth exploration-satellite | 2 |  |
| Earth exploration-satellite (military) | 3 |  |
| ELT | 3 |  |
| Emergency services | 3 | Deletion at next update, term PPDR should be used |
| EPIRBs | 3 |  |
| ESV | 3 |  |
| Eurobalise | 3 |  |
| Euroloop | 3 |  |
| Feeder links | 2 |  |
| Fixed | 1 |  |
| Fixed radio relay (military) | 3 |  |
| Flying model control | 3 |  |
| FM sound analogue | 3 |  |
| FSS Earth stations | 2 |  |
| GALILEO | 3 |  |
| GBSAR | 3 |  |
| GLONASS | 3 |  |
| GMDSS | 2 |  |
| GNSS Pseudolites | 2 | Used in ECC/REC/(11)08 |
| GPS | 3 |  |
| GPR/WPR | 3 |  |
| GSM | 3 |  |
| GSM-R | 3 |  |
| HAPS | 2 |  |
| HEST | 3 |  |
| IFF | 3 |  |
| ILS | 3 |  |
| IMT-2000 satellite component | 3 |  |
| IMT | 3 |  |
| Inductive applications | 2 |  |
| In-ear monitor systems | 3 |  |
| Inland waterway communications | 2(3) |  |
| Inland waterway radar | 3 |  |
| INMARSAT | 3 |  |
| INMARSAT C |  |  |
| Inter-satellite links | 2 |  |
| ISM | 2 |  |
| ITS | 2 |  |
| JTIDS/MIDS | 3 |  |
| Land military systems | 2 |  |
| LAES | 3 | Used in ECC/REC/(11)10 |
| Land mobile | 1 |  |
| Land radionavigation | 3 |  |
| LEST | 3 |  |
| Loran C | 3 |  |
| LP-AMI | 3 |  |
| LPR | 3 | Used in ECC/DEC/(11)02 |
| LT2 | 3 | Used in ECC/REC/(11)09 |
| Maritime | 1 |  |
| Maritime communications | 2 |  |
| Maritime military systems | 2 |  |
| Maritime navigation | 2 |  |
| Maritime radar | 3 |  |
| Material Sensing | 3 |  |
| MCA | 3 |  |
| MCV | 3 |  |
| Medical implants | 3 |  |
| Medical telemetry | 3 |  |
| Meteor scatter communications | 2 |  |
| Meteorological aids (military) | 2 |  |
| Meteorology | 1 |  |
| Meter reading | 3 |  |
| MFCN | 2 |  |
| Microwave ovens | 3 |  |
| MLS | 3 |  |
| Model control | 2 |  |
| MSI | 3 |  |
| MSS Earth stations | 2 |  |
| MWS | 3 |  |
| Narrow band analogue voice devices | 3 |  |
| NAVTEX | 3 |  |
| NMT | 3 |  |
| Non-specific SRDs | 2 |  |
| Oceanographic buoys | 2 |  |
| On-board communications | 3 |  |
| On-site paging | 3 |  |
| Other | 1 |  |
| Paging | 2 |  |
| PAMR | 3 |  |
| Passive sensors (satellite) | 3 |  |
| Personal hearing aids | 3 |  |
| PLB | 3 |  |
| PMR | 3 |  |
| PMR 446 | 3 |  |
| PMR/PAMR | 2 |  |
| PMSE | 2 |  |
| POCSAG | 3 |  |
| Point-to-Multipoint | 2 |  |
| Point-to-Point | 2 |  |
| PPDR | 2 |  |
| Primary radar | 3 |  |
| Private fixed networks | 3 |  |
| Public fixed networks | 3 |  |
| Public hearing aids | 3 |  |
| Radio astronomy | 1 |  |
| Radio LANs | 3 |  |
| Radio microphones | 3 |  |
| Radio microphones and ALD | 2 |  |
| Radiodetermination applications | 2 |  |
| Radiolocation (civil) | 2 |  |
| Radiolocation (military) | 2 |  |
| Railway applications | 2 |  |
| RFID | 2 |  |
| RTE | 3 |  |
| RTTT | 2 |  |
| SAP/SAB airborne video links | 3 |  |
| SAP/SAB and ENG/OB | 3 |  |
| SAP/SAB engineering links | 3 |  |
| SAP/SAB P to P audio links | 3 |  |
| SAP/SAB P to P video links | 3 |  |
| SAP/SAB portable audio links | 3 |  |
| SAP/SAB portable video links | 3 |  |
| SAP/SAB remote control | 3 |  |
| SAP/SAB telecommand | 3 |  |
| SAP/SAB vehicular audio links | 3 |  |
| SAP/SAB vehicular video links | 3 |  |
| SAR (communications) | 3 |  |
| SAR (navigation) | 3 |  |
| Satellite communications (military) | 3 |  |
| Satellite navigation systems | 2 |  |
| Satellite radio | 3 |  |
| Satellite systems (civil) | 1 |  |
| Satellite systems (military) | 2 |  |
| Satellite TV | 3 |  |
| Scanning telemetry | 3 |  |
| Short Range Devices | 1 |  |
| SIT/SUT | 3 |  |
| SNG | 3 |  |
| Social alarms | 3 |  |
| Sondes | 2 |  |
| Sonobuoy | 3 |  |
| Space operations | 2 |  |
| Space research | 2 |  |
| S-PCS | 3 |  |
| Spectral line observations | 2 |  |
| SRR | 3 |  |
| SSR | 3 |  |
| Standard frequency and time signal | 2 |  |
| Standard frequency and time signal-satellite | 2 |  |
| Subscriber access excluding MWS | 3 |  |
| Synthetic aperture radar | 3 |  |
| TACAN-DME | 3 |  |
| Tactical mobile | 3 |  |
| Tactical radar | 3 |  |
| Tactical radio relay | 3 |  |
| Talkback | 3 |  |
| Talkback pocket unit | 3 |  |
| T-DAB | 3 |  |
| Telecommand (military) | 2 |  |
| Telemetry (civil) | 2 |  |
| Telemetry (military) | 2 |  |
| Telemetry/Telecommand (civil) | 2 |  |
| Telemetry/Telecommand (military) | 2 |  |
| TETRA | 3 |  |
| TETRAPOL | 3 |  |
| TLPR | 3 |  |
| Tracking systems | 2 |  |
| Tracking, tracing and data acquisition | 2 |  |
| TRA-ECS | 1 |  |
| TV analogue (terrestrial) | 3 |  |
| ULP-AID | 3 |  |
| ULP-AMI | 3 |  |
| ULP-MMI | 3 |  |
| Unplanned, uncoordinated fixed links | 3 |  |
| UWB applications | 2 |  |
| Vehicle and infrastructure radar | 3 |  |
| VLBI observations | 2 |  |
| VOR | 3 |  |
| VSAT | 3 |  |
| Weather radar | 2 |  |
| Weather satellites | 2(3) |  |
| Wide area paging | 3 |  |
| Wideband data transmission systems | 2 |  |
| Wind profilers | 2 |  |
| Wireless audio applications | 2 |  |

ABBREVIATIONS

| Abbreviations |  |
| --- | --- |
| ADS | Automatic Dependant Surveillance (Aeronautical) |
| AES | Aircraft Earth Station |
| AGA | Air-Ground-Air |
| AIS | Universal Shipborne Automatic Identification System |
| AM | Amplitude Modulation |
| ALD | Assistive Listening Devices |
| ASDE | Airport Surface Detection Equipment |
| AVI | Automatic Vehicle Identification |
| BBDR | Broad Band Disaster Relief |
| BFWA | Broadband Fixed Wireless Access |
| BWA | Broadband Wireless Access |
| CB | Citizen’s Band |
| CGC | Complementary Ground Component |
| CT | Cordless Telephone |
| DECT | Digital Enhanced Cordless Telecommunications |
| D-GPS | Differential Global Positioning System |
| DME | Distance Measuring Equipment |
| DRM | Digital Radio Mondiale |
| DSC | Digital Selective Calling |
| DVB-T | Digital Video Broadcasting -Terrestrial |
| ELT | Emergency locator transmitter |
| ENG/OB | Electronic News Gathering / Outside Broadcasting |
| EPIRBs | Emergency Position Indicating Radio Beacons |
| ESV | Earth Stations on-board Vessels |
| FM | Frequency Modulation |
| FSS | Fixed-Satellite Service |
| GALILEO | European Global Navigation Satellite System |
| GBSAR | Ground Based Synthetic Aperture Radar |
| GLONASS | Globalnaya Navigatsionnaya Sputnikovaya Sistema |
| GMDSS | Global Maritime Distress and Safety System |
| GNSS | Global Navigation Satellite System |
| GNSS Pseudolites | Global Navigation Satellite System Pseudolites |
| GPS | Global Positioning System |
| GPR | Ground Probing Radar |
| GSM | Global System for Mobile Communications |
| GSM-R | Global System for Mobile Communications on Railways |
| HAPS | High Altitude Platform Station |
| HEST | High e.i.r.p. Satellite Terminal |
| IFF | Identification Friend or Foe |
| ILS | Instrument Landing System |
| IMT-2000 | International Mobile Telecommunications-2000 |
| IMT-Advanced | Systems beyond IMT-2000 |
| IMT | International Mobile Telecommunications (includes IMT-2000 and IMT-Advanced |
| ISM | Industrial, Scientific and Medical Applications |
| ITS | Intelligent Transport Systems |
| JTIDS | Joint Tactical Information Distribution System |
| LAES | Location Application for Emergency Services |
| LANs | Local Area Networks |
| LEST | Low e.i.r.p. Satellite Terminal |
| LP-AMI | Low Power Medical Implants |
| LPD | Low Power Device |
| LPR | Level Probing Radar |
| LT2 | Location Tracking Type 2 |
| MCA | Mobile Communications on Board Aircraft |
| MCV | Mobile Communication Services on Board Vessels |
| MFCN | Mobile/Fixed Communications Networks |
| MIDS | Multifunctional Information Distribution System |
| MLS | Microwave Landing System |
| MSI | Maritime Safety Information |
| MSS | Mobile-Satellite Service |
| MWS | Multimedia Wireless System |
| NAVTEX | Narrow-band direct-printing telegraphy system for transmission of navigational and meteorological warnings and urgent information to ships |
| NMT | Nordic Mobile Telephone |
| PAMR | Public Access Mobile Radio |
| PLB | Personal Locator Beacon |
| PMR | Private (Professional) Mobile Radio |
| PMSE | Programme Making and Special Events |
| POCSAG | Post Office Code Standards Advisory Group |
| PPDR | Public Protection & Disaster Relief |
| RFID | Radio Frequency Identification |
| RTE | Radar Target Enhancer |
| RTTT | Road Transport and Traffic Telematics |
| SAB | Service Ancillary to Broadcasting |
| SAP | Service Ancillary to Programme making |
| SAR | Search and Rescue |
| SIT/SUT | Satellite Interactive Terminal / Satellite User Terminal |
| SNG | Satellite News Gathering |
| S-PCS | Satellite - Personal Communications System |
| SRDs | Short Range Devices |
| SRR | Short Range Radars |
| SSR | Secondary Surveillance Radar |
| TACAN | Tactical Air Navigation |
| T-DAB | Terrestrial Digital Audio Broadcasting |
| TETRA | Terrestrial Trunked Radio |
| TETRAPOL | Digital PMR technology |
| TLPR | Tank Level Probing Radar |
| TRA-ECS | Terrestrial radio applications capable of providing electronic communications services |
| TV | Television |
| ULP-AID | Ultra Low Power Animal Implant Devices |
| ULP-AMI | Ultra Low Power Active Medical Implants |
| ULP-MMI | Ultra Low Power Medical Membrane Implants |
| VLBI | Very Long Baseline Interferometry |
| VOR | VHF Omnidirectional Radio Range |
| VSAT | Very Small Aperture Terminal |
| UWB | Ultra Wide Band |
| WPR | Wall Probing Radar |

searchable radio interface specification

Explanatory Note

The list of parameters for radio interfaces in EFIS is based on the template and the guide developed by TCAM RIG II and adopted by TCAM.

The use of these parameters for entering radio interface information into EFIS allows an efficient and meaningful comparison of interfaces within Europe.

The parameters are divided into normative and an informative parts.

The normative part consists of the following parameters:

- Frequency band

- Country

- Application (ref Annex 2 of this Decision)

- Radiocommunication service (Ref Annex 1 of this Decision)

- Channelling

- Modulation/occupied bandwidth

- Transmit power limit

- Channel access and occupation rules

- Direction/separation

- Authorisation regime

- Additional Article 3.3 requirements

- Frequency planning assumptions

The informative part consists of the following:

- Planned changes

- Reference

- Remarks

- Notification

- .

standard for right of use information

Explanatory Note

At its 15th meeting (20 April 2006), the EFIS Maintenance Group concluded that providing information on the right of use was important for frequency bands of high economic interest where market mechanisms would apply, and that it was important to define a standard for the information required.

The EFIS MG decided to define the standard for right of use information in EFIS as follows:

- Lower frequency  (numeric, EFIS format)

- Upper frequency   (numeric, EFIS format)

- License holder name and contact details (free text format)

- Expiry date / duration of the license

- Information on location

  - National (tick box) or

  - Regional or local (free text field, link to national details) or

  - One transmitter (free text field, link to national details)

- Spectrum trading Yes/No (tick box)

EFIS harmonised interface

The EFIS Harmonised Interface can be used for uploading or downloading data related to spectrum allocations and spectrum applications only. No other data is included from the start. However, if EFIS is developed further other types of data (e.g. radio interface parameters) might be added at a later stage.

The following Harmonised Interface shall be used:

- The interface shall be an XML file;

- The XML file has a defined structure, called the EFIS XML Format, which is defined by the Document Type Definition (DTD) given below;

- The terms used for allocations and applications shall be taken from the List of Radio Services in the ITU RR (see Annex 1) and the List of Searchable Applications (see Annex 2);

- All frequencies shall be written in Hertz, i.e. not in kHz, MHz or GHz.

DTD defining the EFIS XML Format:

<?xml encoding="UTF-8"?>

<!-- This simple DTD defines the import/export interface for use with the EFIS system -->

<!-- Frequency values are to be specified in Hertz (1 - 999999999999 Hz) -->

<!-- Terms are at most 100 characters -->

<!-- TemporalRestrictions are at most 50 characters -->

<!ELEMENT frequencyInformation (frequencyTable*)>

<!ELEMENT frequencyTable (allocation | application | document | radioInterface | Rightofuseinfo)*>

<!ELEMENT allocation EMPTY>

<!ELEMENT application EMPTY>

<!ELEMENT document EMPTY>

<!ELEMENT radioInterface EMPTY>

<!ELEMENT Rightofuseinfo EMPTY>

<!ATTLIST frequencyTable

name ID #REQUIRED

>

<!ATTLIST allocation

lowerFrequency CDATA #REQUIRED

higherFrequency CDATA #REQUIRED

term CDATA #REQUIRED

status (primary | secondary) #REQUIRED

shortComments CDATA #IMPLIED

>

<!ATTLIST application

lowerFrequency CDATA #REQUIRED

higherFrequency CDATA #REQUIRED

term CDATA #REQUIRED

shortComments CDATA #IMPLIED

>

<!ATTLIST document

title CDATA #REQUIRED

lowerFrequency CDATA #IMPLIED

higherFrequency CDATA #IMPLIED

term CDATA #IMPLIED

type (activity | NTFA | other | R&amp;TTE ) #REQUIRED

expiry CDATA #REQUIRED

hyperlink CDATA #REQUIRED

>

<!ATTLIST radioInterface

radiointerfaceid CDATA #IMPLIED

lower_frequency CDATA #REQUIRED

higher_frequency CDATA #REQUIRED

Allocation_Term CDATA #IMPLIED

Application_Term CDATA #IMPLIED

Channeling CDATA #IMPLIED

TransmitPowerLimit CDATA #IMPLIED

ChannelOccupationRules CDATA #IMPLIED

DuplexDirection CDATA #IMPLIED

LicensingRegime CDATA #IMPLIED

Art33Requirements CDATA #IMPLIED

FrequencyPlanning CDATA #IMPLIED

Reference CDATA #IMPLIED

Remarks CDATA #IMPLIED

NotificationNo CDATA #IMPLIED

OccupiedBandwidth CDATA #IMPLIED

PlannedChanges CDATA #IMPLIED

Channeling_notes CDATA #IMPLIED

TransmitPowerLimit_notes CDATA #IMPLIED

ChannelOccupationRules_notes CDATA #IMPLIED

DuplexDirection_notes CDATA #IMPLIED

LicensingRegime_notes CDATA #IMPLIED

Art33Requirements_notes CDATA #IMPLIED

FrequencyPlanning_notes CDATA #IMPLIED

Reference_notes CDATA #IMPLIED

Remarks_notes CDATA #IMPLIED

NotificationNo_notes CDATA #IMPLIED

OccupiedBandwidth_notes CDATA #IMPLIED

PlannedChanges_notes CDATA #IMPLIED

FrequencyBand_notes  CDATA #IMPLIED
    	Allocation_notes  CDATA #IMPLIED
    	Application_notes  CDATA #IMPLIED
  >

<!ATTLIST Rightofuseinfo

Rightofuseinfono CDATA #IMPLIED

LowerFrequency CDATA #REQUIRED

HigherFrequency CDATA #REQUIRED

Application CDATA #IMPLIED

Company CDATA #IMPLIED

Surname CDATA #IMPLIED

Firstname CDATA #IMPLIED

town CDATA #IMPLIED

Address CDATA #IMPLIED

postalcode CDATA #IMPLIED

Faxno CDATA #IMPLIED

Telephoneno CDATA #IMPLIED

Email CDATA #IMPLIED

Website CDATA #IMPLIED

Country CDATA #IMPLIED

Expiry CDATA #IMPLIED

Tradable CDATA #IMPLIED

Nationalcoverage CDATA #IMPLIED

Localcoverage CDATA #IMPLIED

Onetransmitter CDATA #IMPLIED

LONGITUDE CDATA #IMPLIED

LATITUDE CDATA #IMPLIED

>

Subject to approval by the ECO Council