|  |  | FM PT44 | FM44(11)056 |  |
| --- | --- | --- | --- | --- |
| 16-18 November 2011, Pozzallo, Italy |  |  |
| Subject: | LS from WGSE to FM44 on the work carried out in support of a draft new ECC Decision covering Earth Stations on Mobile Platforms in the Ka-band |  |  |
| Date: | 16th November 2011 |  |  |
| Origin: | WGSE |  |  |

| WGSE/SE PT40 |  |  |  |
| --- | --- | --- | --- |
| To: Chairman FM 44 Mr. Benoist Deschamps Cc for action: Chairman TC SES Mr. Jean-Jacques Bloch |  |  |  |
|  |  | 16 November 2011 |

Liaison Statement on work carried out in support of a draft new ECC Decision covering Earth Stations on Mobile Platforms in the Ka-band

Dear Benoîst and Jean-Jacques,

At its meeting in Pozzallo in November 2011, SE 40 continued its work on FSS Earth Stations on Mobile Platforms (ESOMP) operating in the 17.3-30.0 GHz band.

Based on the contributions discussed within SE 40, SE 40 would like to report the following:

- With respect to cross-border sharing between aircraft-mounted ESOMPs and FS, SE 40 considered two different studies on the potential sharing issues between aircraft-mounted ESOMPs and FS in neighbouring countries. These studies take into account updated parameters for FS systems provided by some administrations as well as atmospheric attenuation which was not considered previously. It appears that cross-border compatibility between such aircraft-mounted ESOMPs and FS could be achieved under certain technical conditions. SE40 is currently working on a pfd threshold mask that could be used to trigger coordination between administrations operating ESOMPs and administrations operating FS in the band 28.8365-28.9485 GHz. The same approach could be considered when addressing sharing between aircraft-mounted ESOMPs and FS receivers in any part of the band 27.5-29.5 GHz. Further studies are however needed for the definition of this mask and SE40 plans to work by correspondence on this before its next meeting in March 2012.

- With respect to the request of FM 44 to monitor the development of a Harmonised ETSI standard for ESOMP operations in the 27.0-31.0 GHz band, SE 40 would kindly request the TC SES chairman to provide the latest version of the draft standard as soon as it is considered matured. It should be noted that SE40 was informed during the meeting that the EIRP mask in the current version of the draft ESOMPs standard appears to be relaxed compared to the existing SUT standard for the same frequency band which was used as a basis for SE40 studies with ESOMPs. If confirmed, the assumptions taken in SE40 studies might need to be updated.

- SE40 took into account the request from FM 44 to study the potential impact from BSS feeder links (Earth-to-space) into FSS uncoordinated Earth stations in the band 17.3-17.7 GHz, however, in absence of contributions, SE 40 was unable to progress further on this issue.

Sincerely yours,

Jean-Yves Guyomard

SE PT40 Chairman

Email:	 guyomard@anfr.fr