| WGSE/SE PT40 |  |  | FM44(12)016 |
| --- | --- | --- | --- |
| To: Chairman FM 44 Mr. Benoist Deschamps |  |  |  |
|  |  | 13 March 2012 |

Liaison Statement on work carried out in support of a draft new ECC Decision covering Earth Stations on Mobile Platforms in the Ka-band

Dear Benoîst,

At its meeting in London in March 2012, SE 40 continued its work on FSS Earth Stations on Mobile Platforms (ESOMP) operating in the 17.3-30.0 GHz band.

Based on the contributions discussed within SE 40, we would like to report the following:

- With respect to the impact of ESOMPs on board vessels on FS stations deployed in a limited number of countries, SE40 would recommend to use a pfd threshold at the coastline, taking the same method and assumptions as in recommendation ITU-R SF.1650 dealing with ESV in the bands 5 925 – 6 425 MHz and 14 – 14.5 GHz. This would therefore still be based on coordination, in view of the limited number of countries using FS. This coordination would be triggered by a pfd threshold of – 109 dBW/m² expressed in a reference bandwidth of 14 MHz at a height of 20 m above sea level. In addition, the percentage of time that should be used in the propagation model when assessing compliance with this pfd threshold should be 0.007%.

- With respect to cross-border sharing between aircraft-mounted ESOMPs and FS, SE 40 considered two additionnal studies on the potential sharing issues between aircraft-mounted ESOMPs and FS in neighbouring countries, taking into account information received from SE19 as well as the answers to the FM44 questionnaire. SE40 is developing a pfd threshold mask that could be used to trigger coordination between administrations operating ESOMPs and administrations operating FS in the band 28 GHz, but require more time to conclude on this issue

- SE40 has not received any contribution on the impact from BSS feeder links (Earth-to-space) into FSS uncoordinated Earth stations in the band 17.3-17.7 GHz, and was unable to progress further on this issue.

- The draft ETSI standard on ESOMPs has been provided to SE40 as an information document but was not discussed at this meeting.

Sincerely yours,

Jean-Yves Guyomard

SE PT40 Chairman

Email:	 guyomard@anfr.fr