|  |  | FM PT44 | FM44(11)040 |  |
| --- | --- | --- | --- | --- |
| 6-8 July, Biel, Switzerland |  |  |
| Subject: | Liaison Statement on work carried out in support of a draft new ECC Decision covering Earth Stations on Mobile Platforms in the Ka-band |  |  |
| Date: | 6th July 2011 |  |  |
| Origin: | SE40 |  |  |

| WGSE/SE PT40 |  |  |  |
| --- | --- | --- | --- |
| To: Chairman FM 44 Mr. Benoist Deschamps Cc for action: Chairman TC SES Mr. Jean-Jacques Bloch |  |  |  |
|  |  | 6 July 2011 |

Liaison Statement on work carried out in support of a draft new ECC Decision covering Earth Stations on Mobile Platforms in the Ka-band

Dear Benoist,

At its meeting in Biel/Bienne in July 2011, SE 40 continued its work on FSS Earth Stations on Mobile Platforms (ESOMP) operating in the 17.3-30.0 GHz band.

Based on the contributions discussed within SE 40, SE 40 would like to report the following:

- With respect to cross-border sharing between aircraft-mounted ESOMPs and FS . SE 40 considered a study on the potential sharing issues between aircraft-mounted ESOMPs and FS in neighboring countries.  The following technical additional issues will be considered by SE40 in future studies:

  - Atmospheric attenuation: the study assumed free-space propagation.  A more realistic scenario should take into account atmospheric attenuation.

  - Power control: the study did not take into account power control.  Use of ATPC is mandatory for FS systems in most countries implementing FS links in the band 17.7-19.7 GHz, as described by decision ERC/DEC/(00)07.  The use of ATPC will impact the interference scenario.

  - Consideration of Worst-Case Scenario: the parameters of FS stations used in the study (coming from Rec F.758-4) are typical of FS deployments. However, further studies shall be made using worst-case values. FM44 may encourage administrations to provide other parameters that differ from the ones used in the study before the next SE40 meeting (November 2011).

  - Other mitigating factors that may limit the number of cases in which cross-border sharing may be an issue.

In addition, SE 40 discussed other issues which could be developed with further studies:

- Results of FM 44 survey of administrations’ use of FS in the 28.8365-28.9485 GHz band.  SE 40 would like to be informed of the results of the survey in this band so that this information can be taken into account in its future work.

- PFD threshold values on aircraft-mounted earth stations could be used as a possible trigger for starting a process of cross-border coordination.

- With respect to the request of FM 44 to monitor the development of a Harmonised ETSI standard for ESOMP operations in the 27.0-31.0 GHz band, SE 40 will liaise with ETSI TC SES on this issue and monitor the development of the new EN.

- Regarding the potential impact from BSS feeder links (Earth-to-space) into FSS ESOMP (space-to-Earth), when for example an aircraft mounted ESOMP flies through a BSS feeder link beam, SE 40 agrees that FSS systems would have to deal with any potential interference from BSS feeder links and shall not put any constraints on BSS feeder links.

Sincerely yours,

Jean-Yves Guyomard

SE PT40 Chairman

Email:	 guyomard@anfr.fr

Attachment :