Remarks:

This Draft ECC Decision was adopted for public consultation by ECC #29 in June 2011. The public consultation took place from 29 June - 26 August 2011. The comments, submitted to the ECO, will be discussed during the ECC PT 1 meeting from 12 - 14 September 2011.

Cover letter to

Draft new ECC/DEC/(11)HH on

harmonised frequency arrangements for

mobile/fixed communications networks (MFCN) (including IMT)

operating in the bands 3400-3600 MHz and 3600-3800 MHz

At its latest meeting, ECC approved the draft ECC/DEC/(11)HH for public consultation. This draft ECC Decision is providing a framework intended to facilitate high data rate IMT services supported by larger channel bandwidths as an evolution to the existing framework without the consequential requirement to replace systems that are based on the existing regulatory framework.

The respondents should consider in particular the proposed frequency arrangements for the band 3400-3600 MHz and that there are currently two options, A and B, for Decides 2) of this draft ECC Decision.

CEPT administrations as well as any other interested parties are invited to provide comments on the draft ECC Decision and, in particular, on their preferred option for the wording of Decides 2).

These comments should be forwarded to the European Communications Office: M. Alexander Gulyaev (alexander.gulyaev@eco.cept.org) not later than the deadline indicated in the ECO web page.

ECC Decision (11)HH

ELECTRONIC COMMUNICATIONS COMMITTEE

Harmonised frequency arrangements

for mobile/fixed communications networks (MFCN) (including IMT)

operating in the bands

3400 - 3600 MHz and 3600- 3800 MHz

EXPLANATORY MEMORANDUM

# INTRODUCTION

Currently, the following EC and CEPT regulatory framework is in force for broadband and fixed wireless access systems (BWA/FWA) in the 3400-3800 MHz band:

- The EC Decision 2008/411/ EC, on the harmonisation of the 3400-3800 MHz frequency band for terrestrial systems capable of providing electronic communications services in the Community. This EC Decision is based on conclusions of CEPT report 15 drafted on the basis of ECC/DEC/(07)02 on the availability of frequency bands between 3400 – 3800 MHz for the harmonised implementation of Broadband Wireless Access systems and recommendation ECC/REC/(04)05 “guidelines for accommodation and assignment of Multipoint Fixed Wireless systems in frequency bands 3400-3600 MHz and 3600-3800 MHz”. The least restrictive technical conditions are based on Block Edge Masks defined (BEM) by ECC/REC/(04)05;

- The ECC/DEC/(07)02, on availability of frequency bands between 3400-3800 MHz for the harmonised implementation of Broadband Wireless Access systems (BWA);

- The ECC/REC/(04)05, that offers guidelines for accommodation and assignment of multipoint fixed wireless systems in the frequency bands 3400-3600 MHz and 3600-3800 MHz.

The harmonised frequency arrangements for the 3400-3800 MHz band in this ECC Decision are intended to facilitate high data rate International Mobile Telecommunications (IMT) services supported by larger channel bandwidths as an evolution to the existing framework without the consequential requirement for a replacement of systems based on the existing regulatory framework. It aims at providing the basis to the mobile industry and administrations to respond to the growth of mobile broadband and technological developments for wider channel bandwidths and increased data rates.

Since WRC-07, the 3400-3600 MHz band is allocated on a primary basis to the mobile, except aeronautical mobile, service in a number of Region 1 countries and is identified for IMT.

The term IMT covers IMT-2000 and IMT-Advanced systems.  A wide range of systems are defined: 6 IMT-2000 radio interfaces and 2 IMT-Advanced radio interfaces ensuring a competitive environment.

Recommendation ITU-R M.1036 (on frequency arrangements for implementation of the terrestrial component of IMT) will be revised to include, amongst others, the arrangement(s) for the 3400-3600 MHz band.

In parallel, the IMT-Advanced process is on-going in ITU-R, in cooperation with standardisation organisations.

ERO carried out a survey in 2008 which found diverse implementation of BWA/FWA within 3400-3800 MHz in CEPT countries, including some IMT systems. This is reflected in various licensing coverages (national, regional), various frequency blocks choices (different portion of the 3400-3800 MHz). Moreover, the paired blocks are used in TDD mode.

In so far as is practicable, this frequency arrangement is intended to be technology neutral and capable of facilitating competitive provision of services using a range of technologies and modes (fixed, nomadic and mobile) with sufficient flexibility to accommodate current wireless broadband services deployed in the band.

# BACKGROUND

At the CEPT level, the regulatory framework in force is as follows:

- ECC Decision (07)02, on the availability of frequency bands between 3400-3800 MHz for the harmonised implementation of Broadband Wireless Access systems (BWA). This Decision refers to ECC Recommendation (04) 05 for frequency arrangements

- ECC Recommendation (04)05 providing “Guidelines for accommodation and assignment of multipoint fixed wireless systems in frequency bands 3.4-3.6 GHz and 3.6-3.8 GHz” and gives some examples of frequency arrangements that were initially developed for fixed and nomadic BWA applications. The indicated block edge masks were developed to ensure coexistence between PMP FWS applications.

Following to an EC Mandate on BWA, CEPT concluded (see CEPT Report 15) that the deployment of fixed, nomadic and mobile networks is technically feasible within the 3 400-3 800 MHz frequency band under the technical conditions described in ECC Decision (07)02 and ECC Recommendation (04)05.

Following another Commission Mandate on WAPECS, CEPT included least restrictive technical conditions for 3400-3800 MHz in CEPT Report 19.

The results are reflected in the EC Decision 2008/411/EC on the harmonization of the 3400-3800 MHz frequency band for terrestrial systems capable of providing electronic communications services in the Community, which is implemented in EU Member States.

In this Decision, CEPT took into account the two possible Frequency Division Duplex (FDD) and Time Division Duplex (TDD) duplex modes. In the case of a TDD operation, it is beneficial to synchronise the TDD networks of different operators to avoid restricted blocks / guard bands between operators and therefore facilitates an efficient   use of spectrum.

CEPT took into account existing CEPT results on coexistence with other services and the potential impact on these services, such as FSS usage, in these bands.

The implementation of this ECC Decision will encompass different stages at the national level (e.g. national consultation processes and update of existing authorisations as required) with a varying complexity depending on the legal and regulatory framework of each country.

# REQUIREMENT FOR AN ECC DECISION

The ECC recognises that implementation of IMT systems in the band 3400-3800 MHz based on a harmonised frequency arrangement will maximise the opportunities and benefits for end users and society, will benefit capital expenditure for operators, reduce development and implementation costs of manufacturing equipment and will secure future long terms investments by providing relevant economies of scale. A harmonised frequency arrangement will reduce complexity in cross border coordination. The opportunity to utilize larger channel widths in the 3400-3800 MHz band will assist the provision of high data rates for IMT (especially with IMT-Advanced).

The ECC recognises that for IMT to continue to develop successfully, the regulatory framework needs to provide the confidence and certainty to make the necessary investment. ECC recognises that administrations need flexibility to adapt their use of the bands 3400-3600 / 3600-3800 MHz to national circumstances. Such national measures may need to be studied (e.g. refarming of the band, planning of renewal or extension of authorisations etc). Moreover, the framework defined by this ECC Decision does not supersede the BWA/FWA framework. Instead, it aims at supplementing this framework to facilitate high data rate services supported by larger channel bandwidths as an evolution to the existing framework without the consequential requirement to replace systems that are based on the existing regulatory framework.

ECC Decision

of DD MM 2011

on harmonised frequency arrangements

for mobile/fixed communications networks (MFCN) (including IMT)

operating in the bands 3400-3600 MHz and 3600-3800 MHz

(ECC/DEC/(11)HH)

“The European Conference of Postal and Telecommunications Administrations,

considering

- that WRC-07 allocated the band 3400-3600 MHz to the Mobile, except Aeronautical Mobile, Service on a primary basis in a number of countries in Region 1 subject to provisions of RR 5.430A;

- that RR 5.430A also identifies the 3400-3600 MHz band for IMT;

- that the 3400-3500 MHz and 3500-3600 MHz bands have been allocated to the mobile service and identified for IMT in some countries of Region 3 (RR 5.432A, 5.432B and 5.433A);

- that the 3500-3600 MHz band is allocated to the Mobile, except Aeronautical Mobile, Service on a primary basis in Region 2, and that the 3400-3500 MHz band is allocated on a primary basis to the Mobile, except Aeronautical Mobile, Service in some countries of Region 2 and to the Mobile Service on a secondary basis in the rest of Region 2;

- that the 3600-3800 MHz band is allocated to the Mobile Service in Region 1 on a secondary basis in the Radio Regulations and not identified for IMT;

- that the 3400-3800 MHz band is allocated to the Mobile Service on a primary basis in the European Table of Frequency Allocations (ERC Report 25);

- the European Commission Decision 2008/411/EC on ”The harmonisation of the 3400-3800 MHz frequency band for terrestrial systems capable of providing electronic communications services in the Community”;

- that “mobile/fixed communications networks” (MFCN) for the purpose of this Decision includes IMT and other communications networks in the mobile and fixed services;

- that IMT covers both IMT-2000 and IMT-Advanced, as defined in Resolution ITU-R 56 (Naming for International Mobile Telecommunications );

- that detailed specifications of IMT radio interfaces are described in ITU-R Recommendation M.1457 for IMT-2000 and a new ITU-R Recommendation [IMT.RSPEC] under development for IMT-Advanced;

- that a harmonised frequency arrangement facilitates economies of scale resulting in the availability of affordable equipment;

- that the designation of a frequency band for a specific application does not prevent the designation of the same frequency band for other applications;

- that the bands 3400-3600MHz and 3600-3800 MHz are allocated to the Fixed-Satellite Service (space-to-Earth) on a primary basis in the Radio Regulations and are used in some CEPT countries for that service;

- that in some CEPT countries the band 3400 MHz to 3410 MHz is used by land, airborne and naval military radars;

- that the use of the band 3400-3600 MHz and the band 3600-3800 MHz for Fixed Satellite Service (FSS) varies between these frequency bands. The band 3600-3800 MHz is used for FSS more heavily than the band 3400-3600 MHz;

- that there could be differences in the market demand for spectrum for MFCN, in different CEPT countries, which could lead to different timescales for the introduction of MFCN within the bands 3400- 3600 MHz and 3600-3800 MHz;

- that ECC Decision (07)02 designates spectrum “for BWA deployment within the band 3400-3600 MHz and/or 3600-3800 MHz, subject to market demand and with due consideration of other services deployed in these bands”;

- that ECC Recommendation (04)05 provides “guidelines for accommodation and assignment of multipoint fixed wireless systems in frequency bands 3.4 – 3.6 GHz and 3.6 – 3.8 GHz”;

- that in some CEPT countries the bands 3400-3600 MHz and/or 3600-3800 MHz are already used for BWA, FWA and IMT systems;

- that global roaming is facilitated by common frequency arrangements and measures for free circulation for IMT terminals;

- that wider channel bandwidths such as 10, 20 and 40 MHz that could be accommodated in the bands 3400-3600 MHz and 3600-3800 MHz will enable higher data rates;

- that spectrum should be assigned in multiples of 5MHz, except where this is not possible due to the presence of existing users;

- that measures might be needed to ensure coexistence between unsynchronized TDD networks in adjacent blocks (e.g. additional filtering, site coordination, restricted blocks/guardbands);

- that in case of TDD operation it may be beneficial to synchronise the TDD networks (frame timing and/or uplink/downlink timeslot ratio) of different network operators or add filtering to base stations, to improve the efficient usage of spectrum by avoiding restricted blocks/guardbands between their networks; however, aligning the uplink/downlink timeslot ratio requires agreement between the involved network operators and may thus reduce their freedom to adjust the uplink/downlink ratio to respond to traffic demand;

- that the synchronisation of TDD networks of different operators can be managed at national level (e.g. voluntary agreement between operators or national regulatory measures);

- that studies on sharing between IMT and the Fixed Satellite Service have been carried out by ITU-R, (see Report ITU-R M.2109);

- that TDD allows more efficient spectrum use when taking into account existing fixed satellite usage in case of geographical sharing;

- that in some CEPT countries, the deployment of networks may need a bilateral agreement concerning the use of stations in the mobile service in one country and stations of other primary services in another country (e.g. earth stations of the fixed satellite service) (see RR 5.430A for the band 3400-3600 MHz );

- that in EU/EFTA countries the radio equipment that is under the scope of this Decision shall comply with the R&TTE Directive; Conformity with the essential requirements of the R&TTE Directive may be demonstrated by compliance with the applicable harmonised European standard(s) or by using the other conformity assessment procedures set out in the R&TTE Directive;

- that there is planned a separate ECC Report covering measures to facilitate coexistence between adjacent TDD networks (e.g. synchronisation, additional filtering, site coordination, restricted blocks/guardbands);

- that the FDD frequency arrangement needs further specification work in order to define the potential for harmonised usage of the duplex gap;

- that sharing studies between FDD and TDD are necessary;

- that although there are licensed paired frequency arrangements in many CEPT countries, TDD systems are currently used in a number of those countries in the band 3400 – 3600 MHz due to the better availability of TDD systems;

- that TDD may allow more flexible accommodation of current use of the frequency bands by other services.

DECIDES

- that CEPT administrations shall designate the frequency bands 3400-3600 MHz and 3600-3800 MHz on a non-exclusive basis to mobile/fixed communications networks, without prejudice to the protection and continued operation of other existing users in these bands;

- [OPTION A:

  - that administrations wishing to implement MFCN (including IMT) in the 3400 – 3600 MHz band should follow the harmonised frequency arrangement given in Annex 1 (TDD) or implement the harmonised frequency arrangement (taking into account considering ee) above) given in Annex 2 (FDD); ]

[OPTION B:

  - that administrations wishing to implement MFCN (including IMT) in the 3400 – 3600 MHz band should adhere to the harmonised frequency arrangement given in Annex 1 (TDD);

  - that administrations wishing to implement MFCN (including IMT) in the 3400 – 3600 MHz band with frequency arrangements other than the harmonised arrangement in Annex 1 should follow Annex 2 (FDD);]

- that administrations wishing to implement MFCN (including IMT) in the 3600 – 3800 MHz band should adhere to the harmonised frequency arrangement given in Annex 3 (TDD);

- that administrations should consider facilitating the migration of existing terrestrial networks and authorisations to the frequency arrangements described in the Annexes;

- that this Decision enters into force on DD MM YYYY;

- that the preferred date for implementation of the Decision shall be DD MM YYYY;

- that CEPT administrations shall communicate the national measures implementing this Decision to the ECC Chairman and the Office when the Decision is nationally implemented.”

Note:

Please check the Office web site (http://www.cept.org/eco) for the up to date position on the implementation of this and other ECC Decisions.

ANNEX 1

Frequency arrangement for the 3400-3600 MHz band based on TDD

The frequency arrangement is a TDD arrangement, based on a block size of 5 MHz starting at the lower edge of 3400 MHz.

If blocks need to be offset to accommodate other users, the raster should be 100 kHz. Narrower blocks can be defined adjacent to other users, to allow full use of spectrum. It has to be noted that TDD in one extreme case also covers downlink only operation.

3400 MHz

3600 MHz

5

5

5

5

5

5

5

5

5

5

5

5

5

5

5

5

5

5

5

5

5

5

5

5

5

5

5

5

5

5

5

5

5

5

5

5

5

5

5

5

5

5

5

5

5

5

5

5

5

5

5

5

5

5

5

5

5

5

5

5

5

5

5

5

5

5

5

5

5

5

5

5

5

5

5

5

5

5

5

5

5

5

5

5

5

5

5

5

5

5

5

5

5

5

5

5

5

5

5

5

5

5

5

5

5

5

5

5

5

5

5

5

5

5

5

5

5

5

5

5

5

5

5

5

5

5

5

5

5

5

5

5

5

5

5

5

5

5

5

5

5

5

5

5

5

5

5

5

5

5

5

5

5

5

5

5

5

5

5

5

5

5

5

5

5

5

5

5

5

5

5

5

ANNEX 2

Frequency arrangement for the 3400-3600 MHz band based on FDD

The frequency arrangement is an FDD arrangement, based on a block size of 5 MHz starting at the lower edge of 3410 MHz. The sub-band 3410-3490 MHz is used for the uplink, the sub-band 3510-3590 MHz is used for the downlink. The resulting duplex gap is 20 MHz (3490-3510 MHz).

If blocks need to be offset to accommodate other uses, the raster should be 100 kHz. Narrower blocks can be defined adjacent to other users, to allow full use of spectrum.

ANNEX 3

Frequency arrangement for the 3600-3800 MHz band based on TDD

The frequency arrangement is a TDD arrangement, based on a block size of 5 MHz starting at the lower edge of 3600 MHz.

If blocks need to be offset to accommodate other uses, the raster should be 100 kHz. Narrower blocks can be defined adjacent to other users, to allow full use of spectrum. It has to be noted that TDD in one extreme case also covers downlink only operation.