Annex 4 of FM48(11)018 - Annex 1

Basic parameter assumptions for the DA2GC system proposed in the ETSI SRdoc (FDD mode; based on LTE technology)

| Parameter | Ground station | Aircraft station |
| --- | --- | --- |
| Channel bandwidth [MHz] | 10 | 10 |
| Signal bandwidth [MHz] (related to 50 Ressource Blocks à 180 kHz) | 9 | 9 |
| Maximum Tx power [dBm] | 46 | 40 |
| Typical Rx noise figure [dB] | 5 | 9 |
| Rx thermal noise level [dBm] | -99 | -95 |
| Rx reference sensitivity level [dBm] | -101.5 | -97 |
| Interference protection ratio I/N [dB] | -6 | -6 |
| Interference protection level [dBm] | -105 | -101 |
| Antenna model | 120° sector antenna (90° HPBW) | omni-directional |
| Antenna gain [dBi] | up to 17 | 0 to 5 |
| Antenna height [m] | about 50 | up to 13000 |
| Tx spectrum emission mask / spurious emissions | according to 3GPP TS 36.104 | under consideration (similar to Ground station) |
| Rx in-band / out-of-band blocking | according to 3GPP TS 36.104 | under consideration (similar to Ground station) |
| Rx adjacent channel selectivity (ACS) [dB] | according to 3GPP TS 36.104 | under consideration (similar to Ground station) |

Values in the table according to parameters given in ITU-R Report M.2039 for frequency sharing/interference analyses of IMT-2000 systems.