Annex 4 of FM48(11)018 - Annex 2

1) 2400 – 2483.5 MHz and 5855 – 5875 MHz: ISM bands

1.1)	2400 – 2483.5 MHz: ISM band

According to ERC/REC 70-03 the following limits and restrictions apply for the band 2400 – 2483.5 MHz concerning Short Range Devices:

| Application | Power | Spectrum access and mitigation requirement | Channel spacing | Notes |
| --- | --- | --- | --- | --- |
| Non-specific Short Range Devices | 10 mW e.i.r.p. | No requirement | No spacing | See also EC Decision 2006/771/EC amended by 2010/368/EU. |
| Wideband Data Transmission systems | 100 mW e.i.r.p. | The equipment shall implement an adequate spectrum sharing mechanism in order to facilitate sharing between the various technologies and applications | No spacing | For wide band modulations other than FHSS , the maximum e.i.r.p. density is limited to 10 mW/MHz. See also EC Decision 2006/771/EC amended by 2010/368/EU. |
| Railway applications | 500 mW e.i.r.p. | No requirement |  | Transmitting only in presence of trains. 5 channels, each 1.5 MHz wide within the band 2446-2454 MHz |
| Radio-determination applications | 25 mW e.i.r.p. | No requirement | No spacing | See also EC Decision 2006/771/EC amended by 2010/368/EU. |
| Radio frequency identification applications | ≤500 mW e.i.r.p. | No requirement | No spacing | Within the band 2446-2454 MHz. See also EC Decision 2006/771/EC amended by 2010/368/EU. |
|  | >500 mW up to 4 W e.i.r.p | ≤ 15% duty cycle FHSS techniques should be used | No spacing | Within the band 2446-2454 MHz. Power levels above 500 mW are restricted to be used inside the boundaries of a building and the duty cycle of all transmissions shall in this case be ≤15 % in any 200 ms period (30 ms on /170 ms off). In addition, for an RFID device which can exceed 500 mW, the device should be fitted with an automatic power control to reduce the radiated power below 500 mW; this automatic power control shall guarantee the reduction of the power to a maximum of 500 mW in cases where the device is moved and used outside the boundary of the user's building or premises as described above. |

Designations in the bands 2400 – 2450 MHz and 2450 – 2483.5 MHz
(Source: ERC/REC 70-03 as of 9 February 2011)

Furthermore, different states of implementation (not implemented, limited implementation, etc.) apply in different European countries.

Other Non-SRD-radio services and applications in the band 2400 - 2483.5 MHz:

MOBILE SERVICE:

- ENG-OB equipment (cordless cameras)

Secondary radio services:

- Amateur Service and

- Amateur Satellite Service in the band segment 2400 – 2450 MHz

- Radiolocation services: military radar

Compatibility of DA2GC with existing radio services in the band 2400 – 2483.5 MHz

The DA2GC reverse link (aircraft to ground) might be critical as all short range devices on the ground are potential victims for interference from DA2GC. For the DA2GC forward link (ground to aircraft), the receiver at the aircraft is receiving in-band signals from all the SRDs operating on the ground.

As RLAN systems in the same frequency band are also being used onboard aircraft, there is a potential of interference from DA2GC forward link (ground to aircraft). This could possibly be managed by channel separation as long as not all RLAN channels are needed on board aircraft.

A presentation on a system with alternative characteristics (different to those already provided in the ETSI SRDoc.) that has been trialed in the US in accordance with the current FCC limits for this band (US Regulation 47 CFR Part 15.247) was introduced to FM PT 48. In order to enable studies on this system more detailed technical information will be provided by Lufthansa Systems into the WG SE correspondance activities or to the relevant SE PT.

| RR Region 1 Allocation and RR footnotes applicable to CEPT | European Common Allocation | Major utilisation | ECC/ERC document | Standard | Notes |
| --- | --- | --- | --- | --- | --- |
| 2400 – 2450 MHz FIXED MOBILE Amateur Radiolocation 5.150 5.282 | FIXED MOBILE Amateur-satellite Radiolocation 5.150 5.282 EU2 | Amateur |  | EN 301 783 |  |
|  |  | Amateur Satellite |  | EN 301 783 |  |
|  |  | ISM |  |  |  |
|  |  | Non-Specific SRDs | ERC/REC 70-03 | EN 300 440 | Within the band 2400.0-2483.5 MHz. |
|  |  | Radiodetermina-tion applications | ERC/DEC/(01)08 ERC/REC 70-03 | EN 300 440 | Within the band 2400.0-2483.5 MHz. |
|  |  | Railway applications | ERC/REC 70-03 | EN 300 761 | Within the band 2446-2454 MHz for AVI applications. |
|  |  | RFID | ERC/REC 70-03 | EN 300 440 | Within the band 2446-2454 MHz. |
|  |  | Wideband Data Transmission Systems | ERC/DEC/(01)07 ERC/REC 70-03 | EN 300 328 | Within the band 2400.0-2483.5 MHz. |
| 2450 – 2483.5 MHz FIXED MOBILE Radiolocation 5.150 5.397 | FIXED MOBILE 5.150 EU2 | ISM |  |  |  |
|  |  | Non-Specific SRDs | ERC/REC 70-03 | EN 300 440 | Within the band 2400.0-2483.5 MHz. |
|  |  | Radiodetermina-tion applications | ERC/DEC/(01)08 ERC/REC 70-03 | EN 300 440 | Within the band 2400.0-2483.5 MHz. |
|  |  | Railway applications | ERC/REC 70-03 | EN 300 761 | Within the band 2446-2454 MHz for AVI applications. |
|  |  | RFID | ERC/REC 70-03 | EN 300 440 | Within the band 2446-2454 MHz. |
|  |  | Wideband Data Transmission Systems | ERC/DEC/(01)07 ERC/REC 70-03 | EN 300 328 | Within the band 2400.0-2483.5 MHz. |

| EU2 | Civil-military sharing. |
| --- | --- |
| 5.150 | The following bands: 13 553 - 13 567 kHz (centre frequency 13 560 kHz), 26 957 - 27 283 kHz (centre frequency 27 120 kHz), 40.66 - 40.70 MHz (centre frequency 40.68 MHz), 902 - 928 MHz in Region 2 (centre frequency 915 MHz), 2 400 - 2 500 MHz (centre frequency 2 450 MHz), 5 725 - 5 875 MHz (centre frequency 5 800 MHz), and 24 - 24.25 GHz (centre frequency 24.125 GHz) are also designated for industrial, scientific and medical (ISM) applications. Radiocommunication services operating within these bands must accept harmful interference which may be caused by these applications. ISM equipment operating in these bands is subject to the provisions of No. 15.13. |
| 5.282 | In the bands 435 - 438 MHz, 1 260 - 1 270 MHz, 2 400 - 2 450 MHz, 3 400 - 3 410 MHz (in Regions 2 and 3 only) and 5 650 - 5 670 MHz, the amateur-satellite service may operate subject to not causing harmful interference to other services operating in accordance with the Table (see No. 5.43). Administrations authorising such use shall ensure that any harmful interference caused by emissions from a station in the amateur-satellite service is immediately eliminated in accordance with the provisions of No. 25.11. The use of the bands 1 260 - 1 270 MHz and 5 650 - 5 670 MHz by the amateur-satellite service is limited to the Earth-to-space direction. |
| 5.397 | Different category of service: in France, the band 2 450 - 2 500 MHz is allocated on a primary basis to the radiolocation service (see No. 5.33). Such use is subject to agreement with administrations having services operating or planned to operate in accordance with the Table of Frequency Allocations which may be affected. |

Allocations and major utilisations in the bands 2400 – 2450 MHz and 2450 – 2483.5 MHz
(Source: ERC Report 25, Kyiv 2009)

1.2) 5855 – 5875 MHz: ISM band

In response to Resolution 734 (Rev.WRC-07) calling for sharing studies for spectrum identification of HAPS (High Altitude Platform Station) gateway links in the range from 5850 to 7075 MHz, the CEPT conducted compatibility studies between HAPS system and different other services. Some relevant interference scenarios are shown in the following figures.

HAPS Platform

Urban area coverage (UAC)

θ=30°

H=21 km

Radius=36 km

FWS P-P

Interference

HAPS gateway

D=42 km

Nadir

d=minimum separation distance

d

Interference modelling scenario HAPS gateway towards FWS P-P
(Source: ECC REPORT 156)

Interference from HAPS gateway (ground) station into FSS network satellite receiver
(Source: ECC REPORT 156)

HAPS Platform

Urban area coverage (UAC)

θ=30°

H=21 km

Radius=36 km

Interference

HAPS gateway

D=42 km

Nadir

d=minimum separation distance

d

Methodology HAPS gateway station vs. ITS
(Source: ECC REPORT 156)

Conclusions for the services in the band 5855 – 5875 MHz are given in the table below.

| Services and applications | HAPS as interferer system vs. other Services |
| --- | --- |
| FS | Aeronautical platform (downlink): In order to meet the FWS nominal long term interference criterion of -147.5 dBW/10 MHz taking into account apportionment considerations of the allowable interference into the FS , the maximum e.i.r.p. at HAPS airborne antenna output should be: - for 0° ≤ θ ≤ 20°, θ is the off-axis angle from the nadir e.i.r.p. mask should comply with a range between -0.5 dBW/10 MHz and 0 dBW/10 MHz; - for 20° < θ ≤ 43°, θ is the off-axis angle from the nadir e.i.r.p. mask should comply with a range between 0 dBW/10 MHz and 2.1 dBW/10 MHz; - for 43° < θ ≤ 60°, θ is the off-axis angle from the nadir e.i.r.p. mask should comply with a range between 2.1 dBW/10 MHz and 0.5 dBW/10 MHz. This mask relates to the e.i.r.p. that would be obtained assuming free-space loss. In order to meet the FWS nominal long term interference criterion of −147.5 dBW/10 MHz, an e.i.r.p. limit of −0.5 dBW/10 MHz for HAPS (downlink) is proposed which is invariant to an off-axis angle up to 60° from the nadir, which corresponds to a minimum elevation angle for the gateway station of 30°. Gateway link (uplink): Compatibility is achieved if minimum separation distances are defined between gateway station and FWS systems. - in clear sky conditions the minimum separation distance is 730 m whereas; - in rainy conditions this minimum distance increases to 1850 m. It is assumed that a minimum elevation angle for the HAPS gateway station is limited by 30°. |
| FSS | To protect the geostationary non-Plan FSS networks the maximum e.i.r.p. at HAPS (airborne or ground) depends on the number of the HAPS within service area of the FSS satellite. The identification of HAPS “uplink” channels is not recommended in the frequency band 6725-7025 MHz where there is FSS Plan allotments Appendix 30B RR. However HAPS downlink may be considered in the frequency band 6725-7025 MHz because there is low probability of interference from HAPS downlink into FSS Plan Appendix 30B allotments even for aggregate interference case. At the same time it should be noted that Existing systems of Appendix 30B RR, operating in the frequency band 6725-7025 MHz in accordance with Resolution 148 (Rev.WRC-07), and Additional systems are out of this study. Therefore the study results may not be applicable for these systems that are also a subject of the provisions of the FSS Plan Appendix 30B RR. There is low probability of interference from single HAPS uplink or downlink into non-GSO FSS space station receiver for MOLNIA-type systems. The quite big value of margin, when single entry case of HAPS uplink or downlink impact to non-GSO FSS space station receiver for MOLNIA-type systems is considered, gives opportunity to suppose, that there will not be interference from HAPS gateway links to non-GSO FSS space station receiver of MOLNIA-type systems when aggregate case is considered. |

(Source: ECC REPORT 156)

| Services and applications | HAPS as interferer system vs. other Services (cont.) |
| --- | --- |
| MS | Aeronautical platform (downlink): In order to meet the ITS nominal long term interference criterion of -106 dBm/MHz, the maximum e.i.r.p. at HAPS airborne antenna output should be: - e.i.r.p. = 12.6 dBm/MHz (or -7.4 dBW/10 MHz) for 0° ≤ θ ≤ 22°; - e.i.r.p. linearly increases from 12.6 dBm/MHz (or -7.4 dBW/10 MHz) to 16.2 dBm/MHz (or -3.8 dBW/10 MHz) for 22° < θ ≤ 60°. θ is the off-axis angle from the nadir. This mask relates to the e.i.r.p. that would be obtained assuming free-space loss. It is assumed that the maximum angle of the HAPS airborne antenna deviation from the nadir should be limited to 60 degrees corresponding to the UAC of the HAPS. Gateway link (uplink) : Compatibility is achieved if minimum separation distances are defined between gateway station and ITS systems. - in clear sky conditions the minimum separation distance is 320 m; - in rainy conditions this minimum distance is equal to 800 m. It is assumed that a minimum elevation angle for the HAPS gateway station is limited by 30°. |

(Source: ECC REPORT 156)

Compatibility of DA2GC with existing radio services in the band 5855 - 5875 MHz

The most critical interference scenario is obviously the interference from DA2GC into the FSS (E-S). It is expected that the adequate protection of FSS (E-S) might lead to undue constraints for the parameters of a DA2GC system, thus not allowing sufficient system performance for economic operation. This could be managed by frequency separation.

Further information on the 5.8 GHz is also provided by ECC Report 68 (June 2005) which had been developed by SE PT 38 before ECC Recommendation (06)04 on “Use of the band 5725-5875 MHz for Broadband Fixed Wireless Access (BFWA)” was developed. With regard to the sub-band 5855-5875 MHz the compatibility between BFWA and non-specific SRDs and between BFWA and FSS (Earth-to-space) was considered in ECC Report 68. Annex 3 of ECC REC (06)04 contains e.i.r.p. spectral density limits in the elevation plane to protect GSO satellite receivers in the FSS. The other compatibility aspects as described in ECC Report 68 were related to radio applications below 5855 MHz. Additionally CEPT Report 15 was developed in response to the Mandate from the European Commission on BWA, this Report also covers the most important compatibility aspects.

The characteristics of the FSS systems are described in section 5.4 of ECC Report 68. The methods and results of sharing studies between BFWA and geostationary satellite networks of the FSS are described in section 6.4 in this Report. It was shown in the studies for BFWA that the conditions to make sharing with FSS (Earth-to-space) feasible in the band 5850-5875 MHz are more restrictive for certain types of BFWA than below 5850 MHz. This also may lead to the assumption that the band 5855-5875 MHz is more suitable for the reverse link (link from the aircraft station to the ground station) of Broadband DA2GC.

It should also be noted that the frequency band 5855-5875 MHz has been designated in the meantime for non-safety ITS applications (by ECC Recommendation (08)01). However, it seems to be that ITS will mainly be implemented in the band 5875-5905 MHz, which has been designated for safety-related ITS applications by ECC Decision (08)01 and also by Commission Decision 2008/671/EC. ECC Report 101 details spectrum sharing studies between ITS systems within the frequency band 5855-5925 MHz and the other services and applications in this frequency range and identifies conditions for ITS applications that facilitate spectrum sharing. ECC Reports 109 and 110 supplement these studies.

Lufthansa Systems has indicated that the system with the alternative characteristics discussed above for the 2.4 GHz band is also capable of operating in the 5.8 GHz band. In order to enable studies on this system more detailed technical information will be provided by Lufthansa Systems into the WG SE correspondance activities or to the relevant SE PT.

| RR Region 1 Allocation and RR footnotes applicable to CEPT | European Common Allocation | Major utilisation | ECC/ERC document | Standard | Notes |
| --- | --- | --- | --- | --- | --- |
| 5850 – 5925 MHz FIXED FIXED-SATELLITE (E/S) MOBILE 5.150 | FIXED FIXED-SATELLITE (E/S) MOBILE 5.150 | BFWA | ECC/REC/(06)04 | EN 302 502 | Within the band 5725-5875 MHz. |
|  |  | FSS |  | EN 301 443 | Priority for civil networks. |
|  |  | ISM |  |  | Within the band 5725-5875 MHz. |
|  |  | ITS | ECC/DEC/(08)01 ECC/REC/(08)01 | EN 302 571 | Within the band 5875-5925 MHz. Within the band 5855-5875 MHz. |
|  |  | Non-Specific SRDs | ERC/REC 70-03 | EN 300 440 | Within the band 5725-5875 MHz. |
|  |  | Radiodetermina-tion applications | ERC/REC 70-03 | EN 302 372 | Within the band 4500-7000 MHz for TLPR application. |

| 5.150 | The following bands: 13 553 - 13 567 kHz (centre frequency 13 560 kHz), 26 957 - 27 283 kHz (centre frequency 27 120 kHz), 40.66 - 40.70 MHz (centre frequency 40.68 MHz), 902 - 928 MHz in Region 2 (centre frequency 915 MHz), 2 400 - 2 500 MHz (centre frequency 2 450 MHz), 5 725 - 5 875 MHz (centre frequency 5 800 MHz), and 24 - 24.25 GHz (centre frequency 24.125 GHz) are also designated for industrial, scientific and medical (ISM) applications. Radiocommunication services operating within these bands must accept harmful interference which may be caused by these applications. ISM equipment operating in these bands is subject to the provisions of No. 15.13. |
| --- | --- |

Allocations and major utilisations in the band 5850 – 5925 MHz 
(Source: ERC Report 25, Kyiv 2009)

2) 3400 – 3600 MHz: Broadband Wireless Access (BWA) bands

Commission Decision 2008/411/EC of 21 May 2008 aims at harmonising, without prejudice to the protection and continued operation of other existing use in this band, the conditions for the availability and efficient use of the 3400-3800 MHz band for terrestrial systems capable of providing electronic communications services. The EU Member States shall designate and make available, on a non-exclusive basis, the 3400-3600 MHz band and the 3600-3800 MHz band for terrestrial electronic communications networks.

Technical parameters called block edge mask (BEM) are an essential component of conditions necessary to ensure coexistence in the absence of bilateral or multilateral agreements between neighbouring BWA networks. Less stringent technical parameters than given in Commission Decision 2008/411/EC, if agreed among the operators of such networks, can also be used. Equipment operating in this band may also make use of e.i.r.p. limits other than those set out in Commission Decision 2008/411/EC provided that appropriate mitigation techniques are applied which comply with Directive 1999/5/EC and which offer at least an equivalent level of protection to that provided by these technical parameters.

block

Assigned

block

-59

-47

-6

PMAX

Adjacent block

Frequency offset

A

Definition (% of the size of the assigned block)

20 %

B

NB: The percentages given in the „Definition“ column refer to the smaller of adjacent blocks, if blocks are of unequal size.

35 %

TX Output Power Density

(dBm/MHz)

Limits for OUT-OF-BLOCK emissions (BEM for BWA central stations)
(Source: Commission Decision 2008/411/EC)

ECC Decision (ECC/DEC/(07)02) of 30 March 2007 on availability of frequency bands between 3400-3800 MHz for the harmonised implementation of Broadband Wireless Access systems (BWA).

“DECIDES

- that spectrum shall be designated for BWA deployment within the band 3400-3600 MHz and/or 3600-3800 MHz, subject to market demand and with due consideration of other services deployed in these bands;

- that administrations shall consider allowing flexible usage modes within authorised BWA deployments in the frequency bands identified in Decides 1, taking into account the considerations as described in the Annex;

- that for the deployment of BWA networks in the frequency bands identified in Decides 1, administrations shall take into account the in-band and adjacent band compatibility with other services/systems (e.g. FS, FSS, ENG/OB, etc) and as a result, coordination of the BWA CS with existing services/systems may be required in the concerned area;

- that this Decision enters into force on 30 March 2007;

- that the preferred date for implementation of this Decision shall be 01 July 2007;

- that CEPT administrations shall communicate the national measures implementing this Decision to the ECC chairman and the Office when the Decision is nationally implemented."

The generic technical conditions applicable to fixed and nomadic networks are described in Harmonised Standards EN 302 326-2 and EN 302 326-3, which also include definitions for a central station and a terminal station. The term central station may be considered equivalent to the term base station in the context of mobile cellular networks.

ECC Recommendation (04)05 (adopted in February 2006) provides “Guidelines for accommodation and assignment of multipoint fixed wireless systems in frequency bands 3400-3600 MHz and 3600-3800 MHz”.

ECC Report 100 presents studies of the compatibility between Broadband Wireless Access (BWA) systems in the frequency range 3400-3800 MHz and other existing systems/services. Those other existing systems/services considered in this study were:

- Point-to-point fixed links,

- ENG/OB systems, otherwise referred to as SAB/SAP,

- Fixed-satellite service (Space-to-Earth),

- Radiolocation (with a primary allocation below 3400 MHz and a secondary allocation above 3400 MHz).

The main conclusion of the report is that when deciding on deployment of BWA networks in subject bands, administrations need to take into account the situation regarding the use of the frequency band in the concerned area and that co-ordination with the existing users may be required.

Additionally CEPT Report 15 was developed by ECC in response to the Mandate from the European Commission on BWA. Section 3.5.1 of this Report shows the most important compatibility aspects. In conclusion, the two bands 3400-3600 MHz and 3600-3800 MHz have different sharing considerations due to the different services, other than BWA, utilising these bands.

The band 3400-3800 MHz is also part of the frequency range which has been designated for UWB applications.

ECC PT 1 is currently developing a new draft ECC Decision on a harmonised frequency arrangement for IMT systems operating the bands 3400-3600 MHz and 3600-3800 MHz. ECC PT1 agreed that the objective of the harmonised frequency arrangement in the 3400-3800 MHz band under development is intended to enable high data rate IMT services supported by larger channel bandwidths as an evolution to the existing framework without the need for automatic replacement of systems based on the current regulatory framework.

Compatibility of DA2GC with existing radio services in the band 3400 – 3600 MHz

The studies performed in ECC Report 100 show that coexistence between BWA and other terrestrial services (P-P fixed links, ENG/OB) requires a certain frequency separation. Therefore, it can be expected that frequency separation will also be necessary between DA2GC and existing terrestrial services.

Coexistence with the FSS (S-E) might be feasible by avoiding co-frequency operation as the density of deployment of FSS Earth stations is lower within the band 3400 – 3600 MHz compared to the band 3600 – 4200 MHz.

| RR Region 1 Allocation and RR footnotes applicable to CEPT | European Common Allocation | Major utilisation | ECC/ERC document | Standard | Notes |
| --- | --- | --- | --- | --- | --- |
| 3400 – 3500 MHz FIXED FIXED-SATELLITE (S/E) Mobile 5.430A Radiolocation 5.431 | FIXED FIXED-SATELLITE (S/E) MOBILE 5.430A Amateur Radiolocation EU17 | Amateur |  | EN 301 783 | EU 17 within the band 3400-3410 MHz. |
|  |  | BWA | ECC/DEC/(07)02 ECC/REC/(04)05 ERC/REC 14-03 | EN 302 217 EN 302 326 EN 302 326 | Within the band 3400-3800 MHz. |
|  |  | FSS |  | EN 301 443 |  |
|  |  | IMT |  |  | This band is planned for future mobile applications (IMT), based on the provisions of the Radio Regulations. |
|  |  | Mobile applications |  | EN 302 064 | For coordinated SAB/SAP applications for occasional use. In some countries the mobile service may be on secondary basis. |
|  |  | Radars |  |  | Upper limit for airborne radars is 3410 MHz. |
| 3500 – 3600 MHz FIXED FIXED-SATELLITE (S/E) Mobile 5.430A Radiolocation | FIXED FIXED-SATELLITE (S/E) MOBILE 5.430A | BWA | ECC/DEC/(07)02 ECC/REC/(04)05 ERC/REC 14-03 | EN 302 217 EN 302 326 EN 302 326 | Within the band 3400-3800 MHz. |
|  |  | FSS |  | EN 301 443 |  |
|  |  | IMT |  |  | This band is planned for future mobile applications (IMT), based on the provisions of the Radio Regulations. |
|  |  | Mobile applications |  | EN 302 064 | For coordinated SAB/SAP applications for occasional use. In some countries the mobile service may be on secondary basis. |

| EU17 | In the sub-bands 3400 - 3410 MHz, 5660 - 5670 MHz, 10.36 - 10.37 GHz, 10.45 - 10.46 GHz the amateur service operates on a secondary basis. In making assignments to other services, CEPT administrations are requested wherever possible to maintain these sub-bands in such a way as to facilitate the reception of amateur emissions with minimal power flux densities. |
| --- | --- |
| 5.430A | Different category of service: in Albania, Algeria, Germany, Andorra, Saudi Arabia, Austria, Azerbaijan, Bahrain, Belgium, Benin, Bosnia and Herzegovina, Botswana, Bulgaria, Burkina Faso, Cameroon, Cyprus, Vatican, Côte d'Ivoire, Croatia, Denmark, French Overseas Departments and Communities in Region 1, Egypt, Spain, Estonia, Finland, France, Gabon, Georgia, Greece, Guinea, Hungary, Ireland, Iceland, Israel, Italy, Jordan, Kuwait, Lesotho, Latvia, Macedonia, Liechtenstein, Lithuania, Malawi, Malta, Morocco, Mauritania, Moldova, Monaco, Mongolia, Montenegro, Mozambique, Namibia, Niger, Norway, Oman, Netherlands, Poland, Portugal, Qatar, Syria, Congo, Slovakia, Czech Rep., Romania, United Kingdom, San Marino, Senegal, Serbia, Sierra Leone, Slovenia, South Africa, Sweden, Switzerland, Swaziland, Togo, Chad, Tunisia, Turkey, Ukraine, Zambia and Zimbabwe, the band 3 400-3 600 MHz is allocated to the mobile, except aeronautical mobile, service on a primary basis subject to agreement obtained under No. 9.21 with other administrations and is identified for International Mobile Telecommunications (IMT). This identification does not preclude the use of this band by any application of the services to which it is allocated and does not establish priority in the Radio Regulations. At the stage of coordination the provisions of Nos. 9.17 and 9.18 also apply. Before an administration brings into use a (base or mobile) station of the mobile service in this band it shall ensure that the power flux-density (pfd) produced at 3 m above ground does not exceed −154.5 dBW/(m2 * 4 kHz) for more than 20 per cent of time at the border of the territory of any other administration. This limit may be exceeded on the territory of any country whose administration has so agreed. In order to ensure that the pfd limit at the border of the territory of any other administration is met, the calculations and verification shall be made, taking into account all relevant information, with the mutual agreement of both administrations (the administration responsible for the terrestrial station and the administration responsible for the earth station), with the assistance of the Bureau if so requested. In case of disagreement, the calculation and verification of the pfd shall be made by the Bureau, taking into account the information referred to above. Stations of the mobile service in the band 3 400-3 600 MHz shall not claim more protection from space stations than that provided in Table 21-4 of the Radio Regulations (Edition of 2004).This allocation is efective from 17 November 2010. (WRC-07) |
| 5.431 | Additional allocation: in Germany, Israel and the United Kingdom, the band 3 400 - 3 475 MHz is also allocated to the amateur service on a secondary basis. (WRC-03) |

Allocations and major utilisations in the bands 3400 – 3500 MHz and 3500 – 3600 MHz
(Source: ERC Report 25, Kyiv 2009)