Annex 5 of FM48(11)018rev1

# Candidate bands for Broadband DA2GC

# Category 1: Frequency bands already submitted to WG SE for compatibility/sharing studies

See section 6 of the Minutes (2nd meeting of FM PT 48).

# Category 2: Frequency bands to be considered next or in case the studies for the category 1 bands will not show positive results

| Frequency band | Existing allocation in the RR suitable for Broadband DA2GC (MS or AMS) ? | Current level of harmonisation ? | Would the bandwidth required (2 x 10 MHz FDD or 20 MHz TDD) fit in the band ? | Proposed or future alternative usage under consideration (within CEPT) ? | Actual usage of the band - national level - within CEPT (- worldwide) for future sharing studies ? | Actual usage of adjacent bands - national level - within CEPT for future compatibility studies ? | (Likely oppor-tunity cost) ? | Likely time scales (target date 2012) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2483.5 – 2500 MHz | yes | high | yes (ground to air) | AI 1.18 WRC-12 | MSS, ISM | RLANs, SRDs/RFIDs, ISM, ECS |  | possibly within |
| 5150 – 5170 MHz | yes (1) (see also 5.446C) | high | yes |  | FSS (feeder links), RLANs, BBDR, AMS (Flight Test Telemetry) | MLS, AM(R)S, RLANs, FSS (feeder links) (2) |  | within |

(1) The current AMS allocation would need to be modified to allow Broadband DA2GC

(2) Compatibility studies with MLS, AM(R)S, RLANs and FSS have been completed

# Category 3: Position on frequency bands still open/undecided

| Frequency band | Existing allocation in the RR suitable for Broadband DA2GC (MS or AMS) ? | Current level of harmonisation ? | Would the bandwidth required (2 x 10 MHz FDD or 20 MHz TDD) fit in the band ? | Proposed or future alternative usage under consideration (within CEPT) ? | Actual usage of the band - national level - within CEPT (- worldwide) for future sharing studies ? | Actual usage of adjacent bands - national level - within CEPT for future compatibility studies ? | (Likely oppor-tunity cost) ? | Likely time scales (target date 2012) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1350 – 1375 MHz / 1492 – 1517 MHz | yes, no |  | yes | no | FS, Tactical Radio Relay, Spectral line observations in 1330-1400 MHz, mil. radars | Defence, Radar and Navigation, Radio astronomy, Satellite navigation / FS, Tactical Radio Relay, (S-DAB) |  | beyond (Possibly within?) |
| 1375 – 1400 MHz | yes |  | yes | no | FS, Tactical Radio Relay, Radio astronomy | FS, Tactical Radio Relay, Radio astronomy, Passive sensors (satellite) |  | beyond (Possibly within?) |
| 1452 – 1492 MHz | no | high | yes | TRA-ECS (Mobile downlink) | Mobile Multimedia (WAPECS), Digital Radio, Satellite Digital Audio, Aeronautical telemetry | Aeronautical telemetry, Fixed Links, Tactical Radio Relay |  | beyond for some countries |
| 1900 – 1920 MHz / 2010 – 2025 MHz | yes, yes | high, high | yes | ECS | UMTS (TDD) | DECT, UMTS (FDD), EESS, SRS, FS, SAP/SAB |  | within |
| 3600 – 3800 MHz | yes | high | yes | band plan under consideration within ECC PT 1 | BWA, FSS, FS links | BWA, FSS, FS links |  | possibly within |
| 5905 – 5925 MHz | yes | high | yes (air to ground) | AI 1.20 WRC-12 | FSS (uplink), military systems (on a national level) | FS links |  | within |

# Category 4: Frequency bands with high regulatory obstacles for an introduction of Broadband DA2GC systems

| Frequency band | Existing allocation in the RR suitable for Broadband DA2GC (MS or AMS) ? | Current level of harmonisation ? | Would the bandwidth required (2 x 10 MHz FDD or 20 MHz TDD) fit in the band ? | Proposed or future alternative usage under consideration (within CEPT) ? | Actual usage of the band - national level - within CEPT (- worldwide) for future sharing studies ? | Actual usage of adjacent bands - national level - within CEPT for future compatibility studies ? | (Likely oppor-tunity cost) ? | Likely time scales (target date 2012) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 821 – 832 MHz | no | high | no | PMSE, AI 1.17 WRC-12 | BC, PMSE | MFCN, BC |  | beyond |
| 960 – 1164 MHz | no | high | yes | AI 1.3, 1.4 WRC-12, FCS | UAT, DME, JTIDS, MIDS, SSR, TACAN, RSBN | RNSS, MS (GSM900), DME, TACAN |  | beyond |
| 1427 – 1452 MHz | no |  | yes | no | FS, Tactical Radio Relay | Passive sensors (satellite), (T-DAB) |  | beyond (Possibly within?) |
| 2300 – 2400 MHz | yes | low | yes |  | Aeronautical Telemetry, Amateur, Mobile applications, SAP/SAB (cordless cameras) | FS, MS, SRS |  | within ??? |
| 2570 – 2620 MHz | no | high | yes | --- | ECS | ECS |  | beyond |
| 2700 – 2900 MHz | no | high | yes |  | Primary Surveillance Radar (PSR), meteorological radars, aeronautical telemetry | ECS, PSR, Passive sensors (satellite), Defence systems, Radar and Navigation systems, RAS |  | beyond |
| 4200 – 4400 MHz | no | high | yes | see footnote EU18 | Radio Altimeter (civil/military), Earth exploration-satellite service (passive sensors) | FS, FSS, Defence systems |  | beyond |
| 5091 – 5150 MHz (1) | yes? (see also 5.444B) | high | yes | Aeronautical telemetry, aeronautical security, Airport Surface Datalink, AI 1.3, 1.7 WRC-12 | MLS, FSS (feeder links) | MLS, FSS (feeder links), RLANs, BBDR, aeronautical telemetry |  | within |

(1) Compatibility studies are available for an aeronautical security system in the band 5091-5150 MHz, which were completed prior to the AMS allocation by WRC-2007. See Recommendation ITU-R M.1827.

Remark: The reason behind the footnote EU18 (ERC Report 25) needs to be clarified.

List of abbreviations used in the tables

| Abbreviation | Explanation |
| --- | --- |
| BBDR | Broad Band Disaster Relief |
| BC | Broadcasting |
| BFWA | Broadband Fixed Wireless Access |
| BWA | Broadband Wireless Access |
| DECT | Digital Enhanced Cordless Telecommunications |
| DME | Distance Measuring Equipment |
| ECS | Electronic Communications Services |
| EESS | Earth Exploration-Satellite Service |
| FCS | Future Communications System |
| FDD | Frequency Division Duplex |
| FS | Fixed Service |
| FSS | Fixed-Satellite Service |
| ISM | Industrial, scientific and medical applications |
| ITS | Intelligent Transport Systems |
| JTIDS | Joint Tactical Information Distribution System |
| MFCN | Mobile/fixed communications networks |
| MIDS | Multifunctional Information Distribution System |
| MLS | Microwave Landing System |
| MS | Mobile Service |
| MSS | Mobile Satellite-Service |
| PMSE | Program Making and Special Events |
| PSR | Primary Surveillance Radar |
| RAS | Radio Astronomy Service |
| RFIDs | Radio frequency identification devices |
| RLANs | Radio Local Area Networks |
| RLS | Radiolocation Service |
| RNSS | Radionavigation-Satellite Service |
| RSBN | Radiotechniczny System Bliskiej Nawigacji (Russian radio navigation system) |
| RTTT | Road Transport and Traffic Telematics |
| SAP/SAB | Services Ancillary to Programme making / Services Ancillary to Broadcasting |
| SRDs | Short Range Devices |
| SRS | Space Research Service |
| SSR | Secondary Surveillance Radar |
| TACAN | Tactical Air Navigation |
| T-DAB | Terrestrial Digital Audio Broadcasting |
| TDD | Time Division Duplex |
| TRA-ECS | Terrestrial radio applications capable of providing Electronic Communications Services |
| UAT | Universal Access Transponder |
| UMTS | Universal Mobile Telecommunications System |
| WAPECS | Wireless Access Policy for Electronic Communications Services |

unless combined with another band