| Project Team FM PT 48 |  |
| --- | --- |
| 3rd Meeting of FM PT 48 Paris, 13-14 September 2011 | FM48(11)038 |
| Date issued: 9 September 2011 Source: Aero3G Subject: Compatibility Analysis between DA2G(3G) and Systems in the Band 5850-5870 MHz |  |

N

Password protection required? (Y/N)

Summary This contribution analyses the compatibility between the DA2G(3G) system and other systems operating in this frequency band. It also considers radiolocation systems operating below 5850 MHz as experience with civil radar has shown that any front-end filtering of the radar is unlikely to be effective for the adjacent band. Indeed this particular analysis, assuming no radiolocation filtering, suggests that the first useable centre frequency for DA2G(3) would be 5857.5 MHz. It is concluded that DA2G(3G) is compatible for operation in the frequency band 5850-5870 MHz provided mitigation measures are taken in certain circumstances. These measures are relatively simple to achieve.

Proposal FM PT 48 is invited to consider this contribution and support the use of the frequency band 5850-5870 MHz for the DA2G(3G) system.

Background Project Team FM48 is working on spectrum issues concerning Broadband Direct-Air-to-Ground Communications (DA2GC) intended to provide high performance broadband connectivity between aircraft and a terrestrial radio access network to be deployed in Europe. Among other things it seeks to identify possible candidate bands for harmonised spectrum in Europe. WGFM has indicated that one preferred band is 5850-5870 MHz.

Contents

1.	Executive Summary	5

2.	Introduction	8

3.	Acronyms	8

4.	Current Allocations and Utilisations affecting the Band 5850-5870 MHz	8

5.	Compatibility Scenarios	9

6.	Antenna Patterns	10

6.1	Omni Directional	10

6.2	Sector Antenna	11

6.3	Aircraft Antenna	12

7.	Propagation Assumptions	12

7.1	Free Path Loss	12

7.2	Bandwidth Factor	12

7.3	Duty Cycle Factor	13

7.4	Received Power	13

7.5	Interference Threshold	13

7.6	Aggregate Power	13

8.	DA2G(3G) Parameters	14

9.	Compatibility between DA2G(3G) and BBDR	14

9.1	Technical Parameters of BBDR Receiver (ECC Report 110)	14

9.2	Technical Parameters of BBDR Transmitter (ECC Report 110)	14

9.3	Compatibility between Source DA2G(3G) and Victim BBDR Base Stations	15

9.4	Compatibility between Source DA2G(3G) Aircraft and Victim BBDR Base Station	16

9.5	Compatibility between Source BBDR and Victim DA2G(3G) Base Stations	16

9.6	Compatibility between Source BBDR Base Station and Victim DA2G(3G) Aircraft	17

9.7	Discussion	17

10.	Compatibility between DA2G(3G) and Radiolocation Systems	18

10.1	Technical Parameters of Radiolocation Systems (ECC Report 110) (ECC Report 101)	18

10.2	Compatibility between Source DA2G (3G) and Victim Radiolocation Systems	18

10.2.1	Radar L	19

10.2.2	Radar M	20

10.2.3	Radar N	21

10.2.4	Radar O	22

10.2.5	Radar Q	23

10.2.6	Radar X & Y	24

10.2.7	Radar Z	25

10.3	Compatibility between Source Radiolocation Systems and Victim DA2G(3G)	26

10.3.1	Radar L	26

10.3.2	Radar M	27

10.3.3	Radar N	28

10.3.4	Radar O	29

10.3.5	Radar Q	30

10.3.6	Radar X & Y	31

10.3.7	Radar Z	32

10.4	Discussion	32

10.4.1	DA2G(3G) Base Station Source – Radiolocation Victim	32

10.4.2	DA2G(3G) Aircraft Source – Radiolocation Victim	33

10.4.3	Radiolocation Source - DA2G(3G) Base Station Victim	33

10.4.4	Radiolocation Source - DA2G(3G) Aircraft Victim	33

11.	Compatibility between DA2G(3G) and FSS	34

11.1	Technical Parameters of FSS Satellite (ECC Report 110)	34

11.2	Technical Parameters of GES (ECC Report 110)	34

11.3	Compatibility between Source DA2G (3G) and Victim FSS Satellite	34

11.4	Compatibility between Source GES and Victim DA2G Base Station	35

11.5	Compatibility between Source GES and Victim DA2G Aircraft	35

11.6	Discussion	35

12.	Compatibility between DA2G(3G) and ITS	35

12.1	Technical Parameters of ITS (ECC Report 101)	35

12.2	Compatibility between Source DA2G (3G) Base Station and Victim ITS	36

12.3	Compatibility between Source DA2G (3G) Aircraft and Victim ITS	36

12.4	Compatibility between Source ITS and DA2G(3G) Base Station	37

12.5	Compatibility between Source ITS and DA2G(3G) Aircraft	37

12.6	Discussion	37

13.	Compatibility between DA2G(3G) and BFWA	38

13.1	Technical Parameters of BFWA (ECC Report 110)	38

13.2	Compatibility between Source DA2G (3G) Base Station and Victim BFWA	38

13.3	Compatibility between Source DA2G (3G) Aircraft and Victim BFWA	39

13.4	Compatibility between Source BFWA and Victim DA2G (3G) Base Station	40

13.5	Compatibility between Source BFWA and Victim DA2G (3G) Aircraft	40

13.6	Discussion	41

14.	Compatibility between DA2G(3G) and SRD	41

14.1	Technical Parameters of BFWA (ECC Report 110)	41

14.2	Compatibility between Source DA2G (3G) Base Station and Victim SRD	41

Co-channel compatibility is achieved for separation distances greater than 200 metres.	42

14.3	Compatibility between Source DA2G (3G) Aircraft and Victim SRD	42

Co-channel compatibility is achieved.	42

14.4	Compatibility between Source SRD  and Victim DA2G Base Station	42

14.5	Compatibility between Source SRD  and Victim DA2G Aircraft	43

14.6	Discussion	43

15.	Conclusions	43

Figure 1: Omni Elevation Pattern	10

Figure 2: Omni Azimuth Pattern	10

Figure 3: Sector Elevation Pattern	11

Figure 4: Sector Azimuth Pattern	11

Figure 5: Aircraft Antenna Pattern (Elevation)	12

Figure 6: Source DA2G Base Station (Sector)/Victim BBDR Base Station	15

Figure 7: Source DA2G Base Station (Omni)/Victim BBDR Station	15

Figure 8: Source DA2G Aircraft/Victim BBDR	16

Figure 9: Source BBDR Base Station/Victim DA2G Base Station	16

Figure 10: Source BBDR Base Station/Victim DA2G Aircraft	17

Figure 11: DA2G Base Station (Sector)/Radar L Victim – 7.5 MHz Offset	19

Figure 12: DA2G Base Station (Omni)/Radar L Victim – 7.5 MHz Offset	19

Figure 13: DA2G Aircraft/Radar L Victim – 7.5 MHz Offset	19

Figure 14: DA2G Base Station (Sector)/Radar M Victim – 7.5 MHz Offset	20

Figure 15: DA2G Base Station (Omni)/Radar M Victim – 7.5 MHz Offset	20

Figure 16: DA2G Aircraft/Radar M Victim – 7.5 MHz Offset	20

Figure 17: DA2G Base Station (Sector)/Radar N Victim – 7.5 MHz Offset	21

Figure 18: DA2G Base Station (Omni)/Radar N Victim – 7.5 MHz Offset	21

Figure 19: DA2G Aircraft/Radar N Victim – 7.5 MHz Offset	21

Figure 20: DA2G Base Station (Sector)/Radar O Victim – 7.5 MHz Offset	22

Figure 21: DA2G Base Station (Omni)/Radar O Victim – 7.5 MHz Offset	22

Figure 22: DA2G Aircraft/Radar O Victim – 7.5 MHz Offset	22

Figure 23: DA2G Base Station (Sector)/Radar Q Victim – 27.5 MHz Offset	23

Figure 24: DA2G Base Station (Omni)/Radar Q Victim – 27.5 MHz Offset	23

Figure 25: DA2G Aircraft/Radar Q Victim – 27.5 MHz Offset	23

Figure 26: DA2G Base Station (Sector)/Radar X&Y Victim – 7.5 MHz Offset	24

Figure 27: DA2G Base Station (Omni)/Radar X&Y Victim – 7.5 MHz Offset	24

Figure 28: DA2G Aircraft/Radar X&Y Victim – 7.5 MHz Offset	24

Figure 29: DA2G Base Station (Sector)/Radar Z Victim – 7.5 MHz Offset	25

Figure 30: DA2G Base Station (Omni)/Radar Z Victim – 7.5 MHz Offset	25

Figure 31: DA2G Aircraft/Radar Z Victim – 7.5 MHz Offset	25

Figure 32: Radar L Source/Victim DA2G Base Station – 7.5 MHz Offset	26

Figure 33: Radar L Source/Victim DA2G Aircraft – 7.5 MHz Offset	26

Figure 34: Radar M Source/Victim DA2G Base Station – 7.5 MHz Offset	27

Figure 35: Radar M Source/Victim DA2G Aircraft – 7.5 MHz Offset	27

Figure 36: Radar N Source/Victim DA2G Base Station – 7.5 MHz Offset	28

Figure 37: Radar N Source/Victim DA2G Aircraft – 7.5 MHz Offset	28

Figure 38: Radar O Source/Victim DA2G Base Station – 7.5 MHz Offset	29

Figure 39: Radar O Source/Victim DA2G Aircraft – 7.5 MHz Offset	29

Figure 40: Radar Q Source/Victim DA2G Base Station – 27.5 MHz Offset	30

Figure 41: Radar Q Source/Victim DA2G Aircraft – 27.5 MHz Offset	30

Figure 42: Radar X&Y Source/Victim DA2G Base Station – 7.5 MHz Offset	31

Figure 43: Radar X & Y Source/Victim DA2G Aircraft – 7.5 MHz Offset	31

Figure 44: Radar Z Source/Victim DA2G Aircraft – 7.5 MHz Offset	32

Figure 45: Radar X & Y Source/Victim DA2G Aircraft – 7.5 MHz Offset	32

Figure 46: DA2G Network Source/Victim FSS Satellite	34

Figure 47: GES Source/Victim DA2G Base Station	35

Figure 48: DA2G Base Station Source/Victim ITS	36

Figure 49: DA2G Aircraft Source/Victim ITS	36

Figure 50: ITS Source/Victim DA2G Base Station	37

Figure 51: ITS Source/Victim DA2G Aircraft	37

Figure 52: DA2G Base Station Source/Victim BFWA1 (P-MP) and BFWA2 (Mesh)	38

Figure 53: DA2G Base Station Source/Victim BFWA1 (P-MP) and BFWA2 (Mesh) with down-tilt of 6 degrees	39

Figure 54: DA2G Aircraft Source/Victim BFWA1 (P-MP) and BFWA2 (Mesh)	39

Figure 55: BFWA1 (P-MP) and BFWA2 (Mesh) Source/Victim DA2G Base Station	40

Figure 56: BFWA1 (P-MP) and BFWA2 (Mesh) Source/Victim DA2G Base Station	40

Figure 57: DA2G Base Station Source/Victim SRD	41

Figure 58: DA2G Aircraft Source/Victim SRD	42

Figure 59: SRD Source/Victim DA2G Base Station	42

Figure 60: SRD Source/Victim DA2G Aircraft	43

# Executive Summary

Introduction

This report describes compatibility analyses between DA2G (3G) and other systems operating in, or adjacent to in the case of radiolocation, the frequency band 5850 – 5870 MHz. The DA2G (3G) system is based on UMTS 3G TDD technology that has been modified for aeronautical use in the 5 GHz band.

The results are summarises below:

|  | Compatible |
| --- | --- |
|  | Separation distance less than 1 km – Mitigation possible |
|  | Separation distance greater or equal to 1 km – Mitigation possible |
|  | Compatibility not possible |

| Victim | Type | DA2G(3G) Base Station Source | DA2G(3G) Aircraft Source |
| --- | --- | --- | --- |
| BBDR |  |  |  |
| Radiolocation | L |  |  |
|  | M |  |  |
|  | N |  |  |
|  | O |  |  |
|  | Q |  |  |
|  | X&Y |  |  |
|  | Z |  |  |
| FSS |  |  |  |
| ITS |  |  |  |
| BFWA |  |  |  |
| SRD |  |  |  |

| Source | Type | DA2G(3G) Base Station Victim | DA2G(3G) Aircraft Victim |
| --- | --- | --- | --- |
| BBDR |  |  |  |
| Radiolocation | L |  |  |
|  | M |  |  |
|  | N |  |  |
|  | O |  |  |
|  | Q |  |  |
|  | X&Y |  |  |
|  | Z |  |  |
| FSS |  |  |  |
| ITS |  |  |  |
| BFWA |  |  |  |
| SRD |  |  |  |

Broad Band Disaster Relief

Compatibility between DA2G (Source) and BBDR (Victim) can be achieved in all cases for distances greater than 650 metres. It is noted that the BBDR User Equipment is less sensitive to interference than the BBDR base station. Mitigation for shorter distances is possible because of propagation shadowing by buildings and by the potential for the base station to use adjacent channels. The possible co-channel interference between BBDR and DA2G base stations is acceptable given the probability of a disaster. Interference between BBDR base station and DA2G aircraft is acceptable.

Radiolocation

For the analysis of Radiolocation compatibility it has been assumed that any front-end filtering by the Radiolocation system would not be effective for the adjacent band considered here. This is based on the experience of interference caused to civil radars by systems operating in adjacent bands. If the radiolocation system is operating at the top end of its band then the lowest acceptable centre frequency for DA2G would be 5857.5 MHz.

The minimum separation distances (DA2G base station source/radiolocation system victim) are as follows:

| Radar Type | Minimum Distance (Kms) |
| --- | --- |
| L | 1.3 |
| M | 1.0 |
| N | 1.0 |
| O | 0.6 |
| Q | 0.1 |
| X & Y | 0.2 |
| Z | 0.1 |

Compatibility can be achieved through planning by ensuring the minimum distances are respected. This should not pose any problems given the relatively short separation distances and the flexibility in siting DA2G base stations.

Compatibility is achieved for DA2G aircraft source/Radiolocation victim.

The minimum separation distances (Radiolocation source/DA2G base station victim) are as follows:

| Radar Type | Minimum Distance (Kms) |
| --- | --- |
| L | 1.1 |
| M | 0.75 |
| N | 0.4 |
| O | Compatible |
| Q | Compatible |
| X & Y | Compatible |
| Z | Compatible |

Compatibility can be achieved for Radar L, M and N through planning by ensuring the minimum distance is respected. For instrumentation radar this is straightforward. The other radars are compatible.

Compatibility is achieved for Radiolocation source/DA2G aircraft victim with some minor acceptable interference from Radar L.

Fixed Satellite Service

There is compatibility between the DA2G system and the FSS satellite. The GES could interfere with the DA2G base station but mitigation is easily achieved by the siting of the base station. The GES will cause interference to the DA2G aircraft but the time during which this is experienced is low as is the probability of flying in the vicinity of the GES main beam.

Intelligent Transport Systems

Potential co-channel interference caused between ITS (roadside-to-vehicle) links by the DA2G base station can be mitigated by respecting the minimum separation distance. This can be achieved through siting of the base station. There is no co-channel interference between ITS and the DA2G aircraft.

Broadband Fixed Wireless Access

Co-channel compatibility is achieved for separation distances greater than about 3 kms. ECC Report 68 suggests that 90% of BFWA systems would be Point to Multi-Point. These systems are designed to use directional or sectoral antennas that could have down-tilt. This would reduce the minimum separation distance to about 0.5 km. Hence co-channel compatibility between BFWA systems and DA2G base stations can be achieved through siting of the base stations. Alternatively, using different channels would provide compatibility. There is no co-channel interference between BFWA systems and DA2G aircraft.

Non-specific Short Range Devices

Co-channel compatibility between SRD (victim) and DA2G (source) is achieved provided the DA2G base station is more than 200 metres from the SRD. The SRD (Type I) could cause interference to the DA2G base station for separation distances less than about 2.5 kms but this is not considered significant because the probability of a co-channel Type I (narrow band) source close to the base station is low and mitigation is possible through the use of directional antennas if necessary.

Conclusions

The analyses conclude that the DA2G(3G) is compatible with other systems operating in the frequency band 5850 - 5870 MHz and radiolocation systems operating in the lower adjacent band. Where mitigation is required this can be achieved by appropriate siting of the DA2G (3G) base stations. There are no compatibility issues regarding interference caused by the DA2G (3G) aircraft.

Any interference caused to the DA2G (3G) ground network can be mitigated through base station siting and/or directional antenna considerations.

Interference caused to the DA2G (3G) aircraft caused by the FSS GES main beam is acceptable because the probability of the aircraft being in the vicinity of the main beam is low and the effective transit time through the main beam is very short.

Finally, the aeronautical use of the frequency band 5850 – 5870 MHz would significant increase the overall  spectrum efficiency without disturbing existing systems including radiolocation systems (particularly search radars) operating in the lower adjacent band.

# Introduction

This report describes compatibility analyses between DA2G (3G) and other systems operating in, or adjacent to in the case of radiolocation, the frequency band 5850 – 5870 MHz.

The DA2G (3G) system is based on UMTS 3G TDD technology that has been modified for aeronautical use in the 5 GHz band.

# Acronyms

| Term | Definition |
| --- | --- |
| BBDR | Broad-Band Disaster Relief |
| BFWA | Broadband Fixed Wireless Access |
| CEPT | European Conference of Postal and Telecommunications Administrations |
| FSS | Fixed Satellite Service |
| ITS | Intelligent Transport Systems |
| ITU | International Telecommunications Union |
| SRD | Short Range Devices |
| BFWA | Broadband Fixed Wireless Access |
| WRC | World Radiocommunications Conference |

# Current Allocations and Utilisations affecting the Band 5850-5870 MHz

The following allocations have been made in or encompassing this band:

Fixed-Satellite (Earth/space)

Radiolocation

Amateur

Amateur-Satellite (space/Earth)

Mobile

In Europe, the major utilisation is by:

Defence systems (adjacent band)

Non-specific SRDs

ITS

BBDR

BFWA

FSS

# Compatibility Scenarios

|  | Compatible |
| --- | --- |
|  | Separation distance less than 1 km – Mitigation possible |
|  | Separation distance greater or equal to 1 km – Mitigation possible |
|  | Compatibility not possible |

| Victim | Type | DA2G(3G) Base Station Source | DA2G(3G) Aircraft Source |
| --- | --- | --- | --- |
| BBDR |  |  |  |
| Radiolocation | L |  |  |
|  | M |  |  |
|  | N |  |  |
|  | O |  |  |
|  | Q |  |  |
|  | X&Y |  |  |
|  | Z |  |  |
| FSS |  |  |  |
| ITS |  |  |  |
| BFWA |  |  |  |
| SRD |  |  |  |

| Source | Type | DA2G(3G) Base Station Victim | DA2G(3G) Aircraft Victim |
| --- | --- | --- | --- |
| BBDR |  |  |  |
| Radiolocation | L |  |  |
|  | M |  |  |
|  | N |  |  |
|  | O |  |  |
|  | Q |  |  |
|  | X&Y |  |  |
|  | Z |  |  |
| FSS |  |  |  |
| ITS |  |  |  |
| BFWA |  |  |  |
| SRD |  |  |  |

# Antenna Patterns

For the purposes of this analysis actual antenna patterns were used.

## Omni Directional

The measured elevation pattern is illustrated below:

Figure 1: Omni Elevation Pattern

The measured azimuth pattern is illustrated below:

Figure 2: Omni Azimuth Pattern

## Sector Antenna

A total of 6 antennas, each with an azimuthal coverage of 60 degrees, were used to provide omni directional coverage in azimuth.

The elevation pattern of the sector antennas is illustrated below:

Figure 3: Sector Elevation Pattern

The azimuth pattern of the sector antenna is illustrated below:

Figure 4: Sector Azimuth Pattern

## Aircraft Antenna

The elevation pattern of the aircraft antenna is illustrated below:

Figure 5: Aircraft Antenna Pattern (Elevation)

# Propagation Assumptions

## Free Path Loss

The Free Path Loss (Lf) is given by the equation:

Lf (d) = 32.44 + 20log10(fMHz) + 20log10(d)

Where:

Lf = free path loss in dB

d = distance in kms

fMHz = operating frequency in MHz

## Bandwidth Factor

The Bandwidth Factor, Bf, allows for the source transmitter bandwidth to be different to that of the victim receiver bandwidth and if given by the equation:

If  (BWRx >= BWTx) Bf =  0 dB

If  (BWRx < BWTx) Bf  = 10log10(BWRx/BWTx) dB

## Duty Cycle Factor

The Duty Cycle Factor  reflects the time the transmitter is ON compared to the time it is OFF and is given by the equation:

DCf = 10log10(Ton/Toff) dB

Where:

DCf = duty cycle factor in dB

Ton = transmitter ON period

Toff  = transmitter OFF period

## Received Power

The received power, PRx, from a single transmission is given by the equation:

PRx = PTx + GTxAntenna – LTxFeeder - Lf  + GRxAntenna – LRxFeeder – Lp + Bf

Where:

PRx = receiver power in dBm

PTx = transmitter power in dBm

GTxAntenna = transmitter antenna gain in dBi

LTxFeeder = transmitter feeder loss in dB

Lf = free space loss in dB

GRxAntenna = receiver antenna gain in dBi

LRxFeeder = receiver feeder loss in dB

Lp = polarisation discrimination in dB

Bf = bandwidth factor in dB

## Interference Threshold

The interference threshold, Ithresh, is given by the equation:

Ithresh = 10log10(kTBC) + 30 dBm

Where:

k =  Boltzmann’s constant (1.38 * 10-23  joules/K

T = noise temperature of receiver in degs K

B = receiver bandwidth in Hz

C = percentage interference criterion

## Aggregate Power

The received aggregate power from many transmission sources is given by the equation:

PRxAgg  = 10log10(∑10(PRx /10) dBm)

Where

PRxAgg = aggregate received power in dBm

PRx = received power from a single transmission in dBm

# DA2G(3G) Parameters

The table below details the assumed parameters for the DA2G(3G):

| Parameter | Unit | Value |
| --- | --- | --- |
| Max transmitter power | dBm | 40 |
| Transmitter feeder loss | dB | 4 |
| Transmitter duty cycle | % | 50 |
| Transmitter bandwidth | MHz | 5 |
| Receiver interference threshold | dBm/MHz | -113.9 |

Table 1: UMTS 3G System Parameters

# Compatibility between DA2G(3G) and BBDR

## Technical Parameters of BBDR Receiver (ECC Report 110)

| Receiver Characteristics | Units | Base Station | User Equipment |
| --- | --- | --- | --- |
| Bandwidth | MHz | 10 | 10 |
| Sensitivity | dBm | -82 (-88 to -69) | -82 (-91 to -72) |
| Sensitivity at antenna input | dBm/MHz | -101 (-107 to -88) | -85 (-91 to -72) |
| C/I | dB | 6 | 6 |
| Allowable Interfering Power at antenna input | dBm/MHz | -107 | -91 |

## Technical Parameters of BBDR Transmitter (ECC Report 110)

| Transmitter Characteristics | Units | Base Station | User Equipment |
| --- | --- | --- | --- |
| Bandwidth | MHz | 10 | 10 |
| Transmitter e.i.r.p | dBm | 36 | 23 |
| Assumed value for TPC | dB | 0 | 6 |
| Antenna Gain | dBi | 9 | 0 |
| Body loss | dB | 0 | 6 |
| Antenna loss due to portable usage | dB | 0 | 1 |

## Compatibility between Source DA2G(3G) and Victim BBDR Base Stations

Figure 6: Source DA2G Base Station (Sector)/Victim BBDR Base Station

Figure 7: Source DA2G Base Station (Omni)/Victim BBDR Station

Co-channel compatibility is achieved for distances greater than 650 metres for both sector antenna and omni antenna cases. This distance is likely to be reduced by location factors including siting DA2G base stations on roof tops thereby introducing greater attenuation at negative elevation angles due to shadowing.

## Compatibility between Source DA2G(3G) Aircraft and Victim BBDR Base Station

Figure 8: Source DA2G Aircraft/Victim BBDR

Co-channel compatibility is achieved.

## Compatibility between Source BBDR and Victim DA2G(3G) Base Stations

Figure 9: Source BBDR Base Station/Victim DA2G Base Station

The BBDR base station will cause interference to the DA2G base station for distances less than about 20 kms. Beyond that distance there is compatibility.

However it should be noted that the probability of a co-channel BBDR transmission in the vicinity of a DA2G base station is low give the likelihood of a disaster and also there would probably be further attenuation through shadowing by buildings or terrain.

Regarding the DA2G aircraft, co-channel compatibility is achieved.

Given that DA2G is non-safety of flight, this possible interference by BBDR is acceptable.

## Compatibility between Source BBDR Base Station and Victim DA2G(3G) Aircraft

Figure 10: Source BBDR Base Station/Victim DA2G Aircraft

The BBDR base station will cause some interference to the DA2G aircraft for distances less than about 10 kms. However, the degree of interference is slight and the impact will be a possible reduction in data rate. Given an aircraft speed of 800 kms/hr the duration will be less than 2 minutes.

## Discussion

Compatibility between DA2G (Source) and BBDR (Victim) can be achieved in all cases for distances greater than 650 metres. It is noted that the BBDR User Equipment is less sensitive to interference than the BBDR base station. Mitigation for shorter distances is possible because of propagation shadowing by buildings and by the potential for the base station to use adjacent channels.

The possible co-channel interference between BBDR and DA2G base stations is acceptable given the probability of a disaster. Interference between BBDR base station and DA2G aircraft is acceptable.

# Compatibility between DA2G(3G) and Radiolocation Systems

## Technical Parameters of Radiolocation Systems (ECC Report 110) (ECC Report 101)

| Characteristics | Radar L | Radar M | Radar N | Radar O | Radar Q | Radar X&Y | Radar Z |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Function | Instrumentation | Instrumentation | Instrumentation | Instrumentation | Surface & Air Search | Surface & Air Search | Search |
| Tuning (MHz) | 5350-5850 | 5350-5850 | 5400-5850 | 5400-5850 | 5450-8525 | 5400-5850 | 5250-5850 |
| Eirp (dBm) | 148.5 | 137.8 | 135.9 | 124.2 | 114.5 | 105.8 | 110 |
| Polarisation | Vert/LHC | Vert/LHC | Vert/LHC | Vert/LHC | Horz | Vert | Horz |
| Tx Bandwidth (MHz) | 4.8 | 4 | 8 | 8 | 10 | 4 | 1 |
| Eirp (dBm/MHz) | 141.7 | 131.8 | 126.9 | 115.2 | 104.5 | 99.8 | 110 |
| Noise temperature (deg K) | 290 | 290 | 290 | 290 | 290 | 290 | 290 |
| Unwanted Attenuation Factor (Spurious) (dB) | 60 | 60 | 60 | 60 | 60 | 60 | 60 |
| Antenna Gain (dBi) | 54 | 47 | 45.9 | 42 | 30 | 35 | 31.5 |
| Side lobe Attenuation (dB) | 20 | 20 | 22 | 22 | 25 | 40 | 40 |
| Rx Noise Figure (dB) | 7 | 4 | 2.3 | 3 | 3 | 5 | 13 |
| I/N (dB) | -6 | -6 | -6 | -6 | -6 | -6 | -6 |
| Rx Allowable Interference (dBm/MHz) | -167 | -163 | -164 | -159 | -147 | -150 | -138 |

Table 2: Radiolocation Technical Parameters

## Compatibility between Source DA2G (3G) and Victim Radiolocation Systems

The following analyses have assumed that the radar is operating at 5850 MHz with the exception of Radar Q where the frequency is 5825 MHz.  The possible centre frequencies for DA2G(3G) are 5852.5, 5857.5, 5862.5 and 5867.5 MHz.

Initial analysis demonstrated that the DA2G(3G) lower frequency (5852.5 MHz) would cause marginal interference to search  radar and so the next frequency was selected giving an offset of 7.5 MHz. The exception was Radar Q where the minimum offset was 27.5 MHz.

### Radar L

Figure 11: DA2G Base Station (Sector)/Radar L Victim – 7.5 MHz Offset

Figure 12: DA2G Base Station (Omni)/Radar L Victim – 7.5 MHz Offset

Figure 13: DA2G Aircraft/Radar L Victim – 7.5 MHz Offset

### Radar M

Figure 14: DA2G Base Station (Sector)/Radar M Victim – 7.5 MHz Offset

Figure 15: DA2G Base Station (Omni)/Radar M Victim – 7.5 MHz Offset

Figure 16: DA2G Aircraft/Radar M Victim – 7.5 MHz Offset

### Radar N

Figure 17: DA2G Base Station (Sector)/Radar N Victim – 7.5 MHz Offset

Figure 18: DA2G Base Station (Omni)/Radar N Victim – 7.5 MHz Offset

Figure 19: DA2G Aircraft/Radar N Victim – 7.5 MHz Offset

### Radar O

Figure 20: DA2G Base Station (Sector)/Radar O Victim – 7.5 MHz Offset

Figure 21: DA2G Base Station (Omni)/Radar O Victim – 7.5 MHz Offset

Figure 22: DA2G Aircraft/Radar O Victim – 7.5 MHz Offset

### Radar Q

Figure 23: DA2G Base Station (Sector)/Radar Q Victim – 27.5 MHz Offset

Figure 24: DA2G Base Station (Omni)/Radar Q Victim – 27.5 MHz Offset

Figure 25: DA2G Aircraft/Radar Q Victim – 27.5 MHz Offset

### Radar X & Y

Figure 26: DA2G Base Station (Sector)/Radar X&Y Victim – 7.5 MHz Offset

Figure 27: DA2G Base Station (Omni)/Radar X&Y Victim – 7.5 MHz Offset

Figure 28: DA2G Aircraft/Radar X&Y Victim – 7.5 MHz Offset

### Radar Z

Figure 29: DA2G Base Station (Sector)/Radar Z Victim – 7.5 MHz Offset

Figure 30: DA2G Base Station (Omni)/Radar Z Victim – 7.5 MHz Offset

Figure 31: DA2G Aircraft/Radar Z Victim – 7.5 MHz Offset

## Compatibility between Source Radiolocation Systems and Victim DA2G(3G)

Note: For the analysis of the impact of the Radiolocation systems on the DA2G base stations, the signal levels are those at the base station receiver antenna.

### Radar L

Figure 32: Radar L Source/Victim DA2G Base Station – 7.5 MHz Offset

Figure 33: Radar L Source/Victim DA2G Aircraft – 7.5 MHz Offset

### Radar M

Figure 34: Radar M Source/Victim DA2G Base Station – 7.5 MHz Offset

Figure 35: Radar M Source/Victim DA2G Aircraft – 7.5 MHz Offset

### Radar N

Figure 36: Radar N Source/Victim DA2G Base Station – 7.5 MHz Offset

Figure 37: Radar N Source/Victim DA2G Aircraft – 7.5 MHz Offset

### Radar O

Figure 38: Radar O Source/Victim DA2G Base Station – 7.5 MHz Offset

Figure 39: Radar O Source/Victim DA2G Aircraft – 7.5 MHz Offset

### Radar Q

Figure 40: Radar Q Source/Victim DA2G Base Station – 27.5 MHz Offset

Figure 41: Radar Q Source/Victim DA2G Aircraft – 27.5 MHz Offset

### Radar X & Y

Figure 42: Radar X&Y Source/Victim DA2G Base Station – 7.5 MHz Offset

Figure 43: Radar X & Y Source/Victim DA2G Aircraft – 7.5 MHz Offset

### Radar Z

Figure 44: Radar Z Source/Victim DA2G Aircraft – 7.5 MHz Offset

Figure 45: Radar X & Y Source/Victim DA2G Aircraft – 7.5 MHz Offset

## Discussion

For the analysis of Radiolocation compatibility it has been assumed that any front-end filtering by the Radiolocation system would not be effective for the adjacent band considered here. This is based on the experience of interference caused to civil radars by systems operating in adjacent bands. If the radiolocation system is operating at the top end of its band then the lowest acceptable centre frequency for DA2G would be 5857.5 MHz.

### DA2G(3G) Base Station Source – Radiolocation Victim

The minimum separation distances are as follows:

| Radar Type | Minimum Distance (Kms) |
| --- | --- |
| L | 1.3 |
| M | 1.0 |
| N | 1.0 |
| O | 0.6 |
| Q | 0.1 |
| X & Y | 0.2 |
| Z | 0.1 |

Compatibility can be achieved through planning by ensuring the minimum distances are respected. This should not pose any problems given the relatively short separation distances and the flexibility in siting DA2G base stations.

### DA2G(3G) Aircraft Source – Radiolocation Victim

Compatibility is achieved.

### Radiolocation Source - DA2G(3G) Base Station Victim

The minimum separation distances are as follows:

| Radar Type | Minimum Distance (Kms) |
| --- | --- |
| L | 1.1 |
| M | 0.75 |
| N | 0.4 |
| O | Compatible |
| Q | Compatible |
| X & Y | Compatible |
| Z | Compatible |

Compatibility can be achieved for Radar L, M and N through planning by ensuring the minimum distance is respected. For instrumentation radar this is straightforward. The other radars are compatible.

### Radiolocation Source - DA2G(3G) Aircraft Victim

Compatibility is achieved with some minor acceptable  interference from Radar L.

# Compatibility between DA2G(3G) and FSS

## Technical Parameters of FSS Satellite (ECC Report 110)

| Satellite | A | B | C | D | E | F | G | H | I |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Rx Antenna Gain (dBi) | 34 | 26.5 | 32.8 | 34 | 32.8 | 26.5 | 34 | 34.7 | 32.8 |
| Noise Temperature (deg K) | 773 | 1200 | 700 | 773 | 700 | 1200 | 1200 | 700 | 700 |
| Longitude (Degs) | -5 | -14 | -31.5 | -3 | -18 | 53 | 59.5 | 66 | 359 |

## Technical Parameters of GES (ECC Report 110)

| Earth Station | ST1 | ST2 | ST3 | ST4 |
| --- | --- | --- | --- | --- |
| Antenna Diameter (m) | 4.6 | 32.5 | 4.6 | 32.5 |
| Power (dBW/MHz) | 21.3 | 2.0 | 21.3 | 2.0 |
| Max antenna gain (dBi) | 47.8 | 63 | 47.8 | 63 |
| Height (m) | 4.3 | 18.25 | 4.3 | 18.25 |

## Compatibility between Source DA2G (3G) and Victim FSS Satellite

For this analysis the simulatedDA2G network was assumed. This comprises 786 antennas, some omni directional and some sectoral. The omni antennas had an electrical up-tilt of 20 degrees whereas the sectoral antennas had an up-tilt of 6 degrees.

It was also assumed that all antennas would be radiating on the same channel.

The system ensures that, for a given cell, either the base station or a single aircraft would be transmitting at the same time. The aircraft antenna is located directly under the airframe and, in level flight, would be significantly shielded from the FSS satellite. It is essentially the network of base stations that are the source of interference.

Figure 46: DA2G Network Source/Victim FSS Satellite

It can be seen from the above figure that the interference to the FSS satellite is very small indeed and compatibility is assured.

## Compatibility between Source GES and Victim DA2G Base Station

Figure 47: GES Source/Victim DA2G Base Station

When operating on the same channel the GES would interfere with the DA2G base station within a distance of about 10 kms due to the radio horizon. Compatibility can be achieved through location planning given that the actual siting of base stations is flexible within some 20 kms of the optimum location.

## Compatibility between Source GES and Victim DA2G Aircraft

When subject to main beam co-channel interference the aircraft would experience a significant reduction in data rate or temporary loss of signal due to the saturation of the receiver front-end. However, given the aircraft speed and flight profile, the time spent in the vicinity of the main beam would be very small. Hence this interference is acceptable.

## Discussion

There is compatibility between the DA2G system and the FSS satellite. The GES could interfere with the DA2G base station but mitigation is easily achieved by the siting of the base station. The GES will cause interference to the DA2G aircraft but the time during which this is experienced is low as is the probability of flying in the vicinity of the GES main beam.

# Compatibility between DA2G(3G) and ITS

## Technical Parameters of ITS (ECC Report 101)

| Device | Unit | ITS |
| --- | --- | --- |
| Eirp | dBm | 33 |
| Bandwidth | MHz | 10 |
| Antenna gain | dBi | 8 |
| Side lobe attenuation | dB | 12 |
| TPC | dB | 8 |
| Sensitivity (at antenna input) | dBm | -82 |
| Protection criterion | C/I | 6 (BPSK) |

## Compatibility between Source DA2G (3G) Base Station and Victim ITS

Figure 48: DA2G Base Station Source/Victim ITS

Co-channel compatibility is achieved for distances greater than 1.1 kms. It is noted from ECC Report 101 that the frequency band 5850-5870 MHz is intended for ITS roadside-to-vehicle communication. Full compatibility can be achieved by siting the base station such that the minimum separation distance is respected.

## Compatibility between Source DA2G (3G) Aircraft and Victim ITS

Figure 49: DA2G Aircraft Source/Victim ITS

Co-channel compatibility is achieved.

## Compatibility between Source ITS and DA2G(3G) Base Station

Figure 50: ITS Source/Victim DA2G Base Station

Co-channel compatibility is achieved for separation distances greater than 5 kms assuming a flat terrain with no obstructions. Compatibility can be achieved through location planning with respect to roadside ITS equipment.

## Compatibility between Source ITS and DA2G(3G) Aircraft

Figure 51: ITS Source/Victim DA2G Aircraft

Co-channel compatibility is achieved.

## Discussion

Potential co-channel interference caused between ITS (roadside-to-vehicle) link by the DA2G base station  can be mitigated by respecting the minimum separation distance. This can be achieved through siting of the base station. There is no co-channel interference between ITS and the DA2G aircraft.

# Compatibility between DA2G(3G) and BFWA

## Technical Parameters of BFWA (ECC Report 110)

| Device | Unit | BFWA P-MP | BFWA Mesh |
| --- | --- | --- | --- |
| Eirp | dBm | 36 | 36 |
| Bandwidth | MHz | 20 | 20 |
| Antenna gain | dBi | 18 | 10 |
| Side lobe attenuation | dB | 15 | 15 |
| TPC | dB | 10 | 10 |
| Sensitivity (at antenna input) | dBm | -86 | -86 |
| Protection criterion | C/I | 6 (BPSK) | 6 (BPSK) |

## Compatibility between Source DA2G (3G) Base Station and Victim BFWA

Figure 52: DA2G Base Station Source/Victim BFWA1 (P-MP) and BFWA2 (Mesh)

Co-channel compatibility is achieved for separation distances greater than about 3 kms. ECC Report 68 suggests that 90% of BFWA systems would be Point to Multi-Point. These systems are designed to use directional or sectoral antennas that could have down-tilt. This would reduce the minimum separation distance to about 0.5 km as shown below:

Figure 53: DA2G Base Station Source/Victim BFWA1 (P-MP) and BFWA2 (Mesh) with down-tilt of 6 degrees

Overall it is considered that compatibility can be achieved through a combination of directional antennas and the location of DA2G Base Stations.

## Compatibility between Source DA2G (3G) Aircraft and Victim BFWA

Figure 54: DA2G Aircraft Source/Victim BFWA1 (P-MP) and BFWA2 (Mesh)

Co-channel compatibility is achieved.

## Compatibility between Source BFWA and Victim DA2G (3G) Base Station

Figure 55: BFWA1 (P-MP) and BFWA2 (Mesh) Source/Victim DA2G Base Station

Interference will be experienced by the DA2G base station for separation distances less than about 6 kms. However acceptable mitigation can be achieved by the use of location and sectoral antennas.

## Compatibility between Source BFWA and Victim DA2G (3G) Aircraft

Figure 56: BFWA1 (P-MP) and BFWA2 (Mesh) Source/Victim DA2G Base Station

Co-channel compatibility is achieved.

## Discussion

Co-channel compatibility is achieved for separation distances greater than about 3 kms. ECC Report 68 suggests that 90% of BFWA systems would be Point to Multi-Point. These systems are designed to use directional or sectoral antennas that could have down-tilt. This would reduce the minimum separation distance to about 0.5 km. Hence co-channel compatibility between BFWA systems and DA2G base stations can be achieved through siting of the base stations. Alternatively, using different channels would provide compatibility. There is no co-channel interference between BFWA systems and DA2G aircraft.

# Compatibility between DA2G(3G) and SRD

## Technical Parameters of BFWA (ECC Report 110)

| Parameter | Unit | SRD I | SRD II | SRD III |
| --- | --- | --- | --- | --- |
| Bandwidth | MHz | 0.25 | 20 | 8 |
| Eirp | dBm | 14 | 14 | 14 |
| Antenna gain | dBi | 20 | 24 | 2 |
| Polarisation |  | Circular | Circular | Vertical |
| Receiver sensitivity | dBm | -110 | -91 | -84 |
| Receiver noise | dBm/MHz | -114 | N/A | N/A |
| Protection criterion (I/N) | dB | 0 | 8 | 20 |
| SRD noise figure F | dB | 9 | N/A | N/A |
| FkTB | dBm/MHz | -105 | N/A | N/A |
| Max OoB Rx interference | dBm | -35 | -35 | -35 |
| Wall attenuation | dB | 15 | 15 | 15 |

## Compatibility between Source DA2G (3G) Base Station and Victim SRD

Figure 57: DA2G Base Station Source/Victim SRD

## Co-channel compatibility is achieved for separation distances greater than 200 metres.

## Compatibility between Source DA2G (3G) Aircraft and Victim SRD

Figure 58: DA2G Aircraft Source/Victim SRD

## Co-channel compatibility is achieved.

## Compatibility between Source SRD  and Victim DA2G Base Station

Figure 59: SRD Source/Victim DA2G Base Station

Co-channel compatibility is achieved for SRD types II and III for a separation distance greater than 100 metres. The narrow band type I is compatible for a separation distance greater than 2.2 kms. However, if the SRD device is indoors then there is always compatibility.

## Compatibility between Source SRD  and Victim DA2G Aircraft

Figure 60: SRD Source/Victim DA2G Aircraft

Co-channel compatibility is achieved.

## Discussion

Co-channel compatibility between SRD (victim) and DA2G (source) is achieved provided the DA2G base station is more than 200 metres from the SRD. The SRD (Type I) could cause interference to the DA2G base station for separation distances less than about 2.5 kms but this is not considered significant because the probability of a co-channel Type I (narrow band) source close to the base station is low and mitigation is possible through the use of directional antennas if necessary.

# Conclusions

The analyses conclude that the DA2G(3G) is compatible with other systems operating in the frequency band 5850 - 5870 MHz and radiolocation systems operating in the lower adjacent band. Where mitigation is required this can be achieved by appropriate siting of the DA2G(3G) base stations. There are no compatibility issues regarding interference caused by the DA2G(3G) aircraft.

Any interference caused to the DA2G(3G) ground network can be mitigated through base station siting  and/or directional antenna considerations.

Interference caused to the DA2G(3G) aircraft caused by the FSS GES main beam is acceptable because the probability of the aircraft being in the vicinity of the main beam  is low and the effective  transit time through the main beam is very short.

Finally, the aeronautical use of the frequency band 5850 – 5870 MHz would significant increase the overall  spectrum efficiency without disturbing existing systems including radiolocation systems (particularly search radars) operating in the lower adjacent band.