FM48#3-Temp02rev1

# Candidate bands for Broadband DA2GC

(Revised during the 3rd meeting of FM PT 48)

# Category 1: Frequency bands already submitted to WG SE for compatibility/sharing studies

| Frequency band | Existing allocation in the RR suitable for Broadband DA2GC (MS or AMS) ? | Current level of harmonisation ? | Would the bandwidth required (2 x 10 MHz FDD or 20 MHz TDD) fit in the band ? | Proposed or future alternative usage under consideration (within CEPT) ? | Actual usage of the band - national level - within CEPT (- worldwide) for future sharing studies ? | Actual usage of adjacent bands - national level - within CEPT for future compatibility studies ? |  | Likely time scales (target date 2012) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2400 – 2483.5 MHz | yes | high | yes | --- | RLANs, SRDs/RFIDs, ISM | Aeronautical Telemetry, Amateur, Mobile applications, SAP/SAB (cordless cameras), MSS, ISM |  | possibly within |
| 3400 – 3600 MHz | yes | high | yes | band plan under consideration within ECC PT 1 | BWA, FSS, FS links, RLS | RLS, FSS, BWA, FS links |  | possibly within |
| 5855 – 5875 MHz | yes | medium | yes (air to ground) | AI 1.20 WRC-12 | BFWA, ITS, SRDs, FSS (uplink), military systems (on a national level), ISM | RLS, RTTT, BFWA, SRDs, ITS |  | within |

# Category 2: Frequency bands to be considered next or in case the studies for the category 1 bands will not show positive results

| Frequency band | Existing allocation in the RR suitable for Broadband DA2GC (MS or AMS) ? | Current level of harmonisation ? | Would the bandwidth required (2 x 10 MHz FDD or 20 MHz TDD) fit in the band ? | Proposed or future alternative usage under consideration (within CEPT) ? | Actual usage of the band - national level - within CEPT (- worldwide) for future sharing studies ? | Actual usage of adjacent bands - national level - within CEPT for future compatibility studies ? |  | Likely time scales (target date 2012) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2483.5 – 2500 MHz | yes | high | yes (ground to air) | AI 1.18 WRC-12 | MSS, ISM | RLANs, SRDs/RFIDs, ISM, ECS |  | possibly within |

# Category 3: Position on frequency bands still open/undecided

| Frequency band | Existing allocation in the RR suitable for Broadband DA2GC (MS or AMS) ? | Current level of harmonisation ? | Would the bandwidth required (2 x 10 MHz FDD or 20 MHz TDD) fit in the band ? | Proposed or future alternative usage under consideration (within CEPT) ? | Actual usage of the band - national level - within CEPT (- worldwide) for future sharing studies ? | Actual usage of adjacent bands - national level - within CEPT for future compatibility studies ? |  | Likely time scales (target date 2012) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1452 – 1492 MHz | no | high | yes | TRA-ECS (Mobile downlink) | Mobile Multimedia (WAPECS), Digital Radio, Satellite Digital Audio, Aeronautical telemetry | Aeronautical telemetry, Fixed Links, Tactical Radio Relay |  | beyond for some countries |
| 1900 – 1920 MHz / 2010 – 2025 MHz | yes, yes | high, high | yes | ECS | UMTS (TDD) | DECT, UMTS (FDD), EESS, SRS, FS, SAP/SAB |  | within |
| 3600 – 3800 MHz | yes | high | yes | band plan under consideration within ECC PT 1 | BWA, FSS, FS links | BWA, FSS, FS links |  | possibly within |
| 5905 – 5925 MHz | yes | high | yes (air to ground) | AI 1.20 WRC-12 | FSS (uplink), military systems (on a national level) | FS links |  | within |

# Category 4: Frequency bands with high regulatory obstacles for an introduction of Broadband DA2GC systems

| Frequency band | Existing allocation in the RR suitable for Broadband DA2GC (MS or AMS) ? | Current level of harmonisation ? | Would the bandwidth required (2 x 10 MHz FDD or 20 MHz TDD) fit in the band ? | Proposed or future alternative usage under consideration (within CEPT) ? | Actual usage of the band - national level - within CEPT (- worldwide) for future sharing studies ? | Actual usage of adjacent bands - national level - within CEPT for future compatibility studies ? |  | Likely time scales (target date 2012) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2300 – 2400 MHz | yes | low | yes |  | Aeronautical Telemetry, Amateur, Mobile applications, SAP/SAB (cordless cameras) | FS, MS, SRS |  | within ??? |

List of abbreviations used in the tables

{abbreviations no longer relevant will be deleted}

| Abbreviation | Explanation |
| --- | --- |
| BBDR | Broad Band Disaster Relief |
| BC | Broadcasting |
| BFWA | Broadband Fixed Wireless Access |
| BWA | Broadband Wireless Access |
| DECT | Digital Enhanced Cordless Telecommunications |
| DME | Distance Measuring Equipment |
| ECS | Electronic Communications Services |
| EESS | Earth Exploration-Satellite Service |
| FCS | Future Communications System |
| FDD | Frequency Division Duplex |
| FS | Fixed Service |
| FSS | Fixed-Satellite Service |
| ISM | Industrial, scientific and medical applications |
| ITS | Intelligent Transport Systems |
| JTIDS | Joint Tactical Information Distribution System |
| MFCN | Mobile/fixed communications networks |
| MIDS | Multifunctional Information Distribution System |
| MLS | Microwave Landing System |
| MS | Mobile Service |
| MSS | Mobile Satellite-Service |
| PMSE | Program Making and Special Events |
| PSR | Primary Surveillance Radar |
| RAS | Radio Astronomy Service |
| RFIDs | Radio frequency identification devices |
| RLANs | Radio Local Area Networks |
| RLS | Radiolocation Service |
| RNSS | Radionavigation-Satellite Service |
| RSBN | Radiotechniczny System Bliskiej Nawigacji (Russian radio navigation system) |
| RTTT | Road Transport and Traffic Telematics |
| SAP/SAB | Services Ancillary to Programme making / Services Ancillary to Broadcasting |
| SRDs | Short Range Devices |
| SRS | Space Research Service |
| SSR | Secondary Surveillance Radar |
| TACAN | Tactical Air Navigation |
| T-DAB | Terrestrial Digital Audio Broadcasting |
| TDD | Time Division Duplex |
| TRA-ECS | Terrestrial radio applications capable of providing Electronic Communications Services |
| UAT | Universal Access Transponder |
| UMTS | Universal Mobile Telecommunications System |
| WAPECS | Wireless Access Policy for Electronic Communications Services |

This band is mentioned for completeness. It could be subject for discussion in FM PT 48 after the results of the FM PT 50 study aiming at determining which future use(s) of the 1452-1492 MHz band would be the most appropriate for CEPT.