| Project Team FM PT 48 |  |  | FM48(11)053 |
| --- | --- | --- | --- |
| 4th Meeting, 24-25 November 2011 |  |  |  |
| Sophia-Antipolis / France |  |  |  |
| Date issued: | 23 November 2011 |  |  |
| Source: | Aero3G |  |  |
| Subject: | New ECC Report on Broadband DA2GC, section 6.3 |  |  |
| Group membership required to read? (Y/N): N |  |  |  |
| Summary: |  |  |  |
| At the last WG FM meeting, FM PT 48 proposed to develop a new ECC Report on Broadband DA2GC in which all relevant aspects, besides compatibility and sharing issues for which WG SE is responsible, could be described in detail. Elements for such an ECC Report were provided in Annex 7r1 of the FM PT 48 Progress Report (see FM48(11)043). WG FM agreed that the ECC Report should be finalised by September 2012. |  |  |  |
| Proposal: |  |  |  |
| Proposed text for section 6.3. |  |  |  |
| Background: |  |  |  |
| See extract of WG FM #73 Minutes (FM48(11)045). |  |  |  |

6.3	PROPOSAL FROM AERO3G [Future ETSI System Reference Document xxx ???]

see FM48(11)038

### Introduction

The importance of connectivity between aircraft and ground for the airline business is growing fast. Passengers have made clear that they would benefit from on-board high quality internet access and that they are prepared to pay for the service provided that the price is considered reasonable. Apart from passenger related applications, airlines would also benefit from a high capacity communications link for non-safety of life operational data. Many applications to reduce operating costs and increase efficiency are denied airlines because of the present costs of providing an adequate link.

There have been several attempts to provide connectivity but these have been frustrated mainly due to the cost of deploying and operating the necessary infrastructure; especially the cost of aircraft installation and operation.

Consumer devices, such as iPhone, are developing rapidly and offer easy WiFi connectivity. More and more passengers are carrying pocket devices that are convenient to use during even the shortest of flights.

### Technical Description

### General

The Aero3G system provides a direct air to ground communications link using the transparent Internet Protocol. Co-funded by the European Union, the original system was developed to operate in the frequency band 5091 – 5150 MHz but subsequently the design has been modified to operate in the higher frequency band 5855 – 5875 MHz.

It is a TDD system based on COTS equipment that complies with ETSI 3GPP R8 standards and has been ETSI certified in the 2.5 - 2.7 GHz band. A separate frequency converter is used to support operation in the 5855 – 5875 MHz band. The current UE modem is 3GPP R8 LTE compliant and may be 3GPP R9 LTE compliant via future software upgrade. Signal-in-space characteristics conform to these standards apart from the operating frequency band.

Flight testing of 3G technology has been successfully conducted in the 5091 - 5150 MHz band. Among other things, aircraft sorties were used to validate simulations. These simulations were subsequently used to provide performance predictions for the 5855 - 5875 MHz band.

Through simulation it is estimated that the number of base stations required to provide pan-European coverage is 220. The cell size is approximately 150 kms but this may be extended to 280 kms, using directional antennas, to support continuous coverage over sea.

### Key System Parameters

Receiver Characteristics

| Parameter | Unit | Value | Remarks |
| --- | --- | --- | --- |
| Bandwidth | MHz | 5 or 10 | 5 MHz used for flight testing |
| Receiver sensitivity | dBm | -106 |  |
| Receiver feeder loss | dB | 1 |  |
| Receiver interference threshold | dBm/MHz | -113.9 |  |

Transmitter Characteristics

| Parameter | Unit | Value | Remarks |
| --- | --- | --- | --- |
| Bandwidth | MHz | 5 or 10 | 5 MHz used for flight testing |
| Power | dBm | 40 |  |
| Transmitter feeder loss | dB | 4 |  |
| Transmitter duty cycle | % | 50 |  |
| Max antenna gain (A/C) | dBi | 6 |  |
| Max antenna gain (BS Omni) | dBi | 7 |  |
| Max antenna gain (BS Sector) | dBi | 15 |  |
| Max antenna gain (BS Directional) | dBi | 24 |  |

Ground Antenna Patterns

Aircraft Omni Antenna Pattern

Unwanted Emission Level of Aero3G System

### Quality of Service

Availability:	The system is designed to be available (7/24/365) very year with a probability greater than 98%. The average monthly availability will be better than 99%.

Capacity:	The system will provide a minimum forward link capacity per aircraft of 5 Mbps and a minimum reverse link of 1 Mbps.

Latency:	The system latency will be less than 100 mSec.

### Specific Spectrum Needs

The system can use switch-selectable bandwidths of 5 or 10 MHz. Although single channel operation is possible, the use of additional channels reduces potential inter-cell interference and also any interference of other systems.

The required spectrum is 20 MHz in the 5855 – 5875 MHz candidate band thereby enabling 2x10 MHz or 4x5 MHz channels.

### Timescale

The planned implementation is over a two year timescale following the appropriate regulatory permission. The critical path includes to phased installation of aircraft systems.

### System Implementation

Both airlines and operators have been involved with the development of specifications and validation of business planning. There is a keen desire to implement the system and detailed planning has been made through several non-disclosure agreements.

Retrofit was always an important issue for airlines and the system has been designed to enable installation over a series of night stops thereby ensuring that the aircraft need not be taken out of operational service.

Finally, the necessary investment and operating finance for the project is available subject to certain conditions including regulatory permission to use the candidate band 5855 - 5875 MHz.