| Project Team FM PT 48 |  |  | Proposal 4 Aero3G |
| --- | --- | --- | --- |
| 5th Meeting, 1-2 March 2012 |  |  |  |
| Copenhagen / Denmark |  |  |  |
| Date issued: | 22 February 2012 |  |  |
| Source: | Aero3G |  |  |
| Subject: | Proposed Amendments to Proposal 2 from Deutsche Telekom based on FM(11)056 – Annex 5 Draft ECC Report V01 |  |  |
| Group membership required to read? (Y/N): N |  |  |  |
| Summary: |  |  |  |
| FM48 is developing a draft ECC Report on DA2G. FM(11)056 – Annex 5 provided an initial draft structure with text for certain sections. Recently Deutsche Telekom provided a further contribution with amendments. This paper proposes some additional amendments based on the new proposed draft contributed by Deutsche Telekom. |  |  |  |
| Proposal: |  |  |  |
| FM48 is invited to consider the proposed amendments detailed in this paper and agree to their inclusion in its Draft ECC report. |  |  |  |
| Background: |  |  |  |

Section 4.3.1 Page 16

Current Text:

The most critical interference scenario is obviously the interference from the DA2GC forward link (Ground-to-air) into the FSS (E-S). It is expected that the adequate protection of FSS (E-S) might lead to undue constraints for the parameters of a DA2GC system, thus not allowing sufficient system performance for economic operation.

Comment:

This statement is not correct for DA2G Aero3G. Compatibility studies, including full network simulations, have demonstrated that the DA2G forward link does not cause interference to the FSS (E-s). The actual operating parameters of Aero3G ensure this compatibility.

Indeed, the most critical interference scenario could be the DA2G reverse link (air to ground) into radiolocation systems (air search radar) operating in the lower adjacent band. Again the operating parameters of Aero3G ensure compatibility.

Proposed Text:

Any system using this band must satisfy compatible constraints with the other co-frequency and adjacent frequency systems, namely:

- BBDR

- BFWA

- FSS

- ITS

- Non-specific SRDs

- Defence systems (radiolocation in adjacent band)

Section 5 Pages 20 – 21

Current Text:

The aircraft stations of a DA2GC network system will be under the control of the DA2GC network and hence could be dealt with the exemption from individual licensing and free circulation and use, possibly under the umbrella of a new ECC Decision.

Comment:

Aircraft radio licensing for civil aircraft is on the basis of a single license that covers all radio equipment on-board (excluding passenger equipment). Hence licensing issues for the DA2G aircraft station would de facto be already addressed.

Proposed Text:

The licensing of aircraft stations of a European DA2G network system may be covered by existing provisions thereby avoiding the need for further regulation.

Section 5 Pages 20 – 21

Current Text

Given the high financial risks involved, it seems not reasonable to assume a licence-exempt solution for DA2GC, i.e. without individual rights, providing stable frame conditions at least for a dedicated period of time, and individual obligations.

Comment:

Where minimum separation distances are required to achieve compatibility with other systems, it is reasonable to impose some form of light regulation. For example, the characteristics and location of base stations would need to be registered.

Proposed Text:

Given the need for substantial financial investment, together with the requirement to protect other spectrum users, it is reasonable to envisage some form of light regulation in Europe probably based on individual authorisation. However, it is important that the chosen forms of regulation and licensing do not impose unreasonable restrictions on competition.

Section 6.3.2.1 Page 28

Current Text:

It is a TDD system based on COTS equipment that complies with ETSI 3GPP R8 standards and has been ETSI certified in the 2.5 - 2.7 GHz band. A separate frequency converter is used to support operation in the 5855 – 5875 MHz band. The current UE modem is 3GPP R8 LTE compliant and may be 3GPP R9 LTE compliant via future software upgrade. Signal-in-space characteristics conform to these standards apart from the operating frequency band.

Comment:

In the past, reference to LTE has introduced some confusion.

Proposed Text:

It is a TDD system based on COTS equipment that complies with the 3GPP Release 7 standards and has been ETSI certified in the 2.5 - 2.7 GHz band. A separate frequency converter is used to support operation in the 5855 – 5875 MHz band. Signal-in-space characteristics conform to these standards apart from the operating frequency band including Doppler shift compensation.

Section 6.3.2.2 Pages 28-31

Current text:

The current text comprises a series of tables and figures.

Comment:

Consistency with the overall report numbering needs to be respected.

Proposed Text;

A new Section 6.3.2.2 to be inserted as follows:

### 6.3.2.2	Key System Parameters

Receiver Characteristics

| Parameter | Unit | Value | Remarks |
| --- | --- | --- | --- |
| Bandwidth | MHz | 5 or 10 | 5 MHz used for flight testing |
| Receiver sensitivity | dBm | -106 |  |
| Receiver feeder loss | dB | 1 |  |
| Receiver interference threshold | dBm/MHz | -113.9 |  |

Table 6- 1: Receiver Characteristics

Transmitter Characteristics

| Parameter | Unit | Value | Remarks |
| --- | --- | --- | --- |
| Bandwidth | MHz | 5 or 10 | 5 MHz used for flight testing |
| Power | dBm | 40 |  |
| Transmitter feeder loss | dB | 4 |  |
| Transmitter duty cycle | % | 50 |  |
| Max antenna gain (A/C) | dBi | 6 |  |
| Max antenna gain (BS Omni) | dBi | 7 |  |
| Max antenna gain (BS Sector) | dBi | 15 |  |
| Max antenna gain (BS Directional) | dBi | 24 | Used over sea to achieve maximum range. |

Table 6- 2: Transmitter Characteristics

Base Station Antenna Characteristics

Figure 6 - 1: Omni Antenna Pattern - Elevation

Figure 6 - 2: Omni Antenna Pattern - Azimuth

Figure 6 - 3: Sector Antenna Pattern - Elevation

Figure 6 - 4: Sector Antenna Pattern - Azimuth

Aircraft Antenna Characteristics

Figure 6 - 5: Aircraft Antenna Pattern - Elevation

Figure 6 - 6: Aircraft Antenna Pattern - Azimuth

Spectrum Emission Mask Characteristics

Figure 6 - 7: Unwanted Emission Level of Aero3G System

Section 6.3.3 Page31

Current text:

Latency:	The system latency will be less than 100 mSec.

Comment:

It would be helpful to the reader to provide measured latency information.

Proposed Text:

Latency:	The system latency will be less than 100 mSec.	 Measured round trip latency during testing was less than 45 mSec.

Section 6.3.5 Page32

Current Text:

The planned implementation is over a two year timescale following the appropriate regulatory permission. The critical path includes to phased installation of aircraft systems.

Comment:

It would be helpful to the reader if the timescale for the initial implementation were included.

Proposed Text:

The planned implementation is over a two year timescale following the appropriate regulatory permission. The critical path includes the phased installation of aircraft systems which would commence 18 months after the start.